3GPP TSG-RAN WG2 Meeting #123	R2- 2307767

Toulouse, France, 21 – 25 August 2023

Agenda item:	7.10

Source:	Nokia, Nokia Shanghai Bell

Title:	UE interface for IDC

WID/SID:	NR_IDC_enh-Core - Release 18

Document for:	Discussion and Decision

# 1	Introduction

In RAN2, it is discussed how IDC can be mitigated through NW/UE signalling, please refer to R2-2306365.

One of the items discussed is Time Division Multiplexing, TDM, methods, where sub-systems within the UE are expected to do time multiplexing between them. However, there is no standardized means for exchanging parameters for the time multiplexing between the sub-systems in the UE.

# 2	Discussion

The root problem of In-Device Co-existence, IDC, is outlined in 3GPP TR 36.816, where the main figure illustrating IDC is copied in and shown in Figure 1.

Figure 1 From: 3GPP TR 36.816 V11.2.0 (2011-12)

Figure 2 From: 3GPP TR 36.816 V11.2.0 (2011-12). 3GPP/NR/LTE transmitter disturb the ISM transceiver in the same manner.

As seen in Figure 1, sub-systems may be implemented independently.

MT	Mobile Termination

TA	Terminal Adaptor, e.g. a GSM data card (equal to DCE; Data Circuit terminating Equipment)

TE	Terminal Equipment, e.g. a computer (equal to DTE; Data Terminal Equipment)J

Figure 1: Setup

Figure 3: From: 3GPP TS 27.007 V17.8.0 (2022-12)

The current interface specification for controlling the 3GPP modem is given in 3GPP TS 27.007 and the reference model of the specification is shown in Figure 3.

If there are more sub-systems, one architecture could be that they are all connected to the TE of Figure 3 as illustrated in Figure 4.

Figure 4: Combining Figure 1 and Figure 3, extending to include both 3GPP, WiFi and BT sub-systems

In Figure 4, the TE is the common denominator between sub-systems, but sub-systems can also communicate between them directly.

However, it is clear that the sub-systems can exchange parameters about the radio

Observation 1: There is no standard for exchanging IDC parameters between sub-systems of a device/UE.

Observation 2: There may be a connection between parameters exchanged between sub-systems of the UE and the parameters exchanged between the UE and the NW.

Proposal 1: It is suggested an LS is sent to CT1 with the request to consider adding an interface description for exchange of IDC parameters between UE sub-systems.

Proposal 2: It is suggested that RAN2 and CT1 aligns on the parameters exchanged between UE sub-systems and UE-NW message exchange.

# 3	Conclusion

This document has made the following observations:

Observation 1: There is no standard for exchanging IDC parameters between sub-systems of a device/UE.

Observation 2: There may be a connection between parameters exchanged between sub-systems of the UE and the parameters exchanged between the UE and the NW.

Proposal 1: It is suggested an LS is sent to CT1 with the request to consider adding an interface description for exchange of IDC parameters between UE sub-systems.

Proposal 2: It is suggested that RAN2 and CT1 aligns on the parameters exchanged between UE sub-systems and UE-NW message exchange.

The proposed LS draft is in the Annex.

# Annex: LS to CT1:

3GPP TSG-RAN WG2 Meeting #123	DRAFT R2-230xxxx

Toulouse, France, 21– 25 August 2023

Title:	[DRAFT] LS on UE interface for IDC

Response to:	-

Release:	Release 18

Work Item:	NR_IDC_enh-Core

Source:	Nokia [TSG RAN WG2]

To:	TSG CT WG1

Cc:

Contact Person:

Name: Jarkko Koskela

Email Address: jarkko.t.koskela@nokia.com

Send any reply LS to:	3GPP Liaisons Coordinator, mailto:3GPPLiaison@etsi.org

Attachments:	-

1. Overall Description:

In RAN2, it is discussed how IDC can be mitigated through NW/UE signalling, please refer to R2-2306365.

One of the items discussed is Time Division Multiplexing, TDM, methods, where sub-systems within the UE are expected to do time multiplexing between them. However, there is no standardized means for exchanging parameters for the time multiplexing between the sub-systems in the UE.

The parameters exchanged should be aligned between the sub-system interface and the air interface between the UE and the NW.

2. Actions:

To CT1 group.

ACTION: 	RAN2 respectfully asks CT1 to consider a standard interface between UE sub-systems to exchange parameters to avoid In-Device Co-existence interference issues as an addition to TS 27.007.

3. Date of Next TSG-RAN WG2 Meeting:

RAN2#123bis	from 2023-10-09	to 2023-10-13		Xiamen, CN

RAN2#124	from 2023-11-13	to 2023-11-17		Chicago, US