3GPP TSG-RAN WG2 Meeting #123	R2-2307786

Toulouse, France, Aug 21st – Aug 25th, 2023

Agenda item:	7.4.3

Source:	Qualcomm Incorporated

Title:	SCG Selective Activation in NR-DC

WID/SID:	NR_Mob_enh2-Core – Release 18

Document for:	Discussion and Decision

# Introduction

RAN2 #122 agreements:

- For SN-initiated SCG selective activation, candidate SN generates execution conditions for subsequent CPC.

- FFS if it shall be possible to do something like MN-initiated CPA/CPC where Candidate SN generate execution conditions for subsequent CPC.

- The UE shall skip the condition evaluation for a candidate which is a current PScell.

- The reference configuration is provided to all candidates involved in preparation, FFS which node initially generates it. Assume it can be provided in MN initiated and in SN initiated procedures.

- Will not spend specific efforts for supporting nested configurations for candidate cell configuration.

- Terminology is “Subsequent CPAC”.

In the last RAN2 meeting (RAN2 #122 [1]), there was continued progress on this topic. The following agreements were reached.

In this contribution, we continue to discuss proposals on the following scenarios that were agreed in the previous meetings:

- SN initiated inter-SN Subsequent CPAC,

- MN initiated inter-SN Subsequent CPAC,

# SN initiated inter-SN Subsequent CPAC

Figure 1 shows a signalling flow of the procedure, assuming that initially UE is in DC operation. We describe the steps of the procedure and alongside some proposals.

Steps 1, 2: Based on the UE measurement reports, source SN initiates the procedure by transmitting SN Change Required to the source MN.

It was agreed in a previous RAN2 meeting (RAN2 #121-bis-e [2]) that the source SN generates the execution conditions for the initial CPC.

Proposal 1. In SN Change Required, source SN includes:

- A list of candidate SNs for the initial CPC, and for each candidate SN in the list, a list of candidate PSCells.

  - Execution conditions associated with each candidate PSCell of the initial CPC.

- A list of candidate SNs for the subsequent CPCs, and for each candidate SN in the list, a list of candidate PSCells.

Step 3: It was agreed in the last RAN2 meeting (RAN2 #122 [1]) that candidate SN generates execution conditions for a subsequent CPC. This holds when the serving PSCell of the UE becomes a cell of the candidate SN.

In Step 3, source MN provides the required information for CPC preparation to a candidate SN.

Proposal 2. Source MN includes in the SN Addition Request message to a candidate SN:

- List of all candidate PSCells (i.e., candidate PSCells belonging to all candidate SNs),

- UE measurements.

Step 4: The candidate SN prepares a list of PSCells that belong to it and includes the list in the response message SN Addition Request Acknowledge.

Figure 1: SN initiated inter-SN Subsequent CPAC

The candidate SN also determines a list of candidate PSCells suitable for the subsequent CPC, e.g., the neighbour PSCells; these PSCells are cells of the other candidate SNs. For each candidate PSCell for the subsequent CPC, the candidate SN includes the execution conditions which are A3/A5 events.

Proposal 3. In SN Addition Request Acknowledge, the candidate SN includes:

- List of prepared PSCells and associated target SCG configurations, which include the target SCG measurement configurations,

- For each cell in 1), a list of candidate PSCells for the subsequent CPC (e.g., the neighbour PSCells), and associated execution conditions (events A3/A5).

Steps 5, 6: Upon receiving the SN Addition Request Acknowledge message, source MN checks whether the candidate PSCells for the subsequent CPC as indicated in the message have been prepared by the other candidate SNs. If there are candidate PSCells for the subsequent CPC that have not been prepared, source MN initiates an SN Modification procedure with the candidate SN including only those candidate PSCells that have been prepared. In response, the candidate SN may include target SCG configurations with updated SCG measurement configurations, taking into account the prepared candidate PSCells for the subsequent CPC.

Proposal 4. Source MN may initiate SN Modification with the candidate SN including the prepared candidate PSCells for the subsequent CPC.

Proposal 5. In response, the candidate SN may include target SCG configurations with updated target SCG measurement configurations.

Steps 7-9: It was agreed in the previous RAN2 #121-bis-e [2] meeting that the source MN prepares the initial RRC reconfiguration message containing the Rel-18 CPC configurations. It was also agreed that the following are the possible contents of the initial RRC reconfiguration message containing the Rel-18 CPC configurations.

- The reference SCG configuration.

- The initial list of candidate target PSCells with associated target SCG configurations (this list can be updated later by the network, e.g., cells can be added to or removed from this list).

- The execution conditions associated with each candidate target PSCell.

We think that for each candidate target PSCell in the initial list (2), the MCG configuration associated with the target SCG configuration should be also included, as in legacy PSCell change or addition, or CPA and CPC.

Proposal 6. For each candidate target PSCell in the initial RRC reconfiguration message containing the Rel-18 CPC configurations, the MCG configuration associated with the target SCG configuration is included.

Proposal 7. In the initial RRC reconfiguration message containing the Rel-18 CPC configurations, for each candidate target PSCell when it becomes the serving PSCell, include:

- The set of candidate target PSCells to evaluate for the subsequent CPC.

- The execution conditions (A3/A5) associated with the cells in 1).

- The set of CPC configurations to be released by the UE.

Steps 10-13: Upon triggering of CPC for a candidate target PSCell, UE transmits RRC reconfiguration complete to the MN including the candidate target PSCell ID, similar as in Rel-17 inter-SN CPC.

# MN initiated inter-SN Subsequent CPAC

Figure 2 shows a signalling flow of the procedure, assuming that initially UE is in DC operation.

Steps 1, 2: Upon receiving measurement reports from the UE, source MN initiates the procedure. In a previous RAN2 meeting (RAN2 #121-bis-e [2]), it was agreed that source MN generates the execution conditions for the initial CPC.

We now consider the case of a subsequent CPC. As for the SN initiated inter-SN procedure, for a subsequent CPC, it may be better for candidate SN to determine the execution conditions, i.e., when the serving PSCell of the UE becomes a cell of the candidate SN, the candidate SN can set the execution condition parameters, e.g., event trigger thresholds, times to trigger, more accurately.

Observation 1. Candidate SN generates the execution conditions for subsequent CPC, i.e., when the UE serving PSCell becomes a cell of the candidate SN, the candidate SN can set the execution condition parameters, e.g., event trigger thresholds, times to trigger, more accurately.

Proposal 8. Candidate SN generates execution conditions for a subsequent CPC.

Figure 2: MN initiated inter-SN Subsequent CPAC

The remaining steps are the same as for the SN initiated inter-SN procedure and their description is thus omitted.

Observation 2. A similar set of proposals corresponding to proposals 2-7 for the SN initiated inter-SN procedure hold for the MN initiated inter-SN procedure.

# 4 Conclusions

Based on the above discussions, we recommend that RAN2 discuss the following observations and proposals.

SN initiated inter-SN SCG Subsequent CPAC

Proposal 1. In SN Change Required, source SN includes:

- A list of candidate SNs for the initial CPC, and for each candidate SN in the list, a list of candidate PSCells.

  - Execution conditions associated with each candidate PSCell of the initial CPC.

- A list of candidate SNs for the subsequent CPCs, and for each candidate SN in the list, a list of candidate PSCells.

Proposal 2. Source MN includes in the SN Addition Request message to a candidate SN:

- List of all candidate PSCells (i.e., candidate PSCells belonging to all candidate SNs),

- UE measurements.

Proposal 3. In SN Addition Request Acknowledge, the candidate SN includes:

- List of prepared PSCells and associated target SCG configurations, which include the target SCG measurement configurations,

- For each cell in 1), a list of candidate PSCells for the subsequent CPC (e.g., the neighbour PSCells), and associated execution conditions (events A3/A5).

Proposal 4. Source MN may initiate SN Modification with the candidate SN including the prepared candidate PSCells for the subsequent CPC.

Proposal 5. In response, the candidate SN may include target SCG configurations with updated target SCG measurement configurations.

Proposal 6. For each candidate target PSCell in the initial RRC reconfiguration message containing the Rel-18 CPC configurations, the MCG configuration associated with the target SCG configuration is included.

Proposal 7. In the initial RRC reconfiguration message containing the Rel-18 CPC configurations, for each candidate target PSCell when it becomes the serving PSCell, include:

- The set of candidate target PSCells to evaluate for the subsequent CPC.

- The execution conditions (A3/A5) associated with the cells in 1).

- The set of CPC configurations to be released by the UE.

MN initiated inter-SN SCG Subsequent CPAC

Observation 1. Candidate SN generates the execution conditions for subsequent CPC, i.e., when the UE serving PSCell becomes a cell of the candidate SN, the candidate SN can set the execution condition parameters, e.g., event trigger thresholds, times to trigger, more accurately.

Proposal 8. Candidate SN generates execution conditions for a subsequent CPC.

Observation 2. A similar set of proposals corresponding to proposals 2-7 for the SN initiated inter-SN procedure hold for the MN initiated inter-SN procedure.

# References

[1] Chair notes, RAN2 #122 meeting.