3GPP TSG-RAN WG2 Meeting #123	R2-2307829

Toulouse, France, 21– 25 August 2023

Agenda item:	7.5.3

Source:	Apple

Title:	Views on Configurations of Rational Number-Based DRX Cycles

WID/SID:	NR_XR_enh-Core

Document for:	Discussion / Decision

# Introduction

According to [1], the Rel-18 WI of XR aims to address the issue of DRX timing mismatch caused by non-integer periodicity of packet arrival rate. During RAN2 #122, we have agreed that DRX cycle based on rational numbers can be defined in order to support XR frame rates with non-integer periodicities:

Define DRX cycle based on rational numbers. Inform RAN1/4 about this and ask them to indicate if this causes issues in their specifications.

Furthermore, to ensure the DRX cycle based on rational number is feasible in both RAN1 and RAN4 specifications, RAN2 has sent an LS (R2-2306564) with the following information:

To address the issue of mismatch between legacy DRX cycles and periodicity of XR traffic, RAN2 have agreed to introduce new values of DRX cycles in rational numbers. For example, to support traffic with a periodicity of 60 fps, the network may configure the UE with a DRX cycle of 50/3 msec.

In this contribution, we present some of our views about configurations of ration number-based DRX cycles.

# Discussions

In both of the agreement and the LS, only “DRX cycle” is mentioned. However, a DRX cycle could mean either the long DRX cycle or short DRX cycle, and so it is not too explicit in either the agreement or the LS, whether rational number is applied to only long DRX cycle, or both long and short DRX cycles. In our understanding, Short DRX cycle is mainly used to address the possibility of residual data communications. After the drx-InactivityTimer is expired, the UE starts to use the short cycle by assuming that the gNB may allocate further DL/UL resources:

2> if the PDCCH indicates a new transmission (DL, UL or SL) on a Serving Cell in this DRX group: 3> start or restart drx-InactivityTimer for this DRX group in the first symbol after the end of the PDCCH reception.

1> if drx-InactivityTimer for a DRX group expires: 2> if the Short DRX cycle is configured: 3> start or restart drx-ShortCycleTimer for this DRX group in the first symbol after the expiry of drx-InactivityTimer; 3> use the Short DRX cycle for this DRX group.

Therefore, whether to use the Short DRX cycle depends on if the UE has already received some resource allocation, and this does not seem to be relevant to the non-integer XR frame rate problems that we have been discussing. In the context of XR, Short DRX cycle is mainly used for cases where the packet size that arrives periodically is large or is experiencing some jitter, and the gNB may need to allocate more “follow-up” radio resources before the long DRX cycle in order to complete the transmission in time – this is not related to the time mismatch problem. As long as the gNB knows the traffic pattern and can properly use the rational number based long DRX cycle, the long DRX cycle alone seems to be sufficient to ensure the timing of packet arrival can match the long DRX cycle On-Duration, the configuration of Short DRX cycle may not be needed. Hence, it is questionable whether rational number based DRX cycles should also be applicable to short cycles. We think RAN2 needs to discuss it further.

Proposal 1: RAN2 should discuss whether rational number based DRX cycle is applicable only to Long DRX cycle, or also to Short DRX cycle.

In the current TS 38.331, the long DRX cycles are configured in millisecond, via the parameter drx-LongCycleStartOffset in the IE of DRX-Config:

drx-LongCycleStartOffset CHOICE { ms10 INTEGER(0..9), ms20 INTEGER(0..19), ms32 INTEGER(0..31), ms40 INTEGER(0..39), ms60 INTEGER(0..59), ms64 INTEGER(0..63), ms70 INTEGER(0..69), ms80 INTEGER(0..79), ms128 INTEGER(0..127), ms160 INTEGER(0..159), ms256 INTEGER(0..255), ms320 INTEGER(0..319), ms512 INTEGER(0..511), ms640 INTEGER(0..639), ms1024 INTEGER(0..1023), ms1280 INTEGER(0..1279), ms2048 INTEGER(0..2047), ms2560 INTEGER(0..2559), ms5120 INTEGER(0..5119), ms10240 INTEGER(0..10239)

To support new long DRX cycles based on rational numbers, there could be two options:

- Option 1: Directly extend the parameter of LongCycleStartOffset by adding new DRX cycle values based on rational numbers.

- Option 2: Introduce a new separate parameter in DRX-Config dedicated to configuration of new DRX cycle values based on rational numbers.

We think Option 1 is rather complicated as some re-structuring of the parameter of LongCycleStartOffset may be needed, which could lead to some backward-compatibility issues. Option 2, on the other hand, is a cleaner approach that allows the gNB to configure rational number based DRX cycles without changing the structure of the legacy parameter. Hence, we think a new separate parameter in DRX-Config should be introduced for the purpose of configuring rational number based DRX cycles. Additionally, this new parameter should not be configured together with LongCycleStartOffset in the same DRX group to avoid confusion.

Proposal 2: A new separate parameter in DRX-Config is introduced for configuration of DRX cycles based on rational numbers.

Finally, we would like to discuss what are rational number values that can be configured for DRX cycle. From our perspective, we think we should at least cover all the most common XR rates including 15, 30, 45, 60, 72, 90, and 120 fps (as mentioned in [2]). Thus, we can specify the following rational numbers for the DRX cycle values:

- 15 fps: 200/3 msec

- 30 fps: 100/3 msec

- 45 fps: 200/9 msec

- 60 fps: 50/3 msec

- 72 fps: 125/9 msec

- 90 fps: 100/9 msec

- 120 fps: 25/3 msec

Proposal 3: In Rel-18, specify the rational number based DRX cycles to support common XR frame rates including 15, 30, 45, 60, 72, 90, and 120 fps.

# Conclusions

This contribution presents some of our views on rational number based DRX cycles, We propose the following:

Proposal 1: RAN2 should discuss whether rational number based DRX cycle is applicable only to Long DRX cycle, or also to Short DRX cycle.

Proposal 2: A new separate parameter in DRX-Config is introduced for configuration of DRX cycles based on rational numbers.

Proposal 3: In Rel-18, specify the rational number based DRX cycles to support common XR frame rates including 15, 30, 45, 60, 72, 90, and 120 fps.

# References

- RP-230786, Updated WID on XR Enhancements for NR, Nokia, Qualcomm, 3GPP TSG RAN #99, March 2023, Rotterdam, Netherlands.

- R2-2306920, XR Enhancements, Nokia, Qualcomm (Rapporteurs), 3GPP TSG RAN WG2 Meeting #122, May 2023, Incheon, Korea.