3GPP TSG-RAN2 Meeting #123	R2-2308071

Toulouse France, August 21-25, 2023

Source: 			ZTE Corporation, Sanechips

Title: 	Discussion on inter-RAT LTE measurements without gap

Agenda item:	7.25.1

Document for: 	Discussion and Decision

- Introduction

RAN2 received an LS from RAN4 on the inter-RAT measurement without gaps [1]. And according to the LS, RAN4 has determined how to support inter-RAT LTE measurements without gap in the case that there is vacant RF chains for UE measurements (case b-1) and in the case that LTE CRS to be measured are fully contained within UE’s active BWP (case b-2). Besides, RAN4 also agreed to introduce the effective measurement window for inter-RAT LTE measurement to determine the location of scheduling and measurement restriction. In this paper, we’d like to analyze the impact of these enhancements on the specification.

- Discussion

## 2.1 How to support case b-1?

Regarding case b-1, RAN4 agreed that the existing NeedForGapNCSG-InfoEUTRAN-r17 in the Rel-17 can be reused to support case b-1. The corresponding agreements and IE NeedForGapNCSG-InfoEUTRAN-r17 are as follows:

- For case b-1 - Reuse NeedForGapNCSG-InfoEUTRA-r17 to support Case b-1 - Define requirements for case when UE reports “nogap-noncsg” in NeedForGapNCSG-InfoEUTRA-r17 for indicating no-gap without interruption.

NeedForGapNCSG-InfoEUTRA-r17 ::=   SEQUENCE {

needForNCSG-EUTRA-r17              SEQUENCE (SIZE (1..maxBandsEUTRA)) OF NeedForNCSG-EUTRA-r17

}

NeedForNCSG-EUTRA-r17  ::=         SEQUENCE {

bandEUTRA-r17                      FreqBandIndicatorEUTRA,

gapIndication-r17                  ENUMERATED {gap, ncsg, nogap-noncsg}

}

gapIndication Indicates whether measurement gap or NCSG is required for the UE to perform measurements on the concerned E-UTRA target band while NR-DC or NE-DC is not configured. The UE determines this information based on the resultant configuration of the RRCReconfiguration message or RRCResume message that triggers this response. Value gap indicates that a measurement gap is needed, value ncsg indicates that NCSG is needed, value nogap-noncsg indicates neither a measurement gap nor a NCSG is needed.

In the existing specification, it is unclear that the value nogap-noncsg indicates ‘no gap without interruption’ and/or ‘no gap with interruption’. While according to the RAN4’s discussions in the Rel-18, value ncsg indicates ‘no gap with interruption’, and value nogap-noncsg indicates ‘no gap without interruption’ in the inter-RAT LTE measurement with vacant RF chains available. To make it clear, we suggest to clarify the meaning of the value nogap-noncsg in the specification to support case b-1 in the Rel-18. Additionally, there are the following four scenarios:

- Scenario 1: Rel-17 UE in a Rel-17 network;

- Scenario 2: Rel-17 UE in a Rel-18 network;

- Scenario 3: Rel-18 UE in a Rel-17 network;

- Scenario 4: Rel-18 UE in a Rel-18 network.

For scenario 4, it is obvious that UE and network have the same understanding of the value nogap-noncsg. However, for scenarios 1，2 and 3, due to the fact that the meaning of the value nogap-noncsg in Rel-17 is not specified, UE and NW may have the mismatched understandings. Thus, we suggest to also clarify the meaning of value nogap-noncsg in the Rel-17. And in our view, if the UE reports nogap-noncsg in the Rel-17, the interruption is not allowed.

Proposal 1: RAN2 to clarify that value nogap-noncsg in the NeedForNCSG-InfoEUTRA-r17 means no gap without interruption in the Rel-17 and Rel-18 specifications.

## 2.2 How to support case b-2?

RAN4 has agreed to introduce a new per-UE capability to support inter-RAT LTE measurement when LTE CRS to be measured is completely contained within UE’s active BWP, and only ‘gap’ and ‘no gap no interruption’ are considered for case b-2, as follows:

- For case b-2 - A new per-UE capability to support Case b-2 should be defined. - Introduce a new per-UE capability to support case b-2 similar as Rel-16 inter-frequency measurement without gap - “No gap with interruption” is not considered for case b-2 - No interruption is considered for case b-2

Regarding signaling design, RAN4 thinks interFrequencyMeas-NoGap-r16 can be taken as reference. The IE interFrequencyMeas-NoGap-r16 is as follows:

MeasAndMobParametersFRX-Diff ::=            SEQUENCE {

** skip non-related part **

interFrequencyMeas-NoGap-r16                ENUMERATED {supported}              OPTIONAL,

** skip non-related part **

}

| Definitions for parameters | Per | M | FDD-TDD DIFF | FR1- FR2 DIFF |
| --- | --- | --- | --- | --- |
| interFrequencyMeas-NoGap-r16 Indicates whether the UE can perform inter-frequency SSB based measurements without measurement gaps if the SSB is completely contained in the active BWP of the UE as specified in TS 38.133 [5]. If this parameter is indicated for FR1 and FR2 differently, each indication corresponds to the frequency range of cells to be measured. | UE | No | No | Yes |

Considering the using scenario is that the UE’s active BWP contains LTE CRS, the new capability is only applicable in FR1. Thus, the signaling design and description are as follows:

MeasAndMobParametersCommon::=            SEQUENCE {

** skip non-related part **

interRAT-LTEMeas-NoGapNoInterr-r18     ENUMERATED {supported}       OPTIONAL,

** skip non-related part **

}

| Definitions for parameters | Per | M | FDD-TDD DIFF | FR1- FR2 DIFF |
| --- | --- | --- | --- | --- |
| interRAT-LTEMeas-NoGapNoInterr-r18 Indicates whether the UE can perform inter-RAT LTE no-gap measurements without interruption if the LTE CRS to be measured is completely contained in the active BWP of the UE as specified in TS 38.133 [5]. If this field is absent, a measurement gap is needed. | UE | No | No | FR1 only |

Proposal 2: To introduce a per-UE level capability (e.g. interRAT-LTEMeas-NoGapNoInterr-r18) in MeasAndMobParametersCommon to indicate whether gap and interruption can be avoided when the measured LTE CRS is within UE’s active BWP. The new UE capability is only applicable in FR1.

## 2.3 Effective measurement window

Regarding effective measurement window (EMW), RAN4 has achieved the following agreements:

- Agreement - Introduce the effective measurement window for inter-RAT LTE measurement, including offset, duration and periodicity. - The effective measurement window is used to determine the location of scheduling and measurement restriction. - Candidate values for effective measurement window duration: 5ms - Other values are not precluded - Candidate values for periodicity of effective measurement window: 40ms and 80ms - Other values are not precluded

The motivation of introducing EMW is to determine the location of scheduling and measurement restriction for case b-1 and case b-2. It is noted that the inter-RAT LTE no-gap measurement includes two cases: with interruption and without interruption. Is the network allowed to configure EMW in the above two cases? In our view, EMW is only applicable in the inter-RAT LTE no-gap measurement without interruption. Because in the case that no gap with interruption, NCSG would be applied, and the location of scheduling and measurement restriction can be determined according to the corresponding NCSG pattern. While in the case that no gap no interruption, network is unclear of which symbol the UE can perform measurement on, thus EMW is needed.

Proposal 3: RAN2 confirms that EMW (effective measurement window) is only applicable when UE performs inter-RAT LTE no-gap measurement without interruption.

According to the LS, the periodicity, duration and offset need to be included in the EMW configuration. And it is also noted that the exact values for these parameters have not been completely determined, thus, some reserved spares are needed. The corresponding signaling design for EMW is as follows:

MeasConfig ::=                      SEQUENCE {

measObjectToRemoveList              MeasObjectToRemoveList               OPTIONAL,   -- Need N

measObjectToAddModList              MeasObjectToAddModList               OPTIONAL,   -- Need N

reportConfigToRemoveList            ReportConfigToRemoveList             OPTIONAL,   -- Need N

reportConfigToAddModList            ReportConfigToAddModList             OPTIONAL,   -- Need N

measIdToRemoveList                   MeasIdToRemoveList                     OPTIONAL,   -- Need N

measIdToAddModList                   MeasIdToAddModList                     OPTIONAL,   -- Need N

s-MeasureConfig                       CHOICE {

ssb-RSRP                            RSRP-Range,

csi-RSRP                            RSRP-Range

}                                                                                  OPTIONAL,   -- Need M

quantityConfig                      QuantityConfig                             OPTIONAL,   -- Need M

measGapConfig                       MeasGapConfig                              OPTIONAL,   -- Need M

measGapSharingConfig               MeasGapSharingConfig                     OPTIONAL,   -- Need M

...,

[[

interFrequencyConfig-NoGap-r16      ENUMERATED {true}                     OPTIONAL    -- Need R

]]

effectiveMeasWindowConfig-r18  SetupRelease{EffectiveMeasWindowConfig-r18}     OPTIONAL,   -- Need M

}

EffectiveMeasWindowConfig-r18::= SEQUENCE{

periodicityAndOffset            CHOICE {

ms40                INTEGER(0..39),

ms80                INTEGER(0..79),

...

}

duration               ENUMERATED {ms5, spare3, spare2, spare1}

nonCriticalExtension SEQUENCE {} OPTIONAL

}

effectiveMeasWindowConfig Used to setup and release effective measurement window in NR.

periodicityAndOffset Periodicity and offset for effective measurement window in which to receive LTE CRS, see TS 38.133. Periodicity and offset are given in ms.

Duration Indicates the effective measurement window duration.

Proposal 4: To define the EMW configuration (including periodicityAndOffset and duration) in MeasConfig.

Regarding how the network can know whether the UE supports EMW feature or not, there are the following two cases:

Case 1: If the capability for EMW is mandatory (the UE supporting case b-1 and case b-2 always supports EMW configuration), reuse the capabilities for case b-1 and case b-2.

Case 2: If the capability for EMW is optional (the UE supporting case b-1 and case b-2 optionally supports EMW configuration), introduce a new UE capability for EMW configuration.

Since RAN4 has not finished the RRM requirements design for case b-1 and case b-2, we are not sure whether the capability for EMW is mandatory or optional. However, even if the capability is mandatory, we can not reuse the capabilities for case b-1 and case b-2 directly. Because RAN4 decided to reuse legacy signaling to support case b-1 instead of introducing new UE capability, thus, the Rel-18 network can not distinguish the case that Rel-18 UE can support case b-1 and the case the Rel-18 UE can not support case b-1 but can support NCSG. In this way, we need to introduce new capability for case b-1. Considering no matter the capability for EMW is mandatory or optional, a new capability is always needed. We suggest to introduce new UE capability for EWM configuration, which is applicable in both case 1 and case 2. The corresponding signaling is as follows:

MeasAndMobParametersCommon ::=            SEQUENCE {

** skip non-related part **

effectiveMeasWindow-r18                ENUMERATED {supported}              OPTIONAL,

** skip non-related part **

}

| Definitions for parameters | Per | M | FDD-TDD DIFF | FR1- FR2 DIFF |
| --- | --- | --- | --- | --- |
| effectiveMeasWindow-r18 Indicates whether the UE supports effective measurement window in the inter-RAT LTE measurement. | UE | No | No | No |

Proposal 5: RAN2 to introduce new UE capability for EMW configuration in the MeasAndMobParametersCommon.

- Conclusion and proposals

Proposal 1: RAN2 to clarify that value nogap-noncsg in the NeedForNCSG-InfoEUTRA-r17 means no gap without interruption in the Rel-17 and Rel-18 specifications.

Proposal 2: To introduce a per-UE level capability (e.g. interRAT-LTEMeas-NoGapNoInterr-r18) in MeasAndMobParametersCommon to indicate whether gap and interruption can be avoided when the measured LTE CRS is within UE’s active BWP. The new UE capability is only applicable in FR1.

Proposal 3: RAN2 confirms that EMW (effective measurement window) is only applicable when UE performs inter-RAT LTE no-gap measurement without interruption.

Proposal 4: To define the EMW configuration (including periodicityAndOffset and duration) in MeasConfig.

Proposal 5: RAN2 to introduce new UE capability for EMW configuration in the MeasAndMobParametersCommon.

- Reference

[1] R2-2307041  LS on inter-RAT measurements without gaps (R4- 2310158; contact: Intel) Rel-18 NR_MG_enh2-Core  From RAN4 To RAN2