3GPP TSG-RAN WG2 #123	R2-2308541

Toulouse, France, Aug 21st – 25th, 2023

Agenda Item:	7.6.2.1

Source:	Ericsson

Title:	R18 IoT NTN HARQ enhancements

Document for:	Discussion, Decision

# 1	Introduction

At RAN2#121bis, the following was agreed:

Agreements:

- RAN2#121’s agreement is revised to “For NB-IoT NTN with single HARQ process when the HARQ feedback is disabled, the UE will start/restart drx-inactivity timer in the subframe containing the last repetition of the corresponding PDSCH reception plus 12 subframes plus deltaPDCCH” (Can further check in the NB-IoT session if anything needs to be done for legacy NB-IoT as well, as some timers don’t take deltaPDCCH into account)

- Wait for RAN1’s decision on the RRC signalling of enabling DCI-based solution to indicate HARQ feedback enabled/disabled, and the signalling granularity, e.g. per UE or per HARQ process

- On DCI indication overriding RRC configuration for the HARQ feedback enabled/disabled, wait for RAN1’s progress on DCI-based solution before discussing related DRX impact in RAN2.

- On DL multiple TB scheduling, wait for RAN1’s progress before discussing related DRX impact in RAN2.

- P4 in R2-2302557 is not agreed, i.e. no special handling for single HARQ process for eMTC.

- For eMTC NTN, a parameter harq-FeedbackEnablingforSPSactive could be configured for a UE. If harq-FeedbackEnablingforSPSactive is configured to enable HARQ feedback, UE reports ACK/NACK for the first SPS PDSCH after activation, regardless of if HARQ feedback is enabled or disabled corresponding to the first SPS PDSCH after activation.

- For a NB-IoT UE configured with a single HARQ process in HARQ mode B, send LS to RAN1 and ask for the “processing time for starting drx-InactivityTimer (i.e. start to monitor NPDCCH)”. (can further check the detailed wording of the question)

- Network implementation resolves the issue of ambiguity on start of DRX inactivity timer after the PUSCH transmission by not scheduling the NPDCCH back-to-back during the ambiguity period (i.e., Koffset – UE’s TA)

- Send LS to RAN1 to check for UL multiple TB scheduling, which UL HARQ mode combination(s) are to be supported.

- In the LS to RAN1, we don’t include a question on whether RAN1 intends to introduce the DCI-based solution for the UL HARQ mode

- UL transmission using SPS can be configured with HARQ mode B

- P1 in R2-2303713 is not agreed, i.e. do not enhance the LCP restriction based on uplinkHARQ-Mode for different RLC PDU types

- Send LS to RAN1 informing RAN2’s agreements and also including potential questions to be checked with RAN1

Agreements via email – from offline 103 – second round:

- Add one more question in the LS to check with RAN1 which of the below understandings is correct for the RAN1 agreement.

Understanding 1: For a DL HARQ process with disabled HARQ feedback in NB-IoT, UE is not required to monitor NPDCCH for the same HARQ process in a period of Y=12(ms) from the end of reception of the NPDSCH.

Understanding 2: For a DL HARQ process with disabled HARQ feedback in NB-IoT, UE is not required to monitor NPDCCH for all the HARQ processes in a period of Y=12(ms) from the end of reception of the NPDSCH.

- RAN2 further discuss whether UL transmission using PUR can be configured with HARQ mode B.

Further an LS [1] was sent to RAN1 and they provided a reply [2]:

1. Overall Description:

RAN1 would like to thank RAN2’s LS on HARQ enhancements. Regarding RAN2 raised questions, RAN1 has discussed the corresponding responses in the RAN1#113 meeting. The following RAN1 agreements are achieved.

Agreement Adopt the response below to RAN2’s question 3: - Understanding 2 is the correct understanding. Agreement For a NB-IoT UE operating with two HARQ processes, for an UL HARQ process with HARQ mode B, the minimum time between the end of NPUSCH transmission and the start of NPDCCH monitoring for the same HARQ process is 1 ms. - Note: this implies a RAN1 specification change in Rel-18 Agreement For the response to RAN2’s question 1a: provide the agreement above. Agreement Adopt the response below to RAN2’s question 1b: - For an UL HARQ process with HARQ mode B for eMTC UEs, the minimum time between the end of PUSCH transmission and the start of MPDCCH monitoring for the same HARQ process is 1 ms for Type B half-duplex FDD operation, and there is no minimum time restriction between the end of PUSCH transmission and the start of MPDCCH monitoring for full-duplex FDD operation. Agreement Adopt the response below to RAN2’s question 2: - Whether/how to support case 1, case 2 and/or case 3 for UL multiple TB scheduling for eMTC and NB-IoT is transparent to RAN1 and can be left to RAN2 decision. Agreement For a NB-IoT UE operating with one HARQ process, for an UL HARQ process with HARQ mode B, the minimum time between the end of NPUSCH transmission and the start of NPDCCH monitoring is 1 ms. - Note: this implies a RAN1 specification change in Rel-18 Agreement - For the response to RAN2’s question 1a: provide the agreement above.

Please find RAN1 responses below:

RAN2’s question 1a: For an UL HARQ process with HARQ mode B for NB-IoT UEs, what is the minimum time between the end of NPUSCH transmission and the start of NPDCCH monitoring for the same HARQ process?

RAN1 response:

For a NB-IoT UE operating with two HARQ processes, for an UL HARQ process with HARQ mode B, the minimum time between the end of NPUSCH transmission and the start of NPDCCH monitoring for the same HARQ process is 1 ms.

- Note: this implies a RAN1 specification change in Rel-18.

For a NB-IoT UE operating with one HARQ process, for an UL HARQ process with HARQ mode B, the minimum time between the end of NPUSCH transmission and the start of NPDCCH monitoring is 1 ms.

- Note: this implies a RAN1 specification change in Rel-18.

RAN2’s question 1b: For an UL HARQ process with HARQ mode B for eMTC UEs, what is the minimum time between the end of PUSCH transmission and the start of MPDCCH monitoring for the same HARQ process?

RAN1 response:

For an UL HARQ process with HARQ mode B for eMTC UEs, the minimum time between the end of PUSCH transmission and the start of MPDCCH monitoring for the same HARQ process is 1 ms for Type B half-duplex FDD operation, and there is no minimum time restriction between the end of PUSCH transmission and the start of MPDCCH monitoring for full-duplex FDD operation.

RAN2’s question 2: For UL multiple TB scheduling, which of the following HARQ mode combinations does RAN1 intend to support for eMTC and NB-IoT?

- Case 1: all HARQ processes corresponding to the scheduled multiple TBs are configured with HARQ mode A

- Case 2: all HARQ processes corresponding to the scheduled multiple TBs are configured with HARQ mode B

- Case 3: some HARQ processes corresponding to the scheduled multiple TBs are configured with HARQ mode A and the others are configured with HARQ mode B

RAN1 response:

Whether/how to support case 1, case 2 and/or case 3 for UL multiple TB scheduling for eMTC and NB-IoT is transparent to RAN1 and can be left to RAN2 decision.

RAN2’s question 3: For the above RAN1 agreement, which is the correct understanding?

- Understanding 1: For a DL HARQ process with disabled HARQ feedback in NB-IoT, UE is not required to monitor NPDCCH for the same HARQ process in a period of Y=12(ms) from the end of reception of the NPDSCH.

- Understanding 2: For a DL HARQ process with disabled HARQ feedback in NB-IoT, UE is not required to monitor NPDCCH for all the HARQ processes in a period of Y=12(ms) from the end of reception of the NPDSCH.

RAN1 response:

Understanding 2 is the correct understanding.

2. Actions:

To RAN2

ACTION: 	RAN1 respectfully asks RAN2 to take the above responses into account in the future work.

RAN2’s question 1b: For an UL HARQ process with HARQ mode B for eMTC UEs, what is the minimum time between the end of PUSCH transmission and the start of MPDCCH monitoring for the same HARQ process?

RAN1 response:

For an UL HARQ process with HARQ mode B for eMTC UEs, the minimum time between the end of PUSCH transmission and the start of MPDCCH monitoring for the same HARQ process is 1 ms for Type B half-duplex FDD operation, and there is no minimum time restriction between the end of PUSCH transmission and the start of MPDCCH monitoring for full-duplex FDD operation.

RAN2’s question 2: For UL multiple TB scheduling, which of the following HARQ mode combinations does RAN1 intend to support for eMTC and NB-IoT?

- Case 1: all HARQ processes corresponding to the scheduled multiple TBs are configured with HARQ mode A

- Case 2: all HARQ processes corresponding to the scheduled multiple TBs are configured with HARQ mode B

- Case 3: some HARQ processes corresponding to the scheduled multiple TBs are configured with HARQ mode A and the others are configured with HARQ mode B

RAN1 response:

Whether/how to support case 1, case 2 and/or case 3 for UL multiple TB scheduling for eMTC and NB-IoT is transparent to RAN1 and can be left to RAN2 decision.

RAN2’s question 3: For the above RAN1 agreement, which is the correct understanding?

- Understanding 1: For a DL HARQ process with disabled HARQ feedback in NB-IoT, UE is not required to monitor NPDCCH for the same HARQ process in a period of Y=12(ms) from the end of reception of the NPDSCH.

- Understanding 2: For a DL HARQ process with disabled HARQ feedback in NB-IoT, UE is not required to monitor NPDCCH for all the HARQ processes in a period of Y=12(ms) from the end of reception of the NPDSCH.

RAN1 response:

Understanding 2 is the correct understanding.

2. Actions:

To RAN2

ACTION: 	RAN1 respectfully asks RAN2 to take the above responses into account in the future work.

In this paper we discuss our view on the HARQ enhancements.

# 2	Discussion

## 2.1 Timing based on the RAN1 reply LS

Summary of the timing replies in RAN1 LS reply:

|  | NB-IoT NPDSCH reception on HARQ process with disabled HARQ feedback | NB-IoT NPUSCH transmission on HARQ process with HARQ mode B | NB-IoT NPDSCH reception on HARQ process with enabled HARQ feedback | NB-IoT NPUSCH transmission on HARQ process with HARQ mode A |
| --- | --- | --- | --- | --- |
| One HARQ process operation | Not required to monitor any NPDCCH for 12 ms after end of NPDSCH | Minimum time from end of NPUSCH until monitoring NPDCCH for the same HARQ process is 1 ms |  |  |
| Two HARQ processes operation | Same as for one HARQ process | Same as for one HARQ process |  |  |

|  | eMTC PDSCH reception on HARQ process with disabled HARQ feedback | eMTC NPUSCH transmission on HARQ process with HARQ mode B | eMTC PDSCH reception on HARQ process with enabled HARQ feedback | eMTC NPUSCH transmission on HARQ process with HARQ mode A |
| --- | --- | --- | --- | --- |
| One HARQ process operation |  | Minimum time from end of PUSCH until monitoring MPDCCH for the same HARQ process is 1 ms for half duplex FDD operation and 0 ms otherwise. |  |  |
| Two HARQ processes operation |  | Same as for one HARQ process |  |  |

In light of the information from RAN1, the following RAN2 agreements needs to be revised:

- For NB-IoT NTN with single HARQ process in HARQ mode B, the UE will start/restart drx-inactivity timer in the subframe containing the last repetition of the corresponding PUSCH transmission (can still check whether other alternatives also work)

Therefore we propose

For NB-IoT NTN UE with two HARQ processes, the HARQ feedback of one the two HARQ processes is disabled 7and one is enabled, scheduled with single TB:
	-	stop the drx-InactivityTimer (as in legacy, no spec change expected);
-	if the HARQ process with HARQ feedback disabled is scheduled: start/restart drx-InactivityTimer in the subframe containing the last repetition of the corresponding PDSCH reception plus 12 subframes;
-	if the HARQ process with HARQ feedback enabled is scheduled: start HARQ RTT Timer in the subframe containing the last repetition of the corresponding PDSCH reception (as in legacy no spec change expected) [and then start/restart drx-InactivityTimer when HARQ RTT Timer expires (as in legacy, no spec change expected)].

For NB-IoT NPUSCH transmission on a HARQ process with HARQ mode B, start the

For HARQ processes that has HARQ mode B, the drx-InactivityTimer is (re)started after a period equal to: 
	-	for NB-IoT: 1 ms after end of latest PUSCH repetition;
-	for eMTC with FDD half duplex, 1 ms after latest PUSCH repetition;
-	for eMTC not with FDD half duplex, 0 ms after latest PUSCH repetition.

## 2.2 PUR and HARQ mode B

As Preconfigured Uplink Resources (PUR) shall be rare occasions without need connection setup – there is no need whatsoever to try saving PDCCH resources by not starting the UL HARQ RTT Timer. If the UE has lots of data in its buffer, it needs to enter connected mode instead.

PUR are rare transmissions of small amounts of data without connection setup.

PUR transmissions are not configured with an HARQ mode A nor HARQ mode B.

# 3	Conclusions

In the previous sections we made the following observations:

Observation 1	PUR are rare transmissions of small amounts of data without connection setup.

Based on the discussion in the previous sections we propose the following:

Proposal 1	For NB-IoT NTN UE with two HARQ processes, the HARQ feedback of one the two HARQ processes is disabled 7and one is enabled, scheduled with single TB:  - stop the drx-InactivityTimer (as in legacy, no spec change expected); - if the HARQ process with HARQ feedback disabled is scheduled: start/restart drx-InactivityTimer in the subframe containing the last repetition of the corresponding PDSCH reception plus 12 subframes; - if the HARQ process with HARQ feedback enabled is scheduled: start HARQ RTT Timer in the subframe containing the last repetition of the corresponding PDSCH reception (as in legacy no spec change expected) [and then start/restart drx-InactivityTimer when HARQ RTT Timer expires (as in legacy, no spec change expected)].

Proposal 2	For NB-IoT NPUSCH transmission on a HARQ process with HARQ mode B, start the

Proposal 3	For HARQ processes that has HARQ mode B, the drx-InactivityTimer is (re)started after a period equal to:   - for NB-IoT: 1 ms after end of latest PUSCH repetition; - for eMTC with FDD half duplex, 1 ms after latest PUSCH repetition; - for eMTC not with FDD half duplex, 0 ms after latest PUSCH repetition.

Proposal 4	PUR transmissions are not configured with an HARQ mode A nor HARQ mode B.

# References

R2-2304274 LS on HARQ Enhancements (contact: OPPO)	RAN2	LS out	Rel-18	NR_NTN_enh-Core	To:RAN1

R1-2306182 Reply LS on HARQ Enhancements (contact: OPPO)	RAN1	LS out	Rel-18	NR_NTN_enh-Core	To:RAN2