3GPP TSG-RAN WG2 Meeting #123		R2-2308739

Toulouse, France, Aug. 21th- Aug. 25th, 2023

Agenda Item:	7.3.4

Source:	vivo

Title:	Discussion on cell selection/re-selection

Document for:	Discussion and Decision

Introduction

In RAN2#122 meeting, cell selection/re-selection mechanism for network energy saving was further discussed with the below agreement [1]:

- We will define UE capabilities with signaling.  Details are FFS and will be discussed later during the WI phase.

- Separate camping restrictions for NES-capable and non-NES UEs will be defined.  FFS if it is a single bit or more.

In this contribution, we provide our further consideration on cell selection/re-selection for network energy saving.

Discussion

According to the agreement made in RAN2#122, RAN2 confirms that there is a need to have separate camping restrictions for NES-capable UE and legacy UE. Whether this can be done by a single bit or more is FFS. From both UE and network side, the UE and the network may expect to support or apply one or more Rel-18 NES features (e.g. Cell DTX/DRX, Spatial adaptation) or further NES features that may be introduced in future releases. For example, if the network applies Cell DTX/DRX feature, while the UE only support spatial adaptation, it does not make sense to allow this NES capable UE but bar non-NES capable UE. So, the network should be able to allow the UE who supports specific NES feature (cell DTX/DRX in this example) while bar the others. Based on the discussion so far, we see cell DTX/DRX and spatial adaptation NES features can be considered, since the UE in the cell applying such NES features may have performance loss.

Proposal 1: Introduce two bits cell bar to distinctively restrict camping of NES-capable UEs that support Cell DTX/DRX and Spatial adaptation NES features.

To prevent legacy UE from camping on the NES Cell, the network can configure cellBarred in MIB as barred. The NES-capable UE that supporting any NES features can just ignore cellBarred in MIB. Accordingly, to control the NES-capable UE that supporting different NES features camping on the NES cell applying specific Rel-18 NES features, the network can configure two bits cellBarredNES-r18 in SIB1 (e.g. one bit for Cell DTX/DRX, another bit for Spatial adaptation). If any NES feature in cellBarredNES-r18 is set to ‘not barred’ and the NES-capable UE is capable to the corresponding NES feature, the NES-capable UE considers the cell as not barred.

For example, whether the NES-capable UE considers the NES cell as barred or not can be achieved through the following table.

| Camping restriction setting of the NES cell that supports/applies spatial adaptation | cellBarredCellDTX-DRX-r18 = ‘not barred’ cellBarredSpatialAdapat-r18 = ‘barred’ | cellBarredCellDTX-DRX-r18 = ‘barred’ cellBarredSpatialAdapat-r18 = ‘not barred’ |
| --- | --- | --- |
| NES-capable UE that only supports Cell DTX/DRX | Consider the cell as not barred. | Consider the cell as barred. |
| NES-capable UE that only supports Spatial adaptation | Consider the cell as barred. | Consider the cell as not barred. |
| NES-capable UE that supports Cell DTX/DRX and Spatial adaptation | Consider the cell as not barred | Consider the cell as not barred. |

Table 1: cell barring for different NES features.

Proposal 2: The NES-capable UE ignores the cellBarred in MIB and uses the cellBarredNES-r18 in SIB1.

Proposal 3: If any introduced cell bar cellBarredNES-r18 is set to not barred and the NES-capable UE is capable of the corresponding NES feature, the NES-capable UE considers the cell as not barred.

Conclusion

Based on the discussion, we have the following observation and proposals:

Proposal 1: Introduce two bits cell bar to distinctively restrict camping of NES-capable UEs that support Cell DTX/DRX and Spatial adaptation NES features.

Proposal 2: The NES-capable UE ignores the cellBarred in MIB and uses the cellBarredNES-r18 in SIB1.

Proposal 3: If any introduced cell bar cellBarredNES-r18 is set to not barred and the NES-capable UE is capable of the corresponding NES feature, the NES-capable UE considers the cell as not barred.

References

[1] R2#122 meeting minutes