3GPP TSG-RAN WG2 Meeting #123	R2-2308772

# Toulouse, France, August 21-25, 2023

Agenda item:	7.4.4

Source: 	China Telecom

Title:	Discussion on remaining issues of CHO with candidate SCGs

WID/SID:	NR_Mob_enh2-Core

Document for:	Discussion and Decision

- Introduction

In last RAN2 meeting, CHO including target MCG and candidate SCGs related issues have been widely discussed and much progress has been made. However, there are still some open issues and stage-3 details left for further discussion. Some agreements on the execution of CHO with candidate SCG(s) are listed as below.

Agreements in RAN2#122 When the CPA/CPC execution condition is met but no CHO execution condition is met, the UE continues to evaluate both CHO and CPA/CPC execution conditions. For CHO+CPC we only consider execution when BOTH conditions are met. - (When the CHO execution condition is met but no CPC execution condition is met, if there is an available CHO-only or Rel-17 CHO with SCG configuration for which the CHO condition is met, the UE performs the CHO-only or Rel-17 CHO with SCG execution, and THUS the network can handle such situation by providing proper configurations).

In this contribution, we would like to share our opinions on the following remaining issues of CHO with candidate SCGs based on the agreements reached in previous RAN2 meetings.

- Whether the legacy CHO recovery mechanism applies to the configuration for CHO with candidate SCG(s).

- Whether to stop conditional reconfiguration evaluation for CHO with candidate SCG(s) upon initiating SCG failure information procedure.

- Whether to support condEventA3 or condEventA5 for the execution conditions for candidate PSCells for CHO with candidate SCG(s)

- Discussion

CHO recovery for CHO with candidate SCG(s)

In the post meeting discussion on running CR for CHO with candidate SCG(s), the issue of whether the legacy CHO recovery mechanism applying to CHO with candidate SCG(s) was discussed with no consensus reached. Generally, we tend to agree the motivation of enabling the legacy CHO recovery mechanism also to apply for CHO with candidate SCG(s), but the details may need further discussion.

Based on the agreements reached in the last RAN2 meeting, RAN2 confirms that CHO with candidate SCG(s) can only be executed when both conditions (CHO+CPA/CPC) are met. However, the legacy CHO recovery mechanism is based on cell reselection criteria for a suitable cell, so only the quality of PCell will be evaluated which is not aligned with the principle used in Rel-18 CHO with candidate SCG(s). In this sense, we think the legacy CHO recovery mechanism could not apply to the Rel-18 configuration for CHO with candidate SCG(s) directly. When a candidate PCell with candidate SCG(s) is selected during RRC re-establishment procedure, if there is an available CHO-only or Rel-17 CHO with SCG configuration associated to the selected PCell, then the CHO recovery mechanism could be conducted.

Proposal 1: If there is an available CHO-only or Rel-17 CHO with SCG configuration associated to the selected PCell with a Rel-18 CHO with candidate SCG(s) configuration, then the CHO recovery mechanism could be applied to CHO-only configuration or Rel-17 CHO with SCG configuration.

Stop CHO with candidate SCG(s) evaluation upon SCG failure

Another issue to be addressed in RAN2 is whether to stop conditional reconfiguration evaluation for CHO with candidate SCG(s) upon initiating SCG failure information procedure.

Copied from TS 38.331, clause 5.7.3.2 Upon initiating the procedure, the UE shall: 1> if the procedure was not initiated due to beam failure of the PSCell while the SCG is deactivated: 2> suspend SCG transmission for all SRBs, DRBs and, if any, BH RLC channels; 2> reset SCG MAC; 1> stop T304 for the SCG, if running; 1> stop conditional reconfiguration evaluation for CPC or CPA, if configured; 1> if the UE is in (NG)EN-DC: 2> initiate transmission of the SCGFailureInformationNR message as specified in TS 36.331 [10], clause 5.6.13a. 1> else: 2> initiate transmission of the SCGFailureInformation message in accordance with 5.7.3.5.

Based on the current mechanism listed above, the evaluation of CPC or CPA shall be stopped upon initiating the SCG failure information procedure. To follow the legacy behaviour, the UE shall stop the evaluation of CPC or CPA conditions configured in Rel-18 CHO with candidate SCG(s) configuration. As per the agreements made in last RAN2 meeting, only when BOTH conditions (CHO+CPC/CPA) are met will the UE perform the CHO with candidate SCG(s). Therefore, the execution condition will never be met in this case. We prefer to stop CHO with candidate SCG(s) evaluation upon initiating SCG failure information procedure. And if there is an available CHO-only or Rel-17 CHO with SCG configuration, then the evaluation of CHO conditions is not affected as usual.

Proposal 2: Upon initiating SCG failure information procedure, the UE shall stop conditional reconfiguration evaluation for CHO with candidate SCG(s). If there is an available CHO-only or Rel-17 CHO with SCG configuration, the evaluation of CHO conditions is not affected.

Whether to support condEventA3 or condEventA5

Whether to support condEventA3 or condEventA5 for the execution conditions for candidate PSCells for CHO with candidate SCG(s) is one open issue left for further discussion. We understand the main intention of introducing CHO with candidate SCG(s) in Rel-18 is to have a relatively good enough PSCell together with the target PCell after HO, so that the service data rate and UE experience could be guaranteed. In this sense, CHO with candidate SCG(s) is more like a CPA instead of CPC, then condEventA4 is a good choice.

Besides, supporting condEventA3 or condEventA5 for CHO with candidate SCG(s) may also bring more open issues such as how to generate the execution conditions. We suggest deprioritize the discussion on whether to support condEventA3 or condEventA5 for CHO with candidate SCG(s) in RAN2.

Proposal 3: Deprioritize the support of condEventA3 or condEventA5 for CHO with candidate SCG(s).

- Conclusion

In this contribution, we discuss some remaining issues on CHO with candidate SCG(s) in Rel-18. We kindly ask RAN2 to consider the corresponding proposals listed as below.

Proposal 1: If there is an available CHO-only or Rel-17 CHO with SCG configuration associated to the selected PCell with a Rel-18 CHO with candidate SCG(s) configuration, then the CHO recovery mechanism could be applied to CHO-only configuration or Rel-17 CHO with SCG configuration.

Proposal 2: Upon initiating SCG failure information procedure, the UE shall stop conditional reconfiguration evaluation for CHO with candidate SCG(s). If there is an available CHO-only or Rel-17 CHO with SCG configuration, the evaluation of CHO conditions is not affected.

Proposal 3: Deprioritize the support of condEventA3 or condEventA5 for CHO with candidate SCG(s).

- Reference

RP-22xxxxx	Report of 3GPP TSG RAN WG2 meeting #122

3GPP TS 38.331 17.0.0: "NR; Radio Resource Control (RRC) protocol specification".