3GPP TSG-RAN WG2 #123	R2-2308816

Toulouse, France, 21st – 25th August 2023

Source:	NTT DOCOMO, INC.

Title:	Open issues on Two TAs for multi-DCI multi-TRP

Document for:	Discussion and decision

Agenda Item:	7.20.2

- Introduction

This contribution is to address ongoing open issues on Two TAs for multi-DCI multi-TRP enhancements.

- Discussion

  - Parameter list from RAN1

RAN2 have received the parameter list [1] from RAN1. Parameters related to 2TA enhancements are shown in Annex A. In short, following 4 parameters are listed.

- tag-Id2 (in ServingCellConfig)

- tag-Id-ptr (in TCI-State)

- tag-Id-ptr (in TCI-UL-State)

- n-TimingAdvanceOffset2 (in ServingCellConfig)

There seems to be no functional difficulty, thus we think RAN2 can start to introduce these parameters.

Note that tag-Id-ptr is a field to associate TAG ID to UL/joint TCI State, which RAN2 discussed in the POST e-mail discussion after RAN2#122 and reached a similar proposal [2]:

Proposal 1: each joint/UL TCI state is associated with either TAG1 or TAG2 by RRC configuration.

Proposal 1.	Introduce following fields in TS 38.331 to reflect the RAN1 parameter list. 
- tag-Id2 (in ServingCellConfig)
- tag-Id-ptr (in TCI-State)
- tag-Id-ptr (in TCI-UL-State)
- n-TimingAdvanceOffset2 (in ServingCellConfig)

  - UE feature list from RAN1

RAN2 have received the UE parameter list [3] from RAN1. Features related to 2TA enhancements are shown in Annex B. However, all the features include a lot of FFSes to be discussed more in RAN1. We think RAN2 can wait for RAN1 to address FFS.

Proposal 2.	RAN2 wait for RAN1 progress before starting discussion on implementation of UE features.

  - Maximum number of TAGs

RAN2 asked RAN1 via LS [4] whether 4 TAGs per cell group is enough or extension is needed. RAN1 replied that they reached a following conclusion in May meeting [5].

Q1b: NR currently supports up to 4 TAGs per cell group. Are the 4 TAGs enough or does RAN1 see a need to increase the number of TAGs per cell group? Answer: RAN1 has not reached consensus to increase the current number of TAGs per cell group.

In our understanding, no consensus basically means that it is not pursued. We do not think RAN2 have an enough reason to revert this RAN1 conclusion, so RAN2 can follow RAN1’s intention, i.e., can agree not to increase maximum number of TAGs.

Proposal 3.	RAN2 do not increase maximum number of TAGs per cell group (currently 4).

  - Definition of PTAG/STAG and behavior upon TAT expiry

In the POST e-mail discussion after RAN2#122, we discussed the behavior when both of TATs or one of TATs are expired for SpCell and SCell in intra-cell scenario [2].

- not perform any uplink transmission except the Random Access Preamble and MSGA transmission; - flush all HARQ buffers; - notify RRC to release PUCCH, if configured; - notify RRC to release SRS, if configured; - clear any configured downlink assignments and configured uplink grants; - clear any PUSCH resource for semi-persistent CSI reporting; - maintain NTA (defined in TS 38.211 [8]) of this TAG; - consider all running timeAlignmentTimers as expired.

It should be noted that multiple companies proposed to conclude definition of PTAG and STAG first, especially whether one or two PTAGs are defined for intra-cell two-TA scenario. We have similar understanding, but we rather think we cannot discuss definition by itself because what we want to define for PTAG/STAG is depending on what we want to do upon PTAG/STAG expiry.

Considering RAN2 have continued this kind of chicken-and-egg discussion, to work something out, we would like to start with setting options to be down-selected which enable us to jointly discuss definition of PTAG/STAG and behavior upon TAT expiry. Based on our observation of companies’ opinions, following two options are sufficient.

Option 1. One PTAG

Option 2. Up to two PTAGs

Table 1 shows what behavior upon TAT expiry is expected for each deployment option. These behaviors are mainly derived just from our understanding. We are ok to discuss more and update them appropriately.

Table 1. Comparison of candidate PTAG/STAG frameworks for intra-cell 2TA scenario.

|  | Option 1. One PTAG | Option 2. Up to two PTAGs |
| --- | --- | --- |
| Description | One of two TAGs associated with SpCell is defined as PTAG while the rest of TAGs are defined as STAG even though it is also associated with SpCell. | Both of TAGs associated with SpCell are defined as PTAG and the rest of TAGs are defined as STAG. |
| Behavior when PTAG expires | Conduct 1 ~ 8 for all serving cells. | - If the other PTAG is still running, conduct 1 and 3 ~ 7 for for the TRP in the SpCell. (*1)(*2) - Else (i.e., if the other PTAG is already expired or if both PTAGs are expired simultaneously), conduct 1 ~ 8 for all serving cells. |
| Behavior when STAG expires | - If a serving cell is configured with another TAT which is still running, conduct 1 and 3 ~ 7 for the TRP in the serving cell. (*1)(*2) - Else (i.e., if there is no remaining running TAT configured to a serving cell), conduct 1 ~ 7 for the serving cell. | Same as Option 1. |

*1: On procedure 2 (flush all HARQ buffers), we do not think HARQ buffers for corresponding TRP have to be flushed. This is because, as multiple companies pointed out in [Post122][852][MIMOevo], UE can retransmit HARQ via the other TRP which is associated with running TAT.

*2: On procedures 3 ~ 6 (related to configured grants and semi-persistent scheduling), current spec says resources for CG and SPS are configured per BWP. In our understanding, in intra-cell multi-TRP scenario, the same BWP is shared even between different TRPs (because BWP should be configured per cell). In short, current TS 38.321 would release all resources for CG/SPS including those for the other TRP which has valid TA. Thus, some rewording in TS 38.321 is needed (e.g., clear any configured downlink assignments and configured uplink grants corresponding to the TRP).

Proposal 4.	RAN2 jointly discuss definition of PTAG/STAG and behavior upon TAT expiry. RAN2 will down-select from Option 1/2 in Table 1.

  - Indication of TAG ID for RACH procedure

As discussed in RAN1 so far, two triggering of RA procedure are supported in two TA multi-DCI multi-TRP scenario; PDCCH ordered CFRA and UE-initiated CBRA. For more detail, UE-initiated RACH includes 2-step RACH and 4-step RACH. Then we would like to summarize agreements so far for indication of TAG ID to update through the RA procedure for each type of RA procedure.

Table 2. How UE knows which TAG to apply a new TA obtained in RACH procedure?

|  | Intra-cell multi-TRP | Inter-cell multi-TRP |
| --- | --- | --- |
| UE-initiated 4-step CBA | FFS in RAN1 (*1) | By reading PCI |
| UE-initiated 2-step CBRA | Absolute TAC MAC CE (*2) | By reading PCI |
| PDCCH order 4-step CFRA | FFS in RAN1 (*1) | gNB indication of PRACH config selection (*3) |
| PDCCH order 2-step CFRA (*4) | - | - |

*1: Latest agreement in RAN1 is on alternatives to be down-selected in the upcoming meeting or later.

RAN1#113 [6]

Agreement For intra-cell multi-DCI based Multi-TRP operation with two TA enhancement, down-select one of the following alternatives: - Alt 1: indicate TAG ID as part of TA command in RAR - Alt 3: divide SSBs into two groups, one for each TRP. If a SSB associated to a RACH procedure belongs to the nth group (n=1,2), then the TA obtained via the RACH procedure corresponds to the nth TRP.

*2: Details can be found in Section 2.6.

*3: Details can be found in Section 2.7.

*4: TS 38.321 does not allow 2-step RA for PDCCH ordered RACH [7].

### 5.1.1 Random Access procedure initialization 1> if the Random Access procedure is initiated by PDCCH order and if the ra-PreambleIndex explicitly provided by PDCCH is not 0b000000; or … 2> set the RA_TYPE to 4-stepRA.

Now RAN2 discussed and reached following proposals in the POST e-mail discussion after RAN2#122 [2].

Proposal 11: For intra-cell PDCCH order CFRA, wait for RAN1 progress on which TAG to be applied for PRACH and RAR Proposal 12: For inter-cell PDCCH order CFRA to the additionalPCI, - PDCCH order indicates which additionalPCI’s PRACH configuration to be used (according to RAN1 agreement), - wait for RAN1 progress on which TAG to be applied for PRACH and RAR Proposal 13: For UE initiated CBRA, support SSB partition: for PRACH transmission and TAC in RAR UE applies the TAG that corresponds to the selected SSB.

From our perspective, Proposal 11 and 12 are fine but we have different understanding on Proposal 13, that is, it is still under RAN1 discussion how to select TAG ID to update upon UE-initiated 4-step CBRA. This is why we think RAN2 should wait for RAN1 progress.

Proposal 5.	RAN2 wait for RAN1 discussion on indication of TAG ID to be updated upon UE-initiated 4-step CBRA.

  - Absolute TAC MAC CE

RAN1 agreed to indicate TAG ID via Absolute TAC MAC CE and left details up to RAN2.

RAN1#110bis-e [8]

Agreement For multi-DCI based Multi-TRP operation with two TA enhancement, support enhancements related to indicating TAG ID via absolute TA command: - FFS: whether the indication is implicit or explicit - Detailed indication schemes are FFS - This does not preclude indication of two TAG IDs (if supported) - Note: This applies at least to MSGB in case of C-RNTI

RAN1#112bis-e [9]

Conclusion For multi-DCI based Multi-TRP operation with two TA enhancement, how to indicate the TAG ID via absolute TA command MAC CE is left up to RAN2: - One of two TAG IDs configured in the SpCell can be indicated.

As far as we look back the RAN1#112bis-e discussion, companies raised two solutions:

- Explicit indication: Enhance absolute TAC MAC CE to indicate TAG ID.

- Implicit indication: No enhancement on MAC CE structure. Ensure that absolute TA command always updates TA with TAG ID = 0.

In our view, the explicit indication is preferable as it is more straightforward. If we go with the explicit indication, a simple way is to utilize one reserved bit in Absolute Timing Advance Command MAC CE [7] to indicate which TAG ID is updated, original TAG (associated with a TRP configured of Type-1 CSS) or additional TAG.

Figure 6.1.3.4a-1: Absolute Timing Advance Command MAC CE

Proposal 6.	Utilize one reserved bit in Absolute Timing Advance Command MAC CE to indicate which TAG ID, original TAG (associated with a TRP configured of Type-1 CSS) or additional TAG, is updated in intra-cell two-TA scenario.

  - Additional PRACH configuration

RAN1 agreed to introduce a separate PRACH configuration for additional PCI. RAN1 also agreed to support indication of which PRACH configuration to be used in PDCCH order CFRA.

RAN1#112 [10]

Agreement Confirm the following working assumption: For multi-DCI based inter-cell Multi-TRP operation with two TA enhancement, one additional PRACH configuration is supported for each configured additional PCI - the additional PRACH configuration is used in a RACH procedure triggered by a PDCCH order for the corresponding configured additional PCI

RAN1#112bis-e [9]

Agreement For intercell multi-DCI based Multi-TRP operation with two TA enhancement, support indication of which PRACH configuration to be used in the RACH procedure in the PDCCH order. - FFS: Whether additionalPCI or a generic identifier is indicated in PDCCH order - FFS: The detail of the indication in PDCCH order in terms of whether to support PRACH triggered for inactive additionalPCI.

However, no RRC parameter for additional PRACH configuration is found in RRC parameter list from RAN1 [1]. This is because, as far as we see RAN1 discussion in May (can be found in [11] and [12]), they are still discussing whether it is allowed to transmit PRACH to inactive PCI, in other words, whether additional PRACH configuration can be configured with inactive PCIs. Thus, RAN2 can wait for RAN1 to conclude.

Proposal 7.	RAN2 wait for RAN1 progress before starting to implement additional PRACH configuration.

- Summary and proposal

Proposal 1.	Introduce following fields in TS 38.331 to reflect the RAN1 parameter list. 
- tag-Id2 (in ServingCellConfig)
- tag-Id-ptr (in TCI-State)
- tag-Id-ptr (in TCI-UL-State)
- n-TimingAdvanceOffset2 (in ServingCellConfig)

Proposal 2.	RAN2 wait for RAN1 progress before starting discussion on implementation of UE features.

Proposal 3.	RAN2 do not increase maximum number of TAGs per cell group (currently 4).

Proposal 4.	RAN2 jointly discuss definition of PTAG/STAG and behavior upon TAT expiry. RAN2 will down-select from Option 1/2 in Table 1.

Proposal 5.	RAN2 wait for RAN1 discussion on indication of TAG ID to be updated upon UE-initiated 4-step CBRA.

Proposal 6.	Utilize one reserved bit in Absolute Timing Advance Command MAC CE to indicate which TAG ID, original TAG (associated with a TRP configured of Type-1 CSS) or additional TAG, is updated in intra-cell two-TA scenario.

Proposal 7.	RAN2 wait for RAN1 progress before starting to implement additional PRACH configuration.

- References

[1]	R1-2306270, “LS on Rel-18 higher-layers parameter list,” RAN1#113.

[2]	“Report of [Post122][852][MIMOevo] 2TAs for multi-DCI multi-TRP,” Samsung, RAN2#123.

[3]	R1-2306225, “LS on Rel-18 RAN1 UE features list for NR after RAN1#113,” RAN1#113.

[4]	R2-2304342, “LS on 2TA for multi-DCI multi-TRP,” RAN2#121bis-e.

[5]	R1-2306249, “Reply on LS 2TA for multi-DCI multi-TRP,” RAN1#113.

[6]	“Draft Report of 3GPP TSG RAN WG1 #113 v0.1.0,” RAN1.

[7]	TS 38.321, V17.5.0.

[8]	“Final Report of 3GPP TSG RAN WG1 #110bis-e v1.0.0,” RAN1.

[9]	“Final Report of 3GPP TSG RAN WG1 #112bis-e v1.0.0,” RAN1.

[10]	“Final Report of 3GPP TSG RAN WG1 #112 v1.0.0,” RAN1.

[11]	R1-2306053, “Moderator Summary #1 on Two TAs for multi-DCI,” Ericsson, RAN1#113.

[12]	R1-2306128, “Moderator Summary #2 on Two TAs for multi-DCI,” Ericsson, RAN1#113.

Annex A: Parameter list from RAN1#113

| WI code | Sub-feature group | RAN2 Parent IE | Parameter name in the spec | New or existing? | Description | Value range | Required for initial access or IDLE/INACTIVE | Comment |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| NR_MIMO_evo_DL_UL-Core | mTRP-2TA | ServingCellConfig | tag-Id2 | New | This parameter provides the second TAG ID configured for the servingcell. | (0..maxNrofTAGs-1) | UE-specific | Exact design is up to RAN2 |
| NR_MIMO_evo_DL_UL-Core | mTRP-2TA | TCI-State | tag-Id-ptr | New | This parameter is used to associate a TAG ID with a joint TCI state. This covers the case when joint TCI state is used for UL transmission. | 0, 1 | UE-specific | Note that this parameter either points to either tag-ID or tag-ID2 configured per serving cell depending on the value it indicates. This is only a suggestion/example and the final RRC signaling detail is up to RAN2 to decide. |
| NR_MIMO_evo_DL_UL-Core | mTRP-2TA | TCI-UL-State | tag-Id-ptr | New | This parameter is used to associate a TAG ID with an UL TCI state. This covers the case when UL TCI state is used for UL transmission. | 0, 1 | UE-specific | Note that this parameter either points to either tag-ID or tag-ID2 configured per serving cell depending on the value it indicates. This is only a suggestion/example and the final RRC signaling detail is up to RAN2 to decide. |
| NR_MIMO_evo_DL_UL-Core | mTRP-2TA | ServingCellConfig | n-TimingAdvanceOffset2-r18 | New | This parameter provides the second Timing Advanced Offset configured per cell. | { n0, n25600, n39936 } | UE-specific | Note that the first n-TimingAdvanceOffset is the legacy parameter given in ServingCellConfigCommon/ServingCellConfigCommonSIB |

Annex B: UE feature list from RAN1#113

| Features | Index | Feature group | Components | Prerequisite feature groups | Need for the gNB to know if the feature is supported | Applicable to the capability signalling exchange between UEs (Sidelink WI only)”. | Consequence if the feature is not supported by the UE | Type (the ‘type’ definition from UE features should be based on the granularity of 1) Per UE or 2) Per Band or 3) Per BC or 4) Per FS or 5) Per FSPC) | Need of FDD/TDD differentiation | Need of FR1/FR2 differentiation | Capability interpretation for mixture of FDD/TDD and/or FR1/FR2 | Note | Mandatory/Optional |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 40. NR_MIMO_evo_DL_UL | 40-2-2 | Basic feature for multi-DCI based inter-cell Multi-TRP operation with two TA enhancement | FFS: split joint TCI states and separate DL/UL TCI states into two rows/FGs FFS: what to do/signal when UE also supports multi-DCI based STxMP PUSCH+PUSCH FFS: what to do/signal about absolute TA command MAC CE FFS: Maximum number of n-TimingAdvanceOffset value per serving cell |  |  |  |  |  |  |  |  |  |  |
| 40. NR_MIMO_evo_DL_UL | 40-2-4 | Support of a PDCCH order sent by one TRP triggers RACH procedure (specifically PRACH) towards a different TRP based on CFRA | FFS: merge with 40-2-2 FFS: inter-/intra-cell |  |  |  |  |  |  |  |  |  |  |
| 40. NR_MIMO_evo_DL_UL | 40-2-5 | TCI states of one CORESET Pool associated to both TAGs | This is a WA FFS: split joint TCI states and separate DL/UL TCI states into two rows/FGs |  |  |  |  |  |  |  |  |  |  |
| 40. NR_MIMO_evo_DL_UL | 40-2-6 | Rx timing difference larger than CP length |  |  |  |  |  |  |  |  |  |  |  |
| 40. NR_MIMO_evo_DL_UL | 40-2-8 | The maximum number of TAGs all CCs |  |  |  |  |  |  |  |  |  |  |  |