3GPP TSG RAN WG2 Meeting #123	R2-2308883

Toulouse, France, August 21-25, 2023

Agenda Item:	7.5.4.1 (BSR Enhancements for XR)

Source: 	Samsung

Title: 	Discussion on BSR enhancements for XR

Document for:	Discussion & decision

Introduction

The agreements on BSR enhancements that have been made in the last RAN2 meeting are as follows. In this paper, we propose to discuss on 1) how to use new BS table, and 2) how to report delay information, based on the previous agreements.

UE calculates the remaining time based on the PDCP discard timer value. FFS if UE reports one or multiple values. FFS how this is modelled in PDCP specification. FFS which UEs support this.

When/if UE reports remaining time, the reference time for the remaining time is determined from the point of the first transmission of the information. FFS if intra-UE prioritization can impact this.

Support one static BSR table with 8 bits BS field for Rel-18 XR (for all cases).

New BS Table

For the applications that do not entail large amount of uplink traffic, the legacy BSR with legacy BS table can work without much quantization issue. gNB may configure the LCH(s)/LCG(s) facing the quantization issue to be able to use the new BS table. When reporting BSR, for LCG(s) with more than one candidate BS table, UE needs to select one, and indicates the gNB of the one that had been selected. Otherwise, gNB would have no idea about how to interpret the buffer size field of the LCG(s).

Observation 1: UE may have more than one candidate BS table for a certain LCG when reporting BSR.

Correspondingly, if new BS table is involved, we cannot reuse the legacy BSR format. We should design a new BSR format to include the BS table indicator per LCG.

Proposal 1: RAN2 is kindly asked to consider including BS table indicator per LCG to indicate which BS table is used, when designing new BSR.

For a certain LCG able to use more than one BS tables, it is reasonable to use the BS table that provides the finest granularity. What UE reports in terms of buffer size is the index defined in the BS table, which can indicate a buffer size interval/range, with upper and lower bounds. Correspondingly, there could be three options to consider, in defining/evaluating the buffer size granularity, i.e.,

- defined as the difference between the upper and lower bounds of the reported buffer size interval,

- defined as the difference between the upper bound of the reported buffer size interval and the actual buffer size,

- defined as the difference between the lower bound of the reported buffer size interval and the actual buffer size.

If the definition of the granularity is agreed and confirmed, UE can adopt the definition in selecting the BS table, which, in turn, can be utilized by gNB in estimating the actual buffer size.

Proposal 2: RAN2 is kindly asked to discuss how UE determines which BS table to use for a certain LCG, if it has more than one candidate BS table.

Delay Reporting

The motivation behind the delay reporting is to assist timely UL scheduling for urgent XR traffic. The relevant candidate solutions that have been proposed so far are mainly pertaining to either 1) reporting the urgent buffer size, or 2) reporting remaining time with the associated buffer size.

Observation 2: In achieving the timely UL scheduling, two options can be considered in terms of which information UE should report

- Option 1) the urgent buffer size,

- Option 2) the remaining time with associated buffer size.

Regarding Option 1, the urgent buffer size could indicate the amount of the urgent data buffered in UE’s UL buffer, where the criterion of the urgency is configurable for gNB. For example, gNB can configure a remaining time upper bound, whereby UE reports the amount of the buffered data, whose remaining time is less than the remaining time upper bound. gNB can determine the value of the remaining time upper bound, by taking into account, e.g., minimum UL scheduling delay, the available UL resource, traffic load, etc., based on the gNB’s implementation.

Also, gNB can configure remaining time lower bound. If configured, UE reports the amount of buffered data with remaining time longer than the lower bound, for the purpose of, filtering out the data whose remaining time is too short to be served, considering the minimum UL scheduling delay.

Proposal 3: If Option 1 is agreed, RAN2 is kindly asked to consider that UE reports the amount of the urgent data, whose remaining time satisfies certain condition configured by gNB, e.g., less than a certain threshold.

Considering legacy BSR format, it is natural to report the urgent buffer size per LCG. We can also consider per LCH reporting, since it can provide finer information on urgent data in terms of each LCH, facilitating finer UL scheduling, especially, when UL resource is inadequate. Also, if we want to reduce the signalling overhead caused by urgent buffer size reporting, per MAC entity reporting, i.e., one urgent buffer size field for total amount of urgent data among all LCGs/LCHs configured to report, can also be considered. It seems necessary to discuss, at which level, the amount of urgent data should be reported, considering the signalling overhead and potential gain.

Proposal 4: If Option 1 is agreed, RAN2 is kindly asked to discuss about, at which level, e.g., per LCH/LCG/MAC Entity, the amount of the urgent data should be reported, considering signalling overhead and potential gain.

If Option 2 is agreed, i.e., remaining time is reported, we need to firstly evaluate the tradeoff between the benefit brought by reporting remaining time, and the corresponding signalling overhead. Note that even for a single LCH/LCG, there could be various data in the UE’s buffer, with different remaining time values, resulting from, e.g., different arrival times and/or different delay budgets. If no restrictions are imposed on, the case with overwhelmingly many remaining time values/fields to report will definitely happen, which should be avoided. Hence, we need to take into account how to limit the signalling overhead caused by reporting remaining time when designing the reporting mechanism.

Observation 3: gNB may need a way to limit the signalling overhead caused by reporting remaining time, since there could be overwhelmingly many remaining time values associated with the data in the UE buffer.

For the data with sufficiently long remaining time, gNB may have much more time budget to serve it, compared to the data with short remaining time. That is, gNB has much more reasons to firstly serve data with short remaining time, and may have less/no interest in the exact value of the remaining time which is sufficiently long, when conducting UL scheduling, especially in the case that UL resource is inadequate. Therefore, there may be lack of reasons to inform the gNB of the sufficiently long remaining time.

Observation 4: If remaining time is sufficiently long, gNB may have no interest in such information.

To avoid overwhelmingly much signalling overhead, and filter out the information out of gNB’s interest, gNB should be able to configure the range of remaining time UE can report, e.g., less than a certain threshold, even in the case that only shortest remaining time is reported per LCH/LCG/MAC Entity. It is reasonable, in the sense that, if the (shortest) remaining time is sufficiently long, UE can wait until the corresponding data being served without reporting its remaining time (since gNB is aware of the existence of the data via buffer size reporting), or delay the reporting until the remaining time becomes short enough.

Proposal 5: If Option 2 is agreed, RAN2 is kindly asked to consider that UE reports the (shortest) remaining time value(s) satisfying certain condition configured by gNB, e.g., less than a certain threshold.

In the case of Option 2, similarly, we need a means of compromise between signalling overhead and potential gain.

Considering the way buffer size is reported via legacy BSR, we can follow the same granularity, i.e., on LCG basis, in reporting remaining time. We can also consider per LCH reporting, since it can provide finer information on the remaining time, facilitating finer UL scheduling, especially, when UL resource is inadequate. Also, if we want to reduce the signalling overhead, per MAC entity reporting, i.e., one or multiple remaining time fields reflecting remaining time of all the buffered data among all LCGs/LCHs configured to report, can also be considered. It seems necessary to discuss, at which level, the remaining time should be reported, considering the signalling overhead and potential gain.

Proposal 6: If Option 2 is agreed, RAN2 is kindly asked to discuss about, at which level, e.g., per LCH/LCG/MAC Entity, the remaining time should be reported, considering signalling overhead and potential gain.

In addressing the timely UL scheduling issue among Option 1 and Option 2, a solution relying only on the buffer size information (Option 1) should be given a higher priority than that involving remaining time information (Option 2), since the new BSR design becomes much simpler when only the buffer size information is of interest.

Observation 5: New BSR design may become much simpler with option 1 than option 2.

In Option 1, UE may report [Urgent Buffer Size 1], [Urgent Buffer Size 2], ... , [Urgent Buffer Size M], where M indicates the number of urgent buffer size fields included in the MAC CE.

In Option 2, since with only remaining time information, gNB may have no idea about how much UL resource should be allocated to serve the data with reported remaining time, the associated buffer size information per remaining time value should be accompanied by, i.e., UE may report [Remaining Time 1, Buffer Size 1], [Remaining Time 2, Buffer Size 2], ..., [Remaining Time N, Buffer Size N], where N indicates the number of remaining time fields included in the MAC CE.

For a fair comparison, if M and N are equal, Option 1 entails much less signalling overhead than Option 2, i.e., [Urgent Buffer Size] incurs less signalling overhead than [Remaining Time, Buffer Size].

Normally, what gNB should determine in UL scheduling is, how much UL resource should be allocated to which UE among the available UL resource. Although, with the Option 1, the detailed information on the remaining time provided by Option 2, cannot be informed of, gNB is still able to judge the amount of UL resource required to serve the urgent data reported by a certain UE, and the prioritization between different UEs can be achieved based on the respective remaining time thresholds (implicitly reflecting the remaining time information) configured by gNB.

Observation 6: Signalling overhead may be much less with Option 1 than Option 2, without sacrificing considerable performance loss.

Thus, Option 1 seems more applicable for timely UL scheduling than Option 2, from the perspectives of 1) complexity of new BSR format, 2) signalling overhead, and 3) functional effects for UL scheduling.

Proposal 7: RAN2 is kindly asked to give higher priority to Option 1 than Option 2 in achieving the timely UL scheduling.

Conclusion

Based on the discussion above, we have the following observations and proposals:

Observation 1: UE may have more than one candidate BS table for a certain LCG when reporting BSR.

Proposal 1: RAN2 is kindly asked to consider including BS table indicator per LCG to indicate which BS table is used, when designing new BSR.

Proposal 2: RAN2 is kindly asked to discuss how UE determines which BS table to use for a certain LCG, if it has more than one candidate BS table.

Observation 2: In achieving the timely UL scheduling, two options can be considered in terms of which information UE should report

- Option 1) the urgent buffer size,

- Option 2) the remaining time with associated buffer size.

Proposal 3: If Option 1 is agreed, RAN2 is kindly asked to consider that UE reports the amount of the urgent data, whose remaining time satisfies certain condition configured by gNB, e.g., less than a certain threshold.

Proposal 4: If Option 1 is agreed, RAN2 is kindly asked to discuss about, at which level, e.g., per LCH/LCG/MAC Entity, the amount of the urgent data should be reported, considering signalling overhead and potential gain.

Observation 3: gNB may need a way to limit the signalling overhead caused by reporting remaining time, since there could be overwhelmingly many remaining time values associated with the data in the UE buffer.

Observation 4: If remaining time is sufficiently long, gNB may have no interest in such information.

Proposal 5: If Option 2 is agreed, RAN2 is kindly asked to consider that UE reports the (shortest) remaining time value(s) satisfying certain condition configured by gNB, e.g., less than a certain threshold.

Proposal 6: If Option 2 is agreed, RAN2 is kindly asked to discuss about, at which level, e.g., per LCH/LCG/MAC Entity, the remaining time should be reported, considering signalling overhead and potential gain.

Observation 5: New BSR design may become much simpler with option 1 than option 2.

Observation 6: Signalling overhead may be much less with Option 1 than Option 2, without sacrificing considerable performance loss.

Proposal 7: RAN2 is kindly asked to give higher priority to Option 1 than Option 2 in achieving the timely UL scheduling.