# 3GPP TSG-RAN WG2 Meeting #123	R2-2308907

# Toulouse 21th -25th Aug, 2023

Title:	(Draft) Reply LS on update for “interBandMRDC-WithOverlapDL-Bands-r16” in 38.306

Release:	Rel-16

Work Item:	     TEI16

Source:	RAN2

To:	RAN4

Cc:

Contact Person:

#### Name:	Wenting Li

Tel. Number:

E-mail Address:	li.wenting@zte.com.cn

Attachments:

- Overall Description:

RAN2 thanks RAN4 for the LS on update for “interBandMRDC-WithOverlapDL-Bands-r16” in 38.306.

RAN2 discussed this issue and would like to confirm 2 issues below for the case with FDD-FDD inter-band EN-

DC with overlapping frequency when the UE is not capable of interBandMRDC-WithOverlapDL-Bands-r16.

Issue 1: MRTD

In the Ran4 Feature list 2-19, it says that if the UE is not capable of interBandMRDC-WithOverlapDL-Bands-r16, MRTD<3us (i.e. Table 7.6.3-1 of 38.133 for synchronous operation) and intra-band MR-DC requirements apply.

| 2-19 | FDD-FDD or TDD-TDD inter-band MR-DC with overlapping or partially overlapping DL spectrum | Type 1 UE: supports FDD-FDD or TDD-TDD inter-band operation with overlapping or partially DL bands with MRTD<3us and intra-band MR-DC requirements apply. Type 2 UE: supports FDD-FDD or TDD-TDD inter-band operation with overlapping or partially overlapping DL bands with an MR-DC MRTD according to clause 7.6.2 in 38.133 and applicable inter-band RF requirements. If absent the UE is a type 1 UE. |
| --- | --- | --- |

However in the 38.133 7.6.3, it said that for the asynchronous operation, the Table 7.6.2-1 shall be adopted.

38.133 7.6.3 Minimum Requirements for intra-band EN-DC The UE shall be capable of handling at least a relative receive timing difference between subframe timing of signal from a E-UTRA cell belonging to the MCG and slot timing of signal from a cell belonging to the SCG as shown in Table 7.6.2-1 for E-UTRA FDD-NR FDD intra-band EN-DC provided the UE indicates that it is capable of asynchronous EN-DC operation [2].

Thus RAN2 would like to confirm whether the MRTD as in Table 7.6.2-1 of 38.133 (instead of the Table 7.6.3-1) shall be applied for asynchronous operation with FDD-FDD inter-band EN-DC with overlapping frequency when the UE is not capable of interBandMRDC-WithOverlapDL-Bands-r16.

Issue 2: MTTD

For the MTTD, from both the LS and the feature list table above, it doesn’t mention which requirement shall be supported for the FDD-FDD inter-band EN-DC with overlapping frequency when the UE is not capable of interBandMRDC-WithOverlapDL-Bands-r16.

Thus RAN2 would like to confirm the MTTD requirement for the FDD-FDD inter-band EN-DC with overlapping frequency for the asynchronous and synchronous operation respectively when the UE is not capable of interBandMRDC-WithOverlapDL-Bands-r16.

- 2. Actions:

To RAN4 group:

ACTION: RAN2 respectfully asks RAN4 to confirm the above 2 issues for the MRTD and MTTD respectively.

.

# 3. Date of Next TSG-RAN2 Meetings:

3GPPRAN2#123bis	  9 -13 Oct 2023     	 Xiamen, CN

3GPPRAN2#124         13 - 17 Nov 2023             Chicago, US