3GPP TSG-RAN WG2 #123	R2-2308955

Toulouse, France Aug 21st – 25th, 2023

Agenda Item:	7.24.2

Source:	Ericsson, AT&T, Polaris Wireless, u-blox, T-Mobile

Title:	Adding support for Bluetooth AoA/AoD

Document for:	Discussion, Decision

# 1	Introduction

During Rel-18 WID preparations, it was discussed whether to support the recently (Jan 2019) agreed AoA/AoD feature in Bluetooth 5.1 as part of the Bluetooth positioning in LPP. The recommendation was to consider this as TEI18 instead.

Bluetooth can be seen as a highly relevant positioning method for reduced capability (RedCap) devices but also for IoT devices in general, and therefore, it is natural to extend the existing Bluetooth support in LPP with the Bluetooth AoA/AoD positioning features.

This paper describes the LPP updates needed to introduce support for Bluetooth AoA/AoD.

# 2	Discussion

Bluetooth introduced support for AoA/AoD in 5.1 released January 2019. It is based on a signal extension Continuous Tone Extension (CTE) that can be configured together with periodic advertisements. Two modes are described:

- AoA with a single antenna device transmitter and multiple antenna anchor receivers, where the device is configured to transmit the periodic advertisement signal and the receivers are estimating the incoming AoA. This is similar to the NR UL-AoA positioning method in LPP.

- AoD with a single antenna device receiver and multiple antenna anchor transmitters, where the transmitter will beam sweep periodic advertisement transmissions received by the device to estimate the outgoing AoD from the anchor transmitters. This is similar to NR DL-AoD positioning method in LPP.

Furthermore, the configuration of the receiver and transmitter can be managed via a connection-oriented scenarios based on communication, but also via connectionless scenarios, where each end is configured via a host controller interface.

The connectionless scenarios may be naturally transferred to LPP, where LPP can configure the device for AoA/AoD operation.

The Bluetooth AoA/AoD enhancement for connectionless scenarios are natural to include in LPP Bluetooth positioning as enhancements

The periodic advertisements are associated to two key attributes that needs to be exchanged with the location server:

- Advertisement address, which is a six byte address that can be of different types, either public (MAC address), random-static (random number but static) or random-private-resolvable (random number regularly updated) where the type random-private-non-resolvable is not suitable for positioning. For AoA, the UE needs to provide its address to the location server and for AoD, the UE needs the address of the Bluetooth anchor (already in LPP for BT beacons)

- Advertisement interval can be configured from 20ms to about 10s. For AoA, the UE needs to provide its advertisement interval to the location server or the location server needs to configure the advertisement interval of the UE and for AoD, the UE needs the advertisement interval of the Bluetooth anchor

For AoA/AoD, LPP needs to be extended to exchange BT advertisement address and advertisement interval.

The CTE can be configured with the following parameters

- Length of the CTE in 8us steps, 1-20 steps corresponding to 8us to 160us

- Type AoA, AoD 1 us slot duration and AoD 2us slot duration

- Count - specifies how many packets that include a CTE are to be transmitted each periodic advertising event

- PHY, which can be either 1 Msymbols/s (mandatory) or 2 Msymbols/s (optional)

For AoA/AoD CTE, LPP needs to be extended to CTE configuration information of CTE length, CTE type, CTE count and PHY type.

For AoD, there is also a need to configure the antenna beam sweeping and antenna configuration. For AoD UEB, there is also a need to provide BLE anchor location information.

For AoD LPP needs to be extended with BLE anchor antenna configuration and for UEB also BLE anchor location information.

AoA can support UE-assisted positioning, while AoD can support both UE-assisted and UE-based positioning. Therefore

- AoA needs to be configured via the LPP Location Information procedure

- AoD needs to be configured via LPP Assistance Data procedure and for UEA also via LPP Location Information procedure

For AoA/AoD support, both LPP Location Information and Assistance Data procedures need to be updated.

Based on the discussion, we have the following proposals

Discuss and agree to introduce support for Bluetooth AoA/AoD positioning in the LPP Bluetooth positioning method

Endorse the draft CR in the Appendix

# Conclusion

Based on the discussion in the previous sections we make the following observations:

Observation 1	The Bluetooth AoA/AoD enhancement for connectionless scenarios are natural to include in LPP Bluetooth positioning as enhancements

Observation 2	For AoA/AoD, LPP needs to be extended to exchange BT advertisement address and advertisement interval

Observation 3	For AoA/AoD CTE, LPP needs to be extended to CTE configuration information of CTE length, CTE type, CTE count and PHY type

Observation 4	For AoD LPP needs to be extended with BLE anchor antenna configuration and for UEB also BLE anchor location information

Observation 5	For AoA/AoD support, both LPP Location Information and Assistance Data procedures need to be updated

and propose the following:

Proposal 1	Discuss and agree to introduce support for Bluetooth AoA/AoD positioning in the LPP Bluetooth positioning method

Proposal 2	Endorse the draft CR in the Appendix

# References

Bluetooth Core Specification 5.1

Bluetooth Direction Finding: A Technical Overview, Bluetooth SIG v1.0.3 Oct 2021. Available at https://www.bluetooth.com/bluetooth-resources/bluetooth-direction-finding/

# Appendix A Text Proposal for 37.355 V17.5.0

# 2	References

[…]

[xx]	Bluetooth Special Interest Group: "Bluetooth Core Specification v5.4", February 2023.

[…]

#### 6.5.7.1	Bluetooth Location Information

#### –	BT-ProvideLocationInformation

The IE BT-ProvideLocationInformation is used by the target device to provide measurements for one or more Bluetooth beacons to the location server. It may also be used to provide Bluetooth positioning specific error reason.

-- ASN1START

BT-ProvideLocationInformation-r13 ::= SEQUENCE {

bt-MeasurementInformation-r13		BT-MeasurementInformation-r13	OPTIONAL,

bt-Error-r13						BT-Error-r13					OPTIONAL,

... ,

[[

bt-AoA-Config-r18			BT-AoA-Config-r18	OPTIONAL,

]]

}

BT-AoA-Config-r18 ::= SEQUENCE {

btAddr-r13				BIT STRING (SIZE (48)),

advInterval-r18			INTEGER (32..16777),

cteLength-r18			INTEGER (1..20)				OPTIONAL,

cteCount-r18			INTEGER (1..16)				OPTIONAL,

tx-PHY-r18				ENUMERATED {m1, m2}			OPTIONAL,

...

}

-- ASN1STOP

BT- ProvideLocationInformation field descriptions

btAddr This field specifies the Bluetooth address of the Bluetooth anchor [xx]. In case the device updates its address during an established LPP session, the device shall provide the new address as unsolicited location information to the server

advInterval This field specifies the Bluetooth advertisement periodicity with a scaling factor of 0.625 ms [xx]. This field represents the advertisement interval that the device will use. If the server has included a requested advertisement interval in RequestLocationInformation, then the device will use the nearest supported advertisement interval.

cteLength This field specifies the configured CTE length to be used by the device in number of 8us segments.

cteCount This field specifies the number of Bluetooth packets that include a CTE that the device will transmit each periodic advertising. If the server has included a requested CTE count in RequestLocationInformation, then the device will use the nearest supported CTE count.

tx-PHY-r18 This field specifies the Bluetooth TX PHY rate to be used, where m1 and m2 corresponds to 1 and 2 Megasymbols/s respectively

#### 6.5.7.2	Bluetooth Location Information Elements

#### –	BT-MeasurementInformation

-- ASN1START

BT-MeasurementInformation-r13 ::= SEQUENCE {

measurementReferenceTime-r13		UTCTime						OPTIONAL,

bt-MeasurementList-r13				BT-MeasurementList-r13		OPTIONAL,

...,

bt-MeasurementList-r18				BT-MeasurementList-r18		OPTIONAL,

}

BT-MeasurementList-r13 ::= SEQUENCE (SIZE(1..maxBT-Beacon-r13)) OF BT-MeasurementElement-r13

BT-MeasurementElement-r13 ::= SEQUENCE {

btAddr-r13						BIT STRING (SIZE (48)),

rssi-r13						INTEGER (-128..127)				OPTIONAL,

...

}

BT-MeasurementList-r18 ::= SEQUENCE (SIZE(1..maxBT-Beacon-r18)) OF BT-MeasurementElement-r18

BT-MeasurementElement-r18 ::= SEQUENCE {

btAddr-r18						BIT STRING (SIZE (48)),

btAoD-r18						FFS,

bt-MeasurementPerAntList-r18	BT-MeasurementPerAntList-r18	OPTIONAL,

rssi-r18						INTEGER (-128..127)				OPTIONAL,

...

}

BT-MeasurementPerAntList-r18 ::= SEQUENCE (SIZE(1..maxBT-BeaconAnt-r18)) OF BT-MeasurementperAntElement-r18

BT-MeasurementPerAntElement-r18 ::= SEQUENCE {

btAntId-r18						FFS,

ant-rssi-r18					INTEGER (-128..127)				OPTIONAL,

...

}

-- ASN1STOP

BT-MeasurementInformation field descriptions

measurementReferenceTime This field provides the UTC time when the Bluetooth measurements are performed and should take the form of YYMMDDhhmmssZ.

bt-MeasurementList This field provides the Bluetooth measurements for up to 32 Bluetooth beacons.

btAddr This field specifies the Bluetooth public address of the Bluetooth beacon [25].

rssi This field provides the beacon received signal strength indicator (RSSI) in dBm.

#### 6.5.7.3	Bluetooth Location Information Request

#### –	BT-RequestLocationInformation

The IE BT-RequestLocationInformation is used by the location server to request Bluetooth measurements from a target device.

-- ASN1START

BT-RequestLocationInformation-r13 ::= SEQUENCE {

requestedMeasurements-r13	BIT STRING {

rssi		(0),

aod-r18		(1),

rsrp-r18	(2)} (SIZE(1..8)),

...,

[[

bt-requestedAoA-Config-r18		BIT STRING {

cteLength   (0),

cteCount    (1)} (SIZE(1..8))	OPTIONAL,

bt-providedAoA-Config-r18		BT-ProvidedAoA-Config-r18	OPTIONAL,

]]

}

BT-ProvidedAoA-Config-r18 ::= SEQUENCE {

cteStatus-r18			ENUMERATED {enabled, disabled}	OPTIONAL,

advInterval-r18			INTEGER (32..16777)				OPTIONAL,

cteLength-r18			INTEGER (1..20)					OPTIONAL,

cteCount-r18			INTEGER (1..16)					OPTIONAL,

tx-PHY-r18				ENUMERATED {m1, m2}				OPTIONAL,

...

}

-- ASN1STOP

BT-RequestLocationInformation field descriptions

requestedMeasurements This field specifies the Bluetooth measurements requested. This is represented by a bit string, with a one-value at the bit position means the particular measurement is requested; a zero-value means not requested. The following measurement requests can be included. rssi: Bluetooth beacon signal strength at the target aod: Estimated AoD per Bluetooth anchor antenna ant-rssi: Estimated received signal power per Bluetooth anchor antenna

bt-requestedAoA-Config This field specifies the Bluetooth AoA configuration parameters requested. This is represented by a bit string, with a one-value at the bit position means the particular measurement is requested; a zero-value means not requested. The following measurement requests can be included. advAddress: The Bluetooth advertisement address of the device cteLength: The Bluetooth CTE length in number of 8us segments cteCount: The number of Bluetooth packets that include a CTE transmitted each periodic advertising event

cteStatus This field specifies the Bluetooth AoA CTE status of the device, and is used by the location server to enable or disable transmission of the BLE CTE by the device for AoA estimation at BLE anchors enabled: The Bluetooth CTE shall be enabled by the device disabled: The Bluetooth CTE shall be disabled by the device

advInterval This field specifies the Bluetooth advertisement periodicity that the device is requested to use, scaling factor 0.625 ms [xx].

cteLength This field specifies the configured CTE length to be used by the device in number of 8us segments.

cteCount This field specifies the number of Bluetooth packets that include a CTE that zthe device is requested to transmit each periodic advertising event

tx-PHY-r18 This field specifies the Bluetooth TX PHY rate to be used, where m1 and m2 corresponds to 1 and 2 Megasymbols/s respectively

#### 6.5.7.4	Bluetooth Capability Information

#### –	BT-ProvideCapabilities

The IE BT-ProvideCapabilites is used by the target device to provide its capabilities for Bluetooth positioning to the location server.

-- ASN1START

BT-ProvideCapabilities-r13 ::= SEQUENCE {

bt-Modes-r13			BIT STRING {	standalone		(0),

ue-assisted		(1),

ue-based		(2)}	(SIZE (1..8)),

bt-MeasSupported-r13	BIT STRING {	rssi-r13		(0)}	(SIZE (1..8)),

...,

[[

idleStateForMeasurements-r14

ENUMERATED {	required	}							OPTIONAL,

periodicalReportingSupported-r14

PositioningModes										OPTIONAL

]],

[[	scheduledLocationRequestSupported-r17	ScheduledLocationTimeSupportPerMode-r17		OPTIONAL

]] ,

[[

bt-AoA-Capability-r18			BT-AoA-Capability-r18	OPTIONAL, -- Cond AoAsup

bt-AoD-Capability-r18			BT-AoD-Capability-r18	OPTIONAL -- Cond AoDsup

]]

}

BT-AoA-Capability-r18 ::= SEQUENCE {

bt-AoA-TXadrType-r18			ENUMERATED {public, random-static, random-private-resolvable},

bt-TX-PHY-2M-r18				ENUMERATED {supported}	OPTIONAL,

...

}

BT_AoD-Capability-r18 ::= SEQUENCE {

bt-AoD-slotDuration-r18			ENUMERATED {us1, us2, both},

bt-RX-PHY-2M-r18				ENUMERATED {supported}	OPTIONAL,

...

}

-- ASN1STOP

| Conditional presence | Explanation |
| --- | --- |
| AoAsup | The field is mandatory present if BT AoA is supported; otherwise it is not present. |
| AoDsup | The field is mandatory present if BT AoD is supported; otherwise it is not present. |

BT-ProvideCapabilities field descriptions

bt-Modes This field specifies the Bluetooth mode(s) supported by the target device. This is represented by a bit string, with a one value at the bit position means the Bluetooth mode is supported; a zero value means not supported.

bt-MeasSupported This field specifies the Bluetooth measurements supported by the target device. This is represented by a bit string, with a one-value at the bit position means the particular measurement is supported; a zero-value means not supported. A zero-value in all bit positions in the bit string means only the basic Bluetooth positioning method is supported by the target device which is reporting of the Bluetooth beacon identity. The following bits are assigned for the indicated measurements. rssi: Bluetooth beacon signal strength at the target device

idleStateForMeasurements This field, if present, indicates that the target device requires idle state to perform BT measurements.

periodicalReportingSupported This field, if present, specifies the positioning modes for which the target device supports periodicalReporting. This is represented by a bit string, with a one value at the bit position means periodicalReporting for the positioning mode is supported; a zero value means not supported. If this field is absent, the location server may assume that the target device does not support periodicalReporting in CommonIEsRequestLocationInformation.

scheduledLocationRequestSupported This field, if present, specifies the positioning modes for which the target device supports scheduled location requests – i.e., supports the IE ScheduledLocationTime in IE CommonIEsRequestLocationInformation – and the time base(s) supported for the scheduled location time for each positioning mode. If this field is absent, the target device does not support scheduled location requests.

bt-AoA-TXadrType This field specifies used TX address type in the advertisements - public – the device Bluetooth MAC address - random-static – a random, static address - random-resolvable – a random address made resolvable for the location server

bt-TX-PHY-2M This field, if present, indicates that the device supports 2M PHY mode with 2 Msymbols/s transmission rate (1 Msymbols/s rate is mandatory)

bt-AoD-slotDuration This field specifies which slot durations that are supported by the device, us1 (1 us), us2 (2 us) or both (both 1 us and 2 us).

bt-RX-PHY-2M This field, if present, indicates that the device supports 2M PHY mode with 2 Msymbols/s reception rate (1 Msymbols/s rate is mandatory)

#### 6.5.7.5	Bluetooth Capability Information Request

#### –	BT-RequestCapabilities

The IE BT-RequestCapabilities is used by the location server to request Bluetooth positioning capabilities from a target device.

-- ASN1START

BT-RequestCapabilities-r13 ::= SEQUENCE {

...

}

-- ASN1STOP

[…]

#### –	RequestAssistanceData

The RequestAssistanceData message body in a LPP message is used by the target device to request assistance data from the location server.

-- ASN1START

RequestAssistanceData ::= SEQUENCE {

criticalExtensions		CHOICE {

c1						CHOICE {

requestAssistanceData-r9	RequestAssistanceData-r9-IEs,

spare3 NULL, spare2 NULL, spare1 NULL

},

criticalExtensionsFuture	SEQUENCE {}

}

}

RequestAssistanceData-r9-IEs ::= SEQUENCE {

commonIEsRequestAssistanceData		CommonIEsRequestAssistanceData		OPTIONAL,

a-gnss-RequestAssistanceData		A-GNSS-RequestAssistanceData		OPTIONAL,

otdoa-RequestAssistanceData			OTDOA-RequestAssistanceData			OPTIONAL,

epdu-RequestAssistanceData			EPDU-Sequence						OPTIONAL,

...,

[[	sensor-RequestAssistanceData-r14

Sensor-RequestAssistanceData-r14	OPTIONAL,

tbs-RequestAssistanceData-r14	TBS-RequestAssistanceData-r14		OPTIONAL,

wlan-RequestAssistanceData-r14	WLAN-RequestAssistanceData-r14		OPTIONAL

]],

[[	nr-Multi-RTT-RequestAssistanceData-r16	NR-Multi-RTT-RequestAssistanceData-r16	OPTIONAL,

nr-DL-AoD-RequestAssistanceData-r16		NR-DL-AoD-RequestAssistanceData-r16		OPTIONAL,

nr-DL-TDOA-RequestAssistanceData-r16	NR-DL-TDOA-RequestAssistanceData-r16	OPTIONAL

]],

[[

bt-RequestAssistanceData-r18	BT-RequestAssistanceData-r18		OPTIONAL

]]

}

-- ASN1STOP

#### –	ProvideAssistanceData

The ProvideAssistanceData message body in a LPP message is used by the location server to provide assistance data to the target device either in response to a request from the target device or in an unsolicited manner.

-- ASN1START

ProvideAssistanceData ::= SEQUENCE {

criticalExtensions		CHOICE {

c1						CHOICE {

provideAssistanceData-r9	ProvideAssistanceData-r9-IEs,

spare3 NULL, spare2 NULL, spare1 NULL

},

criticalExtensionsFuture	SEQUENCE {}

}

}

ProvideAssistanceData-r9-IEs ::= SEQUENCE {

commonIEsProvideAssistanceData		CommonIEsProvideAssistanceData		OPTIONAL,	-- Need ON

a-gnss-ProvideAssistanceData		A-GNSS-ProvideAssistanceData		OPTIONAL,	-- Need ON

otdoa-ProvideAssistanceData			OTDOA-ProvideAssistanceData			OPTIONAL,	-- Need ON

epdu-Provide-Assistance-Data		EPDU-Sequence						OPTIONAL,	-- Need ON

...,

[[

sensor-ProvideAssistanceData-r14	Sensor-ProvideAssistanceData-r14	OPTIONAL,	-- Need ON

tbs-ProvideAssistanceData-r14		TBS-ProvideAssistanceData-r14		OPTIONAL,	-- Need ON

wlan-ProvideAssistanceData-r14		WLAN-ProvideAssistanceData-r14		OPTIONAL	-- Need ON

]],

[[	nr-Multi-RTT-ProvideAssistanceData-r16

NR-Multi-RTT-ProvideAssistanceData-r16

OPTIONAL,	-- Need ON

nr-DL-AoD-ProvideAssistanceData-r16

NR-DL-AoD-ProvideAssistanceData-r16	OPTIONAL,	-- Need ON

nr-DL-TDOA-ProvideAssistanceData-r16

NR-DL-TDOA-ProvideAssistanceData-r16

OPTIONAL	-- Need ON

]],

[[

bt-ProvideAssistanceData-r18		BT-ProvideAssistanceData-r18	OPTIONAL	-- Need ON

]]

}

-- ASN1STOP

[…]

#### 6.5.7.7	Bluetooth Assistance Data

#### –	BT-ProvideAssistanceData

The IE BT-ProvideAssistanceData is used by the location server to provide assistance data to enable UE-based BT positioning. It may also be used to provide BT positioning specific error reason.

-- ASN1START

BT-ProvideAssistanceData-r18 ::= SEQUENCE {

bt-DataSet-r14	SEQUENCE (SIZE (1..maxBT-DataSets-r18)) OF BT-DataSet-r18

OPTIONAL,	-- Need ON

bt-Error-r18		BT-Error-r18										OPTIONAL,	-- Need ON

...

}

-- ASN1STOP

WLAN-ProvideAssistanceData field descriptions

bt-DataSet This field provides data for sets of Bluetooth anchors.

bt-Error This field provides error information and may be included when a Provide Assistance Data is sent in response to a Request Assistance Data.

#### 6.5.7.8	Bluetooth Assistance Data Elements

#### –	BT-DataSet

The IE BT-DataSet is used by the location server to provide Bluetooth anchor information for one set of Bluetooth anchors.

-- ASN1START

BT-DataSet-r18 ::= SEQUENCE {

bt-anchor-List-r18				SEQUENCE (SIZE (1..maxBTanchor-r18)) OF BT-Anchor-Data-r18,

...

}

BT-Anchor-Data-r18 ::= SEQUENCE {

btAddr-r13				BIT STRING (SIZE (48)),

bt-AoD-Data-r18			BT-AoD-Data-r18				OPTIONAL,

bt-Anchor-Location-r18	BT-Anchor-Location-r18		OPTIONAL,	-- Need ON

...

}

BT-AoD-Data-r18 ::= SEQUENCE {

advInterval				INTEGER (0..16745),

cteLength-r18			INTEGER (1..20)			OPTIONAL,

cteCount-r18			INTEGER (1..16)			OPTIONAL,

tx-PHY-r18				ENUMERATED {m1, m2}		OPTIONAL,

bt-AntennaConfig-r18	BT-AntennaConfig-r18,

...}

BT-AntennaConfig-r18 ::= SEQUENCE {

FFS

...

}

BT-AP-Location-r18 ::= SEQUENCE {

locationDataLCI-r18					LocationDataLCI-r18,

...

}

LocationDataLCI-r18 ::= SEQUENCE {

latitudeUncertainty-r18				BIT STRING (SIZE (6)),

latitude-r18						BIT STRING (SIZE (34)),

longitudeUncertainty-r18			BIT STRING (SIZE (6)),

longitude-r18						BIT STRING (SIZE (34)),

altitudeUncertainty-r18				BIT STRING (SIZE (6))		OPTIONAL,	-- Need ON

altitude-r18						BIT STRING (SIZE (30))		OPTIONAL,	-- Need ON

datum-r18							BIT STRING (SIZE (8)),

...

}

-- ASN1STOP

BT-DataSet field descriptions

bt-Anchor-List This field provides information for Bluetooth anchors in the data set.

bt-Anchor-List This field provides information for Bluetooth anchors in the data set.

cteLength This field specifies the configured CTE length to be used by the anchor in number of 8us segments.

cteCount This field specifies the number of Bluetooth packets that include a CTE that are transmitted each periodic advertising event

tx-PHY-r18 This field specifies the Bluetooth TX PHY rate used, where m1 and m2 corresponds to 1 and 2 Megasymbols/s respectively

bt-AP-Location - locationDataLCI This field provides the location of the Bluetooth anchor in the form of Location Configuration Information (LCI) defined in [27] and includes the following subfields: latitudeUncertainty: 6-bits quantifying the amount of uncertainty in latitude. A value of 0 is reserved to indicate that the uncertainty is unknown; values greater than 34 are reserved. Its relation with the corresponding value in degrees is expressed with the following formula: latitudeUncertainty = 8 - ceil(log2(uncertainty in degrees)) latitude: A 34-bits fixed point value consisting of 9-bits of integer and 25-bits of fraction indicating the Latitude (+/- 90 degrees) of the AP. longitudeUncertainty: 6-bits quantifying the amount of uncertainty in longitude. A value of 0 is reserved to indicate that the uncertainty is unknown; values greater than 34 are reserved. Its relation with the corresponding value in degrees is expressed with the following formula: longitudeUncertainty = 8 - ceil(log2(uncertainty in degrees)) longitude: A 34-bits fixed point value consisting of 9-bits of integer and 25-bits of fraction indicating the Longitude (+/- 180 degrees) of the AP. altitudeUncertainty: 6-bits value quantifying the amount of uncertainty in the altitude value. A value of 0 is reserved to indicate that the uncertainty is unknown; values greater than 30 are reserved. Its relation with the corresponding value in metres is expressed with the following formula: altitudeUncertainty = 21 - ceil(log2( uncertainty in metres)) altitude: A 30-bit fixed point value consisting of 22-bits of integer and 8-bits of fraction indicating the altitude of the AP in metres. datum: 8-bits indicating the map datum used for the coordinates. Defined codes are: Bit 1: World Geodetic System 1984 (WGS-84) Bit 2: North American Datum 1983 (NAD-83) with North American Vertical Datum 1988 (NAVD-88) Bit 3: North American Datum 1983 (NAD-83) with Mean Lower Low Water (MLLW) vertical datum. Bits 4 – 8 are reserved.

#### 6.5.7.9	Bluetooth Assistance Data Request

#### –	BT-RequestAssistanceData

The IE BT-RequestAssistanceData is used by the target device to request BT assistance data from a location server.

-- ASN1START

BT-RequestAssistanceData-r18 ::= SEQUENCE {

requestedAD-r18			BIT STRING {	aod-config		(0),

anchor-location			(1)}	(SIZE (1..8)),

...

}

-- ASN1STOP

BT-RequestAssistanceData field descriptions

requestedAD This field specifies the Bluetooth assistance data requested. This is represented by a bit string, with a one-value at the bit position means the particular assistance data is requested; a zero-value means not requested. The following assistance data types are included: aod-config: Bluetooth anchor AoD configuration anchor-location: Bluetooth anchor location information