3GPP TSG RAN WG2 Meeting #123	R2-2309005

Toulouse, France, August 21-25, 2023

Title:	[draft] Reply LS on on XR capacity enhancements

Response to:	R2-2307014 (R1-2306233) LS on XR capacity enhancements

Release:	Rel-18

Work Item:	NR_XR_enh-Core

Source:	MediaTek Inc.

To:	RAN1

Cc:

Contact person:	Pradeep Jose

pradeep dot jose at mediatek dot com

# 1	Overall description

RAN2 would like to thank RAN1 for agreeing on RAN2 specification impact of HARQ process ID determination for multi-PUSCH configured grant. RAN2 would like to provide the following feedback.

1. RAN2 has identified an error in the RAN1 agreement as below:

The HARQ process ID of the remaining configured and valid CG PUSCHs in the period is determined by:

HARQ Process ID = [X*floor((CURRENT_symbol ) / periodicity) + (K-1)] modulo nrofHARQ-Processes + harq-ProcID-Offset2

RAN2 will capture a corrected version of the HARQ process ID formula in TS38.321.

2. RAN2 has the following questions on the note below:

Note: A configured CG PUSCH is invalid if the CG PUSCH is dropped due to collision with DL symbol(s) indicated by tdd-UL-DL-ConfigurationCommon or tdd-UL-DL-ConfigurationDedicated or SSB.

RAN2 would like to know if the definition of a valid CG PUSCH will be captured in RAN1 specifications?

If yes, TS38.321 will refer to RAN1 specifications for the definition of a valid CG PUSCH.

If not, could RAN1 please clarify if a CG PUSCH is only deemed invalid if it is dropped due to overlap with DL only indicated by tdd-UL-DL-ConfigurationCommon or tdd-UL-DL-ConfigurationDedicated; or do all cases of CG PUSCH dropping need to be considered (e.g. overlap with DL indicated by SFI DCI, measurement gaps etc.)? Regardless of where invalid/valid CG opportunities are captured, MAC layer needs to be aware of unusable CG opportunities to not obtain MAC PDUs for them, as that data will be lost.

# 2	Actions

To RAN1

RAN2 respectfully asks RAN1 to provide their response to the following questions:

1. Will RAN1 define valid and invalid CG PUSCH occasions in RAN1 specifications?

2. If not, do all cases of CG PUSCH dropping need to be considered for the definition of a valid CG PUSCH occasion?

# 3	Dates of next TSG RAN WG2 meetings

TSG RAN WG2 #123-bis		October 9th – 13th, 2023		Xiamen, China

TSG RAN WG2 #124		November 13th – 17th, 2023	Chicago, U.S.A.