3GPP TSG RAN WG2#123	R2-2309212

Toulouse, France, 21st - 25th August 2023

Title:	[Draft] Reply LS on update for “interBandMRDC-WithOverlapDL-Bands-r16” in 38.306

Response to:	R2-2307043/R4-2310170

Release:	Rel-16

Work Item:	TEI16

Source:	[Apple, To be RAN2]

To:	RAN4

Cc:	-

Contact person:

Name: Yuqin Chen

E-mail Address: yuqin_chen@apple.com

Send any reply LS to:	3GPP Liaisons Coordinator, mailto:3GPPLiaison@etsi.org

Attachments:	None

# 1	Overall description

RAN2 thanks RAN4 for the LS on interBandMRDC-WithOverlapDL-Bands-r16. RAN2 discussed about it and spotted some unclear points on the MTTD/MRTD requirements.

1) The LS indicates the MTTD/MRTD requirements applicability for EN-DC scenario but not for NE-DC. However, in TS38.306, both EN-DC and NE-DC are included (highlighted in yellow) in the field description of interBandMRDC-WithOverlapDL-Bands-r16.

2) If UE does not support interBandMRDC-WithOverlapDL-Bands-r16, for FDD-FDD inter-band EN-DC with overlapping frequency, TS38.306 says for MRTD UE applies intra-band requirements (MRTD<3us, Clause 7.6.3 in TS38.133) (highlighted in green). However, in TS38.133, intra-band requirements are only applied for synchronous operation, while UE follows the inter-band requirements (Table 7.6.2-1 of TS38.133) for asynchronous operation (highlighted in green).

3) Similarly, there isn’t any description in TS38.306 on MTTD requirement for the FDD-FDD inter-band EN-DC with overlapping frequency when the UE doesn’t support interBandMRDC-WithOverlapDL-Bands-r16.

### TS38.306 interBandMRDC-WithOverlapDL-Bands-r16 Indicates the UE supports FDD-FDD or TDD-TDD inter-band (NG)EN-DC/NE-DC operation with overlapping or partially overlapping DL bands with an (NG)EN-DC/NE-DC MRTD according to clause 7.6.2/7.6.5 in 38.133 [5] and inter-band RF requirements (i.e Type 2 UE). If the capability is not reported, the UE supports FDD-FDD or TDD-TDD inter-band operation with overlapping or partially DL bands with (NG)EN-DC/NE-DC MRTD<3us according to clause 7.6.3 in 38.133 [5] and intra-band RF requirements (i.e. Type 1 UE).

### TS38.133 ### 7.6.3 Minimum Requirements for intra-band EN-DC For intra-band EN-DC, only co-located deployment is applied. The UE shall be capable of handling at least a relative receive timing difference between subframe timing of signal from a E-UTRA cell belonging to the MCG and slot timing of signal from a cell belonging to the SCG as shown in Table 7.6.2-1 for E-UTRA FDD-NR FDD intra-band EN-DC provided the UE indicates that it is capable of asynchronous EN-DC operation [2].

# 2	Actions

To RAN4:

ACTION: 	RAN2 respectfully requests RAN4 to provide answers to the questions in order to clarify the ambiguities described above.

Question 1: For UE supporting and not supporting interBandMRDC-WithOverlapDL-Bands-r16, what are the requirements in NE-DC operation respectively?

Question 2: For FDD-FDD inter-band EN-DC with overlapping frequency, if UE does not report interBandMRDC-WithOverlapDL-Bands-r16, what are the MRTD requirements for asynchronous operation? And are there any differences on MRTD requirements in asynchronous operation for UE(s) supporting and not supporting interBandMRDC-WithOverlapDL-Bands-r16?

Question 3: For FDD-FDD inter-band EN-DC with overlapping frequency, if UE does not report interBandMRDC-WithOverlapDL-Bands-r16, what are the MTTD requirements for both synchronous and asynchronous operations? And for asynchronous operation, are there any differences on MTTD requirements for UE(s) supporting and not supporting interBandMRDC-WithOverlapDL-Bands-r16?

# 3	Dates of next TSG-RAN WG2 meetings

TSG-RAN WG2 Meeting #123bis	09th – 13th Oct.  2023	Xiamen, China

TSG-RAN WG2 Meeting #124	11th – 13th Nov. 2023               Chicago, US