3GPP TSG-RAN WG2 Meeting #123                                                                  R2-2309238

Toulouse, France, 21st – 25th Aug 2023

Source:	CATT

Title:	Correction to BLCR of 38.331 for RRC configuration

Agenda Item:	7.25.2‎

Document for:	Discussion and Decision

- Introduction

For Rel-18 UL Tx switching, the agreed-in-principle BLCRs have been finished via the post email discussion[1] after RAN2#122 meeting[2].

This contribution will discuss corrections to the agreed BLCR of 38.331 for RRC configuration[3].

- Discussion

Description for switching2T-DualUL

In RAN4 #107 meeting[5], RAN4 confirmed semi-static RRC configuration is used in indicate whether 2T-2T switching period is applied for a UL Tx switching.

RAN4 agrees that option 2 matches RAN4 understanding with the following understanding: - Option 2 means semi-static RRC configurations and gNB configures which switching cases are applied to 2Tx-2Tx switching period. - Regarding the granularity, RAN4 suggests per band pair granularity, i.e., - if gNB configures as 2Tx-2Tx switching period applies to a band pair between switching (e.g., band A and band B), 2Tx-2Tx switching period applies to the switching cases between the band pair - 2P(band A)+0P(band B) <=> 0P(band A)+2P(band B) - 1P(band A)+0P(band B) <=> 0P(band A)+2P(band B) - 0P(band A)+1P(band B) <=> 2P(band A)+0P(band B) - 1P(band A)+0P(band B) <=> P(band A)+1P(band B) - if the gNB does not configure the band pair for 2Tx-2Tx, then the Tx switching period for 1Tx-2Tx should be applied.

Per band pair IE switching2T-DualUL is introduced for indicating whether 2Tx-2Tx switching period is applied to a band pair. Present description for switching2T-DualUL in the BLCR:

switching2T-DualUL Indicates 2Tx-2Tx switching period is considered as the switching gap duration when UL Tx switching is performed between the two bands within the band pair as specified in TS 38.214 [19].

According to present description, 2Tx-2Tx switching period is applied to the band pair when this IE is configured, however, it’s not clear which switching case is applied to, and what switching period is used when this IE is not configured.

Based on RAN4’s assumption, if gNB configures switching2T-DualUL, all switching cases including 2Tx-2Tx switching will utilize 2Tx-2Tx switching period for the band pair.

When switching2T-DualUL is absent, all 4 switching cases will utilize 1Tx switching period for that band:

- 2P(band A)+0P(band B) <=> 0P(band A)+2P(band B)

- 1P(band A)+0P(band B) <=> 0P(band A)+2P(band B)

- 0P(band A)+1P(band B) <=> 2P(band A)+0P(band B)

- 1P(band A)+0P(band B) <=> P(band A)+1P(band B)

That means when gNB configures 2Tx-2Tx switching mode to the cell group, it should always provide switching2T-DualUL, otherwise, UE will be forced to use 1Tx switching period for 2Tx-2Tx UL Tx switching, which may not be able to be achieved.

So, we have following proposal for updating the BLCR:

Proposal 1: Specify switching2T-DualUL is always present for band pair(s) when 2Tx-2Tx UL Tx switching is configured.

Based on proposal 1, whether 2Tx-2Tx switching period is applied can depend on presence of switching2T-DualUL. That means is gNB does not configure switching2T-DualUL, the switching period for 1Tx-2Tx/1Tx-1Tx should be applied to the band pair. But present BLCR does not clarify that. Propose to add the corresponding description.

Proposal 2: Specify the 1Tx-2Tx/1Tx-1Tx switching period is applied to band pair(s) when switching2T-DualUL is absent.

When gNB does not configures 2Tx-2Tx switching mode to the cell group, i.e., UE is only be able to do 1Tx switching. There are different understandings on the configuration for switching2T-DualUL in that case. Two options are presented by companies:

Option 1: switching2T-DualUL is always absent for band pairs when 2Tx-2Tx UL Tx switching is not configured.

Option 2: switching2T-DualUL can be present for band pairs when 2Tx-2Tx UL Tx switching is not configured.

Option 1 means the 1Tx switching on the band pair can only utilize the 1Tx switching period.

While Option 2 means the 1Tx switching on the band pair can also use 2Tx-2Tx switching period and whether 2Tx-2Tx switching period is used for 1Tx switching depends on the presence of switching2T-DualUL.

As there is no clue in the LS from RAN4[5], RAN2 should confirm RAN4’s intention and decide which option to go before further update to the BLCR.

Proposal 3: RAN2 to determine which option to go in case 2Tx-2Tx UL Tx switching is not configured:

Option 1: switching2T-DualUL is always absent for band pair(s).

Option 2: switching2T-DualUL can be present for band pair(s).

- Conclusion

In this contribution, we discuss corrections to the 38.331 BLCR for RRC configuration, based on which we have the following proposals.

Proposal 1: Specify switching2T-DualUL is always present for band pair(s) when 2Tx-2Tx UL Tx switching is configured.

Proposal 2: Specify the 1Tx-2Tx/1Tx-1Tx switching period is applied to band pair(s) when switching2T-DualUL is absent.

Proposal 3: RAN2 to determine which option to go in case 2Tx-2Tx UL Tx switching is not configured:

Option 1: switching2T-DualUL is always absent for band pair(s).

Option 2: switching2T-DualUL can be present for band pair(s).

- Reference

- [Post122][025][MCE] MCE TX switching CRs (Huawei)

- Minutes report, RAN2 Chairman, RAN2#122

- R2-2306911, RRC configuration for Rel-18 UL Tx switching enhancements, Huawei, NTT DOCOMO, RAN2#122

- Minutes report, RAN2 Chairman, RAN2#121 bis-e

- R4-2310271, Reply LS on report of switching periods in Rel-18 uplink Tx switching, NTT DOCOMO, RAN4#107