3GPP TSG-RAN WG2 Meeting #123	R2-2308961

Toulouse, France, Aug 21-25, 2023

Agenda item:	8.1

Source: 	Vice Chairman (ZTE Corporation)

Title:	Report from Break-out session on NR-NTN and IoT-NTN

Document for:	Approval

General

Recording of voice or video at meetings is not used in 3GPP. This applies also to this e-Meeting. At this e-Meeting, no specific actions are taken to prevent the recording of web conferences. Companies that have concerns related to recordings, if any, may express those by email in the main meeting organizational thread [AT123][000]

Organizational

- All organization emails and notes will be shared over the following email discussion throughout the meeting:

[AT123][100] Organizational – NR-NTN and IoT-NTN session (RAN2 VC)

Scope:

    - Share plans for the meeting and list of ongoing email discussions for the sessions related to NR-NTN and IoT-NTN

    - Share meetings notes and agreements for review and endorsement

Schedule/Plan

|  | Main room | Brk 1 room | Brk 2 room | Brk 3 room |
| --- | --- | --- | --- | --- |
| Monday August 21 |  |  |  |  |
| 09:00 – 10:30 | [1], [2], [2.5] Elections [3] 10-25 min NR1516 CP (Johan) - Common NR17 (Johan) - Common | Breakout to start after formal opening of meeting in main room NR18 fCovEnh [0.5] (Eswar) | Breakout to start after formal opening of meeting in main room: NRLTE1516 Pos (Nathan) NR17 Pos (Nathan) |  |
| 11:00 – 13:00 |  | NR18 XR [2] (Tero), could possibly start earlier TBD |  |  |
| 14:30 – 16:30 | NR18 Closed WIs early items, if needed (Yi, Sasha) NR18 (Johan) - Early items TBD - Common NR17 (Johan) - Common, if time (TBD continue 20min after the break) | NR151617 UP (Diana) NR18 MT-SDT [0.5] (Diana) NR18 Network Energy Saving [1] (Diana) | NRLTE1516 V2X/SL (Kyeongin) NR17 SL (Kyeongin) |  |
| 17:00 – 19:00 | NR18 MIMO evo [0.75] (Erlin) - Early items |  | NR18 SL evolution [1] (Kyeongin) |  |
| Tuesday August 22 |  |  |  |  |
| 08:30 – 10:30 | NR18 feMob [2] (Johan) | NR18 eQoE [1] (Tero) | NR17 (Nathan) - Pos SL Relay TBD |  |
| 11:00 – 13:00 | NR18 Mobile IAB [0.5] (Johan) NR17 NTN Maint (Sergio) - 4.2 - 6.3 | NR18 XR [2] (Tero) | NR18 Pos [2] (Nathan) |  |
| 14:30 - | [2.5] Elections (Voting for Chair: voting tool open 12:30 - 14:00) |  |  |  |
| – 16:30 | NR18 LP WUS [0.5] (Johan) - Short: Early items for offline prep. NR17 Common (Johan), continuation. | Start after common session: NR18 NTN enh [1] (Sergio) - 7.7.1 - 7.7.2 - 7.7.3 - 7.7.4.1.1 - 7.7.4.1.2 - 7.7.4.2 : report of [Post122][114] | Start after common session: NR18 Pos [2] (Nathan) |  |
| 17:00 | [2.5] Elections, if needed (Voting for Chair 2nd round: voting tool open 15:30 - 16:55) |  |  |  |
| – 19:00 | NR18 Other [2] (Johan) | Start after common session: NR18 UAV [1] (Diana) | Start after common session: NR17 (Nathan) NR18 SL relay [1.5] (Nathan) |  |
| Wednesday August 23 |  |  |  |  |
| 08:30 – 10:30 | NR18 feMob [2] (Johan) | 8:30-9:00 EUTRA17+ (Tero) 9:00-10:30 MUSIM | NR 18 MBS [0.75] (Dawid) |  |
| 11:00 – 13:00 | NR17 - Common (Johan), continuation. | NR18 URLLC [0.5] (Diana) NR18 Network Energy Saving [1] (Diana) | NR17 SONMDT (HuNan) |  |
| 14:30 | [2.5] Elections (Voting for Vice Chair: voting tool open 12:30 - 14:00) |  |  |  |
| – 16:30 | NR18 Other [2] (Johan) | Start after common session: NR18 RedCap [1] (Mattias) | Start after common session: NR18 SONMDT [1] (HuNan) |  |
| 17:00 | [2.5] Elections (Voting for Vice Chair: voting tool open 15:30 - 16:55) |  |  |  |
| – 19:00 | NR18 AIML [1] (Johan) | Start after common session: R18 IoT-NTN [1] (Sergio) - 7.6.1 - 7.6.2.1 : report of [101] - 7.6.2.2 - 7.6.3.1: report of [Post122][112] - 7.6.3.2 - 7.6.4 : report of [Post122][113] NTN Self evaluation (Sergio) - 7.25.4 | Start after common session: NR18 SL relay [1.5] (Nathan) |  |
| Thursday August 24 |  |  |  |  |
| 08:30 – 10:30 | CB NR151617 (Johan) | CB Diana | CB Kyeongin |  |
| 11:00 – 13:00 | NR18 TEI [1] (Johan, TBD Nathan) | CB Diana | CB Kyeongin |  |
| 14:30 – 16:30 | CB NR17 (Johan) | CB Sergio, Tero | CB Nathan |  |
| 17:00 – 19:00 | CB NR17 (Johan) CB NR18 (Johan) | CB Tero MUSIM con’t TBD | CB Nathan |  |
| Friday August 25 |  |  |  |  |
| 08:30 – 10:30 | NR18 MIMO evo [0.75] (Erlin) - late items and CBs. CB Dawid | CB Mattias TBD | CB Nathan, Kyeongin TBD |  |
| 11:00 – 13:00 | CB Johan, Eswar TBD | CB Sergio | TBD CB NR18 IDC [0] (Yi) |  |
| 14:30 – 16:00 | CB Johan | CB Tero CB NR18 NCR [0] (Sasha) | CB HuNan |  |
| 16:00 – 17:00 | CB and conclusion (Johan) |  |  |  |

List and status of offline discussions

NOTE:  No offline email discussions will be kicked off before Monday Aug 21th, 09:00 local time

[AT123][101][IoT NTN] HARQ Enhancements (Nokia)

Initial scope: Discuss the proposals in the submitted contributions in AI 7.6.2.1

Initial intended outcome: Summary of the offline discussion with e.g.:

- List of proposals for agreement (if any)

- List of proposals that require online discussions

- List of proposals that should not be pursued (if any)

Deadline for companies' feedback: Wednesday 2023-08-23 08:00

Deadline for rapporteur’s summary (in R2-2308981): Wednesday 2023-08-23 14:00

Status: To be started

## 4.2	NB-IoT and eMTC support for NTN Rel-17

(LTE_NBIOT_eMTC_NTN; leading WG: RAN1; REL-17; WID: RP-211601)

Tdoc Limitation: 1 tdocs

This Agenda Item is treated in the Breakout session that includes NTN

A single CR per TS with miscelaneous corrections is encouraged.  Small editorial corrections should be sent directly to rapporteur.  Big open issues can be discussed with contributions with CR in the appendix of the contribution

Reference point for timing info in IoT-NTN

R2-2307499	Discussion on UTC reference point in IoT NTN	Huawei, HiSilicon	discussion	Rel-17	LTE_NBIOT_eMTC_NTN

R2-2308227	Reference point for UTC timing in SIB16(-NB)	Nokia, Nokia Shanghai Bell	discussion	Rel-17	LTE_NBIOT_eMTC_NTN-Core

R2-2307188	Clarify the reference point for timing info in SIB16(-NB) and DLInformationTransfer in IoT NTN	MediaTek Inc., Qualcomm Inc, Apple, Ericsson, Huawei	CR	Rel-17	36.331	17.5.0	4937	-	F	LTE_NBIOT_eMTC_NTN-Core

Other RRC CRs

R2-2307325	Correct TrackingAreaList for selected PLMN for NB-IoT	MediaTek	CR	Rel-17	36.331	17.5.0	4938	-	F	LTE_NBIOT_eMTC_NTN-Core

R2-2308522	Miscellaneous RRC corrections for IoT NTN	ZTE Corporation, Sanechips	CR	Rel-17	36.331	17.5.0	4945	-	F	LTE_NBIOT_eMTC_NTN-Core

R2-2308893	RRC Correction on including GNSS validity duration and dedicated SIB31	Samsung	CR	Rel-17	36.331	17.5.0	4952	-	F	LTE_NBIOT_eMTC_NTN

MAC CR

R2-2307324	Corrections on the HARQ RTT timer for IoT NTN	MediaTek	CR	Rel-17	36.321	17.5.0	1567	-	F	LTE_NBIOT_eMTC_NTN-Core

Stage 2 CR

R2-2308538	Correction to GNSS acquisition description for IoT NTN	Ericsson	CR	Rel-17	36.300	17.5.0	1386	-	F	LTE_NBIOT_eMTC_NTN-Core

## 6.3	NR Non-Terrestrial Networks (NTN)

(NR_NTN_solutions-Core; leading WG: RAN2; REL-17; WID: RP-211557)

Tdoc Limitation: 1 tdocs

A single CR per TS with miscellaneous corrections is encouraged.  Small editorial corrections should be sent directly to rapporteur.  Big open issues can be discussed with contributions with CR in the appendix of the contribution

RRC CRs

R2-2308253	Clarification of UE configuration in TN and NTN	Ericsson	CR	Rel-17	38.331	17.5.0	4239	-	F	NR_NTN_solutions-Core

R2-2308520	Correction on RRC Release for NR NTN	Samsung	CR	Rel-17	38.331	17.5.0	4257	-	F	NR_NTN_solutions-Core

Other

R2-2307113	Correction on the Capability of TA Reporting	vivo	CR	Rel-17	38.306	17.5.0	0931	-	F	NR_NTN_solutions-Core

R2-2307498	Triggering of TA Report during handover	Huawei, HiSilicon	discussion	Rel-17	NR_NTN_solutions-Core

## 7.6	IoT NTN enhancements

(IoT_NTN_enh-Core; leading WG: RAN1; REL-18; WID: RP-223519)

Time budget: 1 TU

Tdoc Limitation: 4 tdocs

### 7.6.1	Organizational

LSs, rapporteur inputs and other organizational documents. Rapporteur inputs and other pre-assigned documents in this AI do not count towards the tdoc limitation.

Incoming LSs

R2-2307003	LS on GNSS position fix during inactive state of Connected DRX for improved GNSS operations (R1-2304126; contact: MediaTek)	RAN1	LS in	Rel-18	IoT_NTN_enh-Core	To:RAN2

R2-2307005	Reply LS on HARQ Enhancements (R1-2306182; contact: OPPO)	RAN1	LS in	Rel-18	IoT_NTN_enh-Core	To:RAN2

R2-2307012	LS on Rel-18 RAN1 UE features list for LTE after RAN1#113 (R1-2306222; contact: NTT DOCOMO, AT&T)	RAN1	LS in	Rel-18	IoT_NTN_enh-Core	To:RAN2	Cc:RAN4

R2-2307016	LS on NPDCCH monitoring restriction for NB-IoT NTN (R1-2306245; contact: Lenovo)	RAN1	LS in	Rel-18	IoT_NTN_enh-Core	To:RAN2

Running CRs

R2-2308542	Running CR for IoT NTN	Ericsson	draftCR	Rel-18	36.300	17.5.0	B	IoT_NTN_enh-Core

R2-2308939	Rapporteur input on 36.300	Ericsson	draftCR	Rel-18	36.300	17.5.0	B	IoT_NTN_enh-Core

R2-2308046	36331 running CR for IOT NTN	Huawei, HiSilicon	draftCR	Rel-18	36.331	17.5.0	B	IoT_NTN_enh-Core

R2-2308194	36.304 Running CR for Rel-18 IoT NTN        	Nokia Solutions & Networks (I)	draftCR	Rel-18	36.304	17.4.0	IoT_NTN_enh-Core

R2-2308944	Stage-3 running CR for TS 36.321 for Rel-18 IoT-NTN	MediaTek Inc.	draftCR	Rel-18	36.321	17.5.0	B	IoT_NTN_enh-Core	R2-2306962

R2-2307625	Running CR for TS 36.306 for Rel-18 IoT NTN	Qualcomm Incorporated	draftCR	Rel-18	36.306	17.4.0	B	IoT_NTN_enh-Core

R2-2308904	On R18 IoT NTN UE capabilities	Ericsson	discussion	Rel-18	IoT_NTN_enh-Core

### 7.6.2	Performance Enhancements

#### 7.6.2.1	HARQ enhancements

[AT123][101][IoT NTN] HARQ Enhancements (Nokia)

Initial scope: Discuss the proposals in the submitted contributions in AI 7.6.2.1

Initial intended outcome: Summary of the offline discussion with e.g.:

- List of proposals for agreement (if any)

- List of proposals that require online discussions

- List of proposals that should not be pursued (if any)

Deadline for companies' feedback: Wednesday 2023-08-23 08:00

Deadline for rapporteur’s summary (in R2-2308981): Wednesday 2023-08-23 14:00

R2-2308981	[offline-101] HARQ enhancements	Nokia	discussion	Rel-18	IoT_NTN_enh-Core

R2-2307105	Discussion on HARQ Enhancement for IoT NTN	vivo	discussion	Rel-18	IoT_NTN_enh-Core

R2-2307189	On Disabling HARQ Feedback in IoT-NTN	MediaTek Inc.	discussion

R2-2307250	Discussion on HARQ enhancement for IoT NTN	OPPO	discussion	Rel-18	IoT_NTN_enh-Core

R2-2307251	Draft reply LS on NPDCCH monitoring restriction for NB-IoT NTN	OPPO	LS out	Rel-18	IoT_NTN_enh-Core	To:RAN1

R2-2307413	Discussion on HARQ enhancements in IoT NTN	CATT	discussion	Rel-18	IoT_NTN_enh-Core

R2-2307488	Discussion on HARQ mode for PUR	Huawei, Turkcell, HiSilicon 	discussion	Rel-18	IoT_NTN_enh-Core

R2-2307506	Discussion on HARQ enhancement	Xiaomi	discussion	Rel-18

R2-2307587	Further discussion on HARQ enhancements	ZTE Corporation, Sanechips	discussion	Rel-18	IoT_NTN_enh-Core

R2-2307626	HARQ process enhancement	Qualcomm Incorporated	discussion	Rel-18	IoT_NTN_enh-Core

R2-2308228	On HARQ enhancements for IoT NTN	Nokia, Nokia Shanghai Bell	discussion	Rel-18	IoT_NTN_enh-Core

R2-2308288	Discussion on the HARQ enhancement for IoT-NTN	CMCC	discussion	Rel-18	IoT_NTN_enh-Core

R2-2308541	R18 IoT NTN HARQ enhancements	Ericsson	discussion	Rel-18	IoT_NTN_enh-Core

R2-2308576	Remaining Issues on Disabling HARQ feedback for IoT-NTN	Interdigital, Inc.	discussion	Rel-18	IoT_NTN_enh-Core

#### 7.6.2.2	GNSS operation enhancements

R2-2307106	Discussion on GNSS Operation for IoT NTN	vivo	discussion	Rel-18	IoT_NTN_enh-Core

R2-2307190	Enhancements on GNSS operation	MediaTek Inc.	discussion

R2-2307259	Discussion on GNSS operation for IoT NTN	OPPO	discussion	Rel-18	IoT_NTN_enh-Core

R2-2307414	Discussion on GNSS operation in connected mode	CATT	discussion	Rel-18	IoT_NTN_enh-Core

R2-2307477	Discussion on the GNSS Validity Reporting	Google Inc.	discussion	Rel-18

R2-2307489	Discussion on the impact of GNSS measurement	Huawei, Turkcell, HiSilicon 	discussion	Rel-18	IoT_NTN_enh-Core

R2-2307505	Discussion on GNSS operation enhancement	Xiaomi	discussion	Rel-18

R2-2307588	Remaining issues of GNSS enhancements	ZTE Corporation, Sanechips	discussion	Rel-18	IoT_NTN_enh-Core

R2-2307629	GNSS fix in RRC_CONNECTED	Qualcomm Incorporated	discussion	Rel-18	IoT_NTN_enh-Core

R2-2307865	Improved GNSS Operation	Apple	discussion	Rel-18	IoT_NTN_enh

R2-2308008	Some remaining issues of GNSS operations for IoT NTN	Lenovo	discussion	Rel-18

R2-2308229	GNSS operation enhancement in Rel-18 IoT NTN	Nokia, Nokia Shanghai Bell	discussion	Rel-18	IoT_NTN_enh-Core

R2-2308289	Discussion on GNSS enhancement for IoT-NTN	CMCC	discussion	Rel-18	IoT_NTN_enh-Core

R2-2308540	R18 IoT NTN GNSS operation enhancements	Ericsson	discussion	Rel-18	IoT_NTN_enh-Core

R2-2308577	GNSS acquisition and reporting for IoT NTN	Interdigital, Inc.	discussion	Rel-18	IoT_NTN_enh-Core

R2-2308617	Discussion of GNSS operation enhancements	SHARP Corporation	discussion

R2-2308881	GNSS Validity duration Reporting	Nordic Semiconductor ASA	discussion

Moved here from 7.6.2

R2-2308890	On improved GNSS operation and HARQ for IoT NTN	Samsung Electronics Czech	discussion	Rel-18	IoT_NTN_enh

### 7.6.3	Mobility Enhancements

#### 7.6.3.1	Enhancements for neighbour cell measurements

Including outcome of [Post122][112][IoT NTN Enh] Mobility enhancements (other contributions on issues handled in [Post122][112][ might not be treated at RAN2#123)

Remaining issues related to SIB handling

R2-2307192	Report of [Post122][112][IoT NTN Enh] Mobility 	MediaTek Inc.	discussion

A.	Proposals with Consensus:

Proposal 1 (14/14): For eMTC, the new SIB (SIBxx) is not an essential SIB. UE does not need to consider the cell barred if it is unable to acquire the SIB when scheduled.

Proposal 2 (14/14): RAN2 will not consider cell stop time of neighbor cell in the new SIB (SIBxx) in this release.

Proposal 8 (14/14): for NB-IoT NTN, location-based measurement initiation can also be optionally used in RRC_IDLE for cell re-selection purposes (like in NR-NTN), with the assumption that it is up to the UE to update GNSS location.

B.	Proposals with Majority:

Proposal 3 (12/14): The t-ServiceStart for quasi-earth fixed neighbour cells should be per neighbour cell.

Proposal 6 (10/14): validity duration is optional, and if this field is absent, the UE uses validity duration from the serving cell

Proposal 7 (7/9): Re-acquisition of SIBXX can reply on T317/T318 in connected mode.

C.	Proposals for Further Discussions:

Proposal 4: RAN2 further discuss which SIB(s) can be used to broadcasted t-ServiceStart for quasi-earth fixed neighbour cells

Proposal 5: RAN2 further discuss if ephemeris is absent in a list in the new SIB (SIBxx), serving satellite ephemeris applies.

R2-2307191	Remaining Enhancements on Neighbor Cell Measurements in IoT-NTN	MediaTek Inc.	discussion

R2-2307252	Discussion on mobility enhancement for IoT NTN	OPPO	discussion	Rel-18	IoT_NTN_enh-Core

R2-2307511	Discussion on UE behavior when serving cell t-service expires	Xiaomi	discussion	Rel-18

R2-2307589	Remaining issues of mobility enhancements	ZTE Corporation, Sanechips	discussion	Rel-18	IoT_NTN_enh-Core

R2-2307628	Measurement and Mobility enhancements	Qualcomm Incorporated	discussion	Rel-18	IoT_NTN_enh-Core

R2-2307772	On remaining issues of IoT-NTN mobility enhancements	Nokia, Nokia Shanghai Bell	discussion

R2-2307866	Neighbour cell measurements before RLF for NB-IoT	Apple	discussion	Rel-18	IoT_NTN_enh

R2-2308034	Enhancements for neighbour cell measurements	Huawei, Turkcell, HiSilicon	discussion	Rel-18	IoT_NTN_enh-Core

R2-2308578	Open issues on mobility enhancements (not covered by [Post122][112])	Interdigital, Inc.	discussion	Rel-18	IoT_NTN_enh-Core

R2-2308811	Discussion on gaps for neighbour cell measurements in IoT NTN	Ericsson	discussion	Rel-18	IoT_NTN_enh-Core

R2-2308891	On enhancements for neighbour cell measurements	Samsung Electronics Czech	discussion	Rel-18	IoT_NTN_enh

#### 7.6.3.2	Other

R2-2307107	Discussion on Mobility Enhancement for R18 IoT NTN	vivo	discussion	Rel-18	IoT_NTN_enh-Core

R2-2307867	Mobility enhancement in IoT NTN	Apple	discussion	Rel-18	IoT_NTN_enh

R2-2308035	Discussion on CHO enhancements	Huawei, Turkcell, HiSilicon	discussion	Rel-18	IoT_NTN_enh-Core

R2-2308290	Discussion on CHO enhancements for eMTC NTN	CMCC	discussion	Rel-18	IoT_NTN_enh-Core

R2-2308892	On CHO and other mobility enhancements for IoT NTN	Samsung Electronics Czech	discussion	Rel-18	IoT_NTN_enh

### 7.6.4	Enhancements to discontinuous coverage

Including outcome of [Post122][113][IoT NTN Enh] Discontinuous coverage (other contributions on issues handled in [Post122][113][ might not be treated at RAN2#123)

R2-2307497	Report of [Post122][113][IoT NTN Enh] Discontinuous coverage (Huawei)	Huawei, HiSilicon	discussion	Rel-18	IoT_NTN_enh-Core

Paging enhancements

Proposal 1: Send an LS to SA2 to ask for:

-	SA2 opinion about the issue about misalignment between PTW and Coverage Window (e.g., whether it needs to be solved and if yes, whether it can be solved by NW implementation)

-	SA2 opinion about the possible enhancement to UE and NW behaviour to adjust PH and PTW starting point (e.g., based on an offset configured by eNB or CN, or based on an predefined rule between UE and eNB ) , if it cannot be solved by NW implementation

RRC connection release

Proposal 2: UE performs autonomous RRC release based on a timer (e.g., similar as MUSIM mechanism) or upon detection of the coverage gap (e.g., stop AS operation related to NTN). No further enhancement is needed.

UE behaviour related to RLF

Proposal 3: UE may directly go to RRC_IDLE after RLF is triggered, if there is not enough time for the UE to finish the procedure of RRC re-establishement due to the discontinuous coverage. FFS whether a NOTE is needed for this.

Additional information to UE

Proposal 4: Further discuss which (or none) of the following information is needed to be provided additionally:

-	Ephemeris of more satellites (5/15)

-	Footprint information of more satellites (5/15)

-	Carrier frequency for cell searching after a coverage gap (6/15)

R2-2307108	Discussion on Discontinuous Coverage for R18 IoT NTN	vivo	discussion	Rel-18	IoT_NTN_enh-Core

R2-2307319	Discontinuous coverage handling enhancement for IoT NTN	THALES, Telit	discussion	Rel-18	IoT_NTN_enh-Core

R2-2307415	Discussion on enhancements to discontinuous coverage	CATT	discussion	Rel-18	IoT_NTN_enh-Core

R2-2307444	Considerations on Supporting Discontinuous Coverage	NEC	discussion	Rel-18	IoT_NTN_enh-Core

R2-2307590	Remaining issues of discontinuous coverage	ZTE Corporation, Sanechips	discussion	Rel-18	IoT_NTN_enh-Core

R2-2307627	RRC release procedure in discontinuous coverage	Qualcomm Incorporated	discussion	Rel-18	IoT_NTN_enh-Core

R2-2307773	Further discussion on discontinuous coverage enhancements	Nokia, Nokia Shanghai Bell	discussion

R2-2307868	Support on discontinuous coverage in IoT NTN	Apple	discussion	Rel-18	IoT_NTN_enh

R2-2308009	Some remaining issues for discontinuous coverage	Lenovo	discussion	Rel-18

R2-2308217	Discussion on enhancement to discontinuous coverage for IoT NTN	Transsion Holdings	discussion	Rel-18

R2-2308285	Enhancements to discontinuous coverage 	Samsung R&D Institute UK	discussion	Rel-18	IoT_NTN_enh-Core

R2-2308579	Paging in discontinuous coverage	Interdigital, Inc.	discussion	Rel-18	IoT_NTN_enh-Core

R2-2308580	<draft> LS on PTW modification due to UE unreachability	Interdigital, Inc.	LS out	Rel-18	IoT_NTN_enh-Core	To:SA2	Cc:CT1

R2-2308717	Discussion on TN coverage for discontinuous coverage	ASUSTeK	discussion	Rel-18	IoT_NTN_enh-Core

## 7.7	NR NTN enhancements

(NR_NTN_enh -Core; leading WG: RAN1; REL-18; WID: RP-223534)

Time budget: 1 TU

Tdoc Limitation: 4 tdocs

### 7.7.1	Organizational

LSs, rapporteur inputs and other organizational documents. Rapporteur inputs and other pre-assigned documents in this AI do not count towards the tdoc limitation.

Workplan

R2-2307323	R18 WI NR-NTN-enh work plan at RAN1, 2 and 3	THALES	Work Plan	Rel-18	R2-2305391

Incoming LSs

R2-2307008	Reply LS to RAN2 on unchanged PCI (R1-2306210; contact: CATT)	RAN1	LS in	Rel-18	NR_NTN_enh-Core	To:RAN2

R2-2307011	Reply LS on RACH-less Handover (R1-2306217; contact: Samsung)	RAN1	LS in	Rel-18	NR_NTN_enh-Core	To:RAN2

R2-2307035	LS on time-based trigger condition in NG HO for NR NTN (R3-233527; contact: Ericsson)	RAN3	LS in	Rel-18	NR_NTN_enh-Core	To:SA2	Cc:RAN2

Running CRs

R2-2307318	Stage 2 running CR for TS 38.300 for Rel-18 NTN enhancements	THALES (Rapporteur)	draftCR	Rel-18	38.300	17.5.0	B	NR_NTN_enh-Core	R2-2306960

R2-2308523	Stage 3 NTN running CR for 38.321 - RAN2#122	InterDigital	draftCR	Rel-18	38.321	17.5.0	B	NR_NTN_enh-Core	R2-2306955

R2-2308902	Stage 3 Running RRC CR for NR NTN Rel-18	Ericsson	CR	Rel-18	38.331	17.5.0	4293	-	B	NR_NTN_enh-Core

- Revised in R2-2308937

R2-2308937	Stage 3 Running RRC CR for NR NTN Rel-18	Nanjing Ericsson Panda Com Ltd	CR	Rel-18	38.331	17.5.0	4293	1	B	NR_NTN_enh-Core	R2-2308902

R2-2308943	Running 38.304 CR for NTN	ZTE Corporation, Sanechips	draftCR	Rel-18	38.304	17.5.0	B	NR_NTN_enh-Core	R2-2306961

UE capabilities

R2-2308092	UE Capability Discussion for Rel-18 NR NTN Enhancements WI	Intel Corporation	discussion	Rel-18	NR_NTN_enh-Core

R2-2308093	UE capabilities for Rel-18 NR NTN Enhancements WI	Intel Corporation	draftCR	Rel-18	38.306	17.5.0	B	NR_NTN_enh-Core

R2-2308094	UE capabilities for Rel-18 NR NTN Enhancements WI	Intel Corporation	draftCR	Rel-18	38.331	17.5.0	B	NR_NTN_enh-Core

### 7.7.2	Coverage Enhancements

R2-2307195	Discussion on PUCCH repetition for Msg4 HARQ-ACK for NTN	NTT DOCOMO INC.	discussion	Rel-18

< support for higher layer signallling >

Observation 1.	RAN1 assume that the UE requests PUCCH repetition for Msg4 HARQ-ACK via Msg3 higher layer signaling. The request is assumed to be a two-state information.

Observation 2.	Two-state information in Msg3 to request PUCCH repetition for Msg4 HARQ-ACK is available by utilizing LCIDs.

Proposal 1.	RAN2 confirm that the request of PUCCH repetition for Msg4 HARQ-ACK via Msg3 higher layer signaling is feasible.

Proposal 2.	For request of PUCCH repetition for Msg4 HARQ-ACK, LCIDs are utilized.

(some alternative options are listed in other contributions, for instance in R2-2307313 and R2-2308230)

< interaction with other WIs >

Observation 3.	If RAN2 try to book LCIDs for “repetition request & early indication by (e)RedCap”, we head shortage of remaining LCIDs.

Observation 4. eLCID is not desirable for features to extend coverage because basically eLCID increases the length of MAC header.

Proposal 3.	RAN2 discuss how the request of PUCCH repetition for Msg4 HARQ-ACK via Msg3 co-exist with early indication by (e)RedCap UEs.

Proposal 4.	RAN2 confirm that eLCIDs are not used for request of PUCCH repetition for Msg4 HARQ-ACK.

R2-2307313	Discussion on signalling for PUCCH repetition for Msg4 HARQ-ACK	Samsung	discussion	Rel-18	NR_NTN_enh-Core

Proposal 1: RAN2 to discuss the following options for repetition request or UE capability indication of PUCCH repetition for Msg4 HARQ-ACK.

Option 1: PRACH resource partition

Option 2: indicate by one R bit in a MAC subheader in Msg3 MAC PDU

Option 3: new LCIDs for CCCH and CCCH1 for a non-RedCap UE in Msg3 MAC PDU

Option 4: a new MAC CE with fixed size of zero bits in Msg3 MAC PDU

Proposal 2: If Option 1 is selected, NW indicates enabling PUCCH repetition for Msg4 HARQ-ACK enabled in RACH configuration.

Proposal 3: If Option 2 is selected, if UL CCCH MAC SDU is included in Msg3 MAC PDU, the first or the second R bit in MAC subheader of UL CCCH MAC SDU in Msg3 MAC PDU is set to 1 to indicate the request/capability of PUCCH repetition for Msg4 HARQ-ACK.

Proposal 4: If Option 2 is selected, if C-RNTI MAC CE is included in Msg3 MAC PDU, the first or the second R bit in MAC subheader of C-RNTI MAC CE in Msg3 MAC PDU is set to 1 to indicate repetition request/capability of PUCCH repetition for Msg4 HARQ-ACK.

Proposal 5: If option 3 is selected, discuss how to indicate request/capability of PUCCH repetition for Msg4 HARQ-ACK when Msg3 does not contain UL CCCH SDU.

Proposal 6: If option 4 is selected, discuss whether LCID or eLCID is used for the new MAC CE of request/capability indication for PUCCH repetition for Msg4 HARQ-ACK.

Proposal 7: If Msg 3 includes the indication of UE capability of PUCCH repetition for Msg4 HARQ-ACK, discuss whether the indication is needed after initial access.

R2-2308230	On Msg3 indication for PUCCH repetition for Msg4 HARQ-ACK	Nokia, Nokia Shanghai Bell	discussion	Rel-18	NR_NTN_enh-Core

Proposal 3: RAN2 to discuss below options for ‘repetition request or capability report’ for PUCCH repetition for Msg4 HARQ-ACK.

-	Option1: define four new LCID(s) index indicating not only the size of CCCH in the Msg3 (48 and 64 bits) but also PUCCH repetition UE capability or request.

-	Option2: re-purpose the reserved R bits in the MAC subheader of the Msg3.

-	Option3: re-purpose the spare bit currently present in the RRC messages contained in UL CCCH or UL CCCH1.

Proposal 4: If option1 is to be used, whether and how to optimize the LCID usage can be discussed for Rel-18 (e.g., considering the cross WI coordination on LCID allocation).

R2-2307253	Discussion on PUCCH enhancement for Msg4 HARQ-ACK in NR NTN	OPPO	discussion	Rel-18	NR_NTN_enh-Core

R2-2307416	Discussion on PUCCH repetition for Msg4 HARQ-ACK	CATT	discussion	Rel-18	NR_NTN_enh-Core

R2-2307512	Discussion on coverage enhancement for NR NTN	Xiaomi	discussion	Rel-18

R2-2307526	Higher layer signalling for PUCCH repetition for Msg4 HARQ-ACK	Huawei, HiSilicon	discussion	Rel-18	NR_NTN_enh-Core

R2-2307620	UE capability indication for Msg4 ACK repetition	Qualcomm Incorporated	discussion	Rel-18	NR_NTN_enh-Core

R2-2307839	HL signaling design for the PUCCH repetition request	Apple	discussion	Rel-18	DUMMY

R2-2308294	Discussion on the LS on higher layer signaling in Msg3 PUSCH for PUCCH repetition for Msg4 HARQ-ACK	CMCC	discussion	Rel-18	NR_NTN_enh-Core

R2-2308507	Consideration on coverage enhancements	ZTE Corporation, Sanechips	discussion	Rel-18

R2-2308539	R18 NR NTN Coverage enhancements	Ericsson	discussion	Rel-18	NR_NTN_enh-Core

R2-2308604	Discussion on coverage enhancement	LG Electronics Inc.	discussion	NR_NTN_enh-Core

### 7.7.3	Network verified UE location

UE capability and NW behaviour

R2-2307601	UE support of Network Verified UE Location Feature	Samsung R&D Institute UK	discussion	Rel-18	NR_NTN_enh-Core

Observation 1: RAN1 will work on multi-RTT for performing verification of the UE location.

Observation 2: the new feature of location verification is an optional UE feature.

Observation 3: A Rel-18 UE should support multi-RTT as part of its NR NTN capability.

Proposal 1: RAN2 to change the WA on “A Rel-18 UE capability is needed for indicating whether UE supports the feature of network verified UE location in NR NTN network” to a full agreement.

Proposal 2: RAN2 to discuss how the network handles the access to NR NTN cells for UEs that do not support the new Rel-18 NR NTN capability to support the new feature of network verified UE location.

R2-2307487	Discussion on the network verfied UE location	Huawei, Turkcell, HiSilicon 	discussion	Rel-18	NR_NTN_enh-Core

Cell change handling

R2-2307908	Discussion on network verified UE location	Ericsson	discussion	Rel-18	NR_NTN_enh-Core

Observation 1	In the legacy RAT dependent positioning framework, when the requested measurements cannot be obtained, the UE and the gNB can provide an error cause to the LMF indicating that the required positioning measurements cannot be obtained.

Observation 2	Up reception of the error cause, it is up to the LMF’s implementation on how to handle the positioning procedure.

Proposal 1	Limited by work time in R18, RAN2 shall aim for a simple option to address the impact of cell change and/or satellite change on positioning procedures.

Proposal 2	Define a new error cause in LPP indicating that the requested positioning measurements cannot be obtained due to cell change and/or satellite change.

Proposal 3	Send a LS to RAN1 and RAN3 informing them of the RAN2 agreement on the error cause.

Proposal 4	Adopt the LS draft captured in the appendix.

Procedure / assistance information

R2-2308263	Discussion on network verified UE location	Xiaomi	discussion

Proposal 1: The multi-RTT positioning procedure of TN without any neighbouring gNB/TRP involvement is the baseline for multi-RTT positioning of NTN when only a single satellite is visible.

Proposal 2: The serving gNB/TRP should provide multiple gNB Rx-Tx time difference measurements with different measurement time to LMF and UE should provide multiple UE Rx-Tx time difference measurements on the PRS from the serving gNB with multiple measurement times to LMF respectively.

Proposal 3: LMF could configure UE and gNB to report the positioning measurement periodically or configure the UE and gNB to report multiple positioning measurements in one shot report.

Proposal 4: The following solutions are considered for LMF acquiring the ephemeris data:

•	gNB provides the ephemeris data to LMF by NRPPa message

•	UE reports the ephemeris data to LMF by LPP message

•	OAM configures the ephemeris data to LMF

Proposal 5: gNB reports the common TA with positioning measurement by NRPPa measurement response message.

R2-2307320	Discussion on network verified UE location in NR NTN	THALES	discussion	Rel-18	NR_NTN_enh-Core	R2-2305408

Observation 1:

The geometry relating the UE and positioning anchor points (TRP) affects the network verified UE location based on Multi-RTT method.

Proposal 1: For multi-RTT based positioning in NTN, the UE should include the calculated N_"TA,adj" ^"common"  to the measurement results that need be transferred from UE to the LMF.

Proposal 2:  For multi-RTT based positioning in NTN, the UE includes the position of the satellite when DL-PRS measurements are performed to the measurement results that need be transferred from UE to the LMF.

Proposal 3:  For multi-RTT based positioning in NTN, the following assistance data may be transferred from gNB to the LMF:

The value of the 𝑘mac used by gNB

The value of TACommon when the gNB Rx – Tx time difference measurement is performed

Proposal 4: For multi-RTT based positioning in NTN, the gNB includes the position of the satellite when UL-SRS measurements are performed to the assistance data that may be transferred from gNB to the LMF.

Proposal 5: For multi-RTT based positioning in NTN, the gNB may provide the LMF with assistance data including:

Satellite ID

Cell/beam reference point

The ephemeris data in PVT state vector format or Keplerian format along with the associated epoch time.

Proposal 6:  For multi-RTT based positioning in NTN, the LMF indicates to the UE the vTRP positions or the time intervals at which the PRS should be measured.

Proposal 7:  For multi-RTT based positioning in NTN, the LMF indicates to the UE the vTRP positions or the time intervals at which the aperiodic SRS should be activated.

R2-2308196	Discussion on multiple-RTT based positioning in NTN	Quectel	discussion

R2-2308277	Discussion on NTN NW verified UE location	Lenovo	discussion	Rel-18

R2-2308295	Considerations on network verified UE location	CMCC	discussion	Rel-18	NR_NTN_enh-Core

R2-2308450	UE location verification by Network	NEC Telecom MODUS Ltd.	discussion

R2-2308706	Discussion on Network Verified UE Location	TCL	discussion

R2-2308777	On Network verified UE location for NR NTN	MediaTek Inc.	discussion

### 7.7.4	NTN-TN and NTN-NTN mobility and service continuity enhancements

#### 7.7.4.1	Cell reselection enhancements

R2-2307321	Discussion on mobility enhancements for VSAT	THALES	discussion	Rel-18	NR_NTN_enh-Core

R2-2307314	Discussion on Cell Reselection Enhancements	Samsung	discussion	Rel-18	NR_NTN_enh-Core

##### 7.7.4.1.1	NTN-TN enhancements

R2-2308283	Signaling of the TN coverage area and the frequency information	ZTE corporation, Sanechips	discussion	Rel-18	NR_NTN_enh-Core

Proposal 1: Both of the NR TN coverage and EUTRA coverage can be provided.

Proposal 2: Introduce a new SIB to provide the TN coverage information and it is allowed to provide coverage information for frequencies not included in SIB4/5.

Proposal 3: The frequency info should be indicated under each TN coverage area and frequency index bitmap referring to the frequencies in SIB4 and 5 should be introduced to reduce the signaling overhead, i.e. option 1+3.

R2-2307579	On TN Coverage Definition and TN to NTN Reselections	Nokia, Nokia Shanghai Bell	discussion	Rel-18	NR_NTN_enh-Core

Observation 1: As per RAN2#122 decisions, the information on the TN coverage area will be signaled either using Option 2 or the combination of Option 1 and Option 3.

Observation 2: Neither Option 2, nor the combination of Option 1 and 3 allow to signal other frequencies than already included in SIB4 or SIB5.

Observation 3: If the gNB wants the NTN UE to be aware of certain frequency for terrestrial coverage, this frequency can be listed in the legacy SIB4 or SIB5.

Observation 4: There may be up to 8 frequencies in the InterFreqCarrierFreqList in SIB4, each could be linked to more than a single TN coverage area.

Observation 5: In Option 2, 48 bits are needed if two areas per frequency are signaled, and each area ID consumes 3 bits.

Observation 6: Option 1+3 requires to always signal the same number of bits, equal to the number of frequencies in SIB4, even if for particular TN coverage area there is e.g. just a single frequency to indicate.

Proposal 1: Option 2 (A TN coverage area configuration is associated with a TN coverage Area ID. The frequency information for TN coverage area is indicated by adding TN coverage area IDs in SIB4 and SIB5) is adopted for signaling the TN coverage area information in Rel-18 NTN.

Observation 7: As per Rel-18 WID, cell reselection (mobility) enhancements for NTN to TN are prioritized.

Observation 8: NTN-capable UE while being in TN cell can measure on NTN frequencies even without NTN assistance information (such as ephemeris, common TA, etc.).

Observation 9: Providing NTN assistance information in the TN’s SIB, possibly for multiple NTN neighbour cells, will introduce a non-negligible signalling burden.

Proposal 2: In Rel-18 NTN WI RAN2 does not pursue enhancements for TN -> NTN cell reselection.

R2-2307621	TN cell coverage info and measurement relaxation	Qualcomm Incorporated	discussion	Rel-18	NR_NTN_enh-Core

Proposal 1	A new SIB is introduced to provide TN coverage information as a list of area center locations and their corresponding radius distance.

Proposal 2	Introduce coverage Area identity.

Proposal 3	In SIB4/5, if the frequency is TN frequency, the one or more area IDs are provided as inter-frequency carrier information for UE to decide whether the UE is required to perform measurement on this TN frequency.

Proposal 4	Introduce relaxed measurement for TN frequency for which the reselection priority is higher than current NTN cell reselection priority if the UE does not detect the cell for X number of measurement occasions.

Proposal 5	RAN2 consider solution to enable TN cell providing UE assistance information for NTN cell measurements.

R2-2307101	Remaining Issues on Power Saving for NTN-TN Mobility	vivo	discussion	Rel-18	NR_NTN_enh-Core

R2-2307166	NTN neighbour cell information in TN cells	PANASONIC R&D Center Germany	discussion

R2-2307167	Considerations on TN-NTN cell re-selection	Telit Communications S.p.A.	discussion

R2-2307217	Discussion on providing TN coverage area information	LG Electronics France	discussion	Rel-18	38.331	NR_NTN_enh

R2-2307254	Discussion on NTN-TN cell reselection enhancement	OPPO	discussion	Rel-18	NR_NTN_enh-Core

R2-2307417	Discussion on the mechanism for providing TN coverage information	CATT	discussion	Rel-18	NR_NTN_enh-Core

R2-2307739	Discussion on NTN to TN cell reselection enhancements 	TCL	discussion

R2-2307840	NTN-TN cell reselection enhancement	Apple	discussion	Rel-18	DUMMY

R2-2308010	Some remaining issues for  TN area information	Lenovo	discussion	Rel-18

R2-2308054	Discussion on the NTN – TN cell reselection enhancement	Turkcell, Huawei, HiSilicon	discussion	Rel-18

R2-2308116	Discussion on NTN-TN enhancements	NTT DOCOMO, INC.	discussion	Rel-18

R2-2308218	Discussion on remaining issues of NTN-TN cell reselection enhancements	Transsion Holdings	discussion	Rel-18

R2-2308239	Discussion on TN coverage description	ETRI	discussion	Rel-18	NR_NTN_enh-Core

R2-2308264	Cell reselection enhancements for NTN-TN mobility	Xiaomi	discussion

R2-2308296	Discussion on open issues for NTN-TN cell reselection	CMCC	discussion	Rel-18	NR_NTN_enh-Core

R2-2308524	NTN-TN mobility and service continuity	InterDigital	discussion	Rel-18	NR_NTN_enh-Core

R2-2308701	Discussion on NTN-TN Cell re-selection	ITL	discussion	Rel-18

##### 7.7.4.1.2	NTN-NTN enhancements

R2-2307255	Discussion on NTN-NTN cell reselection enhancement	OPPO	discussion	Rel-18	NR_NTN_enh-Core

Observation 1	For earth moving cell, the broadcasted serving cell reference location needs to be updated frequently with the change of serving satellite ephemeris and epochTime.

Observation 2	Regardless of quasi-earth-fixed cell and earth moving cell, time-based measurement initiation works in the same way.

Based on the discussion above, we give the following proposals:

Proposal 1	The change of serving cell reference location for earth moving cell should neither result in system information change notifications nor in a modification of valueTag in SIB1.

Proposal 2	It is up to UE implementation to maintain a valid serving cell reference location in RRC_IDLE mode.

Proposal 3	For the IE used to trigger UE neighbor cell measurements prior to cell replacement due to feeder link switch, re-use the same field of t-Service-17 as in Rel-17 and update the field description accordingly.

Moved here from 7.7.4.1

R2-2308901	Idle mode mobility enhancements	Ericsson	discussion	Rel-18	NR_NTN_enh-Core

Proposal 7	In case of NTN cell hard switch (either changing or keeping the PCI), UE need not to start neighbour cell measurements of the new cell before t-service expires.

Proposal 8	The network informs (either implicitly or explicitly) the UE whether the next NTN cell switch is a soft or a hard switch.

Proposal 9	Use ReferenceLocation IE for the parameter in SIB19 (movingReferenceLocation-r18) used to indicate the serving cell reference location for Earth-moving cells.

R2-2308525	Cell reselection enhancements for Earth moving cell	InterDigital	discussion	Rel-18	NR_NTN_enh-Core

Proposal 1:	Confirm that for Earth-moving cells, location-based measurement initiation and time-based measurement initiation due to feeder-link switch are optional UE features.

Observation 1:	Broadcasting multiple future reference point coordinates and associated timestamp can reduce frequency of SIB update and improve trajectory calculation.

Proposal 2:	For Earth-moving cells, multiple future reference location coordinates and associated timestamp information can be broadcast simultaneously.

Proposal 3:	If multiple future reference location coordinates are supported, timestamp information for each future reference point is provided by broadcasting an offset to epochTime.

Proposal 4:	A new name t-ServiceFeederLink is used for the IE which triggers UE neighbour cell measurements prior to feeder-link switch.

Observation 2:	During a hard switch, the service interruption extends to all cells originating from the satellite. Prior to the switch, UE should only measure cells originating from a different satellite with a stable feeder-link.

Proposal 5:	RAN2 to discuss whether the UE can relax (e.g., not perform) measurements on neighbouring cell(s) originating from the same feeder-link about to be switched.

R2-2307102	Further discussion NTN-NTN Mobility for Earth-moving Cell	vivo	discussion	Rel-18	NR_NTN_enh-Core

R2-2307218	Discussion on NTN-NTN cell reselection enhancements	LG Electronics France	discussion	Rel-18	38.331	NR_NTN_enh	R2-2306032

R2-2307740	Discussion on NTN to NTN cell reselection enhancements	TCL	discussion

R2-2308011	Feeder link switch time and reference location of NTN moving cells	Lenovo	discussion	Rel-18

R2-2308033	Discussion on location-based measurement initiation in moving cells	Huawei, Turkcell, HiSilicon	discussion	Rel-18	NR_NTN_enh-Core

R2-2308124	Discussion on NTN-NTN mobility enhancements	Spreadtrum Communications	discussion	Rel-18

R2-2308265	Cell reselection enhancements for NTN-NTN mobility	Xiaomi	discussion

R2-2308297	Discussion on remaining issues for NTN-NTN reselection	CMCC	discussion	Rel-18	NR_NTN_enh-Core

R2-2308700	Discussion on NTN-NTN cell reselection enhancements	CAICT	discussion	Rel-18	NR_NTN_enh-Core

R2-2308718	Discussion on description of movingReferenceLocation	ASUSTeK	discussion	Rel-18	38.331	NR_NTN_enh-Core

#### 7.7.4.2	Handover enhancements

Including outcome of [Post122][114][NR NTN Enh] Unchanged PCI (other contributions on issues handled in [Post122][114][ might not be treated at RAN2#123)

Unchanged PCI

R2-2308329	Report of [Post122][114][NR NTN Enh] Unchanged PCI	CMCC	discussion	Rel-18	NR_NTN_enh-Core

List of proposals for agreement:

Proposal 2: If new parameter (t-star or t-gap) is supported, system information (option 1) is used to provide the time information.

Proposal 3: If new parameter (t-start or t-gap) is supported, implicit indication manner is enough(17/19).

Proposal 4: If t-service is reused, explicit indication manner is needed.(14/15)

Proposal 5: 4-step RACH or 2-step RACH is up to NW configuration.

Proposal 6:  the selection of CFRA or CBRA is up to NW configuration.

Proposal 7: RACH-less operation(i.e. skip RA) can be combined with PCI unchanged solution in NTN system.

Proposal 8: For back-ward compatibility, network could handle the legacy UE using the existing mechanism and there is no spec. impact.

List of proposals that require online discussions:

Proposal 1: the unchanged PCI mechanism can be applied to both the continuous coverage where the coverage gap can be ignored and the coverage gap is not zero. FFS the indication is t-gap or t-start. (13/19)

R2-2307104	Further Discusison on Service Link Switch with Unchanged PCI	vivo	discussion	Rel-18	NR_NTN_enh-Core

R2-2307193	On Triggering Unchanged PCI for Handover Enhancement in LEO NTN	MediaTek Inc.	discussion

R2-2307418	Discussion on unchanged PCI scenario	CATT	discussion	Rel-18	NR_NTN_enh-Core

R2-2307476	Discussion on the Unchanged PCI Satellite Switch	Google Inc.	discussion	Rel-18

R2-2307581	On Unchanged PCI and Satellite Switching without L3 Mobility	Nokia, Nokia Shanghai Bell	discussion	Rel-18	NR_NTN_enh-Core

R2-2307623	Details on satellite switch with PCI unchange	Qualcomm Incorporated	discussion	Rel-18	NR_NTN_enh-Core

R2-2307741	Discussion on satellite switch with unchanged PCI	Panasonic	discussion

R2-2307841	Hard satellite switching with unchanged PCI	Apple	discussion	Rel-18	DUMMY

R2-2307893	Discussion on gap time of unchanged PCI	ITRI	discussion	NR_NTN_enh-Core

R2-2307896	Discussion on soft satellite switching with PCI unchanged	FGI	discussion

R2-2308373	Satellite Switch, PCI change without L3 handover	NEC	discussion	NR_NTN_enh-Core

R2-2308527	Satellite switching without PCI change	InterDigital	discussion	Rel-18	NR_NTN_enh-Core

R2-2308609	Discussion on NTN handover enhancements	Fujitsu	discussion	Rel-18	NR_NTN_enh-Core

R2-2308752	Discussion on random access in the unchanged PCI scenario	ETRI	discussion	Rel-18	NR_NTN_enh-Core

R2-2308753	“Unchanged PCI” solution vs “PCI change only” solution	Sequans Communications	discussion	Rel-18	NR_NTN_enh-Core	R2-2306517

RACH-less HO

R2-2307315	Discussion on Handover Enhancements	Samsung	discussion	Rel-18	NR_NTN_enh-Core

Proposal 1: The association between pre-allocated grant and SSB(s) is configured for initial UL transmission in RACH-less HO.

Proposal 2: In a pre-allocated grant, CG occasions are mapped to SSB indexes, configured by SSB-Subset and SSB-PerCG-PUSCH.

Proposal 3: A RSRP threshold is configured for SSB selection for initial UL transmission with pre-allocated grant. UE selects an SSB associated to a pre-allocated grant with RSRP above the threshold, and use that pre-allocated grant and the selected SSB for initial UL transmission.

Proposal 4: Beam indication (i.e., SSB index(es)) is provided to monitor target cell PDCCH for dynamic grant for initial UL transmission. Wait for RAN1 further response for the case where multiple beams are indicated.

Proposal 5: Wait for RAN1 further response on power control for initial UL transmission.

Proposal 6: If configured with RACH-less configuration, the MAC entity applies the configured N_TA for the TAG of the serving cell, and starts the timerAlignmentTimer associated with this TAG.

Proposal 7: The pre-allocated grant is configured for NUL or SUL, and UE selects the corresponding UL carrier for the initial UL transmission.

Proposal 8: For dynamic grant provided in PDCCH, NW indicates NUL or SUL carrier for initial UL transmission.

Proposal 9: If SUL is configured and if the UL carrier for initial UL transmission is not explicitly signaled, UE selects NUL or SUL carrier based on a RSRP threshold of the downlink pathloss reference.

Proposal 10: The BWP indicated by firstActiveUplinkBWP is selected for the initial UL transmission. If the firstActiveUplinkBWP is not signaled, initialUplinkBWP is selected.

Proposal 11: If ta-Report enabled is included in ServingCellConfigCommon for RACH-less HO, a TA report shall be triggered.

Proposal 12: For the pre-allocated grant, support autonomous retransmission of the initial UL transmission with a retransmission timer configured. UE retransmits by pre-allocated grant when the timer expires.

Proposal 13: Both HARQ mode A and HARQ mode B are supported for the HARQ process of RACH-less initial UL transmission.

Proposal 14: HARQ process ID 0 is used for initial UL transmission.

Proposal 15: Before the successful completion of the RACH-less HO, the MAC entity shall not select the logical channel(s) corresponding to DRB(s) for the uplink grant for the HARQ process of initial UL transmission.

Proposal 16: If DRX is configured, the Active time for Serving Cells in a DRX group includes the time while UE is monitoring the PDCCH that provides dynamic grant for the initial UL transmission.

Proposal 17: If a PDCCH addressed to C-RNTI scheduling DL/UL is received after the initial UL transmission, consider RACH-less HO completed.

R2-2307219	Discussion on handover enhancements	LG Electronics France	discussion	Rel-18	38.331	NR_NTN_enh	R2-2306033

R2-2307580	Resolving Open Points on RACH-less HO in Rel-18 NTN	Nokia, Nokia Shanghai Bell	discussion	Rel-18	NR_NTN_enh-Core

R2-2307622	RACH-less handover for NTN	Qualcomm Incorporated	discussion	Rel-18	NR_NTN_enh-Core

R2-2307943	RACH-less signaling design for NTN	China Telecom	discussion	Rel-18

R2-2308012	Some remaining issues for RACH-less HO in NTN	Lenovo	discussion	Rel-18

R2-2308032	Remaining issues on RACH-less HO in NTN	Huawei, Turkcell, HiSilicon	discussion	Rel-18	NR_NTN_enh-Core

R2-2308146	Discussion on RACH-less HO	Sharp	discussion	Rel-18	NR_NTN_enh-Core

R2-2308266	Discussion on handover enhancements for NTN-NTN mobility	Xiaomi	discussion

R2-2308374	Support RACH-less CHO	NEC	discussion	NR_NTN_enh-Core

R2-2308526	NTN mobility enhancements for RRC_CONNECTED	InterDigital	discussion	Rel-18	NR_NTN_enh-Core

R2-2308900	Handover enhancements	Ericsson	discussion	Rel-18	NR_NTN_enh-Core

Common HO configuration

R2-2307419	Discussion on RACH-less and common (C)HO configuration	CATT	discussion	Rel-18	NR_NTN_enh-Core

Proposal 6: RAN2 first focuses on intra-gNB scenario for common configuration.

Proposal 7: The serving cell broadcast the target cell’s servingCellConfigCommon (as common (C)HO signalling)

Proposal 8: Define one new SIB, i.e. SIBx, to transfer the common (C)HO configuration.

Proposal 9: The network broadcasts the common configuration of the adjacent target cell(s) based on the trajectory of the satellite, the number is FFS.

Proposal 10： The procedure in Figure 1 can be adopted as the baseline for common (C)HO configuration mechanism:

-	Step 1: The UE acquires the scheduling information for common (C)HO configuration from SIB1 upon the specified time e.g. upon t-service-offsetHO, or upon reception of indication from network.

-	Step 2: The UE starts to monitor SIBx according to the scheduling information in SIB1 if the UE supports common (C)HO configuration.

-	Step 3: The network send (C)HO command without common (C)HO configuration.

-	Step 4: the UE starts HO using the target cell common configuration in SIBx.

Notes: The order of step 2 and step 3 is up to network implementation.

R2-2307103	Discussion on Handover Enhancement with Common HO Configuration in NR NTN	vivo	discussion	Rel-18	NR_NTN_enh-Core

R2-2307258	Discussion on NTN handover enhancements	OPPO	discussion	Rel-18	NR_NTN_enh-Core

R2-2307894	Discussion on common information of group handover	ITRI	discussion	NR_NTN_enh-Core

R2-2308159	Signaling overhead reduction during NTN-NTN HOs	Sony	discussion	Rel-18	NR_NTN_enh

R2-2308219	Discussion on open issues of NTN-NTN handover	Transsion Holdings	discussion	Rel-18

R2-2308755	Common signalling of HO common information	Sequans Communications	discussion	Rel-18	NR_NTN_enh-Core

CHO/Other

R2-2307842	NR NTN specific HO enhancement	Apple	discussion	Rel-18

Observation 1: For the earth moving cell, the target cell for subsequent handover can be predicted.

Observation 2: Subsequent SCG change scheme will be specified in R18.

Proposal 1: Support Subsequent CHO as one of the NR NTN specific handover enhancements in R18.

Proposal 2: In subsequent CHO, UE will keep all the candidate CHO configurations after CHO execution, until receiving the explicit release indication from network.

Proposal 3: R18 subsequent CHO is limited in intra-gNB handover scenario, and HO with security key change is not supported.

R2-2308719	Discussion on moving cell reference location for CHO	ASUSTeK	discussion	Rel-18	NR_NTN_enh-Core

R2-2307343	Handover enhancements	Continental Automotive	discussion	Rel-18

### 7.25.4	Self-Evaluation NTN

(FS_IMT-2020_Sat_eval; leading Group: TSG RAN; REL-18; WID: RP-230754)

This will be treated in NTN breakout session (Sergio).

Study on Self-Evaluation towards the 3GPP submission of a IMT-2020 Satellite Radio Interface Technology, including both NR NTN and IoT-NTN. Note that the time allocated will be very limited, and this is expected to be mostly an offline activity.

R2-2307322	Discussion on IMT-2020 Satellite self-evaluation for Latency and Mobility	THALES	discussion	Rel-18	NR_NTN_enh-Perf	R2-2305410

R2-2307496	Self-Evaluation for NR NTN	Huawei, HiSilicon	discussion	Rel-18	FS_IMT2020_SAT_eval

R2-2307586	On CP and UP Latency for IMT-2020 NTN Self Evaluation	Nokia, Nokia Shanghai Bell	discussion	Rel-18	FS_IMT2020_SAT_eval

R2-2307624	RAN2 aspects on evaluation methodology for IMT-2020 Satellite	Qualcomm Incorporated	discussion	Rel-18	FS_IMT2020_SAT_eval

R2-2308508	Self-Evaluation towards the 3GPP submission of a IMT-2020 Satellite Radio Interface Technology	ZTE Corporation, Sanechips	discussion	Rel-18

R2-2308903	Satellite IMT-2020 self-evaluation: CP latency	Ericsson	discussion	Rel-18

R2-2308905	Satellite IMT-2020 self-evaluation: UP latency	Ericsson	discussion	Rel-18

# Summary

Agreed CRs

NR-NTN

Stage 2

38.306

38.321

38.331

IoT-NTN

Stage 2

36.321

36.331

Approved LSs out

[Post123] Email discussions

Short

Long