3GPP TSG-RAN WG2 Meeting #123bis		R2-2309677

Xiamen, China, 9th –13th October 2023

Source:	vivo

Title:	Discussion on model management

Agenda Item:	7.16.2.3

Document for:	Discussion and Decision

- Introduction

This discussion paper investigates the issues related to model management in terms of monitoring and control.

- Discussion

- 2.1	Model monitoring/assessment

Model monitoring is a process that can be performed at any entity involved in the UE-NW interaction process. Model monitoring usually follows the principle to obtain a so-called performance metric, which is a criterion for model validity. This performance metric can be calculated with the model inference output and the ground truth label i.e. measurement from legacy schemes (except for input-based monitoring for positioning). There exist rules to evaluate the model performance and hence to determine how to control the model with decision of activation/deactivation/switch/fallback.

In order to facilitate the discussion on specification impact, we tend to clarify:

Proposal 1: Model monitoring is performed by the entity which obtains the monitoring performance metrics, and the metrics will be further forwarded to the management entity.

We can categorize monitoring entities into UE and NW for further discussion, where NW can be further classified as RAN nodes and the CN functions (i.e. LMF for positioning).

In this understanding, when UE plays the role of model monitoring, it works out for the UE-sided model. Since the model output is generated by UE itself, UE could generate the performance metric instantly and proceed with succeeding operations. It introduces no impact on the current implementation. On the contrary, if UE monitors the NW-sided model, NW would needlessly transfer the output and label to UE, let alone the limited competence of UE to complete the monitoring task and to react swiftly.

NW, with full capability, is capable of monitoring both the UE-sided model and the NW-sided model. Most importantly, in some cases, the UE-sided model output reports to NW inherently.

One special case for CSI compression is the two-sided model. The monitoring can be performed at either UE or NW towards the model residing at themselves. Therefore, we impel to narrow down the monitoring entity applied to different model deployments based on the contribution on Mapping of functions to physical entities [3]:

Proposal 2: Regarding the model deployment type, model monitoring can be classified as:

- UE monitoring, can apply to UE-sided model and two-sided model

- NW monitoring, can apply to UE-sided, NW-sided model and two-sided model

With a down selection for each entity to perform model monitoring, we analyse the possible signalling enhancement case by case.

Case 1. UE monitoring UE-sided model

UE collects the label and monitors the model output from itself. UE itself decides the performance metric.

Proposal 3: UE monitoring UE-sided model can be based on UE implementation.

Case 2. UE monitoring two-sided model

Based on RAN1’s agreement, The UE-side monitoring performs towards output-CSI-UE and the output of the reconstruction model. There exist two alternatives to obtain the output of the reconstruction model. One is that the output of the NW-sided model needs to be indicated to UE. Another is that UE maintains a reconstruction model itself with no information requested from NW.

Most importantly, since CSI compression calls for the alignment of both the construction and reconstruction models, no matter UE or NW, once decides on the monitoring result and further proceeds on model activation/ deactivation/ switch/ fallback, it should notify the other side about the decision.

Proposal 4: The procedure of UE monitoring two-sided model may include:

- UE may request NW about the NW-sided model output

- NW may inform UE of the requested NW-sided model output

Case 3. NW monitoring NW-sided model

For the NW-sided model, the NW has the model output itself. Even for the input-based monitoring in positioning, the model input is known by NW inherently. However, to calculate the performance metric, it needs to collect the ground truth label from UE or another entity. For example, in the case of Type 1 CSI compression, UE transfers the estimated uncompressed information of CSI to gNB as the label for monitoring. The left monitoring operations can be covered by NW with no UE perception.

Proposal 5: The procedure of NW monitoring NW-sided model may include:

- NW may request UE about the ground truth label

- UE may inform NW of the requested ground truth label

One slightly special use case is positioning, which differentiates NW further into LMF and RAN nodes separately. In the identified positioning cases, the gNB-sided model is employed to obtain the intermediate feature for the AI-assisted location calculation. Under this condition, LMF to monitor gNB’s model output is selected as the promising solution. As this case requires gNB to transfer the output to LMF for the location estimation, the left enhancement for model monitoring is how LMF obtains labels to determine the model performance.

When it comes to monitoring gNB’s model input, LMF needs to collect the distribution of training data input and the model input together. Since model training is expected to be performed at LMF, it only requests model input transfer with gNB. Besides, LMF should indicate gNB whether the model switch/ activation/deactivation/ fallback is implemented consequently.

Proposal 5a: The procedure of LMF monitoring gNB-sided model may include:

- For output-based monitoring:

- LMF may request gNB about the ground truth label

- gNB may inform LMF of the ground truth label

- For input-based monitoring:

- LMF may request gNB about the model input (or the distribution of model input)

- gNB may inform LMF of the model input (or the distribution of model input)

Case 4. NW monitoring UE-sided model

Considering UE’s restricted capability may not be able to collect vast data and make a wise decision, NW could help UE with model monitoring and decisions on succeeding model control. However, it could also be a burden to transfer the model output if NW does not require it. A split of monitoring functions could be the one to calculate the performance metric and the other to make the final decision. In this way, NW is destined to be the one to make decisions, and UE acquires the performance metric and passes to NW with less signaling overhead.

Proposal 6: When NW monitors the UE-sided model based on the model output, UE should provide the calculated performance metric instead of the model output and label, to reduce the signaling overhead.

After making decisions on model validity based on the UE report on model performance, NW may feedback the decision consequently.

Proposal 7: The procedure of NW monitoring UE-sided model may include:

- NW may request UE about the performance metric

- UE may inform NW of the requested performance metric

When it comes to monitoring UE’s model input for positioning, LMF needs to collect the distribution of training data input and the model input together. Since model training is expected to be performed at LMF, it only requests model input transfer with UE. LMF indicates the model control decisions back to UE.

Proposal 7a: The procedure of LMF monitoring UE-sided model may include:

- LMF may request UE about the model input (or the distribution of model input)

- UE may inform LMF of the requested model input (or the distribution of model input)

Case 5. NW monitoring two-sided model

The NW-side monitoring performs towards output-CSI-NW and the realistic channel estimation associated with the CSI report from UE. Since the output is generated by NW with no specification impact. However, it requires for realistic CSI measurement report, even when the reconstructed CSI can be obtained at NW through the AI model. One raises that the realistic CSI can be substituted by UL-CSI measured via SRS based on implementation. On top of that, NW needs to update UE with the monitoring result, for the purpose that UE could do the same operation on the paired model.

Proposal 8: The signaling procedure of NW monitoring two-sided model may include:

- NW may request UE about the ground truth label, e.g., realistic CSI report

- UE may report NW about the ground truth label

- 2.2	Model control

The control of model activation/ deactivation/ switch/ fallback can be performed by UE or NW respectively. We would like to identify the possibility for the application of each cases in regard to the model type. UE is able to control UE-sided model in a functionality-based LCM. On the other hand, NW is suitable for the control of UE/NW/two-sided model in consideration of its competence. Further combining the model monitoring entity, we make the following observation from the perspective of the entity to control model activation/ deactivation/ switch/ fallback:

Observation: The following two scenarios should be considered for model management:

- Scenario 1: UE may perform model management in UE monitoring UE-sided model

- Scenario 2: NW may perform model management in other monitoring types

On the basis of Observation, we further investigate who to initiate the control operation.

Scenario 1

In this scenario, UE itself monitors the model performance and take decisions on control of model activation/ deactivation/ switch/ fallback among the models that it maintains. This can be a total UE independent implementation. Another realization is that NW can configure UE with the trigger event for model activation/ deactivation/ switch/ fallback without report to NW.

Scenario 2

For Case1/2:

UE-initiated operation applies to UE monitoring UE/two-sided model. In this sense, NW can configure UE with the threshold of performance metric to report NW. Besides, with or without the trigger condition, UE can initiate a request for control operation to NW.

For Case 3-5:

As for NW monitoring, the control is inherently made by NW.

In summary, we thus propose:

Proposal 9: UE can decide the control operation in the case of UE monitoring UE-sided model based on UE implementation.

Proposal 10: NW can decide the control operation in the case of:

- UE monitoring in the way that:

- NW may configure UE threshold to report performance metric;

- UE may request NW to control with or without a trigger event

- NW monitoring based on NW implementation.

In regard to the signaling about model activation/ deactivation/ switch/ fallback, the management configuration can be achieved via RRC/LPP messages similar to other use-case specific configurations. As for the operation feedback in response to UE’s request, it can be time-sensitive in some degree. We thus believe RRC/LPP and MAC (at least for CSI compression and beam management) can be considered to transmit a fast request/provision of model operation decision.

Proposal 11: Model management configuration can be transferred via RRC/LPP messages; model management decision can be transferred via RRC/LPP and MAC (at least study for CSI compression and beam management) signaling.

- Conclusion

Based on the above discussion the following observations are made,

Observation: The following two scenarios should be considered for model management:

- Scenario 1: UE may perform model management in UE monitoring UE-sided model

- Scenario 2: NW may perform model management in other monitoring types

We hence propose,

Proposal 1: Model monitoring is performed by the entity which obtains the monitoring performance metrics, and the metrics will be further forwarded to the management entity.

Proposal 2: Regarding the model deployment type, model monitoring can be classified as:

- UE monitoring, can apply to UE-sided model and two-sided model

- NW monitoring, can apply to UE-sided, NW-sided model and two-sided model

Proposal 3: UE monitoring UE-sided model can be based on UE implementation.

Proposal 4: The procedure of UE monitoring two-sided model may include:

- UE may request NW about the NW-sided model output

- NW may inform UE of the requested NW-sided model output

Proposal 5: The procedure of NW monitoring NW-sided model may include:

- NW may request UE about the ground truth label

- UE may inform NW of the requested ground truth label

Proposal 5a: The procedure of LMF monitoring gNB-sided model may include:

- For output-based monitoring:

- LMF may request gNB about the ground truth label

- gNB may inform LMF of the ground truth label

- For input-based monitoring:

- LMF may request gNB about the model input (or the distribution of model input)

- gNB may inform LMF of the model input (or the distribution of model input)

Proposal 6: When NW monitors the UE-sided model based on the model output, UE should provide the calculated performance metric instead of the model output and label, to reduce the signaling overhead.

Proposal 7: The procedure of NW monitoring UE-sided model may include:

- NW may request UE about the performance metric

- UE may inform NW of the requested performance metric

Proposal 7a: The procedure of LMF monitoring UE-sided model may include:

- LMF may request UE about the model input (or the distribution of model input)

- UE may inform LMF of the requested model input (or the distribution of model input)

Proposal 8: The signaling procedure of NW monitoring two-sided model may include:

- NW may request UE about the ground truth label, e.g., realistic CSI report

- UE may report NW about the ground truth label

Proposal 9: UE can decide the control operation in the case of UE monitoring UE-sided model based on UE implementation.

Proposal 10: NW can decide the control operation in the case of:

- UE monitoring in the way that:

- NW may configure UE threshold to report performance metric;

- UE may request NW to control with or without a trigger event

- NW monitoring based on NW implementation.

Proposal 11: Model management configuration can be transferred via RRC/LPP messages; model management decision can be transferred via RRC/LPP and MAC (at least study for CSI compression and beam management) signaling.

- Reference

- RAN1 Session notes for 9.2 (Study on AI/ ML for NR air interface) (CMCC)

- 3GPP TS 23.288	Architecture enhancements for 5G System (5GS) to support network data analytics services

- R2-2308286	Report of [Post122][060][AIML] Mapping of functions to physical entities (CMCC)