3GPP TSG-RAN WG2 Meeting #123bis                            R2-2309706

Xiamen, China, 9 – 13 October, 2023

Source:        CATT

Title:	        Consideration on DSR and BSR

Agenda Item:	7.5.4.1

Document for:	Discussion and Decision

- Introduction

In the RAN2#123 meeting, the agreements are provided as follows [1]:

- DSR Network can configure the UE whether to trigger delay status reporting. FFS if we have some thresholds per LCG. When UE triggers reporting delay information for a LCG, and UE also reports the buffer status associated with the remaining time. RAN2 aims to define a single MAC CE for the DSR reporting (including the buffer status). FFS if this extends BSR MAC CE or is a new MAC CE. Continue offline on MAC CE design options (modified BSR or new MAC CE) Many companies think single value per LCG is sufficient. Some companies think scheduler needs more information. Continue offline in the MAC CE discussion on how many values would be reported and when. Continue discussion on whether we have threshold value for the DSR or not. Working assumption: Define a new separate MAC CE for DSR (remaining delay and associated data volume) reporting, e.g. DSR reporting is not coupled with BSR reporting. Detailed Definition of associated data volume is FFS. Support threshold based DSR reporting, e.g. DSR reporting is triggered when remaining delay of a PDU/PDU set is below a NW configured threshold. The threshold is configured per LCG. FFS whether configuring multiple thresholds for a LCG is supported. Definition of remaining time is FFS. - BSR RAN2 needs to discuss the BSR table definition in the next meeting based on company inputs.

In the contribution, we will discuss the remaining issues of BSR and DSR.

- Discussion

2.1 DSR

2.1.1 Remaining time

Definition of remaining time

In RAN2#122 meeting, it was agreed:

1: UE calculates the remaining time based on the PDCP discard timer value. FFS if UE reports one or multiple values. FFS how this is modelled in PDCP specification. FFS which UEs support this.

In order to leave full flexibility to implementers, the PDCP specification does not specify if the discard timer runs upwards (starting from 0 up to its configured maximum value) or downwards (starting from its configured maximum value down to 0). Hence the discard timer value cannot be used directly to define unambiguously the remaining time (hence the wording “based on” in above agreement). However the intention and purpose of the DSR since day 1 was to provide a quick snapshot to the gNB of the time left to UE’s critical XR data before its delay budget expiry. Hence, the above agreement should be interpreted as the reported value of the discard timer assuming a timer model where the timer runs downwards. We propose clarifying this.

Proposal 1: The remaining time is the value of the running discardTimer in PDCP, assuming a timer model where the timer runs downwards.

Whether one or more values of remaining time are reported for an LCG

Some companies support that DSR would report, for a given LCG, not only the most critical delay before the PDB/PSDB deadline, but also further delay information associated with buffered data that is not (yet) in critical situation, latency-wise. Main argument is to provide full visibility to the gNB of the latency situation of all buffered data so as to allow for some specific delay-aware UL scheduler. However, it was already agreed that such DSR is triggered based on a threshold on the remaining time, hence it is indeed the delay critical data that triggers the DSR and not other data. In other words, the DSR only gives a partial picture on the latency situation of the other data. Moreover such approach can be beneficial to some types of UL schedulers but is not needed for others. For example, as a baseline, gNB should first allocate UL grants for UEs according to the priority and QoS requirements when receiving BSR reports from different UEs. Only when the remaining time is short enough, gNB should prioritize the associated UE over other UEs within the same priority. Hence, we are reluctant in adding complexity to the specification that would only be driven by some specific network implementations. It should be further noted that DSR only provides the amount of the data to serve urgently, while the BSR provides all data amount. Thus, when gNB treats both DSR and BSR reports, it can derive the amount of data not to be considered as urgent, which somehow addresses (at least partly) the intention of the proposals to report multiple DSR values.

On the other hand, reporting multiple values will occupy more UL resources and then decrease the resource efficiency. The worst case is that oversize DSR may delay data transmission.

Hence, one value for the shortest remaining time is enough.

Proposal 2: Support reporting only one value of remaining time for an LCG.

2.1.2 Number of thresholds for an LCG

Whether configuring multiple remaining time thresholds for an LCG is supported for DSR

We understand this proposal as some kind of “double warning” to the gNB: a 1st one with a somehow relaxed remaining time threshold just warning gNB that this LCG is not yet in a danger zone but needs be watched carefully. It is expected that gNB would schedule the UE/LCG to switch-off the warning. If not, a 2nd warning would further inform the gNB this LCG has now entered the “danger zone”. If the network configures two remaining time thresholds for an LCG, UE may then report two data volumes of remaining time smaller than these two thresholds respectively, that is, UE will report multiple DSR for an LCG. We believe this generates unnecessary UL overhead and, similar to the discussion in section 2.1.1, seems to address one particular gNB scheduler implementation. Hence we prefer to keep this tool generic while simple and the threshold for an LCG should be configured only once.

Proposal 3: Support configuring only one DSR triggering threshold for an LCG.

2.1.3 Format

In previous meetings, it was discussed whether the delay status report (DSR) should be included in the legacy BSR MAC CE or in a separate MAC CE. As stated in the working assumption, when we define a new separate MAC CE for DSR reporting, the data amount that is reported in the DSR is associated with the latency that triggered the DSR, i.e. it only represents some specific data buffered in the UE. Hence it is different and complementary to the legacy BSR MAC CE. Furthermore, designing new DSR MAC CE from scratch provides the opportunity for a compact design to minimize the associated overhead. Therefore, another benefit of a separate MAC CE is that it can be multiplexed in MAC PDU easily considering its small size. Hence we propose to confirm the working assumption:

Proposal 4: Confirm the working assumption on separate MAC CE for DSR, which includes remaining delay and associated data volume.

A new LCG ID should be used to indicate the DSR MAC CE to differentiate from other kinds of BSR MAC CEs.

Regarding the MAC CE format details, our proposal is to keep this new MAC CE compact and small and to follow the designs of Figure 1 and 2. Specifically, when one LCG triggers the report, the DSR MAC CE adopts Figure 1.When multiple LCGs trigger the DSR report, the DSR MAC CE adopts either Figure 2(a) or Figure 2(b). The UE selects one of the two based on the number of reported LCGs. DSR MAC CE with two LCG ID uses Figure 2 (a), and DSR MAC CE with four LCG ID uses figure 2 (b).

Figure 1 Short DSR MAC CE

Figure 2 Long DSR MAC CE

Proposal 5: Define DSR for one LCG as Figure 1. If DSR of multiple LCGs can be reported, adopt either Figure 2(a) or Figure 2(b).

How to calculate data volume for DSR i.e. the buffer status associated with the remaining time (Data volume calculation)

When determine which data volume associated with the remaining time, we can consider whether pdu-SetDiscard is configured or not.

When pdu-SetDiscard is not configured, the first SDU which remaining time reaches the threshold triggers a DSR. Since for that LCG, this triggering SDU is the earliest SDU in the buffer, the associated data amount would be this SDU only (or all SDUs that arrived simultaneously with the triggering SDU).

Moreover, in RAN2#122, it was agreed:

When/if UE reports remaining time, the reference time for the remaining time is determined from the point of the first transmission of the information. FFS if intra-UE prioritization can impact this.

Therefore, later SDUs could be added in the data amount reported in the DSR if they also reach the threshold while the DSR is pending and not transmitted. Finally, the buffer status computation method should reuse the legacy computation of BSR in PDCP specification clause 5.6 [2] and RLC specification clause 5.5 [3].

Proposal 6: If pdu-SetDiscard is not configured, the data amount reported in the DSR is calculated based on the legacy data volume calculation methods in PDCP and RLC, but limited to the SDUs which PDCP discard timer (assuming a timer model running downwards) is below or equal to the DSR triggering threshold at the time the DSR is included in the MAC PDU.

When pdu-SetDiscard is configured, considering a PDU Set should be transmitted entirely or discarded entirely, if DSR is triggered by any SDU of a PDU Set, the reported associated data amount should account for the data of the SDU that reached the threshold as well as the later SDUs from the same PDU set, even if their individual remaining time did not yet reach the DSR triggering threshold. And, same as above, while DSR is pending and not transmitted, SDUs from other PDU sets may also reach the threshold, in which case, the associated data amount should also count those PDU sets.

Proposal 7: If pdu-SetDiscard is configured, the data amount reported in the DSR is calculated based on the legacy data volume calculation methods in PDCP and RLC, but limited to the SDUs of the PDU Sets any SDU of which has its PDCP discard timer (assuming a timer model running downwards) below or equal to the DSR triggering threshold at the time the DSR is included in the MAC PDU.

2.1.4 DSR trigger and cancellation

DSR trigger

First, when data needs to be reported, its remaining time must be equal to or below the threshold. Only when the remaining time is short enough, gNB should prioritize the associated UE over other UEs within the same priority.

However, not all data with remaining time equal to or below the threshold should trigger DSR. For example, if the SDU associated with the discard timer has been discarded, the corresponding data does not need to be reported. Also, even if the data has a short remaining time, but a known upcoming UL grant (either a recent dynamic grant or a configured grant) can accommodate its transmission within the remaining time, the UE does not trigger the DSR. Moreover, an SDU which discard timer gets equal to or below the threshold should not trigger a DSR if a DSR is already pending for the same LCG and has not been transmitted yet. Indeed, since the associated data volume reported in the DSR reflects the latency critical data at the time the DSR is transmitted, the new triggering SDU will necessarily be included in the data volume calculation of the initial DSR, when transmitted. Hence, it does not need to be triggered.

Therefore, if DSR should be triggered, some conditions need to be met. The remaining timer is equal to or lower than a configured time threshold, and the SDU associated with the discard timer has not been discarded, and no upcoming UL grant can be used and no DSR has already been triggered for this LCG, which is still pending.

Proposal 8: DSR should be triggered when

- the remaining time is equal to or lower than a configured time threshold , and

- the SDU associated with the discard timer has not been discarded, and

- no upcoming UL grant can be used before the discard timer deadline, and

- no DSR has already been triggered for this LCG which is still pending

DSR cancellation

Similar to the BSR, once a DSR is triggered, it is pending until it is cancelled. We need to agree on which conditions apply for cancelling a DSR. Clearly, similar to the BSR, a triggered DSR should be cancelled when a MAC PDU is transmitted and this PDU includes this DSR. Moreover, another condition for cancelling a pending DSR is when the triggering SDU has been discarded, since in that case, it is too late and useless to send the DSR.

Proposal 9: A triggered DSR should be cancelled when:

- a MAC PDU is transmitted and this PDU includes this DSR, or

- the triggering SDU has been discarded

2.2 BSR

2.2.1 New BSR table

Entries in the new BSR table

In Table 1, the consideration of the value ranges of BSR table mentioned by other companies at the last meeting have been quoted, the Bmax and Bmin are mentioned as follow[4][5][6][7][8][9][10][11][12].

|  | QC7078 | Honor7156 | ZTE7532 | FW7762 | CATT7348 | vivo7297 | CMCC8310 | NEC8372 | LG8875 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Bmin | 53,054(based on exponential distribution) | 141723 | 117409 | 0 | 75598 or 37850 | 44289 | 68K | 94482 | 10.4K |
| Bmax | 781,250(150Mbps/8/24 ) | 2500000 (bitrate = 200Mbps, frame rate = 15 fps) | 16777216 | 260095(has a linear quantization step size of 1024 bytes) | 1,270,912 or 635,456 | 781250 | 341K | 1666578 | 125K |

As can be seen, the Bmax gap mentioned by each company is too large to get a convergent value. On that note, neither SA2 nor SA4 provides the potential data size range of XR services. Since RAN2 do not have expertise in codec, RAN2 should request SA4 to provide reference data for buffer sizes.

Observation: It is difficult to get a convergent Bmin and Bmax based on XR service characteristics in RAN2.

Proposal 10: RAN2 discuss how to get the convergent value of Bmin and Bmax. The recommended reference data for buffer sizes from SA4 may be needed.

2.2.2 How to report

The application of the new BSR table should be configured by gNB. To simplify the BSR configuration and report, when the new BSR table is configured, it can be used for any LCG. That is, UE does not need to identify which LCG(s) can use the new BSR table and which LCG(s) cannot.

Proposal 11: When gNB configures the new BSR table to the UE, the new BSR table can be applied to any LCG for which the data volume is in the scope of new BSR table.

Whether the legacy BSR MAC CE or a new BSR MAC is used to support the use of new BSR table

When the buffered data volume is out of the scope of new BSR table, legacy BSR table should be used. The BSR MAC CE using new BSR table can be distinguished from the BSR MAC CE using legacy BSR table by (e)LCID value.

When the LCGs in the UE buffer need to use different BSR tables, two BSR MAC CEs should be reported and the two BSR MAC CEs can be in one MAC PDU.

Note the design for new BSR table is applicable to the BSR MAC CE containing DSR described in section 2.1 too.

Proposal 12: New (e)LCID(s) should be used to indicate the BSR MAC CE(s) using the new BSR table.

Proposal 13: UE selects BSR table based on the BS values of LCGs.

- If the BS values of all LCGs are in the value range of the new BSR table, one BSR MAC CE with the new BSR table is reported.

- If the BS values of all LCGs are out of the value range of the new BSR table, one BSR MAC CE with the legacy BSR table is reported.

- If part BS values of all LCGs are out of the value range of the new BSR table, two BSR MAC CEs can be reported in one MAC PDU.

- Conclusion

According to the analysis in section 2, it is proposed:

Proposal 1: The remaining time is the value of the running discardTimer in PDCP, assuming a timer model where the timer runs downwards.

Proposal 2: Support reporting only one value of remaining time for an LCG.

Proposal 3: Support configuring only one DSR triggering threshold for an LCG.

Proposal 4: Confirm the working assumption on separate MAC CE for DSR, which includes remaining delay and associated data volume.

Proposal 5: Define DSR for one LCG as Figure 1. If DSR of multiple LCGs can be reported, adopt either Figure 2(a) or Figure 2(b).

Proposal 6: If pdu-SetDiscard is not configured, the data amount reported in the DSR is calculated based on the legacy data volume calculation methods in PDCP and RLC, but limited to the SDUs which PDCP discard timer (assuming a timer model running downwards) is below or equal to the DSR triggering threshold at the time the DSR is included in the MAC PDU.

Proposal 7: If pdu-SetDiscard is configured, the data amount reported in the DSR is calculated based on the legacy data volume calculation methods in PDCP and RLC, but limited to the SDUs of the PDU Sets any SDU of which has its PDCP discard timer (assuming a timer model running downwards) below or equal to the DSR triggering threshold at the time the DSR is included in the MAC PDU.

Proposal 8: DSR should be triggered when

- the remaining time is equal to or lower than a configured time threshold , and

- the SDU associated with the discard timer has not been discarded, and

- no upcoming UL grant can be used before the discard timer deadline, and

- no DSR has already been triggered for this LCG which is still pending

Proposal 9: A triggered DSR should be cancelled when:

- a MAC PDU is transmitted and this PDU includes this DSR, or

- the triggering SDU has been discarded

Observation: It is difficult to get a convergent Bmin and Bmax based on XR service characteristics in RAN2.

Proposal 10: RAN2 discuss how to get the convergent value of Bmin and Bmax. The recommended reference data for buffer sizes from SA4 may be needed.

Proposal 11: When gNB configures the new BSR table to the UE, the new BSR table can be applied to any LCG for which the data volume is in the scope of new BSR table.

Proposal 12: New (e)LCID(s) should be used to indicate the BSR MAC CE(s) using the new BSR table.

Proposal 13: UE selects BSR table based on the BS values of LCGs.

- If the BS values of all LCGs are in the value range of the new BSR table, one BSR MAC CE with the new BSR table is reported.

- If the BS values of all LCGs are out of the value range of the new BSR table, one BSR MAC CE with the legacy BSR table is reported.

- If part BS values of all LCGs are out of the value range of the new BSR table, two BSR MAC CEs can be reported in one MAC PDU.

- References

- RAN2#123 Chair notes;

- 3GPP TS 38.323 V17.1.0 (2022-06) Packet Data Convergence Protocol (PDCP) specification;

- 3GPP TS 38.322 V17.1.0 (2022-06) Radio Link Control (RLC) protocol specification;

- R2-2307078 BSR and delay status reporting for XR  Qualcomm Incorporated;

- R2-2307156 Discussion on BSR enhancements for XR Honor;

- R2-2307532 BSR enhancements for XR  ZTE Corporation, Sanechips;

- R2-2307762 Discussions on new Buffer Status table design for XR  Futurewei;

- R2-2307348 Consideration on DSR and BSR CATT;

- R2-2307297 Discussion on BSR enhancements for XR vivo;

- R2-2308310 Consideration on BSR enhancement for XR CMCC;

- R2-2308372 New static BS table and BSR trigger(s) NEC;

- R2- 2308875 Discussion on new BSR table and delay status report LG Electronics Inc.