3GPP TSG-RAN WG2 Meeting #123bis	R2-2309870

Xiamen, China, 9 - 13 Oct, 2023

Title: 	Rapporteur proposals to address open issues in MAC running CRs (open issue list)

Source: 	Huawei, HiSilicon

Agenda item:	7.4.2.2

Document Type:	Discussion and Decision

- Open issue list

Following are the open issue list before RAN2#123-bis meeting.

1.1 Early TA acquisition

| Topics | Issue description | Rapporteur suggestion/proposal |
| --- | --- | --- |
| Power ramping | How to handle the power ramping counter, when the candidate cell, indicated in the PDCCH order indicating retransmission, is same (but with different SSB) from that indicated in the last PDCCH order? This is already implemented in the endorsed running CR. | Proposal 1: RAN2 confirm that power ramping counter is neither reset nor increased, when the candidate cell, indicated in the PDCCH order indicating retransmission, is same (but with different SSB) from that indicated in the last PDCCH order. |
| UE based TA measurement | How does the UE apply the TA value based on UE TA measurement? | Rapporteur think no MAC CE information is needed (i.e. RRC configuration and UE capability signalling are sufficient ) Proposal 2: If UE is configured by RRC to perform UE based TA measurement, UE applies the measured TA value and performs RACH-less LTM, upon LTM cell switch. |
|  | How to determine RACH-less LTM for the UE based TA measurement case? |  |

1.2 RACH-less cell switch

| Topics | Issue description | Rapporteur suggestion/proposal |
| --- | --- | --- |
| CG for RACH-less LTM | Whether UE/target cell is allowed to use both DG and CG for RACH-less LTM? | Proposal 3: No specification impact/restriction on the UE to use both DG and CG for RACH-less LTM. |
|  | Whether the CG configured for RACH-less LTM can be used for subsequent UL data transmission, after the successful transmission of first UL data (if not released by NW)? If yes, how to select the CG occasion for subsequent CG transmission (e.g. using any CG occasion or based on the indicated SSB)? | Proposal 4: RAN2 to define the UE behaviour on the R18 CG for RACH-less LTM, if it is not released by NW after LTM completion (to close one EN in MAC running CR): - Option 1: UE skips/stops using those CG; - Option 2: UE can continue use those CG. FFS UE can use any CG occasion (no need to further consider the SSB and CG occasion association). |
|  | Whether UE considers RACH-less LTM failure, if the configuredGrantTimer expires and if a PDCCH addressed to the MAC entity's C-RNTI has not been received after initial transmission (similar as in CG-SDT)? | Proposal 5: RAN2 to select either way (to close one EN in MAC running CR): whether the UE considers RACH-less LTM failure, if the configuredGrantTimer expires before LTM completion/T304 expiry. |
| LTM completion | “RAN2 assumes For RACH-less LTM, the UE determines successful reception of its first UL data based on receiving a PDCCH addressing the UE’s C-RNTI in the target cell scheduling a new transmission after the first UL data” Does the agreement includes the DL assignment for “new transmission”? This is already implemented in the endorsed running CR. | Proposal 6: RAN2 agree that it can be either DL assignment or UL grant for the “new transmission”, in the last meeting agreement: “For RACH-less LTM, the UE determines successful reception of its first UL data based on receiving a PDCCH addressing the UE’s C-RNTI in the target cell scheduling a new transmission after the first UL data.” |
|  | “FFS if specified contents should be transmitted with this transmission, e.g. as LTE MAC CE” Any further issue to address for LTM completion and FFS LTE MAC CE is always include? | Proposal 7: Not to revert the agreement, i.e. the LTE MAC CE is not introduced/not always included. |
| MAC reset | How to capture the trigger of MAC reset in RACH-less LTM? This is already implemented in the endorsed running CR. | Proposal 8: In RACH-less LTM, the MAC reset operation is triggered by MAC layer before applying the TA value of target cell (as captured in the running CR) [Still pending on RRC running CR]. |

1.3 LTM MAC CE format

| Topics | Issue description | Rapporteur suggestion/proposal |
| --- | --- | --- |
| CFRA information | FFS on the detailed CFRA resource related information in LTM MAC CE. | Proposal 9: As to the CFRA resource related information in LTM MAC CE, RAN2 to select either option: - Option 1: The information similar to those in the legacy PDCCH order triggered RACH (e.g. preamble index. etc.); - Option 2: one bit indication about the CFRA resource availability; |
|  | FFS CFRA resource related information in LTM MAC CE is supported in inter-DU. | Proposal 10: CFRA resource related information in LTM MAC CE is limited to only intra-DU case. |
| TA value/indication | “When the UE shall apply the same TA value as the source (already agreed) FFS how the UE knows this. ” FFS on whether and how is the UE aware of the “same TA value as the source” case? | Proposal 11: As for providing the TA for “same TA value as source” case, RAN2 to select either option: - Option 1: Implicit way by providing the TA value; - Option 2: Explicit 1bit flag. |
|  | The presence of TA value field, by considering the “same as source” and “UE based TA measurement” cases. | TBD, after RAN2 concludes other issues. |
| TCI state ID | Whether the TCI state ID should be provided in RACH-less LTM? | Proposal 12: The presence of TCI state field and TA value field are decoupled in the LTM cell switch MAC CE. Proposal 13: In RACH-less LTM, TCI state field should be provided in the LTM cell switch MAC CE, i.e. UE uses the beam indicated by the NW in RACH-less LTM. |
|  | Whether the TCI state ID should be provided in RACH-based LTM? If yes, FFS on the UE behaviours on the SSB used for RO selection of RACH (CBRA and CFRA case)? | Proposal 14a: In case TCI state field is included in RACH-based LTM, if further agreed by RAN1 (e.g. to activate the TCI state), RAN2 to discuss whether this is the SSB used for RO selection during RACH： Options could be considered for CBRA: - Option 1a: UE performs SSB selection for RACH based on RSRP as legacy; - Option 2a: UE uses the SSB (indicated by TCI state field in MAC CE) for RACH. Options could be considered for CFRA: - Option 1b: UE performs SSB selection for RACH based on RSRP as legacy; - Option 2b: UE uses the indicated SSB for RACH (FFS the one indicated by TCI state field or the SSB index if agreed as CFRA related information in MAC CE). |

- Stage-3/CR implementation open issue list

Following are the minor CR implementation open issue list before RAN2#123-bis meeting.

| Topics | Issue description | Rapporteur suggestion/proposal |
| --- | --- | --- |
| Cross-layer inter-action | Editor’s note: FFS on the need of RRC layer indication on the time when the target configuration has been applied, so that MAC layer can start to process the later fields e.g. TA/TCI state. FFS on the need of indication to RRC layer that those fields has been applied in MAC, so that RRC can submit the RRCReconfigurationComplete message for transmission. | See the running CR update. |
|  | Editor’s note: FFS on the need of MAC indicating to RRC to trigger/skip the Random Access procedure for this LTM cell switch (since RA will not be triggered if the TA is valid when sending the RRCReconfigurationComplete message). | See the running CR update. |
| Minor clarification | Editor’s note: The information other than Target Configuration ID in this MAC CE (e.g. Timing Advance Command, the TCI state ID) refers to the (target) RRC configuration after the UE has applied the candidate configuration indicated by Target Configuration ID. It does not refer to the (source) RRC configuration in use upon reception of this MAC CE. FFS how to capture this clarification on UE behaviours. | See the running CR update. |

- Conclusion

In this paper, rapporteur propose following proposals for the MAC open issues:

Early TA acquisition

Power ramping

Proposal 1: RAN2 confirm that power ramping counter is neither reset nor increased, when the candidate cell, indicated in the PDCCH order indicating retransmission, is same (but with different SSB) from that indicated in the last PDCCH order.

UE based TA measurement

Proposal 2: If UE is configured by RRC to perform UE based TA measurement, UE applies the measured TA value and performs RACH-less LTM, upon LTM cell switch.

RACH-less cell switch

CG for RACH-less LTM

Proposal 3: No specification impact/restriction on the UE to use both DG and CG for RACH-less LTM.

Proposal 4: RAN2 to define the UE behaviour on the R18 CG for RACH-less LTM, if it is not released by NW after LTM completion (to close one EN in MAC running CR):

- Option 1: UE skips/stops using those CG;

- Option 2: UE can continue use those CG. FFS UE can use any CG occasion (no need to further consider the SSB and CG occasion association).

Proposal 5: RAN2 to select either way (to close one EN in MAC running CR): whether the UE considers RACH-less LTM failure, if the configuredGrantTimer expires before LTM completion/T304 expiry.

LTM completion

Proposal 6: RAN2 agree that it can be either DL assignment or UL grant for the “new transmission”, in the last meeting agreement: “For RACH-less LTM, the UE determines successful reception of its first UL data based on receiving a PDCCH addressing the UE’s C-RNTI in the target cell scheduling a new transmission after the first UL data.”

Proposal 7: Not to revert the agreement, i.e. the LTE MAC CE is not introduced/not always included.

MAC reset

Proposal 8: In RACH-less LTM, the MAC reset operation is triggered by MAC layer before applying the TA value of target cell (as captured in the running CR). [Still pending on RRC running CR].

LTM MAC CE format

CFRA information

Proposal 9: As to the CFRA resource related information in LTM MAC CE, RAN2 to select either option:

- Option 1: The information similar to those in the legacy PDCCH order triggered RACH (e.g. preamble index. etc.);

- Option 2: one bit indication about the CFRA resource availability;

Proposal 10: CFRA resource related information in LTM MAC CE is limited to only intra-DU case.

TA value/indication

Proposal 11: As for providing the TA for “same TA value as source” case, RAN2 to select either option:

- Option 1: Implicit way by providing the TA value;

- Option 2: Explicit 1bit flag.

TCI state ID

Proposal 12: The presence of TCI state field and TA value field are decoupled in the LTM cell switch MAC CE.

Proposal 13: In RACH-less LTM, TCI state field should be provided in the LTM cell switch MAC CE, i.e. UE uses the beam indicated by the NW in RACH-less LTM.

Proposal 14a: In case TCI state field is included in RACH-based LTM, if further agreed by RAN1 (e.g. to activate the TCI state), RAN2 to discuss whether this is the SSB used for RO selection during RACH：

Options could be considered for CBRA:

- Option 1a: UE performs SSB selection for RACH based on RSRP as legacy;

- Option 2a: UE uses the SSB (indicated by TCI state field in MAC CE) for RACH.

Options could be considered for CFRA:

- Option 1b: UE performs SSB selection for RACH based on RSRP as legacy;

- Option 2b: UE uses the indicated SSB for RACH (FFS the one indicated by TCI state field or the SSB index if agreed as CFRA related information in MAC CE).

- Reference