3GPP TSG-RAN WG2 Meeting #123bis                                R2-2309919

Xiamen, China, 9th October - 13th October, 2023

Agenda item:		6.4.1 (NR Positioning enhancements – Stage 3 correction)

Source:				Samsung

Title:					Issue on dl-prs-ResourceSetPeriodicityReq-r17

Document for:		Discussion & Decision

1	Introduction

In Rel-17, On-demand-PRS feature was introduced to allow the LMF to control whether PRS is transmitted or not and to change the characteristics of an ongoing PRS transmission. The on-demand PRS procedure can be initiated either by the UE for LMF.

In case of UE-initiated On-Demand PRS, the UE sends an On-Demand PRS request to the LMF via LPP Request Assistance Data message. The On-Demand PRS request can be a request for a pre-defined PRS configuration indicated with pre-defined PRS configuration ID or explicit parameter for PRS configuration. For the explicit parameters, DL PRS periodicity, resource bandwidth, and QCL information are included in the request (NR-On-Demand-DL-PRS-Information IE) with the current LPP [1].

Among the parameters in the request from UE, dl-prs-ResourceSetPeriodicityReq-r17 indicates the requested DL PRS periodicity in unit of slot. However, we found some problem on how the LMF interprets the requested periodicity correctly. In this paper, we further discuss what the problem is and propose how to handle this.

2	Discussion

In the latest LPP [1], dl-prs-ResourceSetPeriodicityReq-r17 indicates the requested DL PRS periodicity in unit of slot as below:

NR-On-Demand-DL-PRS-PerFreqLayer-r17 ::= SEQUENCE {

dl-prs-FrequencyRangeReq-r17			ENUMERATED { fr1, fr2, ...},

dl-prs-ResourceSetPeriodicityReq-r17	ENUMERATED { p4, p5, p8, p10, p16, p20, p32, p40,

p64, p80, p160, p320, p640, p1280, p2560,

p5120, p10240, p20480, p40960, p81920, ...}

OPTIONAL,

dl-prs-ResourceBandwidthReq-r17			INTEGER (1..63)							OPTIONAL,

dl-prs-ResourceRepetitionFactorReq-r17	ENUMERATED {n2, n4, n6, n8, n16, n32, ...}

OPTIONAL,

dl-prs-NumSymbolsReq-r17				ENUMERATED {n2, n4, n6, n12, ...}		OPTIONAL,

dl-prs-CombSizeN-Req-r17				ENUMERATED {n2, n4, n6, n12, ...}		OPTIONAL,

dl-prs-QCL-InformationReqTRPlist-r17	DL-PRS-QCL-InformationReqTRPlist-r17	OPTIONAL,

...

}

NR-On-Demand-DL-PRS-Information field descriptions

dl-prs-FrequencyRangeReq This field specifies the frequency range for which the on-demand DL-PRS is requested.

dl-prs-ResourceSetPeriodicityReq This field specifies the requested periodicity of the DL-PRS Resource Set in slots. The periodicity depends on the subcarrier spacing (SCS) and takes values 2μ4, 5, 8, 10, 16, 20, 32, 40, 64, 80, 160, 320, 640, 1280, 2560, 5120, 10240 slots, where μ=0, 1, 2, 3 for SCS of 15, 30, 60 and 120 kHz respectively. μ refers to the target devices current primary cell.

Since dl-prs-ResourceSetPeriodicityReq-r17 indicates the requested periodicity in unit of slots, the periodicity in absolute time (e.g., xy msec) depends on the considered SCS values as described in the field description. For example, let’s assume that the UE sets dl-prs-ResourceSetPeriodicityReq-r17 as 32 slots. Then, the requested periodicity can be interpreted differently as 32, 16, 8, or 4 msec depending on the referred SCS value (e.g., 15, 30, 60, 120 kHz). To handle this ambiguity, the field description specifies that μ (SCS value) refers to the target device’s current primary cell.

The field dl-prs-ResourceSetPeriodicityReq-r17 indicates the requested periodicity of DL-PRS in unit of slots. Thus, the requested periodicity (in absolute time, e.g., xy msec) can be interpreted differently depending on the referred μ (SCS value).

The current field description of dl-prs-ResourceSetPeriodicityReq-r17 specifies that the μ (SCS value) refers to the target device’s current primary cell.

From UE perspective, if the target UE has some preferred value of DL-PRS periodicity, it can set the value of dl-prs-ResourceSetPeriodicityReq-r17 in slots considering its PCell’s SCS as the reference SCS. Then, the information is delivered to the LMF via LPP Request Assistance data and then forwarded also to the serving or non-serving gNBs/TRPs via NRPPa PRS Configuration Request. However, from LMF/(non-serving)gNB/TRP perspective, they can not interpret the requested periodicity correctly, since there is no way for them to know the SCS (μ) of target UE’s Pcell with the current specification.

The LMF/(non-serving)gNB/TRP can not correctly interpret the periodicity requested by UE using dl-prs-ResourceSetPeriodicityReq-r17 since there is no way for them to know the SCS (μ) of target UE’s Pcell with the current specification.

Meanwhile, during the offline discussion 	[AT123][421] at the last meeting, one company comments that the numerology of DL-PRS should be decoupled with that of target UE’s PCell and we also share the comment. In our view, the numerology (i.e., SCS) of DL-PRS should be determined by the gNB/TRP anyway and there is no reason why the SCS of target UE’s Pcell should be used as the reference SCS for the requested periodicity. Moreover, there can be multiple BWPs using different numerology (SCS) within the Pcell, which makes it even more awkward to use SCS of target UE’s Pcell as the reference SCS.

Considering the discussion above, one simple solution to resolve the problem is to allow the UE to explicitly indicate the reference SCS (μ) for dl-prs-ResourceSetPeriodicityReq-r17. For that, a new field (e.g., dl-prs-ReferenceSCS-v17xy) can be introduced in NR-On-Demand-DL-PRS-PerFreqLayer-r17 IE as below.

NR-On-Demand-DL-PRS-PerFreqLayer-r17 ::= SEQUENCE {

dl-prs-FrequencyRangeReq-r17			ENUMERATED { fr1, fr2, ...},

dl-prs-ResourceSetPeriodicityReq-r17	ENUMERATED { p4, p5, p8, p10, p16, p20, p32, p40,

p64, p80, p160, p320, p640, p1280, p2560,

p5120, p10240, p20480, p40960, p81920, ...}

OPTIONAL,

dl-prs-ResourceBandwidthReq-r17			INTEGER (1..63)							OPTIONAL,

dl-prs-ResourceRepetitionFactorReq-r17	ENUMERATED {n2, n4, n6, n8, n16, n32, ...}

OPTIONAL,

dl-prs-NumSymbolsReq-r17				ENUMERATED {n2, n4, n6, n12, ...}		OPTIONAL,

dl-prs-CombSizeN-Req-r17				ENUMERATED {n2, n4, n6, n12, ...}		OPTIONAL,

dl-prs-QCL-InformationReqTRPlist-r17	DL-PRS-QCL-InformationReqTRPlist-r17	OPTIONAL,

...

[[

dl-prs-ReferenceSCS-v17xy				ENUMERATED {kHz15, kHz30, kHz60, kHz120, ...}	OPTIONAL

]]

}

NR-On-Demand-DL-PRS-Information field descriptions

dl-prs-FrequencyRangeReq This field specifies the frequency range for which the on-demand DL-PRS is requested.

dl-prs-ResourceSetPeriodicityReq This field specifies the requested periodicity of the DL-PRS Resource Set in slots. The periodicity depends on the subcarrier spacing (SCS) and takes values 2μ4, 5, 8, 10, 16, 20, 32, 40, 64, 80, 160, 320, 640, 1280, 2560, 5120, 10240 slots, where μ=0, 1, 2, 3 for SCS of 15, 30, 60 and 120 kHz respectively. μ refers to dl-prs-ReferenceSCS-v17xy.

…

dl-prs-ReferenceSCS This field specifies the reference SCS for dl-prs-ResourceSetPeriodicityReq-r17.

Please, note that the new field indicates neither the SCS of the target UE’s Pcell nor the DL PRS SCS requested by UE, but it just indicates the reference SCS (μ) for the requested periodicity (i.e., dl-prs-ResourceSetPeriodicityReq-r17).

Based on the discussion above, we would like to propose the following.

Agree with the CR R2-2309920 to allow the UE to explicitly indicate the reference SCS for dl-prs-ResourceSetPeriodicityReq-r17.

3	Conclusion

Based on the above, RAN2 is requested to discuss and agree on the following proposals:

- The field dl-prs-ResourceSetPeriodicityReq-r17 indicates the requested periodicity of DL-PRS in unit of slots. Thus, the requested periodicity (in absolute time, e.g., xy msec) can be interpreted differently depending on the referred μ (SCS value).

The current field description of dl-prs-ResourceSetPeriodicityReq-r17 specifies that the μ (SCS value) refers to the target device’s current primary cell.

The LMF/(non-serving)gNB/TRP can not correctly interpret the periodicity requested by UE using dl-prs-ResourceSetPeriodicityReq-r17 since there is no way for them to know the SCS (μ) of target UE’s Pcell with the current specification.

- Agree with the CR R2-2309920 to allow the UE to explicitly indicate the reference SCS for dl-prs-ResourceSetPeriodicityReq-r17.

4	Reference

- 3GPP TS 37.355 v17.5.0, “LTE Positioning Protocol (LPP)”.