3GPP TSG-RAN WG2 Meeting #123bis	R2-2309982

Xiamen, China, October 9-14, 2023

Source: 			Qualcomm Incorporated

Title: 	Summary of email discussion Post123][043][NR17] UE caps Maximum aggregated bandwidth (Qualcomm)

Document for: 	Decision

Agenda Item: 	6.1.3.2

# Introduction

This document provides a summary of the following email discussion and proposes a way forward.

[Post123][043][NR17] UE caps Maximum aggregated bandwidth (Qualcomm)

Scope: Based on agreements, continue the discussion on Max Aggregated BW for FR1 and FR2, address the FFS on how/if to take into account MIMO layers. converge on CRs (as far as possible)

Intended outcome: Report, Agreeable CRs, for next meeting

Deadline: Long

Note the this email discussion was meant to continue the discussion based on the agreements in RAN2#123 below.

FR1 inter-band CA - RAN2 considers implementation scenarios 1 and 3 below in future discussion. - 1. Legacy UE/NW not supporting BCS5 - 2. UE/NW supporting BCS5, but not the maximum aggregated BW signalling - 3. UE/NW supporting BCS5 and the maximum aggregated BW signalling - On top of the solution currently discussed, further discuss signalling solution according to the following principles. - Introduce a new CC BW UE capability at FSPC level, which is only applicable to BCS5. - For BCS5, NW ignores the existing supportedBandwidth in FSPC but look at the new CC BW capability and the maximum aggregated BW. - UE populates the existing supportedBandwidth in FSPC only for the purpose of legacy BCS. - Introduce new “Total aggregated BW” UE capability signalled per band combination, including FDD+TDD - FFS whether to introduce “Total number of MIMO layers” signalled per band combination. FR2 FBG5 - Agree that RAN2 considers implementation scenarios 1 and 3 below in future discussion. - 1 Legacy UE/NW not supporting FBG5 - 2 UE/NW supporting FBG5, but not the maximum aggregated BW signalling - 3 UE/NW supporting FBG5 and the maximum aggregated BW signalling - Introduce new “Total aggregated BW” UE capability signalled per band per Band Combination, - On top of the solution currently discussed, further discuss signalling solution according to the following principle. - FFS whether to introduce “Total number of MIMO layers” signalled per band per band combination.

# Summary

The email discussion discussed two alternative signalling approaches and the alternative with the following principle seemed agreeable.

- Signal the “total/maximum aggregated BW capability parameters” in the band combination list.

- Introduce new maximum CC BW parameter as an extension within feature set per CC. This means the UE can signal the legacy CC BW and the new CC BW in a single feature set per CC entry.

- Specify that the new maximum CC BW above must always be used together with the additional total/maximum aggregated BW signaled in band combination list.

There was no consensus on the introduction of “Total number of MIMO layers” per band combination (for FR1 inter-band CA) and per band per band combination (for FR2 intra-band CA with FBG5 BW class).

For FR1 inter-band CA, questions were raised on the applicability of new UE capability signalling in NR-DC, EN-DC and NE-DC scenarios. It was pointed out that some inter-node coordination may be necessary if it is supported in NR-DC. No conclusion due to lack of time. Moderator however found the following text in TR38.862.

### 5.5.2 Guidelines for band combination with BCS4/BCS5 The following are the rules for applying BCS4/BCS5 for band combination request: - BCS4/BCS5 apply to SUL, NR CA, NR DC and SUL and/or NR CA part of inter-band MR-DC while it does not apply to intra-band MR DC. […]

The email discussion did not discuss the following points, due to lack of time.

- UE capability parameter values for “total/maximum aggregated BW”s.

- AOB, e.g. UE capability filtering.

# Proposed way forward

Based on the discussion, email discussion moderator proposes the following.

Proposal 1:	Endorse the running CRs in [1] and [2] capturing UE capability signalling solution with the following principles.

- Signal the “total/maximum aggregated BW capability parameters” in the band combination list.

- Introduce a new maximum CC BW parameter as an extension within feature set per CC. This means the UE can signal the legacy CC BW and the new CC BW in a single feature set per CC entry.

- Specify that the new maximum CC BW above must always be used together with the additional total/maximum aggregated BW signaled in band combination list.

Proposal 2:	RAN2 is requested to discuss whether to introduce the following UE capability parameters.

- “Total number of MIMO layers” per band combination for FR1 inter-band CA

- “Total number of MIMO layers” per band per band combination for FR2 intra-band CA with FBG5 BW class

Proposal 3:	For FR1 inter-band CA, RAN2 is requested to discuss the applicability of the new UE capability signalling in NR-DC, EN-DC and NE-DC.

Proposal 4:	RAN2 is requested to discuss the following points based on company contributions.

- UE capability parameter values for “total/maximum aggregated BW”s.

- AOB, e.g. UE capability filtering.

# Reference

[1]	R2-2309983	CR to 38.331 on Introduction of maximum aggregated bandwidth for FR1 inter-band CA and for FR2 intra-band CA		Qualcomm Incorporated

[2]	R2-2309984	CR to 38.306 on Introduction of maximum aggregated bandwidth for FR1 inter-band CA and for FR2 intra-band CA		Qualcomm Incorporated