3GPP TSG-RAN WG2 Meeting #123bis	R2-2310334

Xiamen, China, 9 – 13 October 2023

Agenda item:	7.5.4.3

Source:	Apple

Title:	UTO for Multi-PUSCH Configured Grant

WID/SID:	NR_XR_enh-Core

Document for:	Discussion / Decision

# Introduction

According to the revised WID for NR XR [1], RAN2 should address the following objectives to specify the enhancements for capacity improvement:

- Multiple Configured Grant (CG) PUSCH transmission occasions in a period of a single CG PUSCH configuration (RAN1, RAN2); - Dynamic indication of unused CG PUSCH occasion(s) based on Uplink Control Information (UCI) by the UE (RAN1, RAN2); - Buffer Status Report (BSR) enhancements including at least new Buffer Status Table(s) (RAN2); - Delay reporting of buffered data in uplink (RAN2); - Discard operation of PDU Sets for DL and UL (RAN2, RAN3);

RAN1 has already started discussing multi-PUSCH CG and made several agreements relating to resource allocation, HARQ PID determination, and UTO-UCI design etc. Essentially, HARQ PID determination is only applicable to “valid” CG PUSCHs, and RAN2 has asked RAN1 to specify the definitions about whether a transmission occasion can be considered as valid or invalid.

In addition to HARQ PID determination, we have also identified some other RAN1 agreements that may have some impacts to MAC operations:

Agreement - When a CG PUSCH occasion is indicated as “unused”, the UE is not allowed to transmit CG PUSCH on that CG PUSCH occasion. - For any other CG PUSCH occasion that is NOT indicated as “unused”, the UE is allowed to transmit or not to transmit CG PUSCH on that CG PUSCH occasion as per legacy specification. - No RAN1 specification impact Agreement - A CG PUSCH occasion indicated as “unused” earlier, is not allowed to be indicated as “NOT unused later”. - A CG PUSCH occasion indicated as “NOT unused” earlier, can be indicated as “unused” later. - FFS: Whether there is specification impact Agreement The UTO-UCI indication for a CG configuration is applicable to only valid CG PUSCH TOs, if any. - Note: A configured CG PUSCH is invalid if the CG PUSCH is dropped due to collision with DL symbol(s) indicated by tdd-UL-DL-ConfigurationCommon or tdd-UL-DL-ConfigurationDedicated or SSB. Otherwise, it is valid. Agreement Indication of UTO-UCI by CG PUSCHs associated to a CG configuration, is enabled by configuration of an RRC parameter. - FFS on whether/how to extend to multiple CG configurations Agreement - Configure the RRC parameter Nu (Nu is the size of bit-map) - FFS range value of Nu - UTO_offset is the offset value. - Alt-1: UTO_Offset is provided by configuration. - FFS range value of UTO_offset - Alt-2: UTO_Offset = 0 - A transmitted CG PUSCH, carries UTO-UCI that is applicable to the Nu consecutive and valid CG PUSCH TOs, starting with UTO_offset from the end of the transmitted CG PUSCH. Agreement For a multi-PUSCH CG configuration, the range value of the higher layer parameter indicating number of consecutive slots (N in previous agreements) is: - Max value=16 or 32 - Up to UE capability - Min value=2 Agreement For a CG configuration with UTO-UCI indication enabled: - For the range value for the RRC parameter Nu (Nu is the size of bit-map): (3, …, 8) Conclusion There is no consensus to introduce RRC parameter UTO_offset. This over-rides earlier RAN1 agreements.

This paper aims to provide some of our views on how UTO-UCI should be derived, and when the UE should check if any update of previously signaled UTO-UCI is needed.

# Discussions

## UTO Determination

The key motivation of the Rel-18 WI objectives relating to configured grant (CG) with multiple PUSCH resources in one CG period is to handle potentially large packets. In particular, although XR traffic is typically periodic and hence the CG is a suitable way for resource allocation, there may be shortcomings when dealing with XR traffic of varying packet size. Since the TBS for every CG occasion is fixed, and it could be too small to fit a large packet in one CG period – it means the UE may need additional resources to complete the delivery of this packet (or PDU Set), which causes potential latency. On the other hand, one could configure a large TBS in the CG to ensure it can accommodate the largest possible packet of this XR traffic in every period, but it may be over-provisioning as a lot of pre-allocated resources are not needed and wasted. With multiple PUSCH per CG cycle, the UL resource can be used by the UE more flexibly. When the packet size is large, the UE may fully exploit all the PUSCHs. When the packet size is relatively small, the UE can only use a fraction of resources in one CG cycle. Moreover, when the PUSCHs in one CG cycle are not fully used, some indication can be provided for the gNB to know which resources are not used, and therefore the gNB could allocate such resources to other UEs to improve the system capacity. Currently, RAN1 has confirmed that such indication is called “unused transmission occasion UCI” or UTO-UCI, which is a new type of UCI which can be multiplexed into PUSCH, akin to CG-UCI. Since MAC has a better visibility of the amount of buffered data, we think UTO should be determined by MAC. Once the UTO information is determined at MAC, it can further deliver the determined UTO information PHY for UTO-UCI signaling.

Proposal 1: RAN2 should confirm that MAC can determine the information of unused transmission occasions (UTO), and provide it to PHY for UTO-UCI signaling.

Apparently, the UTO determination is highly relevant to the amount of data that is currently in the uplink buffer, as the UE may decide how many PUSCHs it needs to transmit the buffered data. Nevertheless, it is also possible for the UE to use more resources than what is needed for the data that is already in the buffer. Since the UE should send the UTO-UCI relating to the subsequent Nu (where Nu = 3…8) CG occasions, the expected/predicted changes of buffer status within the time interval of these Nu CG occasions should be considered.

More specifically, we think the UE may determine UTO based on the several factors, such as:

- The amount of data that is already in the buffer (e.g. the buffer of LCHs that are allowed to use this CG)

- The amount of data that the UE expects to arrive in the buffer within the time interval covering the subsequent Nu CG occasions.

- The amount of data that the UE expects to discard from the buffer within the time interval covering the subsequent Nu CG occasions.

- The MAC CEs that the UE expects to trigger within the interval covering the subsequent Nu CG occasions.

Since it is difficult to specify the UE behaviors involving predictions about how the buffer status will be changed, or what MAC CEs will be triggered in the near future, we tend to think UTO-UCI derivation should be left to UE implementation. One can argue that, for simplicity we could just specify UTO-UCI derivation based on the existing buffer status (i.e. the UE does not take any “prediction” or “expectation” into account). However, RAN1 has agreed that a CG resource indicated as “unused” cannot be changed to “used” later, so it is not too flexible if the UE changes its mind subsequently in cases it tends to use more resources than what it has indicated earlier.

Proposal 2: It is up to UE implementation on how to determine/update UTO-UCI based on factors such as the current buffer status, expected changes of the buffer status, and/or potential UL MAC CEs to be triggered.

Having said that, we think some rules should be defined in the specifications to make sure the UTO-UCI is determined in a sensible way. In particular, this does not make sense for the UE to still indicate a PUSCH as “used”, if we already know that such PUSCH resource cannot be used anyway due to certain reasons. In particular, if the configuredGrantTimer associating to the HARQ PID of a PUSCH in a multi-PUSCH CG would be running when the PUSCH is to be processed or transmitted, the UE should first exclude this PUSCH before deriving the UTO. Consider an example where the UE requires 3 out of 4 CG occasions for UL transmission, and one of upcoming four CG occasions cannot be used as the configuredGrantTimer associating to its HARQ PID will be running. The examples without and with consideration of configuredGrantTimer status for UTO derivations are illustrated in Figure 1 and Figure 2 respectively. In these examples, the UTO-UCI to be derived is bitmap that indicate the used/unused state for Nu=4 subsequent valid CG occasions that respectively have HARQ PID = 0, 1, 2, and 3. Also, we assume that the CG timer associating to HARQ PID = 2 would be running (due to a dynamic grant scheduled earlier with the conflicting HARQ PID) at the PUSCH timing of the CG occasion with HARQ PID = 2, so in such case the MAC entity will not deliver this CG PUSCH to the HARQ entity for MAC PDU generation.

Figure 1 An illustration of UTO-UCI derivation without considering the status of Configured Grant Timer.

Figure 2 An illustration of UTO-UCI derivation that first excludes CG PUSCHs that will be blocked by Configured Grant Timer.

In Figure 1, the UE derives UTO-UCI without considering the fact that the CG PUSCH with HARQ PID = 2 cannot be used due to the running the configuredGrantTimer, and thereby wrongly indentify/indicate the CG PUSCH with HARQ PID = 3 as “unused” by assuming that all data can be transmitted by the first three CG PUSCHs. Note that once a CG occasion indicated as “unused”, the UE can no longer use this resource. Consequently, the UE can only use 2 out of the 4 CG occasions, and therefore unable to transmit all buffered data within the interval of these 4 CG occasions – which results in longer latency and resource wastage. On the other hand, in Figure 2 we show the cases where the UE derives UTO-UCI by first excluding the CG PUSCH with HARQ PID = 2 that will be blocked by the configuredGrantTimer, and so it is able to correctly identify the other 3 CG occasions as “used” to accommodate the buffered data. Hence, all the data can be transmitted within the interval consisting these 4 CG occasions without any unnecessary delay.

Therefore, before deriving the UTO-UCI, the UE should first identify (based on their HARQ PIDs) and exclude the CG resources that will be blocked by the running configuredGrantTimer. Moreover, such CG resources should be indicated as “unused” in the UTO-UCI always.

Proposal 3: When deriving the UTO-UCI bitmap corresponding to 8Nu3 CG PUSCHs, the UE should first identify and exclude the CG PUSCHs that cannot be used because the configuredGrantTimer associating to their HARQ PIDs would be running. These CG resources should be indicated as “unused” in the UTO-UCI.

## UTO Update

According to RAN1 agreement, the UE is able to “update” the UTO information. In particular, a PUSCH that has been indicated as “used” earlier could be indicated as “unused” later, but a PUSCH that has been indicated as “unused” earlier cannot be changed to “used”. This leads to an open question for RAN2: When should the UE MAC check if an UTO update is needed ?

It came to our attention that, RAN1 has agreed a “sliding window” approach for UTO-UCI signaling, which means when UTO-UCI is signalled on a PUSCH, it always indicates the UTO information corresponding the subsequent Nu valid CG occasions. With such a framework, essentially each UTO-UCI bitmap comprises information for two subsets of valid CG occasions, namely:

- A subset of X valid CG occasions whose used/unused state have already been indicated in one or more previously signalled UTO-UCI, and

- A subset of Y valid CG occasions whose used/unused state have not been indicated in any of the previously signalled UTO-UCI.

Note that X + Y = Nu.  Obviously, the UE will always need to derive the used/unused state for the Y valid CG occasions whenever a UTO-UCI is to be signalled. However, we are not sure if the UE should also try to check if any of the X valid CG occasions need to be updated jointly.

In our view, there are three options:

- Option 1: The UE checks if an update is needed for the X valid CG occasions whenever a UTO-UCI is to be signaled.

- Option 2: The UE checks if an update is needed for the X valid CG occasions when certain event(s) have occurred.

- Option 3: Leave it to UE implementation.

Option 1 requires the UE to check if an update is needed whenever it needs to transmit the UTO-UCI, which ensures that the gNB can always get the latest UTO information. Nonetheless, since UTO-UCI should be included in every valid CG PUSCH, it means the UE needs to check whether an update is needed for the X valid CG occasions before every CG PUSCH occasion that can be transmitted, which may result in a heavier burden for the UE. This is not desirable in terms of UE complexity.

Option 2 seems to be a better compromise, as the UE would only check if the UTO update is needed for the X valid CG occasions when specific triggering events have occurred. Based on RAN1 agreements, UTO update is solely relating to reducing (rather than increasing) the CG resource that the UE intends to use, and therefore the events should be mainly relevant to the situations where the uplink buffer size experiences a disruptive decrease. For examples, when the buffered packets (e.g. PDU Sets) are discarded, or when a certain amount of data is transmitted using another resource (such as a dynamic grant). This option guarantees that the UTO to be updated in time for the gNB to perform some resource optimization (if needed), while minimizing the UE complexity for UTO re-evaluation.

Option 3 has no specification impact, and it makes sense if Proposal 2 can be agreed.  Thus, we think RAN2 can also leave it to UE implementation.

Proposal 4: RAN2 can discuss if we can leave checking for the need of UTO update to UE implementation, or if some triggering conditions should be specified.

# Conclusions

This contribution discusses some of our views on UTO for Multi-PUSCH CG. We have proposed the following:

Proposal 1: RAN2 should confirm that MAC can determine the information of unused transmission occasions (UTO), and provide it to PHY for UTO-UCI signaling.

Proposal 2: It is up to UE implementation on how to determine/update UTO-UCI based on factors such as the current buffer status, expected changes of the buffer status, and/or potential UL MAC CEs to be triggered.

Proposal 3: When deriving the UTO-UCI bitmap corresponding to 8Nu3 CG PUSCHs, the UE should first identify and exclude the CG PUSCHs that cannot be used because the configuredGrantTimer associating to their HARQ PIDs would be running. These CG resources should be indicated as “unused” in the UTO-UCI.

Proposal 4: RAN2 can discuss if we can leave checking for the need of UTO update to UE implementation, or if some triggering conditions should be specified.

# References

- RP-230786, Updated WID on XR Enhancements for NR, Nokia, Qualcomm, 3GPP TSG RAN #99, March 2023, Rotterdam, Netherlands.