3GPP TSG-RAN WG2 #123bis                                                                                  R2-2310362

Xiamen, China, 9th Oct. – 13th Aug. 2023

Agenda Item:	    7.25.1

Source: 	    MediaTek Inc.

Title:		    Discussion on further measurement gap enhancement

Document for:	    Discussion and decision

# 1 Introduction

In the last few meetings, RAN2 has discussed further measurement gap enhancement and reached the following agreements.

In RAN2#121bis

- In the current R2 discussion/CRs there is no intention to change legacy definitions or behaviour (It is understood that there may be difference of opinions).

R2-2304199	Report of [AT121bis-e][023][MGE] Measurements without gap with interruption	MediaTek Inc.

- [023] Noted, agreements reflected below

- [023] Introduce UE capability and indication for the Rel-18 case where interruption is needed for NR SSB based measurement without gap. The UE reports Rel-18 indication only if network requests it.

- The Rel-18 indication (e.g. NeedForInterruptionInfoNR) can be included in in RRCReconfigurationComplete and RRCResumeComplete message.

- The Rel-18 indication is in addition to the legacy NeedForGaps information. The UE may report 3 different cases:

--- If gap is needed, the UE reports “gap” in Rel-16 field and empty field in corresponding R18 IE.

---- If gap is NOT needed and there is no interruption, the UE reports “no-gap” in Rel-16 field and “no-gap-no-interruption” in Rel-18 field

---- If gap is NOT needed but there is interruption, the UE reports “no-gap” in Rel-16 field and “no-gap-with-interruption” in Rel-18 field

- If the NW does not request Rel-18 NeedForInterruptionInfoNR, the UE only reports NeedForGaps in the legacy way.

- [023] RAN2 understands that no need to extend the concept of “no-gap measurement with interruption” to Rel-17 NeedForGapNCSG reporting. RAN2 assumes "nogap-noncsg" in Rel-17 NeedForGapNCSG signalling implies "no gap and no interruptions".

- [023] For inter-RAT NR measurement in LTE, introduce a new UE indication (e.g. interRAT-NeedForInterruptionNR-r18) to indicate whether interruption is needed (no-gap-with-interruption) or not (no-gap-no-interruption) when UE reports FALSE (i.e. no gap) in interRAT-NeedForGapsNR-r16.

- [023] No need to send Reply LS for R2-2302431 in this meeting.

In RAN2#122

- R2 signalling will allow full backwards compatibility (details may be up to RAN4)

R2-2306802	Introduction of measurements without gap with interruption	MediaTek Inc., Huawei, HiSilicon	CR	Rel-18	36.331	17.4.0	4929	2	B	NR_MG_enh2-Core	R2-2304432

R2-2306803	Introduction of measurements without gap with interruption	MediaTek Inc., Huawei, HiSilicon	CR	Rel-18	36.306	17.4.0	1870	2	B	NR_MG_enh2-Core	R2-2304433

R2-2306804	Introduction of measurements without gap with interruption	MediaTek Inc., Huawei, HiSilicon	CR	Rel-18	38.331	17.4.0	4063	2	B	NR_MG_enh2-Core	R2-2304434

R2-2306805	Introduction of measurements without gap with interruption	MediaTek Inc., Huawei, HiSilicon	CR	Rel-18	38.306	17.4.0	0906	2	B	NR_MG_enh2-Core	R2-2304435

- 4 CRs above are endorsed, for the purpose of informing RAN4 by LS

R2-2306876 	[Draft] Reply LS on measurements without gap	Mediatek Inc.	LS out	Rel-18	NR_MG_enh2-Core	To:RAN4

- LS is approved in R2-2306905 but then revised in R2-2306919 (attachments not listed in the liaison header part)

R2-2306919 Reply LS on measurements without gap RAN2 LS out Rel-18 NR_MG_enh2-Core To:RAN4

=> Approved

In RAN2#123

- RAN2 introduce new terminology of effective measurement window and new configuration MeasWindowConfig for the effective measurement window, which includes WindowOffset, WindowDuration, and WindowPeriodicity.

- The effective measurement window is a per-UE configuration (can revisit if need is found).

- Other values of configuration for effective measurement window are pending on RAN4 further discussion.

- RAN2 to clarify that value nogap-noncsg in the NeedForNCSG-InfoEUTRA-r17 means no gap without interruption in the Rel-17 and Rel-18 specifications. (this is a rel-17 change, and separate CR is needed, can be provided next meeting).

RAN4 has sent replied LS R2-2309459 for RAN2 question in LS R2-2306919. In this paper, we would like to discuss the impact of RAN2 running CR based on incoming LS.

# 2 Discussion

In RAN2 LS R2-2306919 to RAN4, RAN2 asked RAN4 whether interruption is allowed according to Rel-16 RAN4 specifications when UE reports “no gap” in Rel-16 field for LTE inter-RAT measurement to NR. In the replied LS R2-2309459, RAN4 indicated that there is no define requirement for Rel-16 no-gap feature (i.e. not clear on whether interruption is needed).

RAN4 achieved the following conclusion during the RAN4 #108 meeting. - RAN4 did not define requirements for “no-gap” feature in Rel-16. - When UE reports “no-gap” in Rel-16 field for LTE inter-RAT measurement to NR, the UE behavior is undefined from RAN4 perspective (i.e., RAN4 does not specify whether interruptions are allowed or not). - The new features introduced in Rel-18 are not directly related to “no-gap” Rel-16 feature.

The reason for RAN2 to ask RAN4 above question is because it is unclear whether the UE reported “no-gap-with-interruption” in R18 shall report “gap” in R16. RAN2 has an editor note in the endorsed LTE running CR saying that “It is FFS whether UE indicating “no-gap-with-interruption” shall indicate TRUE for the corresponding NR band entry in the interRAT-NeedForGapsNR.” With the clarification from RAN4 that the new features introduced in Rel-18 are not directly related to “no-gap” Rel-16 feature, we think that it is clear that the EN could be removed and there is no need to add reporting relationship between R16 filed and R18 field.

Proposal 1: Remove below editor note in endorsed LTE running CR (R2-2306802) for MGE; and endorse the update LTE running CR in R2-2310393 (38.331) and R2-2310395 (38.306).

- Editor Note: It is FFS whether UE indicating “no-gap-with-interruption” shall indicate TRUE for the corresponding NR band entry in the interRAT-NeedForGapsNR.

For the NR part, the endorsed NR running CRs should be updated according to the agreement from last meeting.

- RAN2 introduce new terminology of effective measurement window and new configuration MeasWindowConfig for the effective measurement window, which includes WindowOffset, WindowDuration, and WindowPeriodicity.

- The effective measurement window is a per-UE configuration (can revisit if need is found).

- Other values of configuration for effective measurement window are pending on RAN4 further discussion.

Furthermore, the NR capability part should be separated from procedure part based on RAN2 procedure to handle NR UE capability.

Therefore, we updated the NR running CR with below two main changes

- Add effective measurement window

- Divide 38.331 running CR into capability part and procedure part.

The updated CRs are provided in R2-2310397 (38.331 procedure), R2-2310403 (38.331 capability), and R2-2310404 (38.306). We assume that it is endorsable.

Proposal 2: Endorse the update NR running CR in R2-2310397 (38.331 procedure), R2-2310403 (38.331 capability), and R2-2310404 (38.306).

# 3 Conclusions

Based on the discussion in section 2, we propose the following:

Proposal 1: Remove below editor note in endorsed LTE running CR (R2-2306802) for MGE; and endorse the update LTE running CR in R2-2310393 (38.331) and R2-2310395 (38.306).

- Editor Note: It is FFS whether UE indicating “no-gap-with-interruption” shall indicate TRUE for the corresponding NR band entry in the interRAT-NeedForGapsNR.

Proposal 2: Endorse the update NR running CR in R2-2310397 (38.331 procedure), R2-2310403 (38.331 capability), and R2-2310404 (38.306).

# 4 References

[1] R2-2309459, “Reply LS on measurements without gaps”, RAN4