3GPP TSG-RAN WG2 Meeting #123-bis                                                          R2-2310626

Xiamen China, October 9 th – 13 th, 2023

Agenda item:	7.7.4.1.1

Source: 	FGI

Title:                     Discussion on TN Measurement Relaxation Issues

WID/SID:                   NR_NTN_enh (Release 18)

Document for:	Discussion and Decision

Introduction

In RAN2#123 meeting [1], it was agreed that TN coverage configurations would be composed by TN coverage Area IDs and associated (NR/E-UTRA) frequency list, which would be delivered via SIB4 and SIB5 respectively. In addition, the association between the TN coverage information and TN coverage area ID would be transmitted via a new SIB, which is defined as a ‘SIBX’ in this paper for the convenience of discussion.

Agreements: - Both of the NR TN coverage and EUTRA TN coverage can be provided. - We introduce a new SIB to provide the TN coverage information - A TN coverage area configuration is associated with a TN coverage Area ID. The frequency information for TN coverage area is indicated by adding TN coverage area IDs in SIB4 and SIB5.

In addition, during RAN2#121-bis-e meeting [2], RAN2 WG also agreed the acquired TN area information would remain valid until the next system information update of the SIB including TN coverage info.

However, based on the agreements above, it may create a misalignment issue while SIB4/5 and the new SIBX are updated by the serving (NTN) cell closely in time domain (e.g., during the same System information (SI) modification period).

In addition, to increase the power saving gain of UEs in RRC_IDLE/INACTIVE UEs, we further suggest RAN2 WG to further discuss the following issues:

- whether the TN measurement relaxation can be applicable for UEs staying in the ‘camped on any cell state’ (e.g., while the serving NTN cell is an acceptable cell to the UE) and/or for UEs staying in the ‘any cell selection state’.

- Whether UE need to delete the stored SIBX, which contains TN coverage area information, after 3 hours from the moment it was successfully confirmed as valid.

Discussion

2.1 Misalignment of TN Coverage Area Configuration Update

Based on the agreements on RAN2#123 meeting [1] & RAN2#121-bis-e meeting [2], there would be a corner issue for TN coverage configuration update procedure while SIB4/SIB5 is updated jointly with SIBX, which includes the TN coverage area information & TN coverage ID.

Fig. 1 Misalignment Issue while SIB4/SIB5 is updated with SIBX

As shown in Fig. 1, the serving cell may broadcast SI changes indicators during the 1st SI Modification Period (e.g., SI  Modification Period#1 as shown in Fig. 1) to inform one or more UEs (e.g., UEs in RRC Inactive/Idle/Connected state) that the serving cell would update one or more SIBs in the next SI Modification Period (e.g., SI Modification Period#2 as shown in Fig. 1) via short message transmission. Then, in the SI Modification Period#2, the serving cell would inform which SIB(s) would be updated by modifying the valuetag(s) of those SIB(s). During the SI Modification Period#2, UEs would update system information based on the valuetag(s) and SI scheduling information transmitted by the serving cell via broadcasting SIB1(s).

Then, in some cases, the serving cell may update both SIB4 and SIBX during the same SI modification period, as shown in Fig. 1. Based on the agreement in RAN2#121-bis-e meeting [2], the acquired TN area coverage information would remain valid until the next system information update of the SIB including TN coverage info.

Agreements online: - The discussion on how to indicate the frequency information for each TN coverage area should be combined with the discussion on which SIB will be used to indicate the TN coverage area, possibly based on evaluation of the signalling overhead - The acquired TN area coverage information remains valid until the next system information update of the SIB including TN coverage info

As shown in Fig. 1, after the UE updates SIB4 successfully at T1, the UE would modify its implementations on TN measurement relaxation based the updated SIB4 and stored SIBX, which may create misalignment of TN coverage area configuration update during the gap period between T1 and T2. This misalignment  may trigger the UE to modify TN measurement relaxation procedure based on erroneous TN coverage area information while the stored SIBX is still considered as valid before T2. The same problem would also happen while both SIB5 and SIBX are going to be updated during same SI modification period.

To avoid the misalignment issue of TN coverage area update, we suggest that, in the cases that the serving cell is going to update both SIB4 and SIBX (e.g., during the same SI modification period), the previously acquired SIB4 and SIBX related to TN measurement relaxation should remain valid until the UE has updated both SIB4 and SIBX successfully. Similarly, in the cases that the serving cell is going to update both SIB5 and SIBX (e.g., during the same SI modification period), the previously acquired SIB5 and SIBX related to TN measurement relaxation should remain valid until the UE has updated both SIB5 and SIBX successfully.

Proposal#1 The acquired SIB4 and SIBX for TN measurement relaxation procedure remains valid until the next system information update of both SIB4 and SIBX jointly while the UE knows both SIB4 and SIBX are going to be updated.

Proposal#2 The acquired SIB5 and SIBX for TN measurement relaxation procedure remains valid until the next system information update of both SIB5 and SIBX jointly while the UE knows both SIB5 and SIBX are going to be updated.

To continue, also based on the observations in Fig. 1, it would be observed that the UE may update the TN coverage area information/TN coverage area configuration (via SIBX reception) and SIB4/SIB5 at different timings. For example, UE may (re)decide intra-frequency TN measurement relaxation procedure by receiving SIBX alone. In comparison, UE may (re)decide inter-frequency TN measurement relaxation procedure after receiving both SIB4 & SIBX successfully. Furthermore, UE may (re)decide inter-RAT frequency TN measurement relaxation procedure after receiving both SIB5 & SIBX successfully. So, RAN2 WG is suggested to confirm that the UE would implement the following procedures: a) intra-frequency TN measurement relaxation procedure; b) inter-frequency TN measurement relaxation procedure; and c) inter-RAT TN measurement relaxation procedure individually.

Proposal#3 RAN2 WG to confirm that UE would implement following procedures individually:

  - intra-frequency TN measurement relaxation procedure,

  - inter-frequency TN measurement relaxation procedure,

  - inter-RAT TN measurement relaxation procedure.

2.2 Sub-states of RRC_IDLE/INACTIVE states

One of the objectives of R-18 NR NTN enhancements is to discuss cell reselection enhancements for RRC_IDLE/INACTIVE UEs to reduce UE power consumption. However, by referring to 3GPP TS 38.304 [4], there are three different sub-states for UEs in RRC_IDLE and RRC_INACTIVE state:

- Camped Normally state

Camped normally state is applicable for RRC_IDLE and RRC_INACTIVE state. During previous RAN2 WG discussions, it seems that RAN2 WG was discussing TN measurement relaxation procedure based on an assumption that UE is staying in the camped normally state. However, there is no agreement to specify whether TN measurement relaxation procedure (which includes UE implementations about TN coverage area information reception) is limited to UEs staying in camped normally state and RAN2 WG is suggested to clarify this point.

Proposal#4 RAN2 WG to clarify whether TN measurement relaxation procedure is limited to UEs staying in ‘camped normally state’.

- Camped on Any Cell state

Camped on any cell state is only applicable for RRC_IDLE state. A UE under camped on any cell state shall perform the following tasks[4]:

- monitor relevant system information;

- perform necessary measurement for the cell reselection evaluation procedure;

- regularly attempt to find a suitable cell by trying all frequency bands of all RATs supported by the UE.

Fig. 2 UE tasks while staying in camped on any cell state[4]

Therefore, in the NTN-TN mobility scenario, it may happen that a UE camps on an acceptable NTN cell while the UE could not find out any suitable (TN) cell. In addition, the acceptable NTN cell may further broadcast SIB4/SIB5 and SIBX for TN measurement relaxation. Based on the scenario above, some questions need to be discussed in RAN2 WG:

Q1: should a UE consider SIB4/SIB5/SIBX as ‘relevent system information’ to a UE in camped on any cell state?

Q2: should a UE decide what is ‘necessary measurement for the cell reselection evaluation’ by applying TN measurement relaxation procedure?

Q3: should the UE decide the regularity of the suitable cell searching attempt by applying TN measurement relaxation procedure?

By considering the potential power saving gain of TN measurement relaxation, we would suggest RAN2 WG to discuss whether TN measurement relaxation procedure is applicable for UEs staying in the camped on any cell state.

Proposal#5 RAN2 WG to discuss whether TN measurement relaxation procedure is applicable for UEs staying in ‘camped on any cell state’.

- Any Cell Selection state

Any cell selection state is applicable for RRC_IDLE and RRC_INACTIVE state. Based on [4], a UE which is not camped on any cell yet should staying in this state. While staying in any cell selection state, the UE should perform cell selection process to find a suitable cell via a complete scan of all RATs and all frequency bands supported by the UE. If no suitable cell is found after the thorough search, then the UE would try to search for any acceptable cell instead.

Fig. 3 UE tasks while staying in any cell selection state[4]

By referring to NTN-TN mobility scenario, it may also happen that a UE may move to a network coverage hole without any support from NTN/TN. In this condition, the UE may move to ‘any cell selection state’ with the following potential questions:

Q4: Should a UE staying in any cell selection state implement TN measurement relaxation procedure based on stored SIB4/SIB5/SIBX, which are received by the UE from previous serving NTN cell?

Q5: Should a UE staying in any cell selection state keep the stored TN coverage area information?

Also based on the potential power saving gain of TN measurement relaxation, we would suggest RAN2 WG to discuss whether TN measurement relaxation procedure is applicable for UEs staying in the any cell selection state.

Proposal#6 RAN2 WG to discuss whether TN measurement relaxation procedure is applicable for UEs staying in ‘any cell selection state’.

2.3 Regularity of SIBX deletion

Based on [3], a UE has to delete any stored version of a SIB after 3 hours from the moment it was successfully confirmed as valid. One of the reason to create such a life cycle for SIB is to make sure that the RAN could reuse the valuetag to indicate the update of a specific SIB without considering an wrap around error happens. However, to TN coverage area information, network vendors may expect such TN coverage information may be quite stable under the coverage of an NTN cell. If this expectation is true, then it may create un-necessary UE power consumption to delete and receive the same SIBX from its serving NTN cell again. We suggest RAN2 WG to discuss whether UE need to delete the stored SIBX after 3 hours from the moment it was successfully confirmed as valid.

Proposal#7 RAN2 WG to discuss whether UE need to delete the stored SIBX, which contains TN coverage area information, after 3 hours from the moment it was successfully confirmed as valid.

Conclusion

In this paper, we have discussed three issues for TN measurement relaxation:

- Misalignment of TN Coverage Area Configuration Update

A misalignment issue for TN coverage area configuration update may happen while SIB4/SIB5 would be updated jointly with SIBX (as shown in Fig. 1). To solve this issue, both Proposal#1 and Proposal#2 are provided respectively:

Proposal#1 The acquired SIB4 and SIBX for TN measurement relaxation procedure remains valid until the next system information update of both SIB4 and SIBX jointly while the UE knows both SIB4 and SIBX are going to be updated.

Proposal#2 The acquired SIB5 and SIBX for TN measurement relaxation procedure remains valid until the next system information update of both SIB5 and SIBX jointly while the UE knows both SIB5 and SIBX are going to be updated.

In addition, since the TN coverage area information for intra-frequency TN measurement relaxation/inter-frequency TN measurement relaxation/inter-RAT measurement relaxation would be provided by serving cell via different set of SIBs, we also proposed RAN2 WG to confirm that UE would implement those TN measurement relaxation procedures individually.

Proposal#3 RAN2 WG to confirm that UE would implement following procedures individually:

  - intra-frequency TN measurement relaxation procedure,

  - inter-frequency TN measurement relaxation procedure,

  - inter-RAT TN measurement relaxation procedure.

- Sub-states of RRC_IDLE/INACTIVE states

Based on the discussion thread in RAN2 WG and possible power saving gain to UEs under different sub-states, we proposed the following Proposal#4 ~ Proposal#6:

Proposal#4 RAN2 WG to clarify whether TN measurement relaxation procedure is limited to UEs staying in ‘camped normally state’.

Proposal#5 RAN2 WG to discuss whether TN measurement relaxation procedure is applicable for UEs staying in ‘camped on any cell state’.

Proposal#6 RAN2 WG to discuss whether TN measurement relaxation procedure is applicable for UEs staying in ‘any cell selection state’.

- Regularity of SIBX deletion

By considering the stability of TN coverage area information and potential signalling reception overhead, we proposed RAN2 WG to discuss whether UE should also delete the stored SIBX after a 3 hours life cycle as other SIBs.

Proposal#7 RAN2 WG to discuss whether UE need to delete the stored SIBX, which contains TN coverage area information, after 3 hours from the moment it was successfully confirmed as valid.

Reference

[1] R2-2308961 Report from Break-out session on NR-NTN and IoT-NTN

[2] R2-2304201 Report from Break-out session on NR-NTN and IoT-NTN

[3] 3GPP TS 38.331 (Release-17) v17.5.0 (2023-July)

[4] 3GPP TS 38.304 (Release-17) v17.5.0 (2023-July)