3GPP TSG-RAN WG2 Meeting #123-bis	R2-2310780

# Xiamen, China, Oct 9 – Oct 13, 2023

Source:	Qualcomm Incorporated

Title:	Layer-2 specific part on U2U Relay

Agenda Item:	7.9 Enhanced NR Sidelink Relay

Document for:	Discussion and Decision

# Introduction & Background

This contribution discusses the open issues on Layer-2 specific part for U2U relay based on RAN2 and SA2 progress.

# Discussion

#### 2.1 QoS handling and E2E SL-DRB configuration

It was agreed that the Relay UE splits E2E QoS parameters and QoS split should be per E2E QoS flow.

Same as L3 based U2U relay, the QoS split should be per e2e QoS flow, and RAN2 expect that the source UE will inform the Relay UE QoS flow(s) and corresponding QoS profiles.  FFS if this requires AS signalling or can be done in upper layers.

Split PDB is sent to the source (TX) Remote UE from the Relay UE.

One issue is how the Relay UE obtain E2E QoS profiles. There are a lot of QoS parameters to be sent to Relay UE, the QoS parameters are shown in Annex. In Layer-3 based U2U relay, the source Remote UE sends E2E QoS profiles and QoS flow ID(s) to the Relay UE using PC5-S message. There could be a lot of QoS parameters, and the PC5-S message including E2E QoS parameters can be reused for Layer-2 based U2U relay, and RAN2 does not need to introduce QoS parameters in PC5-RRC message.

Proposal 1: The source Remote UE reuses the existing PC5-S message to send E2E QoS profiles and QoS flow ID(s) to Relay UE.

Proposal 2: The Relay UE sends split PDB to the source (TX) Remote UE using PC5-S signalling.

SA2 sent LS to RAN2 in [3] and asks RAN2 to inform SA2 if any impact.

SA2 asks RAN2 to define the AS solution/signalling to address QoS splitting, and if RAN2 identify any impact to SA2 to inform SA2.

Then it is proposed to send LS reply to SA2 that PC5-S message is reused to transmit QoS parameters between the source Remote UE and the Relay UE.

Proposal 3: Send relay LS to SA2 and CT1 that QoS parameters are sent between Remote UE and the Relay UE reusing PC5-S message.

#### 2.2 E2E SL-DRB configuration

In the existing direct communication, CONNECTED state UE reports the QoS profiles to gNB and the gNB provide the DRB configuration to the UE. In U2U relay, if the CONNECTED remote UE want to get the configuration from the gNB, then the Remote UE needs to report E2E QoS profiles and the per-hop PDB to the serving gNB, and then gNB provide the E2E configuration and per-hop configuration to the Remote UE. For CONNECTED Relay UE, the Relay UE needs to provide the mapping of QoS flow and E2E bearer, QoS flows and corresponding QoS profiles to the gNB and the gNB provides the second hop configuration to the Relay UE. This brings more complex on gNB and Uu interface enhancement. Considering the limit time in Rel-18, it proposed do not introduce the enhancement for E2E DRB configuration in dedicated signalling.

Proposal 4: No enhancement is introduced for the E2E and per-hop configuration for SL DRB in dedicated signalling.

For in coverage TX remote UE including all RRC states, the Remote UE should derive E2E SDAP and PDCP configuration from SIB 12.

Proposal 5: In coverage TX Remote UE (including all RRC states) derives E2E SDAP and PDCP configuration from SIB12 based on E2E QoS profiles of the E2E QoS flow.

Currently, the mapping of the QoS profile and the bearer is provided to the UE in SL SDAP configuration, and for each bearer there is corresponding PC5 relay RLC configuration in SL-RLC-BearerConfig. Since the Remote UE needs to derive the first hop PC5 relay RLC Channel configuration, then the mapping of PC5 relay RLC Channel configuration and QoS profiles needs to be provided to the UE.

Proposal 6: The TX Remote UE derives the first hop PC5 Relay RLC channel configuration based on the first hop QoS profiles. It is FFS whether existing SIB2 and UE behavior are needed to be enhanced.

RAN2 agree that the Relay UE derives the second hop configuration

The Relay UE derives the second hop configuration (e.g. PC5 relay RLC Channel configuration) for each SL-DRB.

But since SDAP is E2E configuration and the Relay UE does not know the E2E SL-DRB for each QoS flow, then the Relay UE cannot does not know how to generate and map the RLC Channel configuration to the E2E SL-DRB on the second hop.

Observation 1: Without knowing the mapping of QoS flow and E2E bearer, Relay UE does not know how to generate and map the RLC Channel configuration to the E2E SL-DRB on the second hop.

The Relay UE has already known the QoS flow ID(s) as RAN2 agreed, then one simple way is that the Remote UE sends the mapping of E2E-SL-DRB and QoS flows to the Relay UE. Relay UE knows the per-hop QoS profiles on the second hop for the QoS flows, then the Relay UE can derive the SL-DRB info and corresponding QoS profiles on the second hop, and then can derive the RLC Channel for the second hop.

Proposal 7: The Remote UE sends the mapping of E2E-SL-DRB and QoS flow ID(s) to the Relay UE; and the Relay UE derives the second hop RLC Channel configuration for the SL-DRB based on the second hop QoS profiles.

Considering the mapping of QoS profiles and SL-DRB/PC5 RLC Channel configuration could be different in TX remote UE and Relay UE, there could be case that multiple QoS flows are mapped into one E2E SL-DRB in the TX Remote UE, but in the Relay UE, the multiple QoS flows are mapped into multiple PC5 RLC Channel configurations. In this case, it can be left to Relay UE implementation to select the applied PC5 RLC Channel configuration.

Proposal 8: In case that the multiple QoS flows for the E2E SL-DRB are mapped into multiple PC5 RLC Channel configurations, it is Relay UE implementation to determine which PC5 RLC Channel configuration is used for the E2E SL-DRB.

#### 2.2.4 RLF handling and recovery

Currently, the sidelink RLF is detected in the following conditions, and when any condition is met, UE will release PC5-RRC connection.

1>	upon indication from sidelink RLC entity that the maximum number of retransmissions for a specific destination has been reached; or

1>	upon T400 expiry for a specific destination; or

1>	upon indication from MAC entity that the maximum number of consecutive HARQ DTX for a specific destination has been reached; or

1>	upon integrity check failure indication from sidelink PDCP entity concerning SL-SRB2 or SL-SRB3 for a specific destination

In U2U relay, since there per-hop connection and E2E connection, the RLF could be detected on current hop (e.g. the condition on RLC or MAC entity maximum retransmission number) or on E2E connection (e.g. the condition on T400 expiry and PDCP integrity failure).

When the per-hop PC5 RLF is detected on first hop, it could be wasteful to release and establish again the per-hop PC5 connection on the second hop and the E2E PC5 connection. In SA2 specification TS23.304, E2E connection can be kept and modified if needed during U2U relay reselection, please see Annex (B). Then, for RLF handling, one reasonable way is to keep the second hop connection and E2E connection unaffected, let the Remote UE try to discover and reselect a candidate Relay UE. If the old Relay UE is selected, then the PC5 link on the second hop is not impacted, and if a new Relay UE is selected, then the second hop will be released by the peer Remote UE once the new PC5 link is established. During the procedure the E2E connection is not impacted.  This can be shown in the following procedure.

In case that the source Remote UE is indicated per-hop PC5 RLF by the Relay UE, the Remote UE assumes the old relay path is not available, then the source Remote UE should release the per-hop PC5 link for the S-Remote UE/D-Remote UE pair, and try to discover and reselect a candidate Relay UE.

Proposal 9: When per-hop RLF is detected on one hop, E2E PC5 connection can be kept during Relay reselection and per-hop PC5 connection recovery.

Proposal 10: When per-hop RLF is detected on one hop, per-hop PC5 connection on another hop can be kept if the old Relay UE is reselected; if a new Relay UE is reselected, the destination Remote UE release the old per-hop PC5 connection.

Proposal 11: If E2E PC5 RLF is detected, the Remote UE should release the E2E link connection.

# Conclusion

This contribution discusses Layer-2 specific part and provides the following proposals.

QoS,

Proposal 1: The source Remote UE reuses the existing PC5-S message to send E2E QoS profiles and QoS flow ID(s) to Relay UE.

Proposal 2: The Relay UE sends split PDB to the source (TX) Remote UE using PC5-S signalling.

Proposal 3: Send relay LS to SA2 and CT1 that QoS parameters are sent between Remote UE and the Relay UE reusing PC5-S message.

DRB handling,

Proposal 4: No enhancement is introduced for the E2E and per-hop configuration for SL DRB in dedicated signalling.

Proposal 5: In coverage TX Remote UE (including all RRC states) derives E2E SDAP and PDCP configuration from SIB12 based on E2E QoS profiles of the E2E QoS flow.

Proposal 6: The TX Remote UE derives the first hop PC5 Relay RLC channel configuration based on the first hop QoS profiles. It is FFS whether existing SIB2 and UE behavior are needed to be enhanced.

Observation 1: Without knowing the mapping of QoS flow and E2E bearer, Relay UE does not know how to generate and map the RLC Channel configuration to the E2E SL-DRB on the second hop.

Proposal 7: The Remote UE sends the mapping of E2E-SL-DRB and QoS flow ID(s) to the Relay UE; and the Relay UE derives the second hop RLC Channel configuration for the SL-DRB based on the second hop QoS profiles.

Proposal 8: In case that the multiple QoS flows for the E2E SL-DRB are mapped into multiple PC5 RLC Channel configurations, it is Relay UE implementation to determine which PC5 RLC Channel configuration is used for the E2E SL-DRB.

RLF handling,

Proposal 9: When per-hop RLF is detected on one hop, E2E PC5 connection can be kept during Relay reselection and per-hop PC5 connection recovery.

Proposal 10: When per-hop RLF is detected on one hop, per-hop PC5 connection on another hop can be kept if the old Relay UE is reselected; if a new Relay UE is reselected, the destination Remote UE release the old per-hop PC5 connection.

Proposal 11: If E2E PC5 RLF is detected, the Remote UE should release the E2E link connection.

# Reference

[1]    RP-221262: New WID on NR sidelink relay enhancements

[2]    R2-2304652 Reply LS on 5G ProSe Layer-2 UE-to-UE Relay QoS enforcement (S2-2305915; contact: Qualcomm)	SA2	LS in

# Annex (A)

QoS parameters sent to Relay UE in PC5-S message (TS 24.554)

| 8 | 7 | 6 | 5 | 4 | 3 | 2 | 1 |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| PC5 QoS flow descriptions IEI |  |  |  |  |  |  |  | octet 1 |
| Length of PC5 QoS flow descriptions contents |  |  |  |  |  |  |  | octet 2 octet 3 |
| PC5 QoS flow description 1 |  |  |  |  |  |  |  | octet 4 octet u |
| PC5 QoS flow description 2 |  |  |  |  |  |  |  | octet u+1 octet v |
| ... |  |  |  |  |  |  |  | octet v+1 octet w |
| PC5 QoS flow description n |  |  |  |  |  |  |  | octet w+1 octet x |

Figure 11.3.5.1: PC5 QoS flow descriptions information element

| 8 | 7 |  | 6 |  | 5 |  | 4 |  | 3 | 2 |  | 1 |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 0 Spare |  | 0 Spare | PQFI |  |  |  |  |  |  |  |  |  | octet 4 |
| Operation code |  |  |  | 0 Spare |  | 0 Spare |  | 0 Spare |  | 0 Spare | PII |  | octet 5 |
| 0 Spare |  | E | Number of parameters |  |  |  |  |  |  |  |  |  | octet 6 |
| Associated ProSe identifiers |  |  |  |  |  |  |  |  |  |  |  |  | octet 7* octet k* |
| Parameters list |  |  |  |  |  |  |  |  |  |  |  |  | octet k+1* octet u* |

Figure 11.3.5.2: PC5 QoS flow description

# Annex (B)

#### 6.7.4.2	Negotiated 5G ProSe Layer-2 UE-to-UE Relay reselection

Depicted in Figure 6.7.4.2-1 is the procedure for the negotiated 5G ProSe Layer-2 UE-to-UE Relay reselection.

Figure 6.7.4.2-1: Negotiated 5G ProSe Layer-2 UE-to-UE Relay reselection

1.	A PC5 unicast link is established between the 5G ProSe End UEs via a 5G ProSe UE-to-UE Relay, based on the procedure defined in clause 6.7.2.

2.	The initiating 5G ProSe End UE determines, e.g. based on PC5 signal strength, to perform UE-to-UE Relay reselection and obtains a list of candidate UE-to-UE Relays. The initiating 5G ProSe End UE may receive UE-to-UE Relay Discovery Announcement messages from 5G ProSe UE-to-UE Relays or initiate the 5G ProSe UE-to-UE Relay discovery procedures to find the candidate 5G ProSe UE-to-UE Relays. The initiating 5G ProSe End UE determines the candidate 5G ProSe UE-to-UE Relays e.g., based on the PC5 signal strength of the received UE-to-UE Relay Discovery Announcement message, RSC within the UE-to-UE Relay Discovery Announcement message.

3.	The initiating 5G ProSe End UE sends a Link Modification Request message to the responding 5G ProSe End UE which includes a Relay re-selection indication, the User Info ID(s) of the candidate 5G ProSe UE-to-UE Relay(s), and optionally the Layer-2 ID(s) of the candidate 5G ProSe UE-to-UE Relay(s) and security information.

4.	The responding 5G ProSe End UE selects a new 5G ProSe UE-to-UE Relay from the candidate 5G ProSe UE-to-UE Relays, based on the Relay re-selection indication in the Link Modification Request message. If the responding 5G ProSe End UE has not received a UE-to-UE Relay Discovery Announcement message from a candidate 5G ProSe UE-to-UE Relay or does not have a PC5 connection with the candidate 5G ProSe UE-to-UE Relay associated with the same RSC, then the responding 5G ProSe End UE may perform the 5G ProSe UE-to-UE Relay discovery procedure with the User Info ID of a candidate 5G ProSe UE-to-UE Relay in discovery message, and may set the Layer-2 ID of the candidate 5G ProSe UE-to-UE Relay, if received at step 3, as the Destination Layer-2 ID to carry the discovery message, by using the procedure defined in clause 6.3.2.4.4. The PC5 signal strength of the UE-to-UE Relay Discovery Announcement message or UE-to-UE Relay Discovery Response message may be used to select the new 5G ProSe UE-to-UE Relay.

5.	The responding 5G ProSe End UE sends a Link Modification Accept message to the initating 5G ProSe End UE, including the User Info ID of the new 5G ProSe UE-to-UE Relay and security information.

6.	5G ProSe End UEs set up PC5 unicast links, if not already set up, with the new 5G ProSe UE-to-UE Relay, by reusing the procedure defined in clause 6.7.2 and the PC5 unicast is link established between 5G ProSe End UEs via the new 5G ProSe UE-to-UE Relay. The 5G ProSe End UEs switch the data traffic via the new 5G ProSe UE-to-UE Relay. The security information is used to verify that the new link has been set up successfully.