3GPP TSG-RAN WG2 Meeting #123bis	R2-2310799

Xiamen, China, 09 – 13 October 2023

Agenda item:	7.25.1

Source:	Nokia, Nokia Shanghai Bell

Title:	Unknown SCell activation enhancement

WID/SID:	NR_HST_FR2_Enh - Release 18

Document for:	Discussion and Decision

# 1	Introduction

RAN4 is discussing how to reduce the FR2 unknown SCell activation delay for R18 eFeRRM WI, and in RAN4 #106-bis-e meeting there were some agreements achieved for the enhancements.

When SCell activation command is received and UE has valid L3 measurement results available but not reported , RAN4 agreed that SCell activation delay can be shortened if the available L3 measurement results are reported to NW. RAN4 agreed that following aspects of the above said report of L3 measurement results need RAN2 expertise and requests RAN2 to design solution for following aspects.

How to configure the report of L3 measurement result

How to trigger the report of L3 measurement result

Contents of the report of L3 measurement result and how to send the report.

RAN2 discussed these aspects in RAN2#122 and answered RAN4 with following in R2-2306863:

Based on the discussion in RAN2, RAN2 thinks that the existing L3 mesaurement framework can be reused for the unkown SCell measurement reporting based on SCell activaton command.

Based on RAN2 understanding, RAN2 would like to provide the following answers to RAN4 questions:

Question 1: How to configure the report of L3 measurement result for unknown FR2 SCell activation enhancement

[RAN2 Answer to Q1]

- The unkown SCell measurement report is configured via RRC measurement configuration (i.e. measConfig).

Question 2: How to trigger the report of L3 measurement result for unknown FR2 SCell activation enhancement

[RAN2 Answer to Q2]

- The unkown SCell measurement report is triggered by SCell activation command MAC CE.

Question 3: Contents of the report of L3 measurement result for unknown FR2 SCell activation enhancement and how to send the report

[RAN2 Answer to Q3]

- Legacy RRC MeasurementReport message is used for the unkown SCell measurement report, which can already carry the beam level L3 measurement result and beam index.

RAN2 will further discuss the detailed signaling design and spec impact in next meeting.

In this paper we consider further how to realize these from RAN2 perspective..

# 2	Discussion

In Toulouse we crafted baseline CR for implementing unknown SCell measurement reporting- this can be found in R2-2309254. We further discuss open issues and other aspects of the draft CR.

## 2.1	Configuration

We have following Editor’s Note in the CR:

Editor Note 1: FFS on whether the reporting configuration with the reportType set to reportOnScellActivation is associated with a CG or a SCell (servingCellMO).

In our view configuration for Unknown SCell Status Reporting (USSR) is not linked to any measurement events but trigger/configuration for uknown SCell reporting is done separately:

Proposal 1: Unknown SCell status reporting (USSR) is not linked to existing measurement event configuration but can be configured to occur independent of existing measurement events/periodic reporting

As a simplest one would configure Unknown SCell reporting per UE and e.g. not per SCell. Drawback of this is lack of flexibility but that does not seem necessary as we already agreed to use measurementReport as structure for reporting which can always include all the serving cells.

Proposal 2: USSR is configured per CG (not per SCell) as part of reportType

## 2.2	Triggering

We have following editor’s notes regarding triggering:

Editor Note 1: The requirement refers to the SCell is unknown and has a valid measurement, which may be updated based on RAN4 progress.

Editor Note 2: FFS on whether and how to specify the new report is only triggered upon receiving the SCell activation indication from lower layer.

Regarding the triggering of sending MeasurementReport nowadays triggers are based on radio conditions e.g. event A1 to A6 but report for the unknown SCell measurement report is sent when a SCell is activated. One could consider several optimizations which would reduce amount of reporting e.g. Only USSR is triggered if there is unknown SCell. Problem with this is that there is no such a terminology in RAN2 as unknown SCell i.e. either that would need to be somehow defined in RAN2 or we could have some sort of timer defined in RAN2 e.g. if no measurements have been reported for corresponding SCell for X milliseconds then UE will only trigger measurement reporting.

Alternatively UE will just simply everytime trigger measurement reporting (if configured) whenever SCell is activated with MAC CE.

We are not too worried about UE sending measurement reporting when SCell is activated even if the SCell is “known” thus we would consider it simplest that:

Proposal 3: When USSR is configured UE will trigger measurement reporting whenever a SCell is activated with MAC CE and they fulfil whatever RAN4 requirements are defined. Editor’s Notes can be removed.

If it is seen urgent to have some optimizations for USSR to avoid always reporting when SCell is activated then in order to avoid defining timers/unknown SCell in RAN2 for USSR we can leave it to UE implementation to omit reporting if not seen necessary from UE point of view but if this omitting is done by the UE when USSR is configured then UE should still activated SCell with new performance requirements defined in RAN4

Proposal 4: Whenever USSR is configured and SCell is activated with MAC CE UE may omit USSR sending but still UE needs to follow requirements for known SCell activation

# 3	Conclusion

Proposal 1: Unknown SCell status reporting (USSR) is not linked to existing measurement event configuration but can be configured to occur independent of existing measurement events/periodic reporting

Proposal 2: USSR is configured per CG (not per SCell) as part of reportType

Proposal 3: When USSR is configured UE will trigger measurement reporting whenever a SCell is activated with MAC CE and they fulfil whatever RAN4 requirements are defined. Editor’s Notes can be removed.

Proposal 4: Whenever USSR is configured and SCell is activated with MAC CE UE may omit USSR sending but still UE needs to follow requirements for known SCell activation