3GPP TSG-RAN WG2 Meeting #124	DRAFT R2-2311058

Xiamen, China, 09 October – 13 October 2023

Title:	Draft Reply LS on report quantity parameter setting for CQI reporting with 1Tx

Response to:	R4-2313998

Release:	Release 15

Work Item:	NR_newRAT-Perf

Source:	TSG RAN WG2

To:	TSG RAN WG4

Cc:

Contact Person:

#### Name:	TBD

E-mail Address:	TBD

Send any reply LS to:	3GPP Liaisons Coordinator, mailto:3GPPLiaison@etsi.org

Attachments:	-

- Overall Description:

In RAN4 #108, RAN4 discussed a test parameter called reportQuantity for demodulation CQI reporting with 1Tx and noticed that PMI specification is not defined in TS 38.214 cl.5.2.2.2.1 in the case of 1 antenna port. It is not clear how PMI for a Type I codebook is reported from a UE when cri-RI-PMI-CQI is configured.

After further investigation of the RAN1 specification, TS 38.212, we noticed that there is a description in clause 6.3.1.1.2 about CSI sequence generation in UCI over PUCCH. According to the sentence, the bitwidth for PMI with 1 CSI-RS port is 0. Therefore, there may be a chance that we configure the report quantity as ‘cri-RI-PMI-CQI’ and UE will not report the PMI. Though, the intended behavior for UEs not supporting the “csi-ReportWithoutPMI” capability when dealing with zero length PMI is not clear, and neither is it clear why the zero length PMI is not specified in UCI over PUSCH.

Furthermore, another issue was identified with TS38.331 on codebookConfig IE that it is undefined for one CSI/antenna port. Since the codebookConfig IE is optional according to TS 38.331, there is a possibility that under the 1Tx scenario where no precoder is applied, gNB can omit the codebookConfig field, which means there would be no signaling of precoder Type information and codebook restriction. RAN4 also analyzed that at least for PUSCH part, codebook type will be undefined if CodebookConfig is not signalled (as it is conditioned on codebookType) and consequently PMI bitwidth will be undefined.

We do note that all the CSI tests with 1Tx in the latest RAN4 UE demodulation performance requirements assume the CSI report via PUCCH, and all the test cases do not configure codebook information. However, it may not be aligned with RAN1’s intention as codebook config is conditioned on codebookType.

RAN4 kindly asked RAN2 to take the above information into account and clarify how RAN4 should configure codebook for 1Tx.

RAN2 would like to thank RAN4 for the LS.

RAN2 has discussed the questions raised by RAN4 and would like to answer as follows:

- The CodebookConfig IE is defined for two or more antenna ports (nrOfAntennaPorts ≥ 2), and is undefined for 1 antenna port scenario.

- The CodebookConfig IE is a optional field in CSI-ReportConfig, hence under the one antenna port (nrOfAntennaPorts = 1) scenario, where no precoder is applied, gNB can omit the CodebookConfig field.

2. Actions:

To RAN4 group.

ACTION: 	RAN2 respectfully asks RAN4 to take the above responses into account.

3. Date of Next TSG-RAN WG2 Meeting:

TSG-RAN WG2#123bis	                October 9 - 13, 2023		Xiamen, China

TSG-RAN WG2#124	                November 13 - 17, 2023	Chicago, US