3GPP TSG-RAN WG2 #123bis	R2-2311099

Xiamen, 9th October – 13th October 2023

Source:                    	DENSO CORPORATION

Title:  	Procedures for MUSIM temporary capability restriction

Document for:        	Discussion and decision

Agenda Item:         	7.17.2	Procedures for MUSIM temporary capability restriction

- Introduction

In the WID of Dual Tx/Rx Multi-SIM for NR [1], the following is stated as one of objective.

1. Enhancements for MUSIM procedures to operate in RRC_CONNECTED state simultaneously in NW A and NW B. [RAN2, RAN3, RAN4]. - Specify mechanism to indicate preference on temporary UE capability restriction and removal of restriction (e.g. capability update, release of cells, (de)activation of configured resources) with NW A when UE needs transmission or reception (e.g., start/stop connection to NW B) for MUSIM purpose - RAT Concurrency: Network A is NR SA (with CA) or NR DC. Network B can either be LTE or NR. - Applicable UE architecture: Dual-RX/Dual-TX UE 2.Specify MN-SN coordination of R17 MUSIM gaps when network A is NR-DC in Rel-18 [RAN2]

During the MUSIM discussions for Rel-17, applicable UE architecture was mainly focused to Single-Tx/Rx UE. As mentioned in the above WID, for Rel-18 discussion to support Dual Tx/Rx UE architecture, it would be necessary to introduce the mechanism to indicate preference on temporary UE capability restriction and its removal.

Based on the chair’s note for past meetings, the following agreements were made related to “early indication”:

- RAN2#121bis-e [2]:

- Support “early indication” from UE to network during RRC connection setup/resume procedure. - FFS how to indicate this and in which message. The indication will tell network that UE capabilities are temporarily restricted. - FFS on details (i.e. when UE can indicate this, what does it indicate, how does it relate to UAI, etc.)

- RAN2#123 [3]:

- Use Msg5 for early indication of MUSIM capability restriction for UEs in IDLE. - Using LCIDs would avoid any problems for RRC resume procedure. However, there are not many LCIDs left for UL and some other Rel-18 WIs also intend to use them. - FFS whether there is a need to use the LCIDs or whether we can reuse the legacy LCIDs. - Whether we can use the LCIDs (given that multiple WIs may be trying to use them) will be discussed in the main session. How to proceed LCID usage for MUSIM can be discussed in the next meeting based on the main session decision. - UE sends early indication of MUSIM temporary capability restriction only if the network indicates that it is allowed in SIB1. - No support to use RRCReconfigurationComplete for the early indication of MUSIM capability restriction. Can come back if sufficient support.

This paper attempts to investigate about the procedures for “early indication” based on the above agreements and FFS, and remaining consideration on MN-SN coordination of R17 MUSIM gaps.

- Discussion

  - Consideration on “early indication”

As mentioned above, it was agreed to introduce the indication called “early indication” which tells network that the UE capabilities are temporarily restricted. Then, it was also agreed that “early indication” will be included in msg5 at least for setup. In case of setup, network usually sends UECapabilityEnquiry message to UE, and then UE responds to the message to inform its capability via UECapabilityInformation. At this point, we think RAN2 has not yet discussed explicitly whether UE should inform either full capability or restricted capability on UECapabilityInformation, in case of “early indication”.

Observation 1:  RAN2 has not yet discussed explicitly whether UE should inform either full capability or restricted capability on UECapabilityInformation, in case of “early indication”

From the perspective of consistency with normal scenario, we think UE should inform full (non-restricted) capability to network even though the UE has restricted capability, and then capability restriction information itself should be sent by using UAI. Additionally, this sequence would be effective to support a case where UE completely leaves network A and removes capability restriction in network B.

Proposal 1:  UE should inform full (non-restricted) capability on UECapabilityInformation even though the UE has restricted capability

Proposal 2:  The capability restriction information itself should be sent by using UAI anyway

In the typical sequence, network sends RRCReconfiguration message to configure UE based on the capability information received by UECapabilityInformation. However, in case of early indication, the network cannot complete the configuration to the UE since capability restriction information is not available before UAI is reveived (if proposal 1 and 2 are applied). Therefore, RRCReconfiguration before UAI for capability restriction information might be redundant. We think this redundant RRCReconfiguration could be avoided if UE can send UAI for capability restriction information without RRCReconfiguration somehow. For example, UE assumes to be allowed to send UAI if “early indication” is allowed in SIB, or network allows UE to send UAI by msg4. Then, the network can postpone the configuration for the UE until capability restriction information is available.

Observation 2: In case of early indication, RRCReconfiguration before UAI for capability restriction information might be redundant

Proposal 3: RAN2 to discuss if UE can send UAI for capability restriction information without RRCReconfiguration in case of “early indication”, such as the following options:

- Option 1: UE assumes to be allowed to send UAI if “early indication” is allowed in SIB

- Option 2: Network allows UE to send UAI by msg4

Figure 1. Sequence diagram for “early indication”

  - MN-SN coordination of R17 MUSIM gaps when network A is NR-DC

As mentioned in the introduction of this paper, it was decided to have discussion for specifying MN-SN coordination of R17 MUSIM gaps when network A is NR-DC. We understand that one of the purposes on this MN-SN coordination is to configure CG specific MUSIM gap (not per-UE like Rel-17) to avoid performing unnecessary gap either in SN or MN. The typical use case for the CG specific MUSIM gap would be configuring the MUSIM gap on a RF chain which will be used for the communication with the network B. If that is the case, the network A is not able to judge which RF chain (associated with either of a CG) will be used for the communication with the network B without UE indication. Therefore, the following can be observed and proposed:

Observation 3: To configure CG specific MUSIM gap, UE may need to indicate its preference on which CG to configure the MUSIM gap to the network A, since the network A is not able to know which RF chain will be used for the communication with the network B.

Proposal 4: RAN2 to introduce UE indication for its preference on which CG to configure the MUSIM gap.

- Summary and proposal

This paper discussed about the procedures for MUSIM temporary capability restriction of Dual Tx/Rx Milti-SIM devices. In summary, the followings were observed and proposed:

Observation 1:  RAN2 has not discussed explicitly whether UE should inform either full capability or restricted capability on UECapabilityInformation, in case of “early indication”

Proposal 1:  UE should inform full (non-restricted) capability on UECapabilityInformation even though the UE has restricted capability

Proposal 2:  The capability restriction information itself should be sent by using UAI anyway

Observation 2: In case of early indication, RRCReconfiguration before UAI for capability restriction information might be redundant

Proposal 3: RAN2 to discuss if UE can send UAI for capability restriction information without RRCReconfiguration in case of “early indication”, such as the following options:

- Option 1: UE assumes to be allowed to send UAI if “early indication” is allowed in SIB

- Option 2: Network allow UE to send UAI by msg4

Observation 3: To configure CG specific MUSIM gap, UE may need to indicate its preference on which CG to configure the MUSIM gap to the network A, since the network A is not able to know which RF chain will be used for the communication with the network B.

Proposal 4: RAN2 to introduce UE indication for its preference on which CG to configure the MUSIM gap.

- References

[1] RP-230751, “Revised WID: Dual Transmission/Reception (Tx/Rx) Multi-SIM for NR”

[2] R2-2304202, “Report on LTE legacy, XR, QoE and MUSIM”

[2] R2-2308962, “Report on LTE legacy, XR, QoE and MUSIM”