3GPP TSG-RAN WG2 Meeting #123bis	R2-23xxxxx

Xiamen, China, October 9th – 13th, 2023

Source: 	RAN2 Vice Chairman (CATT)

Title:	Report from session on NR MIMO evolution and Multi-SIM

## Status of At-Meeting Email Discussions

This subclause is not an Agenda Item. It contains a running summary of the email discussions assigned to take place during the meeting weeks.

[AT123bis][200] Organizational – NR MIMO evolution and Multi-SIM (RAN2 VC)

Scope:

    - Share plans and list of ongoing email discussions for the related sessions

    - Share meetings notes and agreements for review and endorsement

## 7.17	Dual Transmission/Reception (Tx/Rx) Multi-SIM for NR

(NR_DualTxRx_MUSIM-Core; leading WG: RAN2; REL-18; WID: RP-231461)

Time budget: 1 TU

Tdoc Limitation: 3 tdocs

### 7.17.1	Organizational

Including LSs and any rapporteur inputs (e.g. work plan)

Running CR rapporteurs of 37.340 (ZTE), 38.300 (China Telecom) and 38.331 (vivo) specifications are requested to provide latest versions running CRs as rapporteur input (which are not counted against the Tdoc limits)

R2-2309461	LS on Dual Tx/Rx Multi-SIM (R4-2314465; contact: MediaTek)	RAN4	LS in	Rel-18	NR_DualTxRx_MUSIM-Core	To:RAN2

R2-2309789	[Post123][MUSIM] Remaining Open Issues (vivo)	vivo	other	Rel-18	NR_DualTxRx_MUSIM-Core

R2-2309790	Running RRC CR for NR MUSIM enhancements	vivo	draftCR	Rel-18	38.331	17.6.0	NR_DualTxRx_MUSIM-Core

R2-2309891	Draft running CR to 38.331 for MUSIM UE Capabilities	Huawei, HiSilicon	draftCR	Rel-17	38.331	17.6.0	NR_DualTxRx_MUSIM-Core

R2-2309892	Draft running CR to 38.306 for MUSIM UE Capabilities	Huawei, HiSilicon	draftCR	Rel-17	38.306	17.6.0	NR_DualTxRx_MUSIM-Core

R2-2310918	38.300 Running CR for NR MUSIM enhancements	China Telecommunications	draftCR	Rel-18	38.300	17.6.0	NR_DualTxRx_MUSIM-Core

R2-2310921	38.300 Running CR for NR MUSIM enhancements	China Telecommunications	draftCR	Rel-17	38.300	17.6.0	NR_DualTxRx_MUSIM-Core	Withdrawn

R2-2311040	37.340 Running CR for Introduction of MUSIM	ZTE Corporation, Sanechips	draftCR	Rel-18	37.340	17.6.0	NR_DualTxRx_MUSIM-Core

### 7.17.2	Procedures for MUSIM temporary capability restriction

Discussion on LCID usage is handled in the main session jointly

Remaining aspects for “proactive” and “reactive” procedures, including output of [Post123][234][MUSIM] UE preferred frequency (vivo)

Including discussion on how UE-network interaction works when UE requests capability restriction (e.g. is there a timer to control when UE applies capability restrictions if network doesn’t provide a reply)

Email discussion report (proposals on the procedure)

R2-2309791	Report of [Post123][234][MUSIM] UE preferred frequency (vivo)	vivo	report	Rel-18	NR_DualTxRx_MUSIM-Core

Proposal 1	[13/13] For proactive UE temporary capability reporting, UE reporting of its impacted frequency ranges is sufficient and there is no need for UE to additionally report preferred.

Proposal 2	[10/13] UE can indicate impacted band(s) in a BC for the proactive reporting, detailed signalling is FFS.

Proposal 3	[11/13] UE is allowed to only report the impacted band(s) based on a frequency/band filter list (e.g. frequencies/bands) configured by the network.

Proposal 4	[13/13] UAI based signalling is also used for proactive reporting of temporary UE capability restriction.

Proposal 5	[8/13] A single enable/disable configuration is applied for both “Proactive approach” and “Reactive approach”.

Proposal 6	[12/13] One configuration is used to control all temporary capabilities update.

Timer related

Chair: different timer proposals will be discussed seperately, e.g., wait timer, prohibit timer, etc.

R2-2309715	Further discussion on the MUSIM temporary capability restriction	China Telecom Corporation Ltd.	discussion	Rel-18	NR_DualTxRx_MUSIM-Core

Proposal 4: Introduce the prohibit timer for MUSIM capability restriction report to avoid overloading signalling messages .

Proposal 5: Introduce the wait timer for reactive MUSIM capability restrictions reporting to address the UE behavior when no response was received from NW.

Proposal 6: When UE receives RRCReconfiguration message in which the configuration related to MUSIM capability is different from the previous one, the wait time can be stopped.

Proposal 7: In reactive approach, Upon expiry of the wait timer, UE can reduce the capability locally, FFS whether the UE can request the RRC release on network A.

R2-2311108	Timer based restriction in MUSIM	LG Electronics Deutschland	discussion	Rel-18	NR_DualTxRx_MUSIM-Core

Proposal 1.	A timer is introduced to allow the UE to perform the temporary capability restriction upon expiry of the timer.

Proposal 2.	The UE starts the timer when the UE requests a temporary restriction to the network if the timer is configured. The UE stops the timer when receiving a response to the request for a temporary restriction.

Proposal 3.	Upon the timer expiry, if the UE requests SCell release or SCG release, the UE performs the SCell release or the SCG release as requested earlier.

Proposal 4.	Upon the timer expiry, if the UE requests a MIMO capability update for maximum MIMO layer or DL/UL MIMO layer information on affected serving cell(s), the UE applies the MIMO configuration to the related serving cell(s) as requested earlier.

R2-2309890	Discussion on procedures for temporary capability restriction	Huawei, HiSilicon	discussion	Rel-18

Proposal 2: Introduce a timer for reactive approach for reporting of temporary capability restriction in UAI. The timer starts when the UE sends the UAI, and stops when a response matching the UAI from the NW is received. The timer length is configured by the NW.

Proposal 3: When the timer is running:

-	if the UE transmits the preference for SCell(s)/SCG to be affected in the UAI, and the NW provides the RRC reconfiguration or MAC CE to release or deactivate the corresponding SCell/SCG; and

-	if the UE transmits the preference for maximum MIMO layers in the UAI, and the NW provides the RRC reconfiguration with maximum MIMO layer configuration as requested in the UAI; and

-	if the UE transmits the preference for measurement gap requirement in the UAI, and the NW provides the RRC reconfiguration compatible with the UAI (i.e. RRC reconfiguration with gap configured for the measured frequencies/serving cells, which requires gap in the UAI);

-	the UE stops the timer.

Proposal 4: After the timer expires:

-	if the UE transmits the preference for SCell(s)/SCG to be released or deactivated in the UAI, the UE releases or deactivates the SCell/SCG;

-	if the UE transmits the preference for maximum MIMO layers in the UAI, the UE applies the maximum MIMO layer configuration suggested in the UAI for the corresponding serving cell or for all the BWPs of the corresponding serving cell;

-	if the UE transmits the preference for measurement gap requirement in the UAI, the UE may not perform gap-less measurement on the frequencies/serving cells if requires gap in the UAI.

Proposal 6: No prohibit timer is defined for the UAI for R18 MUSIM purpose.

Removal of capacity restriction

R2-2309793	Procedures for MUSIM temporary capability restriction	vivo	discussion	Rel-18	NR_DualTxRx_MUSIM-Core

Proposal 4 The UE can remove the MUSIM capability restriction information by not including the detailed fields in Rel-18 MUSIM field in the UAI.

R2-2311014	Further analysis on signalling procedure for capability restriction	Nokia, Nokia Shanghai Bell	discussion

Proposal 8: Indication to the restoration of full capability is included as an additional parameter in RRC Reconfiguration completion and measurement report instead of a separate UAI for this scenario.

Proposal 9: A simple indication of the change in capability is triggered on partial removal of capability restriction to allow NW to obtain the complete restricted capability information via separate signaling.

Chair: other critical issues (if any) can be discussed in the CB session. Details TBD.

R2-2309553	Remaining Issues on Procedures for MUSIM Temporary Capability Restriction	OPPO	discussion	Rel-18	NR_DualTxRx_MUSIM-Core

R2-2310031	Early capability restriction indication in ResumeRequest	Intel Corporation	discussion	Rel-18	NR_DualTxRx_MUSIM-Core

R2-2310171	Control signaling for Dual-Active MUSIM	Qualcomm Incorporated	discussion

R2-2310318	Procedures for MUSIM temporary capability restriction	Apple	discussion	Rel-18	NR_DualTxRx_MUSIM-Core

R2-2310583	Timer control for capability restriction	Xiaomi	discussion	Rel-18	NR_DualTxRx_MUSIM-Core

R2-2310592	Discussion on temporary capability restriction	Samsung	discussion

R2-2310648	Procedures for MUSIM temporary capability restriction	NEC	discussion	Rel-18	NR_DualTxRx_MUSIM-Core

R2-2310966	Indication of restricted capabilities at RRC Setup and Resume by MUSIM UE	Ericsson	discussion	Rel-18	NR_DualTxRx_MUSIM-Core	Revised

R2-2310967	Discussion on MUSIM timers	Ericsson	discussion	Rel-18	NR_DualTxRx_MUSIM-Core

R2-2311041	Consideration on the Temporary Capability Reporting procedure	ZTE Corporation, Sanechips	discussion	Rel-18	NR_DualTxRx_MUSIM-Core

R2-2311098	Supporting Proactive cases in other scenarios	LG Electronics Inc.	discussion	Rel-18	NR_DualTxRx_MUSIM-Core	R2-2308788

R2-2311099	Procedures for MUSIM temporary capability restriction	DENSO CORPORATION	discussion	NR_DualTxRx_MUSIM-Core

R2-2311107	Timer based restriction in MUSIM	LG Electronics France	discussion	Rel-18	NR_DualTxRx_MUSIM-Core	Late

R2-2311238	Indication of restricted capabilities at RRC Setup and Resume by MUSIM UE	Ericsson	discussion	Rel-18	R2-2310966

### 7.17.3	Allowed MUSIM temporary capability restrictions

Remaining aspects for the allowed capabiltity restriction reporting (e.g. which capabilities can be coordinated, how are the restrictions signalled, etc.)

Signaling design

R2-2311015	On specific capabilities for restriction and feature interworking scenarios	Nokia, Nokia Shanghai Bell	discussion

Proposal 1: Change in measurement gap requirements is reported using UAI.  The modification in gap requirement is reported as a change over the first NeedForGapInfo reported from UE.

Proposal 2: NW may optionally configure the UE to report the change in gap requirements for the frequencies /bands configured for measurements.

R2-2309843	Details of allowed MUSIM temporary capability restrictions	Huawei, HiSilicon	discussion	Rel-18	NR_DualTxRx_MUSIM-Core

Proposal 4: The UE can indicate the temporary capability restriction of measurement gap for R18 MUSIM purpose in the UAI by using the existing needForGapInfoNR.

Proposal 5: The UE indicates the change of measurement gap capability based on the current RRC configuration.

On maximum MIMO layers/bandwidth restriction

R2-2310965	On some restricted capabilities for Rel-18 MUSIM UE	Ericsson	discussion	Rel-18	NR_DualTxRx_MUSIM-Core

Proposal 1	Rel-18 MUSIM UE indicates restricted frequencies to be avoided for serving cells based on Nw-provided candidate serving frequency list.

Proposal 2	Rel-18 MUSIM UE indicates preference on reduced max number of CCs in UL/DL

Proposal 3	Rel-18 MUSIM UE indicates preference on max number of MIMO layers per FRx and per UL/DL

R2-2311042	Consideration on the Temporary Capability Reporting	ZTE Corporation, Sanechips	discussion	Rel-18	NR_DualTxRx_MUSIM-Core

Proposal 5: For the MIMO layer and Bandwidth reporting in the proactive case, if per FSPC (per cc per BC) granularity can’t be accepted because of the complexity, per Band per BC granularity can be considered.

Chair: other critical issues (if any) can be discussed in the CB session. Details TBD.

R2-2309554	Allowed MUSIM Temporary Capability Restrictions	OPPO	discussion	Rel-18	NR_DualTxRx_MUSIM-Core

R2-2309792	Discussion on temporary capability restriction for Rel-18 Multi-SIM	vivo	discussion	Rel-18	NR_DualTxRx_MUSIM-Core

R2-2310090	Allowed MUSIM temporary capability restrictions	Samsung R&D Institute India	discussion	Rel-18

R2-2310319	Aspects of allowed MUSIM temporary capability restriction	Apple	discussion	Rel-18	NR_DualTxRx_MUSIM-Core

R2-2310582	Clarification on srs-TxSwitch and MIMO-layer for MUSIM	Xiaomi, vivo	discussion	Rel-18	NR_DualTxRx_MUSIM-Core

### 7.17.4	Other

Other remaining aspects, including e.g., aspects related to the RAN4 incoming LS, and UE capabilit(ies).

This agenda item may be deprioritized in this meeting.

Chair: if time allows, we will start with gap priority related aspect in Monday’s session.

Gap prioirty realted

R2-2311135	Discussion on MUSIM gap priority	MediaTek Inc.	discussion

Proposal 1: RAN2 confirms that no need to request gap priority or configure gap priority for aperiodic gap. Below editor note from running CR could be removed.

•	Editor’s Note: FFS musim-GapPriorityToAddModList-r18 is for aperodic MUSIM gap.

Proposal 2: Introduce single bit indication in MUSIM assistance information to indicate the UE preference of “keep” option.

Proposal 3: Reuse existing control flag (i.e. musim-GapPriorityAssistanceConfig-r18 in running CR) to indicate whether the UE could include “keep” option for MUSIM gap.

Proposal 4: The prohibit timer configuration for R17 MUSIM gap preference (i.e. musim-GapProhibitTimer) is also apply to R18 MUSIM gap priority preference.

UE Capabilities

R2-2309844	Discussion on MUSIM gaps and UE capabilities	Huawei, HiSilicon	discussion	Rel-18	NR_DualTxRx_MUSIM-Core

Proposal 6: For temporary capability restriction mechanism, the following UE capabilities are introduced in per-UE level without xDD/FRx differentiation:

-	1 optional bit to indicate the support of “proactive solution”;

-	1 optional bit to indicate the support of “reactive solution”;

-	The support of “early MUSIM indication” is optional without capability signalling, a UE supporting “early MUSIM indication” shall at least support “proactive solution” or “reactive solution”.

R2-2311043	Consideration on the MN-SN Coordination for the MUSIM	ZTE Corporation, Sanechips	discussion	Rel-18	NR_DualTxRx_MUSIM-Core

R2-2309794	Discussion on MUSIM gap priorities	vivo	discussion	Rel-18	NR_DualTxRx_MUSIM-Core

R2-2310032	Feature interaction between R17 and R18 MUSIM	Intel Corporation	discussion	Rel-18	NR_DualTxRx_MUSIM-Core

R2-2310038	Further discussion on MUSIM gap priorities 	Samsung Electronics Czech	discussion	Rel-18	NR_DualTxRx_MUSIM-Core

R2-2310584	Remaining issues of MUSIM gap	Xiaomi	discussion	Rel-18	NR_DualTxRx_MUSIM-Core

R2-2310917	MUSIM gap priority handlling	China Telecommunications	discussion

R2-2311016	MUSIM Gap collision handling	Nokia, Nokia Shanghai Bell	discussion

## 7.20	NR MIMO evolution

(NR_MIMO_evo_DL_UL-Core; leading WG: RAN1; REL-18; WID: RP-223276)

Time budget: 0.75 TU

Tdoc Limitation: 3 tdoc

### 7.20.1	Organizational

Rapporteur input (e.g., work plan, running CRs submitted by the spec rapporteurs for discussions and endorsements), incoming LS etc.

Email report from [Post123][851][MIMOevo] RRC running CR for MIMO evo (Ericsson)

R2-2309410	LS to RAN2 on CBSR for Rel-18 MIMO (R1-2308396; contact: Samsung)	RAN1	LS in	Rel-18	NR_MIMO_evo_DL_UL	To:RAN2

=> Withdrawn

R2-2309850	Running CR for TS 38.321 for MIMO Evolution	Samsung	draftCR	Rel-18	38.321	17.6.0	B	NR_MIMO_evo_DL_UL-Core

R2-2310611	Running CR for Introduction of MIMO Evolution	Ericsson	draftCR	Rel-18	38.331	17.6.0	NR_MIMO_evo_DL_UL-Core

R2-2310819	Report for Post 123 MIMOevo RRC	Ericsson	discussion	NR_MIMO_evo_DL_UL-Core

R2-2311127	Draft 38.300 CR for introduction of 2-TA enhancement	NTT DOCOMO, INC.	draftCR	Rel-18	38.300	17.6.0	B	NR_MIMO_evo_DL_UL-Core

R2-2311167	L1 parameter Excel with rapporteur comments MIMOevo	Ericsson	discussion	Rel-18	NR_MIMO_evo_DL_UL-Core

### 7.20.2	Two TAs for multi-DCI multi-TRP

Remaining open issues on two TAs for multi-DCI multi-TRP operation

2-PTAG model and related behaviours

R2-2310587	Remaining issues for 2 TA handling of mTRP	Xiaomi	discussion	Rel-18	NR_MIMO_evo_DL_UL-Core

Proposal 1: RAN2 is kindly requested to confirm that the 2-PTAG model (i.e., both TAGs of SpCell are PTAGs) is used.

Proposal 2: When the TAT for STAG is expired and the other TAT is running for a serving cell (i.e., SCell), the UE does not perform the following behaviours for this serving cell:

	flush all HARQ buffers;

	notify RRC to release PUCCH, if configured;

	notify RRC to release SRS, if configured;

	clear any configured downlink assignments and configured uplink grants;

	clear any PUSCH resource for semi-persistent CSI reporting;

Proposal 3: when the TAT for PTAG is expired and the other TAT is running for a serving cell (SpCell or SCell), the UE does not perform the following behaviours for this serving cell:

	flush all HARQ buffers;

	notify RRC to release PUCCH, if configured;

	notify RRC to release SRS, if configured;

	clear any configured downlink assignments and configured uplink grants;

	clear any PUSCH resource for semi-persistent CSI reporting;

R2-2310847	UL time alignment in multi-DCI based multi-TRP with two TAs	InterDigital	discussion	Rel-18	NR_MIMO_evo_DL_UL-Core

Proposal 1:	Agree to the following modified working assumption from RAN2#123:

-	We will use the 2-PTAG model, i.e., both TAGs of SpCell are PTAGs;

o	When the TAT for STAG is expired and the other TAT is running for a serving cell (i.e., SCell), no impact to the TRP with running TAT; 1 and 3-7 are applied to the TRP with TAT expired,

o	when the TAT for PTAG is expired and the other TAT is running for a serving cell (SpCell or SCell), no impact to the TRP with running TAT; 1 and 3-7 are applied to the TRP with TAT expired.

R2-2311188	Handling of two TAGs associated with a Serving Cell	Nokia, Nokia Shanghai Bell	discussion	Rel-18	NR_MIMO_evo_DL_UL-Core

Proposal 1: If a TAT of a TAG associated with a Serving Cell expires (while the Serving Cell is configured with two TAGs), the UE performs all the same actions for the Serving Cell as in the legacy.

TAG ID indication in RACH procedure (intra-cell case)

R2-2309690	TAG modeling and TAT expiry in 2TAs mTRP	LG Electronics Inc.	discussion	Rel-18	NR_MIMO_evo_DL_UL-Core

Proposal 5. One R bit in Absolute TAC MAC CE is used to indicate TAG ID, i.e. which TAG’s TA is updated.

Proposal 6. One R bit in RAR is used to indicate TAG ID, i.e. which TAG’s TA is updated.

TAG ID indication in RACH procedure (inter-cell case)

R2-2311003	Remaining issues on 2TA for mTRP	Huawei, HiSilicon	discussion	Rel-18	NR_MIMO_evo_DL_UL-Core

Proposal 4: For inter-cell multi-DCI based multi-TRP operation, TAG ID is not included in the RAR and absolute TA command MAC CE, i.e. the UE can know the TAG information based on the RACH configuration which the UE has used.

R2-2309913	Discussions on Two TAs for Multi-DCI Multi-TRP	CATT	discussion	Rel-18	NR_MIMO_evo_DL_UL-Core

Proposal 3: For the inter-cell scenario, reuse the mechanism agreed for intra-cell case, i.e., use the RA RAR to indicate the TAG.

Proposal 4: Using one reserved bit to indicate the TAG ID via the Absolute TA Command MAC CE.

RRC aspects (e.g., PRACH configuration for inter-cell CFRA)

R2-2311251	On 2TA operation RRC parts	Ericsson	discussion	Rel-18	NR_MIMO_evo_DL_UL-Core

Proposal 5	Do not extend RACH-Configdedicated for additionalPCI.

Proposal 6	RAN2 extend the PRACH configuration in the BWP-UplinkCommon.

Proposal 7	RAN2 to adopt as content for RACH configuration per additionaPCI(7 of these) IE RACH-ConfigGeneric, ssb-perRACH-OccasionAndCB-PreamblesPerSSB and prach-RootSequenceIndex

Chair: other critical issues (if any) can be discussed in the CB session. Details TBD.

R2-2309665	Discussion on multiple TAG	OPPO	discussion	Rel-18	NR_MIMO_evo_DL_UL-Core

R2-2309912	Consideration on the TAG Indication upon CBRA	CATT	discussion	Rel-18	NR_MIMO_evo_DL_UL-Core

R2-2309953	Discussion on the impacts of Two TAs for multi-DCI multi-TRP operation	Lenovo	discussion	Rel-18

R2-2309954	Discussion on the UE-initiated RACH procedure in multi-TRP operation	Lenovo	discussion	Rel-18

R2-2309955	Consideration on RLF in multi-TRP operation	Lenovo	discussion	Rel-18

R2-2310063	Open issues on two TAs for multi-DCI multi-TRP	Samsung Research America	discussion	Rel-18	NR_MIMO_evo_DL_UL-Core

R2-2310101	Further Discussion on RACH for mDCI mTRP with 2TA enhancement	ZTE Corporation,Sanechips	discussion	Rel-18	NR_MIMO_evo_DL_UL-Core

R2-2310103	On RRC parameter for mDCI mTRP with 2TA enhancement	ZTE Corporation,Sanechips	discussion	Rel-18	NR_MIMO_evo_DL_UL-Core

R2-2310193	Discussion on two TAs for multiple TRPs	SHARP Corporation	discussion	NR_MIMO_evo_DL_UL-Core

R2-2310315	Support of Two TAs for multi-DCI multi-TRP	Apple	discussion	Rel-18	NR_MIMO_evo_DL_UL-Core

R2-2310809	Discussion on multi-DCI multi-TRP with two TAs	Qualcomm Incorporated	discussion	NR_MIMO_evo_DL_UL-Core

R2-2310932	MAC issues for multi-DCI multi-TRP with two TAs	Ericsson	discussion	Rel-18	NR_MIMO_evo_DL_UL-Core

´: Revised in R2-2311252

R2-2311252	MAC issues for multi-DCI multi-TRP with two TAs	Ericsson	discussion	Rel-18	NR_MIMO_evo_DL_UL-Core

R2-2311125	Current status of functional issues on 2TA enhancement	NTT DOCOMO, INC.	discussion	Rel-18

R2-2311169	On 2TA operation RRC parts	Ericsson	discussion	Rel-18	NR_MIMO_evo_DL_UL-Core

´: Revised in R2-2311251

R2-2311187	RA procedure while SpCell is configured with 2 TAGs	Nokia, Nokia Shanghai Bell	discussion	Rel-18	NR_MIMO_evo_DL_UL-Core

### 7.20.3	Unified TCI extension to mTRP operation

Remaining open issues on unified TCI extension to mTRP operation, including the cases for sDCI and mDCI

Chair: if time allows, we will discuss sDCI related aspects in the Monday’s session.

sDCI

R2-2309849	Remaining Issues on Single-DCI based unified TCI extension to multi-TRP operation	Samsung	discussion	Rel-18	NR_MIMO_evo_DL_UL-Core

Proposal 1: RAN2 confirm that separate MAC CEs on the enhanced unified TCI state for Single-DCI based multi-TRP operation are introduced for joint TCI State and separate DL/UL TCI States, respectively.

Proposal 2: The simultaneous TCI state update for the RRC configured serving cell sets are supported in Rel-18 enhanced unified TCI state activation/deactivation MAC CE(s) for multi-TRP operations.

R2-2310906	Design of MAC CE for Rel-18 MIMO	Nokia. Nokia Shanghai Bell	discussion	Rel-18	NR_MIMO_evo_DL_UL-Core

Proposal 1: Reuse MAC CE for unified TCI state activation as defined TS38.321 clause 6.1.3.47 for mDCI by replacing one R-bit with CORESET pool index (e.g. as shown below).

Proposal 2: Define new MAC CE (with eLCID) for joint TCI state activation with sDCI (e.g. as shown below).

Proposal 3: Define another new MAC CE (with eLCID) for separate TCI state activation with sDCI (e.g. as shown below).

R2-2309666	Discussion on MAC CE design for mTRP	OPPO	discussion	Rel-18	NR_MIMO_evo_DL_UL-Core

R2-2309691	Discussion on Unified State MAC CE for mDCI	LG Electronics Inc.	discussion	Rel-18	NR_MIMO_evo_DL_UL-Core

R2-2309693	Discussion on Unified State MAC CE for sDCI	LG Electronics Inc.	discussion	Rel-18	NR_MIMO_evo_DL_UL-Core

R2-2309743	Discussion on MAC CE design for MTRP	CEWiT	discussion

R2-2309914	Discussion on Unified TCI Framework Extension for sDCI and mDCI based Multi-TRP	CATT	discussion	Rel-18	NR_MIMO_evo_DL_UL-Core

R2-2310064	Open issues on MIMO RRC parameters	Samsung Research America	discussion	Rel-18	NR_MIMO_evo_DL_UL-Core

R2-2310102	Further DIscussion on remaining UP issues for MIMO-evo	ZTE Corporation,Sanechips	discussion	Rel-18	NR_MIMO_evo_DL_UL-Core

R2-2311004	Extension of unified TCI framework for mTRP	Huawei, HiSilicon	discussion	Rel-18	NR_MIMO_evo_DL_UL-Core

R2-2311128	Discussion on remaining issues on Unified TCI framework extension	NTT DOCOMO INC.,	discussion	Rel-18

### 7.20.4	Other

Other issues if not covered by 7.20.1, 7.20.2, and 7.20.3.

Depending on the number of contributions/proposals, a summary of this agenda item may be used.

Chair: the following can be discussed in the CB session if time allows. Details TBD.

R2-2311005	Overlapping UL grants handling for STxMP	Huawei, HiSilicon	discussion	Rel-18	NR_MIMO_evo_DL_UL-Core

R2-2311130	Discussion on overlapping UL grants in STxMP	NTT DOCOMO, INC.	discussion	Rel-18

R2-2311165	On other than 2TA parameters MIMOevo Rel18	Ericsson	discussion	Rel-18	NR_MIMO_evo_DL_UL-Core

## List of post meeting email discussions

TBD