3GPP TSG-RAN WG2 Meeting #123bis	draft R2-2311272

Xiamen, China, October 9th – 13th, 2023

Source: 	RAN2 Vice Chairman (CATT)

Title:	Report from session on NR MIMO evolution and Multi-SIM

## Status of At-Meeting Email Discussions

This subclause is not an Agenda Item. It contains a running summary of the email discussions assigned to take place during the meeting weeks.

[AT123bis][200] Organizational – NR MIMO evolution and Multi-SIM (RAN2 VC)

Scope:

    - Share plans and list of ongoing email discussions for the related sessions

    - Share meetings notes and agreements for review and endorsement

## 7.17	Dual Transmission/Reception (Tx/Rx) Multi-SIM for NR

(NR_DualTxRx_MUSIM-Core; leading WG: RAN2; REL-18; WID: RP-231461)

Time budget: 1 TU

Tdoc Limitation: 3 tdocs

### 7.17.1	Organizational

Including LSs and any rapporteur inputs (e.g. work plan)

Running CR rapporteurs of 37.340 (ZTE), 38.300 (China Telecom) and 38.331 (vivo) specifications are requested to provide latest versions running CRs as rapporteur input (which are not counted against the Tdoc limits)

R2-2309461	LS on Dual Tx/Rx Multi-SIM (R4-2314465; contact: MediaTek)	RAN4	LS in	Rel-18	NR_DualTxRx_MUSIM-Core	To:RAN2

-	Xiaomi asks the clarify whether the 2nd part is only for keep solution. MTK think it is for all the approaches.

Noted

R2-2309789	[Post123][MUSIM] Remaining Open Issues (vivo)	vivo	other	Rel-18	NR_DualTxRx_MUSIM-Core

Noted

R2-2309790	Running RRC CR for NR MUSIM enhancements	vivo	draftCR	Rel-18	38.331	17.6.0	NR_DualTxRx_MUSIM-Core

-	vivo clarifies this include also some potential agreements from the long email discussions.

-	Ericsson comments that some wording refinement would be necessary in a later phase.

Take as baseline for further updates.

R2-2309891	Draft running CR to 38.331 for MUSIM UE Capabilities	Huawei, HiSilicon	draftCR	Rel-17	38.331	17.6.0	NR_DualTxRx_MUSIM-Core

Endorsed.

R2-2309892	Draft running CR to 38.306 for MUSIM UE Capabilities	Huawei, HiSilicon	draftCR	Rel-17	38.306	17.6.0	NR_DualTxRx_MUSIM-Core

Endorsed.

R2-2310918	38.300 Running CR for NR MUSIM enhancements	China Telecommunications	draftCR	Rel-18	38.300	17.6.0	NR_DualTxRx_MUSIM-Core

Take as baseline for further updates.

R2-2310921	38.300 Running CR for NR MUSIM enhancements	China Telecommunications	draftCR	Rel-17	38.300	17.6.0	NR_DualTxRx_MUSIM-Core	Withdrawn

R2-2311040	37.340 Running CR for Introduction of MUSIM	ZTE Corporation, Sanechips	draftCR	Rel-18	37.340	17.6.0	NR_DualTxRx_MUSIM-Core

Take as baseline for further updates.

Email discussions after the meeting

[Post123bis][205][MUSIM] RRC Running CR and further discussions (vivo)

Scope: Update and review the RRC running CR, also discussions on the RRC open issues based on the progress in this meeting

Intended outcome: RRC running CR for endorsement, and discussion report with proposals

Deadline: Long (2 weeks for running CR, November 3rd for open issue)

[Post123bis][206][MUSIM] Stage 2 Running CR (China Telecom)

Scope: Update and review the 38.300 running CR

Intended outcome: 38.300 running CR in R2-2311294 for endorsement

Deadline: Short (1 week)

[Post123bis][207][MUSIM] 37.340 Running CR (ZTE)

Scope: Update and review the 37340 running CR

Intended outcome: 37.340 running CR in R2-2311295 for endorsement

Deadline: Short (1 week)

### 7.17.2	Procedures for MUSIM temporary capability restriction

Discussion on LCID usage is handled in the main session jointly

Remaining aspects for “proactive” and “reactive” procedures, including output of [Post123][234][MUSIM] UE preferred frequency (vivo)

Including discussion on how UE-network interaction works when UE requests capability restriction (e.g. is there a timer to control when UE applies capability restrictions if network doesn’t provide a reply)

Email discussion report (proposals on the procedure)

R2-2309791	Report of [Post123][234][MUSIM] UE preferred frequency (vivo)	vivo	report	Rel-18	NR_DualTxRx_MUSIM-Core

Proposal 1	[13/13] For proactive UE temporary capability reporting, UE reporting of its impacted frequency ranges is sufficient and there is no need for UE to additionally report preferred.

Proposal 2	[10/13] UE can indicate impacted band(s) in a BC for the proactive reporting, detailed signalling is FFS.

Proposal 3	[11/13] UE is allowed to only report the impacted band(s) based on a frequency/band filter list (e.g. frequencies/bands) configured by the network.

Proposal 4	[13/13] UAI based signalling is also used for proactive reporting of temporary UE capability restriction.

Proposal 5	[8/13] A single enable/disable configuration is applied for both “Proactive approach” and “Reactive approach”.

Proposal 6	[12/13] One configuration is used to control all temporary capabilities update.

P1:

-	OPPO fine with the intention, but want to understand the meaning of frequency ranges.

For proactive UE temporary capability reporting, UE reporting of the information regarding its impacted frequency is sufficient.

P2:

UE can indicate impacted band(s)/frequencies in a BC for the proactive reporting, detailed signalling is FFS.

P3:

-	UE is allowed to only report the impacted band(s) based on a frequency/band filter list (e.g. frequencies/bands) configured by the network.

-	QC thinks this is an optimization. Samsung agrees, and think this helps the UE to determine what to report.

-	CATT want to understand how the filter works.

-	QC thinks this is optional configuration.

UE is allowed to only report the impacted band(s)/frequencies based on a frequency/band filter list (e.g. frequencies/bands), if configured by the network.

P4:

UAI based signalling is also used for proactive reporting of temporary UE capability restriction.

One configuration is used to control all temporary capabilities update

??Proposal 5	[8/13] A single enable/disable configuration is applied for both “Proactive approach” and “Reactive approach”.

-	Samsung thinks this depends on other discussions, and we should not agree for now.

-	Rapp thinks it is fine the consider this in later stage.

Timer related

Chair: different timer proposals will be discussed seperately, e.g., wait timer, prohibit timer, etc.

R2-2309715	Further discussion on the MUSIM temporary capability restriction	China Telecom Corporation Ltd.	discussion	Rel-18	NR_DualTxRx_MUSIM-Core

Proposal 4: Introduce the prohibit timer for MUSIM capability restriction report to avoid overloading signalling messages .

Proposal 5: Introduce the wait timer for reactive MUSIM capability restrictions reporting to address the UE behavior when no response was received from NW.

Proposal 6: When UE receives RRCReconfiguration message in which the configuration related to MUSIM capability is different from the previous one, the wait time can be stopped.

Proposal 7: In reactive approach, Upon expiry of the wait timer, UE can reduce the capability locally, FFS whether the UE can request the RRC release on network A.

R2-2311108	Timer based restriction in MUSIM	LG Electronics Deutschland	discussion	Rel-18	NR_DualTxRx_MUSIM-Core

Proposal 1.	A timer is introduced to allow the UE to perform the temporary capability restriction upon expiry of the timer.

Proposal 2.	The UE starts the timer when the UE requests a temporary restriction to the network if the timer is configured. The UE stops the timer when receiving a response to the request for a temporary restriction.

Proposal 3.	Upon the timer expiry, if the UE requests SCell release or SCG release, the UE performs the SCell release or the SCG release as requested earlier.

Proposal 4.	Upon the timer expiry, if the UE requests a MIMO capability update for maximum MIMO layer or DL/UL MIMO layer information on affected serving cell(s), the UE applies the MIMO configuration to the related serving cell(s) as requested earlier.

R2-2309890	Discussion on procedures for temporary capability restriction	Huawei, HiSilicon	discussion	Rel-18

Proposal 2: Introduce a timer for reactive approach for reporting of temporary capability restriction in UAI. The timer starts when the UE sends the UAI, and stops when a response matching the UAI from the NW is received. The timer length is configured by the NW.

Proposal 3: When the timer is running:

-	if the UE transmits the preference for SCell(s)/SCG to be affected in the UAI, and the NW provides the RRC reconfiguration or MAC CE to release or deactivate the corresponding SCell/SCG; and

-	if the UE transmits the preference for maximum MIMO layers in the UAI, and the NW provides the RRC reconfiguration with maximum MIMO layer configuration as requested in the UAI; and

-	if the UE transmits the preference for measurement gap requirement in the UAI, and the NW provides the RRC reconfiguration compatible with the UAI (i.e. RRC reconfiguration with gap configured for the measured frequencies/serving cells, which requires gap in the UAI);

-	the UE stops the timer.

Proposal 4: After the timer expires:

-	if the UE transmits the preference for SCell(s)/SCG to be released or deactivated in the UAI, the UE releases or deactivates the SCell/SCG;

-	if the UE transmits the preference for maximum MIMO layers in the UAI, the UE applies the maximum MIMO layer configuration suggested in the UAI for the corresponding serving cell or for all the BWPs of the corresponding serving cell;

-	if the UE transmits the preference for measurement gap requirement in the UAI, the UE may not perform gap-less measurement on the frequencies/serving cells if requires gap in the UAI.

Proposal 6: No prohibit timer is defined for the UAI for R18 MUSIM purpose.

Discussion on ‘wait timer’ based on the three papers above

-	Intel wonders why network does not respond in the first place. QC thinks this is useful in some cases and thus support to introduce a wait timer.

-	Ericsson asks whether UE just goes back to idle, don't think it is good to do reduced configuration.

-	Regarding the stop condition, Intel thinks ‘any reconfiguraiton’ is not a good way, since configuration can be for various purposes. LG thinks there should be some condition, e.g., the reconfiguration solves the capability restriction issue for the UE. CT thinks we can add ‘if the reconfig’ contains something related to the capabilities that UE reported restriction.

-	For behaviour upon expiry, Xiaomi do not think it is necessary to specify detailed behaviour. QC think this is necessary. ZTE also think it useful to specify detailed behaviour. CATT thinks expiry is not a normal case so prefer a simple solution. Samsung shares this view.

-	Nokia thinks if UE do something locally, e.g., releasing SCG, it should be known by the network.

-	China Telecom think it useful to specify UE behaviour than simply going to Idle. LG also think so.

-	OPPO think it is up to UE implementation and thinks some high level description is sufficient.

-	WI rapp: It is OK to stay on high level, seems to be a good compromise.

We will introduce ‘wait timer’ for the reactive approach

    - The UE starts the timer when the UE requests a temporary restriction to the network if the timer is configured. We assume network configures the length for this timer.

    - Stop: if UE receives reconfiguration that does not exceed the capabilities that UE suggested via capability restriction report

    - Expiry: UE can apply the temporary UE capability restriction upon the timer expiry.

Discussion on ‘prohibit timer’ based on the CT and Huawei papers

-	QC do not think it useful. Nokia think this is different from R17 since UE may request different things. Samsung think we can consider zero value for the timer as a compromise.

-	 HW asks whether we have this timer also for reactive case

We will introduce ‘prohibit timer’ for the proactive approach (Network can set zero value for this timer, details can be handled in spec drafting phase)

Removal of capacity restriction

R2-2309793	Procedures for MUSIM temporary capability restriction	vivo	discussion	Rel-18	NR_DualTxRx_MUSIM-Core

Proposal 4 The UE can remove the MUSIM capability restriction information by not including the detailed fields in Rel-18 MUSIM field in the UAI.

R2-2311014	Further analysis on signalling procedure for capability restriction	Nokia, Nokia Shanghai Bell	discussion

Proposal 8: Indication to the restoration of full capability is included as an additional parameter in RRC Reconfiguration completion and measurement report instead of a separate UAI for this scenario.

Proposal 9: A simple indication of the change in capability is triggered on partial removal of capability restriction to allow NW to obtain the complete restricted capability information via separate signaling.

-	Samsung want to clarify how it works in details, e.g., what if UE send another different report? Apple thinks the vivo proposal works and it is simple.

The UE can remove the MUSIM capability restriction information by not including any fields in capability restriction report (details will be handled in the specification drafting).

Early indication

R2-2310031	Early capability restriction indication in ResumeRequest	Intel Corporation	discussion	Rel-18	NR_DualTxRx_MUSIM-Core

Proposal #1: Early capability restriction indication is provided in RRC ResumeRequest messages.

Discussions:

-	Nokia do not see a need to have this early indication as in P1. QC agrees, since msg5 based mechanism is sufficient. Samsung agree as well.

-	Samsung want to clarify what is the UE behaviour if the NW provides configurations not according to its capability in this phase. ZTE think as per spec UE goes to IDLE, and also agree that msg5 is good. LGE and Ericsson also share the view of Samsung.

-	vivo think UE does not need to go to IDLE in this particular time period, and ok to capture some UE behaviour, details FFS. Intel in this case want to know how exactly we capture this. Nokia think this is a simple procedure.

-	HW think msg3 based way is already agreed in this meeting, so no need to discuss msg5 based way.

-	Intel and Apple have concern on the new UE behaviour.

Working assumption: Early capability restriction indication is provided in Msg5. Detailed UE behaviour, if any, can be further discussed.

R2-2309553	Remaining Issues on Procedures for MUSIM Temporary Capability Restriction	OPPO	discussion	Rel-18	NR_DualTxRx_MUSIM-Core

R2-2310171	Control signaling for Dual-Active MUSIM	Qualcomm Incorporated	discussion

R2-2310318	Procedures for MUSIM temporary capability restriction	Apple	discussion	Rel-18	NR_DualTxRx_MUSIM-Core

R2-2310583	Timer control for capability restriction	Xiaomi	discussion	Rel-18	NR_DualTxRx_MUSIM-Core

R2-2310592	Discussion on temporary capability restriction	Samsung	discussion

R2-2310648	Procedures for MUSIM temporary capability restriction	NEC	discussion	Rel-18	NR_DualTxRx_MUSIM-Core

R2-2310966	Indication of restricted capabilities at RRC Setup and Resume by MUSIM UE	Ericsson	discussion	Rel-18	NR_DualTxRx_MUSIM-Core	Revised

R2-2310967	Discussion on MUSIM timers	Ericsson	discussion	Rel-18	NR_DualTxRx_MUSIM-Core

R2-2311041	Consideration on the Temporary Capability Reporting procedure	ZTE Corporation, Sanechips	discussion	Rel-18	NR_DualTxRx_MUSIM-Core

R2-2311098	Supporting Proactive cases in other scenarios	LG Electronics Inc.	discussion	Rel-18	NR_DualTxRx_MUSIM-Core	R2-2308788

R2-2311099	Procedures for MUSIM temporary capability restriction	DENSO CORPORATION	discussion	NR_DualTxRx_MUSIM-Core

R2-2311107	Timer based restriction in MUSIM	LG Electronics France	discussion	Rel-18	NR_DualTxRx_MUSIM-Core	Late

R2-2311238	Indication of restricted capabilities at RRC Setup and Resume by MUSIM UE	Ericsson	discussion	Rel-18	R2-2310966

### 7.17.3	Allowed MUSIM temporary capability restrictions

Remaining aspects for the allowed capabiltity restriction reporting (e.g. which capabilities can be coordinated, how are the restrictions signalled, etc.)

Signaling design

R2-2311015	On specific capabilities for restriction and feature interworking scenarios	Nokia, Nokia Shanghai Bell	discussion

Proposal 1: Change in measurement gap requirements is reported using UAI.  The modification in gap requirement is reported as a change over the first NeedForGapInfo reported from UE.

Proposal 2: NW may optionally configure the UE to report the change in gap requirements for the frequencies /bands configured for measurements.

R2-2309843	Details of allowed MUSIM temporary capability restrictions	Huawei, HiSilicon	discussion	Rel-18	NR_DualTxRx_MUSIM-Core

Proposal 4: The UE can indicate the temporary capability restriction of measurement gap for R18 MUSIM purpose in the UAI by using the existing needForGapInfoNR.

Proposal 5: The UE indicates the change of measurement gap capability based on the current RRC configuration.

-	OPPO thinks this is only used for reactive case and the legacy mechanism works.

-	Samsung agrees with P4 in HW proposal. QC agree as well. HW thinks the legacy mechasnism is not always sufficient.

- 	Ericsson wants to add separate information in UAI to trigger the NW’s reconfig. Samsung understands this takes more time and it not necessary.

The UE can indicate the temporary capability restriction of measurement gap for R18 MUSIM purpose in the UAI by using the existing NeedForGapInfoNR.

On maximum MIMO layers/bandwidth restriction

R2-2310965	On some restricted capabilities for Rel-18 MUSIM UE	Ericsson	discussion	Rel-18	NR_DualTxRx_MUSIM-Core

Proposal 1	Rel-18 MUSIM UE indicates restricted frequencies to be avoided for serving cells based on Nw-provided candidate serving frequency list.

Proposal 2	Rel-18 MUSIM UE indicates preference on reduced max number of CCs in UL/DL

Proposal 3	Rel-18 MUSIM UE indicates preference on max number of MIMO layers per FRx and per UL/DL

R2-2311042	Consideration on the Temporary Capability Reporting	ZTE Corporation, Sanechips	discussion	Rel-18	NR_DualTxRx_MUSIM-Core

Proposal 4: Ran 2 to confirm the below agreement is only for the reactive case.

- Maximum MIMO layers/bandwidth restriction is reported per CC

Proposal 5: For the MIMO layer and Bandwidth reporting in the proactive case, if per FSPC (per cc per BC) granularity can’t be accepted because of the complexity, per Band per BC granularity can be considered.

P4:

-	HW thinks per CC is for proactive case. QC thinks it is for both proactive approach and reactive approach.

-	vivo thinks it is OK to confirm at least for reactive case, per cc granularity is supported. For the proactive case, it requires further discussion.

It is confirmed that the previous agreement that Maximum MIMO layers restriction (and bandwidth restriction, if supported) is reported per CC at least applies for the reactive approach.

Discussion on the granularity for the proactive case:

-	Ericsson thinks per CC per band is too complicated report. Samsung share this view and on the other hand the granularity is different to decide for now, i.e., after the report details for the proactive case have been discussed.

-	QC thinks per CC is OK and we can start from here.

Baseline for the proactive approach: Maximum MIMO layers restriction (and bandwidth restriction, if supported) is reported per FSPC (per cc per BC).

Further discussions on the signaling design

R2-2310090	Allowed MUSIM temporary capability restrictions	Samsung R&D Institute India	discussion	Rel-18

Proposal 4: For Rel-18 MUSIM dual active operation, UE is configured with the band-filter list by the NW A in the OtherConfig for MUSIM band conflict signalling. Existing filter for UE capability reporting could be reused for band filter.

Proposal 5: For Rel-18 MUSIM dual active operation, to address MUSIM band conflict, UE indicates its forbidden/affected UL/DL bands or band combinations based on the network configured band-filter list, in the UAI signalling to NW A.

Proposal 6: For Rel-18 MUSIM dual active operation, UE signals its temporary capabilities restictions as forbidden bands indexed to the band-filter list and/or affected bands indexed to the band-filter list along with explicit fields for restricted (lower) capabilities e.g. maximum MIMO layers.

Discussions:

-	OPPO OK with the intention, but P4 is not so clear, the part on ‘existing filter for…’.

For Rel-18 MUSIM dual active operation, UE is configured with the band-filter list by the NW A in the OtherConfig for forbidden/affected band signalling.

For Rel-18 MUSIM dual active operation, UE indicates its forbidden/affected band combinations (or band(s)) based on the network configured band-filter list, in the UAI signalling to NW A.

For Rel-18 MUSIM dual active operation, UE signals its temporary capabilities restrictions as forbidden band combinations with band indexed to the band-filter list and/or affected band combinations with band indexed to the band-filter list along with explicit fields for restricted (lower) capabilities e.g. maximum MIMO layers.

R2-2309554	Allowed MUSIM Temporary Capability Restrictions	OPPO	discussion	Rel-18	NR_DualTxRx_MUSIM-Core

R2-2309792	Discussion on temporary capability restriction for Rel-18 Multi-SIM	vivo	discussion	Rel-18	NR_DualTxRx_MUSIM-Core

R2-2310319	Aspects of allowed MUSIM temporary capability restriction	Apple	discussion	Rel-18	NR_DualTxRx_MUSIM-Core

R2-2310582	Clarification on srs-TxSwitch and MIMO-layer for MUSIM	Xiaomi, vivo	discussion	Rel-18	NR_DualTxRx_MUSIM-Core

Proposal 1: The SRS Tx switching capability restriction is reported for the restricted band.

Proposal 2: The values for SRS Tx switching capability restriction (i.e. supportedSRS-TxPortSwitch) include: {t1r2, t1r4, t2r4, t1r1, t2r2, t4r4, notSupported}.

Discussions:

-	Xiaomi think this is critical to discuss in this meeting, and think it is important to at least have some high level principle. QC agrees and think MIMO layers is in some cases not sufficient. Vivo agrees. CT agrees.

-	Samsung think this report is not needed, since we already have info related to MIMO layer, so it is not essential. Apple/Mediatek/Nokia/HW share this view.

### 7.17.4	Other

Other remaining aspects, including e.g., aspects related to the RAN4 incoming LS, and UE capabilit(ies).

This agenda item may be deprioritized in this meeting.

Gap prioirty realted

R2-2311135	Discussion on MUSIM gap priority	MediaTek Inc.	discussion

Proposal 1: RAN2 confirms that no need to request gap priority or configure gap priority for aperiodic gap. Below editor note from running CR could be removed.

•	Editor’s Note: FFS musim-GapPriorityToAddModList-r18 is for aperodic MUSIM gap.

Proposal 2: Introduce single bit indication in MUSIM assistance information to indicate the UE preference of “keep” option.

Proposal 3: Reuse existing control flag (i.e. musim-GapPriorityAssistanceConfig-r18 in running CR) to indicate whether the UE could include “keep” option for MUSIM gap.

Proposal 4: The prohibit timer configuration for R17 MUSIM gap preference (i.e. musim-GapProhibitTimer) is also apply to R18 MUSIM gap priority preference.

Discussions:

-	HW agree with all the proposals. For P2, HW wants to clarify what if the bit is absent. MTK think as per R4 it is just based on priority.

-	MTK thinks if there is no collision then no need to consider ‘keep’ or not.

RAN2 confirms that no need to request gap priority or configure gap priority for aperiodic gap. Below editor note from running CR could be removed.

- •Editor’s Note: FFS musim-GapPriorityToAddModList-r18 is for aperodic MUSIM gap.

Introduce single bit indication in MUSIM assistance information to indicate the UE preference of “keep” option.

Reuse existing control flag (i.e. musim-GapPriorityAssistanceConfig-r18 in running CR) to indicate whether the UE could include “keep” option for MUSIM gap.

The prohibit timer configuration for R17 MUSIM gap preference (i.e. musim-GapProhibitTimer) is also apply to R18 MUSIM gap priority preference.

FFS if any other configuration or related behaviour is needed.

UE Capabilities

R2-2309844	Discussion on MUSIM gaps and UE capabilities	Huawei, HiSilicon	discussion	Rel-18	NR_DualTxRx_MUSIM-Core

Proposal 6: For temporary capability restriction mechanism, the following UE capabilities are introduced in per-UE level without xDD/FRx differentiation:

-	1 optional bit to indicate the support of “proactive solution”;

-	1 optional bit to indicate the support of “reactive solution”;

-	The support of “early MUSIM indication” is optional without capability signalling, a UE supporting “early MUSIM indication” shall at least support “proactive solution” or “reactive solution”.

Discussions:

-	QC do not think this is needed. Samsung agree that there is no need to separately indicate these two. OPPO, xiaomi, intel and Ericsson agree as well.

-	HW think this helps the network to know and configure, and think this saves some interaction after this. HW think we can just postpone. HW think the 3rd bullet is agreeable.

Chair: there is wide support to have ‘single bit’ capability.

R2-2311043	Consideration on the MN-SN Coordination for the MUSIM	ZTE Corporation, Sanechips	discussion	Rel-18	NR_DualTxRx_MUSIM-Core

Proposal 1: The MN shall indicate the forbidden band entries (for the MUSIM purpose) info to the SN.

Proposal 1a: As an implementation method, the existing selectedBandEntriesMNList can be reused to include both the selected band entries by the MN and the forbidden band entries (for the MUSIM purpose).

Proposal 2: For the affected bands with restricted capabilities, the MN shall also indicate the SN about the capability restriction info if the corresponding band is allowed for the SN.

Discussions:

-	ZTE think this is critical to discuss in this meeting, and think it is important to at least have some high level principle.

-	Samsung think we just agree MN-SN coordination is needed.

For the proactive approach, the MN can indicate the forbidden/affected band information (for the MUSIM purpose) to the SN. FFS for the reactive case.

R2-2309794	Discussion on MUSIM gap priorities	vivo	discussion	Rel-18	NR_DualTxRx_MUSIM-Core

R2-2310032	Feature interaction between R17 and R18 MUSIM	Intel Corporation	discussion	Rel-18	NR_DualTxRx_MUSIM-Core

R2-2310038	Further discussion on MUSIM gap priorities 	Samsung Electronics Czech	discussion	Rel-18	NR_DualTxRx_MUSIM-Core

R2-2310584	Remaining issues of MUSIM gap	Xiaomi	discussion	Rel-18	NR_DualTxRx_MUSIM-Core

R2-2310917	MUSIM gap priority handlling	China Telecommunications	discussion

R2-2311016	MUSIM Gap collision handling	Nokia, Nokia Shanghai Bell	discussion

## 7.20	NR MIMO evolution

(NR_MIMO_evo_DL_UL-Core; leading WG: RAN1; REL-18; WID: RP-223276)

Time budget: 0.75 TU

Tdoc Limitation: 3 tdoc

### 7.20.1	Organizational

Rapporteur input (e.g., work plan, running CRs submitted by the spec rapporteurs for discussions and endorsements), incoming LS etc.

Email report from [Post123][851][MIMOevo] RRC running CR for MIMO evo (Ericsson)

R2-2309410	LS to RAN2 on CBSR for Rel-18 MIMO (R1-2308396; contact: Samsung)	RAN1	LS in	Rel-18	NR_MIMO_evo_DL_UL	To:RAN2

=> Withdrawn

R2-2309850	Running CR for TS 38.321 for MIMO Evolution	Samsung	draftCR	Rel-18	38.321	17.6.0	B	NR_MIMO_evo_DL_UL-Core

- OPPO think this version is for info and we can check later when we had more discussions.

- LG has comment on ‘per TRP’ vs ‘per serving cell’. Samsung clarifies the current behaviour captured is for the case when both TAT are expired.

Noted. Will be used as baseline for further updates.

R2-2310611	Running CR for Introduction of MIMO Evolution	Ericsson	draftCR	Rel-18	38.331	17.6.0	NR_MIMO_evo_DL_UL-Core

Noted. Will be used as baseline for further updates.

R2-2310819	Report for Post 123 MIMOevo RRC	Ericsson	discussion	NR_MIMO_evo_DL_UL-Core

Revised in R2-2311290

R2-2311290	Report for Post 123 MIMOevo RRC

Proposal 1 In IE ControlResourceset, add value “None”  to applyIndicatedTCI-State (first, second, both, none) and do not configure with followUnifiedTCI-State-r17. FFS if the same can be achieved if this field is not included at all.

Proposal 2 Configure the parameter applyIndicatedTCIState-r18 per PUCCH resource and not additionally to PUCCH groups

Proposal 4 In IE SRS-Config applyIndicatedTCIState-r18 and followUnifiedTCI-StateSRS are configured separately for r18 mTRP and r17 sTRP. -Cond is added:

FollowUTCI	The field is absent if the field followUnifiedTCI-State is present. Otherwise, it is optionally present, Need R.

Proposal 5   Do not use numberOfSDCombinations and numberOfSDCombinations-PS

Proposal 6 IN IE TDCP(new) in IE CSI-ReportConfig The value of Y can be deferred from the list length of delayDSetofLenghtY-r18

Proposal 7  Move parameter m (aperiodicResourceOffset-r18) from IE NZP-CSI-RS-ResourceSet to IE CodebookConfig-r18 under typeII-Doppler-r1 and specify relation in field description.

Proposal 9   For codebook config:

As baseline assumption CodebookConfig is critically extended.

Field description for n1-n2-codebookSubsetRestrictionList includes configuration restriction for same n1-n2 for each element.

Optionality of n1-n2-codebookSubsetRestrictionList is removed to ensure at least one element is included.

Optionality and field description need further review and it can be considered to send LS from next meeting for RAN1 to review the optionality of the parameters, FFS other things.

Proposal 10   Wait for Ran1 input for   ”applyIndicatedTCIState should be added within the PDCCH-ConfigCommon to indicate whether/which TCI state to be applied for corset 0”

In IE ControlResourceset, add value “None”  to applyIndicatedTCI-State (first, second, both, none) and do not configure with followUnifiedTCI-State-r17. FFS if the same can be achieved if this field is not included at all.

P2, P4-7, P9-P10 are taken as baseline for further stage 3 specification development.

The filed description for CSI-AssociatedReportConfigInfo will be updated, using P3 in R2-2311290 as baseline. Details to be further checked.

Chair: we can send LS to RAN1 on the RRC parameters if needed.

Proposal 3 In IE CSI-AssociatedReportConfigInfo configure:

applyIndicatedTCI-State-r18    CHOICE {

perset-r18             ENUMERATED {first, second}

perresource-r18        SEQUENCE (SIZE(1..maxNrofAP-CSI-RS-ResourcesPerSet)) OF   ENUMERATED {first, second}

qcl-info, qcl-info2

List of references to TCI-States for providing the QCL source and QCL type for each NZP-CSI-RS-Resource listed in nzp-CSI-RS-Resources of the NZP-CSI-RS-ResourceSet indicated by resourceSet within nzp-CSI-RS. Each TCI-StateId refers to the TCI-State which has this value for tci-StateId and is defined in tci-StatesToAddModList or in dl-OrJointTCI-StateList in the PDSCH-Config included in the BWP-Downlink corresponding to the serving cell and to the DL BWP to which the resourcesForChannelMeasurement (in the CSI-ReportConfig indicated by reportConfigId above) belong to. First entry in qcl-info corresponds to first entry in nzp-CSI-RS-Resources of that NZP-CSI-RS-ResourceSet, second entry in qcl-info corresponds to second entry in nzp-CSI-RS-Resources, and so on (see TS 38.214 [19], clause 5.2.1.5.1). When the UE is configured with two SRS resource sets with usage set to Codebook or nonCodebook and this field is absent for aperiodic CSI RS, the UE shall use QCL information included in the "indicated" DL only/Joint TCI state as specified in TS 38.214 or when the UE is configured with more than one value for the field coresetPoolIndex in the DL BWP used to trigger the CSI-report and this field is absent for aperiodic CSI RS, the UE shall use QCL information included in the "indicated" DL only/Joint TCI state as specified in TS 38.213 [13], clause 10.1

R2-2311127	Draft 38.300 CR for introduction of 2-TA enhancement	NTT DOCOMO, INC.	draftCR	Rel-18	38.300	17.6.0	B	NR_MIMO_evo_DL_UL-Core

- DCM explains that only the part related to 2TA has been captured. And we can send LS to RAN1 for them to check this draft CR.

-	CATT think we can have a LS to RAN1.

Noted. Will be used as baseline for further updates. Will send to RAN1 for checking.

R2-2311167	L1 parameter Excel with rapporteur comments MIMOevo	Ericsson	discussion	Rel-18	NR_MIMO_evo_DL_UL-Core

Noted

Post meeting email discussions

[Post123bis][201][MIMOevo] LS to RAN1 on Rel-18 MIMO evolution (Ericsson)

Scope: Collect and discuss potential questions to RAN1, including RRC (and MAC, if any) aspects

Intended outcome: Draft LS in R2-2311291

Deadline: Short (1 week)

[Post123bis][202][MIMOevo] Stage-2 Running CR and LS to RAN1 (Docomo)

Scope: Update and review the stage 2 running CR

Intended outcome: Stage-2 running CR in R2-2311292 for endorsement, and draft LS in R2-2311293 (to send the endorsed running CR to RAN1 for checking)

Deadline: Short (1 week)

[Post123bis][203][MIMOevo] RRC Running CR and further discussions (Ericsson)

Scope: Update and review the RRC running CR, also discussions on the RRC open issues based on the progress in this meeting

Intended outcome: RRC running CR for endorsement, and discussion report with proposals Deadline: Long (2 weeks for running CR, November 3rd for open issue)

[Post123bis][204][MIMOevo] MAC Running CR and further discussions (Samsung)

Scope: Update and review the MAC running CR, also discussions on the MAC open issues based on the progress in this meeting

Intended outcome: MAC running CR for endorsement, and discussion report with proposals

Deadline: Long (2 weeks for running CR, November 3rd for open issue)

### 7.20.2	Two TAs for multi-DCI multi-TRP

Remaining open issues on two TAs for multi-DCI multi-TRP operation

2-PTAG model and related behaviours

R2-2310587	Remaining issues for 2 TA handling of mTRP	Xiaomi	discussion	Rel-18	NR_MIMO_evo_DL_UL-Core

Proposal 1: RAN2 is kindly requested to confirm that the 2-PTAG model (i.e., both TAGs of SpCell are PTAGs) is used.

Proposal 2: When the TAT for STAG is expired and the other TAT is running for a serving cell (i.e., SCell), the UE does not perform the following behaviours for this serving cell:

	flush all HARQ buffers;

	notify RRC to release PUCCH, if configured;

	notify RRC to release SRS, if configured;

	clear any configured downlink assignments and configured uplink grants;

	clear any PUSCH resource for semi-persistent CSI reporting;

Proposal 3: when the TAT for PTAG is expired and the other TAT is running for a serving cell (SpCell or SCell), the UE does not perform the following behaviours for this serving cell:

	flush all HARQ buffers;

	notify RRC to release PUCCH, if configured;

	notify RRC to release SRS, if configured;

	clear any configured downlink assignments and configured uplink grants;

	clear any PUSCH resource for semi-persistent CSI reporting;

R2-2310847	UL time alignment in multi-DCI based multi-TRP with two TAs	InterDigital	discussion	Rel-18	NR_MIMO_evo_DL_UL-Core

Proposal 1:	Agree to the following modified working assumption from RAN2#123:

-	We will use the 2-PTAG model, i.e., both TAGs of SpCell are PTAGs;

o	When the TAT for STAG is expired and the other TAT is running for a serving cell (i.e., SCell), no impact to the TRP with running TAT; 1 and 3-7 are applied to the TRP with TAT expired,

o	when the TAT for PTAG is expired and the other TAT is running for a serving cell (SpCell or SCell), no impact to the TRP with running TAT; 1 and 3-7 are applied to the TRP with TAT expired.

R2-2311188	Handling of two TAGs associated with a Serving Cell	Nokia, Nokia Shanghai Bell	discussion	Rel-18	NR_MIMO_evo_DL_UL-Core

Proposal 1: If a TAT of a TAG associated with a Serving Cell expires (while the Serving Cell is configured with two TAGs), the UE performs all the same actions for the Serving Cell as in the legacy.

Discussions based on the above papers:

-	OPPO asks for clarification on P1 of Nokia paper and think if we go this way the benefit of 2TA is gone. LGE shares this thinking and think it is not good to release/stop all the TRPs. ZTE also think P1 is not good.

-	Nokia think TAT expiry is not a normal case so do not see a need to specify too much behaviours and want to have simple specification. Ericsson support Nokia’s proposal 1.

Chair: there is good support to specify some behaviour along the line of xiaomi/IDT proposals.

-	Apple agrees we do not need to release RRC configured resources, and think we can have the others.

-	CATT agree with Apple on 3 and 4, but also think the other actions are also beneficial. ZTE agrees as well.

-	OPPO think the agreed CFRA procedure handles the case of one TAT expires, and support to keep the HARQ buffer. ZTE agrees.

-	For 5 and 6, ZTE wonders what is the UE behaviour if we keep these configured resources.

-	LGE not sure if HARQ buffer can be flushed per TRP, so support to keep it.

-	Nokia thinks it is acceptable not to flush HARQ buffers, but the other actions are needed.

Confirmed: We will use the 2-PTAG model, i.e., both TAGs of SpCell are PTAGs;

Chair: can we take the following as a possible compromised wayforward

-	Ericsson wonders if these are totally different than today’s MAC procedure, i.e., how we manage the TAT/TAG.

-	Samsung thinks there may some difficulties to capture the release of the RRC configured resources in the MAC.

-	OPPO and LGE are not sure about how to capture the actions per TRP.

The following are taken as baseline

- When the TAT for STAG is expired and the other TAT is running for a serving cell (i.e., SCell), no impact to the TRP with running TAT; 1 and 3-7 are applied to the TRP with TAT expired, i.e., 2 is not applied.

- when the TAT for PTAG is expired and the other TAT is running for a serving cell (SpCell or SCell), no impact to the TRP with running TAT; 1 and 3-7 are applied to the TRP with TAT expired, i.e., 2 is not applied.

TAG ID indication in RACH procedure (intra-cell case)

R2-2309690	TAG modeling and TAT expiry in 2TAs mTRP	LG Electronics Inc.	discussion	Rel-18	NR_MIMO_evo_DL_UL-Core

Proposal 5. One R bit in Absolute TAC MAC CE is used to indicate TAG ID, i.e. which TAG’s TA is updated.

Proposal 6. One R bit in RAR is used to indicate TAG ID, i.e. which TAG’s TA is updated.

Discussions on P5:

-	ZTE think it is good to use two bits for this, but OK to have 1 bit only.

-	OPPO think the discussions are about connected state and think P5 is according to R1 agreements.

-	Nokia think P5 and P6 should be both agreed together.

One R bit in Absolute TAC MAC CE is used to indicate TAG ID, i.e. which TAG’s TA is updated.

Discussions on P6:

- 	LGE explains there are the cases when NW cannot know which UEs are legacy or new UEs supporting 2TA.

-	CATT think legacy UE just ignores this bit so no problem. For UEs support 2TAs it can be solved by NW implementation, so think it is OK to have this baseline agreed.

-	ZTE do not want to have restriction to NW implementation and think it is not easy to release.

-	OPPO think for IDLE there is only one TAG and think NW always set the bit to zero, and wonders how NW set the bit properly for different cases/UEs. Nokia think for IDLE case network can set the bit to 0 or 1, so there is no problem for legacy UEs. QC agree to keep the baseline, and do not think this is huge restriction to NW implementation.

-	Nokia think it is possible for NW to configure the TAG IDs so not big issue. And think in RRC we can configure which TAG is mapped to which reserved bit. HW thinks this is only feasible if we have the related configuration in the RRC. OPPO wants to agree on a complete solution, so do not think it can be confirmed as agreement for now.

-	Ericsson think this baseline is OK and agree with HW to discuss the RRC part of the solution.  CATT agree.

The baseline is confirmed as agreement: One R bit in RAR is used to indicate TAG ID, i.e. which TAG’s TA is updated. FFS if the association between the TAGs and value of the R bit (0 or 1) need to be configured by RRC.

TAG ID indication in RACH procedure (inter-cell case)

R2-2311003	Remaining issues on 2TA for mTRP	Huawei, HiSilicon	discussion	Rel-18	NR_MIMO_evo_DL_UL-Core

Proposal 4: For inter-cell multi-DCI based multi-TRP operation, TAG ID is not included in the RAR and absolute TA command MAC CE, i.e. the UE can know the TAG information based on the RACH configuration which the UE has used.

R2-2309913	Discussions on Two TAs for Multi-DCI Multi-TRP	CATT	discussion	Rel-18	NR_MIMO_evo_DL_UL-Core

Proposal 3: For the inter-cell scenario, reuse the mechanism agreed for intra-cell case, i.e., use the RA RAR to indicate the TAG.

Proposal 4: Using one reserved bit to indicate the TAG ID via the Absolute TA Command MAC CE.

RRC aspects (e.g., PRACH configuration for inter-cell CFRA)

R2-2311251	On 2TA operation RRC parts	Ericsson	discussion	Rel-18	NR_MIMO_evo_DL_UL-Core

Proposal 5	Do not extend RACH-Configdedicated for additionalPCI.

Proposal 6	RAN2 extend the PRACH configuration in the BWP-UplinkCommon.

Proposal 7	RAN2 to adopt as content for RACH configuration per additionaPCI(7 of these) IE RACH-ConfigGeneric, ssb-perRACH-OccasionAndCB-PreamblesPerSSB and prach-RootSequenceIndex

Discussions

-	CATT think it should be in the dedicated BWP configuration. And for P7, think there is no need to include RootSequenceIndex.

-	Ericsson not sure if we directly follow the conclusion from the mobility topic.

-	Ericsson think it can also be put in the ServingCellConfig.

-	OPPO agree with P6 and think it is cell level configuration. CATT think this does not work.

-	On P7, OPPO wonders if we can just reuse the legacy configuration of RootSequenceIndex. Ericsson think it is a lot and there is possibility of repeated configurations / more overhead.

Chair: we can discuss in a post meeting email discussion.

We will discuss P5-P7 in the email discussions.

R2-2309665	Discussion on multiple TAG	OPPO	discussion	Rel-18	NR_MIMO_evo_DL_UL-Core

R2-2309912	Consideration on the TAG Indication upon CBRA	CATT	discussion	Rel-18	NR_MIMO_evo_DL_UL-Core

R2-2309953	Discussion on the impacts of Two TAs for multi-DCI multi-TRP operation	Lenovo	discussion	Rel-18

R2-2309954	Discussion on the UE-initiated RACH procedure in multi-TRP operation	Lenovo	discussion	Rel-18

R2-2309955	Consideration on RLF in multi-TRP operation	Lenovo	discussion	Rel-18

R2-2310063	Open issues on two TAs for multi-DCI multi-TRP	Samsung Research America	discussion	Rel-18	NR_MIMO_evo_DL_UL-Core

R2-2310101	Further Discussion on RACH for mDCI mTRP with 2TA enhancement	ZTE Corporation,Sanechips	discussion	Rel-18	NR_MIMO_evo_DL_UL-Core

R2-2310103	On RRC parameter for mDCI mTRP with 2TA enhancement	ZTE Corporation,Sanechips	discussion	Rel-18	NR_MIMO_evo_DL_UL-Core

R2-2310193	Discussion on two TAs for multiple TRPs	SHARP Corporation	discussion	NR_MIMO_evo_DL_UL-Core

R2-2310315	Support of Two TAs for multi-DCI multi-TRP	Apple	discussion	Rel-18	NR_MIMO_evo_DL_UL-Core

R2-2310809	Discussion on multi-DCI multi-TRP with two TAs	Qualcomm Incorporated	discussion	NR_MIMO_evo_DL_UL-Core

R2-2310932	MAC issues for multi-DCI multi-TRP with two TAs	Ericsson	discussion	Rel-18	NR_MIMO_evo_DL_UL-Core

´: Revised in R2-2311252

R2-2311252	MAC issues for multi-DCI multi-TRP with two TAs	Ericsson	discussion	Rel-18	NR_MIMO_evo_DL_UL-Core

R2-2311125	Current status of functional issues on 2TA enhancement	NTT DOCOMO, INC.	discussion	Rel-18

R2-2311169	On 2TA operation RRC parts	Ericsson	discussion	Rel-18	NR_MIMO_evo_DL_UL-Core

´: Revised in R2-2311251

R2-2311187	RA procedure while SpCell is configured with 2 TAGs	Nokia, Nokia Shanghai Bell	discussion	Rel-18	NR_MIMO_evo_DL_UL-Core

### 7.20.3	Unified TCI extension to mTRP operation

Remaining open issues on unified TCI extension to mTRP operation, including the cases for sDCI and mDCI

sDCI

R2-2309849	Remaining Issues on Single-DCI based unified TCI extension to multi-TRP operation	Samsung	discussion	Rel-18	NR_MIMO_evo_DL_UL-Core

Proposal 1: RAN2 confirm that separate MAC CEs on the enhanced unified TCI state for Single-DCI based multi-TRP operation are introduced for joint TCI State and separate DL/UL TCI States, respectively.

Proposal 2: The simultaneous TCI state update for the RRC configured serving cell sets are supported in Rel-18 enhanced unified TCI state activation/deactivation MAC CE(s) for multi-TRP operations.

-	Samsung clarifies that there is no real issue if we use separate MAC CEs for the joint and separate cases.

R2-2310906	Design of MAC CE for Rel-18 MIMO	Nokia. Nokia Shanghai Bell	discussion	Rel-18	NR_MIMO_evo_DL_UL-Core

Proposal 1: Reuse MAC CE for unified TCI state activation as defined TS38.321 clause 6.1.3.47 for mDCI by replacing one R-bit with CORESET pool index (e.g. as shown below).

Proposal 2: Define new MAC CE (with eLCID) for joint TCI state activation with sDCI (e.g. as shown below).

Proposal 3: Define another new MAC CE (with eLCID) for separate TCI state activation with sDCI (e.g. as shown below).

RAN2 confirm that separate MAC CEs on the enhanced unified TCI state for Single-DCI based multi-TRP operation are introduced for joint TCI State and separate DL/UL TCI States, respectively.

The current running CR for MAC spec is used as the base line. Details can be further discussed.

R2-2309666	Discussion on MAC CE design for mTRP	OPPO	discussion	Rel-18	NR_MIMO_evo_DL_UL-Core

R2-2309691	Discussion on Unified State MAC CE for mDCI	LG Electronics Inc.	discussion	Rel-18	NR_MIMO_evo_DL_UL-Core

R2-2309693	Discussion on Unified State MAC CE for sDCI	LG Electronics Inc.	discussion	Rel-18	NR_MIMO_evo_DL_UL-Core

R2-2309743	Discussion on MAC CE design for MTRP	CEWiT	discussion

R2-2309914	Discussion on Unified TCI Framework Extension for sDCI and mDCI based Multi-TRP	CATT	discussion	Rel-18	NR_MIMO_evo_DL_UL-Core

R2-2310102	Further DIscussion on remaining UP issues for MIMO-evo	ZTE Corporation,Sanechips	discussion	Rel-18	NR_MIMO_evo_DL_UL-Core

R2-2311004	Extension of unified TCI framework for mTRP	Huawei, HiSilicon	discussion	Rel-18	NR_MIMO_evo_DL_UL-Core

R2-2311128	Discussion on remaining issues on Unified TCI framework extension	NTT DOCOMO INC.,	discussion	Rel-18

### 7.20.4	Other

Other issues if not covered by 7.20.1, 7.20.2, and 7.20.3.

Depending on the number of contributions/proposals, a summary of this agenda item may be used.

R2-2311005	Overlapping UL grants handling for STxMP	Huawei, HiSilicon	discussion	Rel-18	NR_MIMO_evo_DL_UL-Core

R2-2311130	Discussion on overlapping UL grants in STxMP	NTT DOCOMO, INC.	discussion	Rel-18

R2-2311165	On other than 2TA parameters MIMOevo Rel18	Ericsson	discussion	Rel-18	NR_MIMO_evo_DL_UL-Core

Proposal 1	RAN2 to discuss how to implement the additional channel resources for TDCP and use the above TP as baseline.

Proposal 2	RAN2 to discuss if parameters cmrDopplerK-r18 and cmrCJT-K-r18 are needed or not, and whether these are specified as configuration limitations in TS 38.331 and in which IE.

Proposal 3	RAN2 to discuss need codes and field descriptions for tci-SelectionPresentIn-DCI and applyIndicatedTCI-StateDCI-1-0.

Proposal 4	RAN2 to discuss whether the current field description and ASN1 is ok or the restrictions should be hardcoded/modified.

Proposal 5	RAN2 to ask RAN1 to provide the exact values for the CBSR fields for Release-18.

Proposal 6	RAN2 to discuss how to enable unified TCI state configuration for mDCI mTRP in Release-18.

- Ericsson think some questions need to be asked to RAN1 and suggest to have email discussions.

P6:

-	Chair: do we need to do other things than simply dropping this restriction sentence?

-	Ericsson think this need careful checking. HW think this related to UE capability.

RRC configuration restriction that ‘The network does not configure the field in a serving cell that is configured with more than one value for the coresetPoolIndex’ for unifiedTCI-StateType need to be removed. FFS how, and FFS if this also impact other conditions and configurations.

R2-2310064	Open issues on MIMO RRC parameters	Samsung Research America	discussion	Rel-18	NR_MIMO_evo_DL_UL-Core

Proposal 1: Capture the restriction on the number of NZP-CSI-RS resources configured in a resource set in the field description of nzp-CSI-RS-Resources (i.e., K=1,2,3,4 for CJT/CJT-PS, K=1 for P/SP NZP CSI-RS resource and K=4,8,12 for AP NZP CSI-RS resources for Doppler/Doppler-PS), instead of introducing numberOfCMR-r18, cmrCJT-K-r18, or cmrDopplerK-r18.

Proposal 2: Clarify in the field description that the RI restriction is applied to all NZP-CSI-RS resources in a resource set for Rel-18 CJT and CJT-PS.

Proposal 3: For n1-n2-codebookSubsetRestrictionList-r18, clarify in the field description that

1) the elements in n1-n2-codebookSubsetRestrictionList-r18 are configured with same n1-n2 value;

2) n1-n2-codebookSubsetRestrictionList-r18 always includes at least one element.

Proposal 4: Discuss if to capture in RRC for CJT and CJT-PS codebooks that 10 and 01 are not configured for amplitude restriction in n1-n2-codebookSubsetRestrictionList-r18 as specified in TS 38.214 clause 5.2.2.2.8 and 5.2.2.2.10.

Proposal 5: For enhanced DMRS type selection in DL and UL, capture in RRC the following description and Need code S.

•	If the field is absent, the UE uses DMRS type 1 or DMRS type 2 depending on dmrs-Type.

•	If the field is present, the UE uses DMRS eType 1 if dmrs-Type is absent. If the field is present, the UE uses DMRS eType 2 if dmrs-Type is present.

Discussions

-	Ericsson think P2/P3/P5 are already included in the current RRC running CR.

-	Samsung think P1 and P4 can be discussed further in the email disc.

We will discuss P1 and P4 in the email discussion.

## List of post meeting email discussions

Rel-18 MIMO evo

[Post123bis][201][MIMOevo] LS to RAN1 on Rel-18 MIMO evolution (Ericsson)

Scope: Collect and discuss potential questions to RAN1, including RRC (and MAC, if any) aspects

Intended outcome: Draft LS in R2-2311291

Deadline: Short (1 week)

[Post123bis][202][MIMOevo] Stage-2 Running CR and LS to RAN1 (Docomo)

Scope: Update and review the stage 2 running CR

Intended outcome: Stage-2 running CR in R2-2311292 for endorsement, and draft LS in R2-2311293 (to send the endorsed running CR to RAN1 for checking)

Deadline: Short (1 week)

[Post123bis][203][MIMOevo] RRC Running CR and further discussions (Ericsson)

Scope: Update and review the RRC running CR, also discussions on the RRC open issues based on the progress in this meeting

Intended outcome: RRC running CR for endorsement, and discussion report with proposals Deadline: Long (2 weeks for running CR, November 3rd for open issue)

[Post123bis][204][MIMOevo] MAC Running CR and further discussions (Samsung)

Scope: Update and review the MAC running CR, also discussions on the MAC open issues based on the progress in this meeting

Intended outcome: MAC running CR for endorsement, and discussion report with proposals

Deadline: Long (2 weeks for running CR, November 3rd for open issue)

Rel-18 MU-SIM

[Post123bis][205][MUSIM] RRC Running CR and further discussions (vivo)

Scope: Update and review the RRC running CR, also discussions on the RRC open issues based on the progress in this meeting

Intended outcome: RRC running CR for endorsement, and discussion report with proposals

Deadline: Long (2 weeks for running CR, November 3rd for open issue)

[Post123bis][206][MUSIM] Stage 2 Running CR (China Telecom)

Scope: Update and review the 38.300 running CR

Intended outcome: 38.300 running CR in R2-2311294 for endorsement

Deadline: Short (1 week)

[Post123bis][207][MUSIM] 37.340 Running CR (ZTE)

Scope: Update and review the 37340 running CR

Intended outcome: 37.340 running CR in R2-2311295 for endorsement

Deadline: Short (1 week)