3GPP TSG RAN WG2#123bis	R2-2309427

Xiamen, China, 9th - 13th October 2023

3GPP TSG RAN WG1 Meeting #114	R1-2308644

Toulouse, France, August 21-25, 2023

Title:	Reply LS on PRU Procedures

Response to:	R1-2301915 (S2-2303861)

Release:	Rel-18

Work Item:	NR_pos_enh2-Core, 5G_eLCS_Ph3

Source:	RAN1

To:	SA2, RAN2, RAN3

Cc:	RAN4

Contact person:

Name:	Ren Da

E-mail Address: renda@catt.cn

Send any reply LS to:	3GPP Liaisons Coordinator, mailto:3GPPLiaison@etsi.org

Attachments:	None

# Overall Description

RAN1 thanks SA2 for their LS on PRU procedures, which includes the following request:

SA2 request:

Regarding the procedure for location of a target UE using PRUs (clause 6.x.4 of the attached CR), SA2 noticed that RAN had agreements to support some positioning methods that require simultaneous measurements of the target UE and PRU. In the SA2 agreed procedures, it is assumed that the LMF can make the decision and realize such simultaneous measurements by using a common scheduled location time. SA2 would like to seek RAN1 and RAN2’s feedback on such assumption and whether additional enhancement would be needed to support the simultaneous measurements for the target UE and PRU. SA2 would update the specification according to the RAN1 and RAN2 feedbacks if necessary.

Regarding above request, RAN1 has provided the initial response to SA2 request in R1- 2302146, which is copied in the following:

RAN1 reply:

Current RAN1 specifications do not support a mechanism to ensure simultaneous measurements/transmissions (e.g. in the same slot(s)) for multiple UEs, including a target UE and a PRU. RAN1 will continue discussions on what enhancements to LPP, NRPPa, and/or RAN signaling are necessary to support simultaneous measurements of the same DL-PRS for multiple UEs, including a target UE and a PRU; and to support simultaneous transmission of SRS for multiple UEs, including a target UE and a PRU. Note: The enhancements might or might not have RAN1 specification impact.

In above reply, RAN1 indicated that RAN1 would continue the discussions on the enhancements that are necessary to support simultaneous measurements of the same DL-PRS for multiple UEs, and the simultaneous transmission of SRS for multiple UEs, including a target UE and a PRU.

In this LS, RAN1 informs SA2 that RAN1 has continued the discussion, and reached the following agreements related to the support of simultaneous measurements of the same DL-PRS for multiple UEs and the simultaneous transmission of SRS for multiple UEs, including a target UE and a PRU.

RAN1#112-bis Agreements Agreement Introduce DL reference carrier phase (DL RSCP) and NR DL reference carrier phase difference (DL RSCPD) as DL carrier phase measurements. - Note: It is up to RAN4 to decide whether and how to define the requirements for DL RSCP and/or DL RSCPD. No LS needed to RAN4 for this note. - DL RSCP can be reported together with UE Rx – Tx time difference measurement - DL RSCPD can be reported together with RSTD measurement - FFS: details on how to eliminate unknown initial Rx phase with RSCP/RSCPD reporting can be further discussed - Note: Whether to support standalone DL RSCP and/or DL RSCPD reporting, or DL RSCP/DL RSCPD reporting with other new types of measurements (if agreed), can be further discussed. Agreement Support one of the following options for the definition of the reference point of the UE/TRP carrier phase measurements (down-selection in RAN1#113). - Option 1: - The reference point of the UE carrier phase measurements is defined the same as the reference point of RSTD for frequency range 1 and frequency range 2. - The reference point of the TRP carrier phase measurements is defined the same as the reference point of RTOA for frequency range 1 and frequency range 2. - Note: It is up to UE/TRP’s implementation on how to map the carrier phase to the reference point for reporting. - Option 2: - The reference point of the UE/TRP carrier phase measurements is defined as the antenna phase center of the UE/TRP Rx antenna for frequency range 1 and frequency range 2. - UE/TRP should provide the antenna phase center offset (PCO), i.e., the relative position between the antenna phase center and the antenna connector to LMF - FFS: the more details of the PCO reporting, e.g., in LCS or GCS frame Agreement To enable simultaneous transmission of UL SRS for positioning by a target UE and a PRU, support the following enhancements: - Enabling LMF to request the serving gNB of a UE to configure the transmission of the [indicated] UL SRS resources from the UE within indicated time window(s). - FFS: the details of the time window, e.g., the start time, duration, periodicity for the time window(s), within the vicinity of a reference SRS configuration or use the existing message of Scheduled Location time - Enabling LMF to request the serving gNB and neighboring gNBs of the UE to measure the [indicated] UL SRS resources from the UE within indicated time window(s). - Note: this may be a different indicated time window Agreement To enable simultaneous measurements on same DL PRS by a target UE and a PRU, support the following enhancements: - Enabling LMF to request the UEs, including target UE and PRU(s), to perform measurements on [indicated] DL PRS resources occurring within indicated time window(s). - FFS: the details of the configuration of the indicated time window(s), e.g., the start time, duration, periodicity for the time window(s), as well as the relationship with the Scheduled Location time. Agreement Support the reuse of existing physical layer procedures for DL positioning (e.g., DL-TDOA) with the necessary enhancements in measurement configuration, request and report (e.g., adding the configuration related to the NR DL CPP) for both UE-based and UE-assisted NR DL carrier phase positioning, including - UE in RRC_CONNECTED state with measurement gap. - FFS: UE in RRC_CONNECTED state without measurement gap - UE in RRC_ INACTIVE state Agreement The specific RF frequency associated with a DL carrier phase measurement is defined as the center frequency of the DL PFL by default. - Note: It is open to further discussion whether a frequency other than the center frequency of the DL PFL can also be the specific RF frequency for non-default case(s), if RAN1 agrees to introduce them. Agreement The specific RF frequency associated with a UL carrier phase measurement is defined, by default, as the center frequency of the transmission bandwidth of the SRS for positioning purpose. - Note: It is open to further discussion whether a frequency other than the center frequency of the UL carrier can also be the specific RF frequency for a non-default case(s), if RAN1 agrees to introduce them. Agreement - Support enabling a TRP to report UL RSCP together with RTOA and/or gNB Rx-Tx time difference measurements to LMF - Note 1: The report of UL carrier phase measurement with gNB Rx – Tx time difference does not necessarily require the report of DL carrier phase measurement with UE Rx – Tx time difference. - Note 2: This doesn’t preclude standalone UL carrier phase measurements reporting. Agreement Further study whether and how to support a UE/TRP to report the carrier phase measurement quality indication for corresponding the phase measurements. Agreement For NR UL carrier phase positioning for UE in RRC_CONNECTED and RRC_INACTIVE states, support reuse of existing physical layer procedures for UL positioning (e.g., UL-TDOA), with necessary enhancements in the measurement configuration, measurement request and measurement report (e.g., the configuration related to the NR UL CPP). - FFS: the details of the enhancements. Agreement Adopt one of the following options for a timestamp associated with a reported RSCP/RSCPD measurement (make the decision in RAN1#113): - Option 1: - NR-TimeStamp, currently defined in TS 37.355, is reused as the timestamp with the granularity of a slot. - FFS: Whether to clarify in the specification the reported RSCP/RSCPD value presents the RSCP/RSCPD of a specific OFDM symbol within the slot identified by the NR-TimeStamp. - Option 2: - NR-TimeStamp, currently defined in TS 37.355, should be enhanced to include the OFDM symbol index in a slot, as the timestamp for RSCP/RSCPD measurements. Agreement To address the impact of the phase delays on Tx/Rx RF chains, support one or more of the following options (down-selection in RAN1#113): - Option 1a: introduce the definition of UE/TRP Tx/Rx phase error groups (PEGs) for the Tx/Rx of DL PRS/UL SRS signals - Rel-17 definitions of UE/TRP Tx/Rx TEGs can be used as the starting point for defining UE/TRP Tx/Rx PEGs. - FFS: the details of \the UE/TRP Tx/Rx PEGs - Option 1b: Introduce Tx/Rx RF antenna IDs or Tx/Rx RF chain IDs to identify the individual Tx/Rx RF chains for transmitting/receiving the DL PRS/UL SRS signals. - FFS: the details of the Tx/Rx RF antenna IDs or Tx/Rx RF chain IDs - Note: Device transmitting PRS or positioning SRS provides Tx antenna ID or Tx Chain ID. Device receiving PRS or positioning SRS provides Rx antenna ID or Rx Chain ID. - Option 1c: introduce the report of ARP ID for the Rx/Tx of DL PRS/UL SRS signals. - The transmission/reception associated with the same ARP ID is assumed from the same ARP. - FFS: the maximum number of ARP IDs. - Option 2: reuse or enhance the existing Rel-17 definitions of UE/TRP Tx/Rx TEGs with smaller margin value. - Option 3: RAN1 sends an LS to RAN4, requesting RAN4 to consider whether there is a need to define the new UE/TRP Tx/Rx phase error groups (PEGs), introduce new IDs (e.g., Tx/Rx RF antenna IDs ) to present the phase delays for the Tx/Rx of DL PRS/UL SRS signals, or reuse or enhance the existing Rel-17 definitions of UE/TRP Tx/Rx TEGs with smaller margin value, and provide the definitions if RAN4 decides it is needed.

RAN1#113 Agreements Agreement Adopt the following modifications on the agreements made in RAN1#112bis-e: To enable simultaneous transmission of UL SRS for positioning by a target UE and a PRU, support the following enhancements: - Enabling LMF to request the serving gNB of a UE to configure the transmission of the UL SRS resources from the UE within indicated time window(s). - FFS: the details of the time window, e.g., the start time, duration, periodicity for the time window(s), within the vicinity of a reference SRS configuration or use the existing message of Scheduled Location time - Enabling LMF to request the serving gNB and neighboring gNBs of the UE to measure the UL SRS resources from the UE within indicated time window(s). - Note: this may be a different indicated time window To enable simultaneous measurements on same DL PRS by a target UE and a PRU, support the following enhancements: - Enabling LMF to request the UEs, including target UE and PRU(s), to perform measurements on indicated DL PRS resource sets occurring within indicated time window(s). - FFS: the details of the configuration of the indicated time window(s), e.g., the start time, duration, periodicity for the time window(s), as well as the relationship with the Scheduled Location time. Agreement For UE-based carrier phase positioning, support enabling LMF to forward the DL carrier phase measurement reported by a PRU, with additional information of the same PRU to a target UE for UE-based carrier phase positioning in the positioning assistance data. - Note: Whether the forwarded DL carrier phase measurement is DL RSCP and/or DL RSCPD depends at least on which of them is (are) supported by UE capability. - additional information of the same PRU includes at least PRU location. - FFS: additional PRU information, e.g. the AoD of PRU to each TRP, etc. Working assumption - To enable LMF to optionally request the serving gNB of a UE to configure the transmission of the UL positioning SRS resources from the UE within indicated time window(s), support: - Option 1D: Each of the time windows is defined with the following parameters: - The start of the time window, which is indicated by a combination of subframe number, slot offset and symbol index with respect to the SFN initialization time - The duration of the time window, which is given by a number of consecutive slots/symbols - FFS: the number of the consecutive slots/symbols - (Optional) The periodicity of the time window, which is defined similar to IE PeriodicitySRS in “Requested SRS Transmission Characteristics” in TS 38.455. - FFS: the maximum number of the windows Agreement To enable LMF to request the serving gNB and neighboring gNBs of a UE to measure the UL SRS resources from the UE within indicated time window(s), each time window is defined with the following parameters: - The start of the time window, which is indicated by a combination of subframe number, slot offset and symbol index with respect to the SFN initialization time - The duration of the time window, which is given by a number of consecutive slots/symbols - FFS: the number of consecutive slots/symbols - (Optional) The periodicity of the time window, which is defined similar to IE Measurement Periodicity in MEASUREMENT REQUEST in TS 38.455. - FFS: the maximum number of the windows Agreement To enable LMF to request the UEs, including target UE and PRU(s), to perform measurements on indicated DL PRS resource set(s) occurring within indicated time window(s), each time window is defined with the following parameters: - The start of the time window, which is indicated by a combination of subframe number, slot offset and symbol index - The duration of the time window, which is given by a number of consecutive slots/symbols - FFS: the number of consecutive slots/symbols - (Optional) The periodicity of the time window, which is defined similar to IE NR-DL-PRS-Periodicity-and-ResourceSetSlotOffset in TS 37.355.FFS: the maximum number of the windows

RAN1#114 Agreements: Agreement - Confirm the following working assumption with modification made in RAN1#113: Working assumption - To enable LMF to optionally request the serving gNB of a UE to configure the transmission of the UL positioning SRS resources from the UE within indicated time window(s), support: - Option 1D: Each of the time windows is defined with the following parameters: - The start of the time window, which is indicated by a combination of system frame number, slot offset and symbol index with respect to the SFN initialization time - The duration of the time window, which is given by a number of consecutive slots/symbols - FFS: the number of the consecutive slots/symbols - (Optional) The periodicity of the time window, which is defined similar to IE PeriodicitySRS in “Requested SRS Transmission Characteristics” in TS 38.455. - FFS: the maximum number of the windows Agreement - When a LMF requests the serving gNB of a UE to configure the transmission of the UL positioning SRS resources from the UE within indicated time window(s), - the duration of a time window can be configured by one of the following values: - {1, 2, 4, 8, 12} OFDM symbols - {1, 2, 4, 6, 8, 12, 16} slots - FFS: additional values - the number of the time windows can be configured as: - {1, 2, …, 16} Agreement When a LMF requests the serving gNB and neighboring gNBs of a UE to measure the UL SRS resources from the UE within indicated time window(s): - The duration of a time window can be configured as follows: - {1, 2, 4, 6, 8, 12, 16} slots. - the number of the time windows can be: - {1, 2, …, 16} Agreement When an LMF requests the UEs, including target UE and PRU(s), to perform measurements on indicated DL PRS resource set(s) occurring within indicated time window(s) - The duration of a time window can be configured as follows: - {1, 2, 4, 6, 8, 12, 16} slots. - the number of the time windows can be: - {1, 2} - FFS: {4, 8}

Furthermore, it is RAN1’s understanding that the simultaneous measurements/transmissions for multiple UEs, including a target UE and a PRU, is applicable to RAN1’s on-going work related to NR carrier phase positioning, and is also applicable to the remaining uplink and downlink positioning measurements and methods. Therefore, RAN1’s agreements related to the NR carrier phase positioning are provided in Section 4 in this LS for information.

# Actions

To SA2:

Action: RAN1 respectfully asks SA2 to take the above reply into account for future work.

To RAN2/RAN3:

Action: RAN1 respectfully asks RAN2/RAN3 to work on the necessary higher-layer signalling support, and provide feedback if there is any concern.

# Dates of Next TSG-RAN1 Meetings:

RAN1 #114-bis	9 Oct – 13 Oct 2023	Xiamen, China

RAN1 #115	13 Nov – 17 Nov 2023	Chicago, US

# RAN1’s agreements of NR carrier phase positioning

RAN1#112:

Agreement To enable UE-based and UE-assisted NR carrier phase positioning (CPP), one or both of the following new measurements should be introduced: - DL carrier phase (CP), which is obtained by a UE measuring the DL PRS signal(s) from a TRP. - FFS: The detailed definition of the DL CP - DL carrier phase difference (CPD), which is the difference of two DL CPs from two TRPs - FFS: The detailed definition of the DL CPD To enable NG-RAN node-assisted NR carrier phase positioning (CPP), the following new measurement should be introduced: - UL carrier phase (CP), which is obtained by a TRP measuring the UL SRS for positioning or MIMO SRS from a UE. - FFS: The detailed definition of the UL CP Agreement NR DL reference signal carrier phase (RSCP) (of i-th path) is defined as the phase of the channel response at the i-th path delay derived from the resource elements (REs) that carry the DL PRS signals configured for the measurement. A RSCP is associated with a specific RF frequency. - FFS: the reference point of the RSCP - FFS: whether/how the measurement timing is defined - Note: the i-th path is used for the sake of definition, whether only the first path or additional paths will be supported is subject to further discussion - Note: Whether to capture the above definition into TS 38.215 depends on whether RAN1 decides to introduce DL carrier phase measurement for NR CPP Agreement For NR DL reference signal carrier phase difference (RSCPD) measurement for NR CPP, the RSCPD is defined as the difference of RSCPs measured from the DL PRS signals from target TRP and reference TRP. - FFS: whether/how to define per path RSCPD - Note: Whether/how to capture the above definition into TS 38.215 depends on whether RAN1 decides to introduce DL carrier phase difference measurement for NR CPP Agreement For NR carrier phase positioning, at least support the following approach: enable a UE/TRP to report carrier phase measurements together with the legacy positioning measurements to LMF - FFS: which legacy positioning measurements among RSTD, RTOA, UE Rx-Tx time difference measurements, gNB Rx-Tx time difference measurements Agreement NR UL reference signal carrier phase (RSCP) (of i-th path) is defined as the phase of the channel response at the i-th path delay derived from the resource elements (REs) that carry the UL SRS signal for positioning purpose configured for the measurement. A UL RSCP is associated with a specific RF frequency. - FFS: the reference point of the UL RSCP - FFS: whether/how the measurement timing is defined - Note: the i-th path is used for the sake of definition, whether only the first path or additional paths will be supported is subject to further discussion - Note: The support of MIMO SRS for positioning is transparent to UE Agreement To support NR carrier phase positioning, further consider the following options: - Option 1: Support a UE/TRP to report the carrier phase measurements of more than one frequency within a PFL/carrier to LMF - NOTE: the frequency can be the carrier frequency or the frequency of a subcarrier - FFS: the details of reporting, e.g., the maximum number of reported frequencies within a PFL/ carrier - Option 2: Introduce and report a new type of UE/TRP measurement based on carrier phase differentials across multiple subcarriers within a PFL/carrier - NOTE: carrier phase differentials across multiple subcarriers within a carrier can be related to time of arrival - Option 3: Support a UE/TRP to optionally report an estimated integer ambiguity and/or search range of the integer ambiguity to LMF - Option 4: Support LMF to provide the expected integer ambiguity range at least for UE-based NR CPP in the positioning assistance data. Agreement Rel-17 LOS/NLOS indication (when indicated) applies for the carrier phase measurement(s) in the same report.

RAN1#112-bis:

Agreement Introduce DL reference carrier phase (DL RSCP) and NR DL reference carrier phase difference (DL RSCPD) as DL carrier phase measurements. - Note: It is up to RAN4 to decide whether and how to define the requirements for DL RSCP and/or DL RSCPD. No LS needed to RAN4 for this note. - DL RSCP can be reported together with UE Rx – Tx time difference measurement - DL RSCPD can be reported together with RSTD measurement - FFS: details on how to eliminate unknown initial Rx phase with RSCP/RSCPD reporting can be further discussed - Note: Whether to support standalone DL RSCP and/or DL RSCPD reporting, or DL RSCP/DL RSCPD reporting with other new types of measurements (if agreed), can be further discussed. Agreement Support one of the following options for the definition of the reference point of the UE/TRP carrier phase measurements (down-selection in RAN1#113). - Option 1: - The reference point of the UE carrier phase measurements is defined the same as the reference point of RSTD for frequency range 1 and frequency range 2. - The reference point of the TRP carrier phase measurements is defined the same as the reference point of RTOA for frequency range 1 and frequency range 2. - Note: It is up to UE/TRP’s implementation on how to map the carrier phase to the reference point for reporting. - Option 2: - The reference point of the UE/TRP carrier phase measurements is defined as the antenna phase center of the UE/TRP Rx antenna for frequency range 1 and frequency range 2. - UE/TRP should provide the antenna phase center offset (PCO), i.e., the relative position between the antenna phase center and the antenna connector to LMF - FFS: the more details of the PCO reporting, e.g., in LCS or GCS frame Agreement To enable simultaneous transmission of UL SRS for positioning by a target UE and a PRU, support the following enhancements: - Enabling LMF to request the serving gNB of a UE to configure the transmission of the [indicated] UL SRS resources from the UE within indicated time window(s). - FFS: the details of the time window, e.g., the start time, duration, periodicity for the time window(s), within the vicinity of a reference SRS configuration or use the existing message of Scheduled Location time - Enabling LMF to request the serving gNB and neighboring gNBs of the UE to measure the [indicated] UL SRS resources from the UE within indicated time window(s). - Note: this may be a different indicated time window Agreement To enable simultaneous measurements on same DL PRS by a target UE and a PRU, support the following enhancements: - Enabling LMF to request the UEs, including target UE and PRU(s), to perform measurements on [indicated] DL PRS resources occurring within indicated time window(s). - FFS: the details of the configuration of the indicated time window(s), e.g., the start time, duration, periodicity for the time window(s), as well as the relationship with the Scheduled Location time. Agreement Support the reuse of existing physical layer procedures for DL positioning (e.g., DL-TDOA) with the necessary enhancements in measurement configuration, request and report (e.g., adding the configuration related to the NR DL CPP) for both UE-based and UE-assisted NR DL carrier phase positioning, including - UE in RRC_CONNECTED state with measurement gap. - FFS: UE in RRC_CONNECTED state without measurement gap - UE in RRC_ INACTIVE state Agreement The specific RF frequency associated with a DL carrier phase measurement is defined as the center frequency of the DL PFL by default. - Note: It is open to further discussion whether a frequency other than the center frequency of the DL PFL can also be the specific RF frequency for non-default case(s), if RAN1 agrees to introduce them. Agreement The specific RF frequency associated with a UL carrier phase measurement is defined, by default, as the center frequency of the transmission bandwidth of the SRS for positioning purpose. - Note: It is open to further discussion whether a frequency other than the center frequency of the UL carrier can also be the specific RF frequency for a non-default case(s), if RAN1 agrees to introduce them. Agreement - Support enabling a TRP to report UL RSCP together with RTOA and/or gNB Rx-Tx time difference measurements to LMF - Note 1: The report of UL carrier phase measurement with gNB Rx – Tx time difference does not necessarily require the report of DL carrier phase measurement with UE Rx – Tx time difference. - Note 2: This doesn’t preclude standalone UL carrier phase measurements reporting. Agreement Further study whether and how to support a UE/TRP to report the carrier phase measurement quality indication for corresponding the phase measurements. Agreement For NR UL carrier phase positioning for UE in RRC_CONNECTED and RRC_INACTIVE states, support reuse of existing physical layer procedures for UL positioning (e.g., UL-TDOA), with necessary enhancements in the measurement configuration, measurement request and measurement report (e.g., the configuration related to the NR UL CPP). - FFS: the details of the enhancements. Agreement Adopt one of the following options for a timestamp associated with a reported RSCP/RSCPD measurement (make the decision in RAN1#113): - Option 1: - NR-TimeStamp, currently defined in TS 37.355, is reused as the timestamp with the granularity of a slot. - FFS: Whether to clarify in the specification the reported RSCP/RSCPD value presents the RSCP/RSCPD of a specific OFDM symbol within the slot identified by the NR-TimeStamp. - Option 2: - NR-TimeStamp, currently defined in TS 37.355, should be enhanced to include the OFDM symbol index in a slot, as the timestamp for RSCP/RSCPD measurements. Agreement To address the impact of the phase delays on Tx/Rx RF chains, support one or more of the following options (down-selection in RAN1#113): - Option 1a: introduce the definition of UE/TRP Tx/Rx phase error groups (PEGs) for the Tx/Rx of DL PRS/UL SRS signals - Rel-17 definitions of UE/TRP Tx/Rx TEGs can be used as the starting point for defining UE/TRP Tx/Rx PEGs. - FFS: the details of \the UE/TRP Tx/Rx PEGs - Option 1b: Introduce Tx/Rx RF antenna IDs or Tx/Rx RF chain IDs to identify the individual Tx/Rx RF chains for transmitting/receiving the DL PRS/UL SRS signals. - FFS: the details of the Tx/Rx RF antenna IDs or Tx/Rx RF chain IDs - Note: Device transmitting PRS or positioning SRS provides Tx antenna ID or Tx Chain ID. Device receiving PRS or positioning SRS provides Rx antenna ID or Rx Chain ID. - Option 1c: introduce the report of ARP ID for the Rx/Tx of DL PRS/UL SRS signals. - The transmission/reception associated with the same ARP ID is assumed from the same ARP. - FFS: the maximum number of ARP IDs. - Option 2: reuse or enhance the existing Rel-17 definitions of UE/TRP Tx/Rx TEGs with smaller margin value. - Option 3: RAN1 sends an LS to RAN4, requesting RAN4 to consider whether there is a need to define the new UE/TRP Tx/Rx phase error groups (PEGs), introduce new IDs (e.g., Tx/Rx RF antenna IDs ) to present the phase delays for the Tx/Rx of DL PRS/UL SRS signals, or reuse or enhance the existing Rel-17 definitions of UE/TRP Tx/Rx TEGs with smaller margin value, and provide the definitions if RAN4 decides it is needed.

RAN1#113:

Agreement Support the following definition of the reference point of the UE/TRP carrier phase measurements: - The reference point of the UE carrier phase measurements is defined the same as the reference point of RSTD for both frequency range 1 and frequency range 2. - The reference point of the TRP carrier phase measurements is defined the same as the reference point of RTOA for both frequency range 1 and frequency range 2. - Note: It is up to UE/TRP’s implementation on how to map the carrier phase to the reference point for measurement reporting. Agreement Adopt the following modifications on the agreements made in RAN1#112bis-e: To enable simultaneous transmission of UL SRS for positioning by a target UE and a PRU, support the following enhancements: - Enabling LMF to request the serving gNB of a UE to configure the transmission of the UL SRS resources from the UE within indicated time window(s). - FFS: the details of the time window, e.g., the start time, duration, periodicity for the time window(s), within the vicinity of a reference SRS configuration or use the existing message of Scheduled Location time - Enabling LMF to request the serving gNB and neighboring gNBs of the UE to measure the UL SRS resources from the UE within indicated time window(s). - Note: this may be a different indicated time window To enable simultaneous measurements on same DL PRS by a target UE and a PRU, support the following enhancements: - Enabling LMF to request the UEs, including target UE and PRU(s), to perform measurements on indicated DL PRS resource sets occurring within indicated time window(s). - FFS: the details of the configuration of the indicated time window(s), e.g., the start time, duration, periodicity for the time window(s), as well as the relationship with the Scheduled Location time. Agreement For UE-based carrier phase positioning, support enabling LMF to forward the DL carrier phase measurement reported by a PRU, with additional information of the same PRU to a target UE for UE-based carrier phase positioning in the positioning assistance data. - Note: Whether the forwarded DL carrier phase measurement is DL RSCP and/or DL RSCPD depends at least on which of them is (are) supported by UE capability. - additional information of the same PRU includes at least PRU location. - FFS: additional PRU information, e.g. the AoD of PRU to each TRP, etc. Agreement If a UE reports RSCPD measurements together with RSTD measurements in a measurement report element, the reference TRP for RSCPD is the same as the reference TRP reported for RSTD. - The target and the reference TRP are in the same PFL Agreement - From RAN1’s perspective, carrier phase positioning for UE in RRC_IDLE state is supported for UE-based and UE-assisted positioning in Rel-18. - Note: No additional specification work is expected specifically related to carrier phase positioning for UE in RRC_IDLE state in RAN1. Conclusion From RAN1’s perspective, carrier phase positioning for UE in RRC_CONNECTED state without measurement gap is not supported in Rel-18. Working assumption - To enable LMF to optionally request the serving gNB of a UE to configure the transmission of the UL positioning SRS resources from the UE within indicated time window(s), support: - Option 1D: Each of the time windows is defined with the following parameters: - The start of the time window, which is indicated by a combination of subframe number, slot offset and symbol index with respect to the SFN initialization time - The duration of the time window, which is given by a number of consecutive slots/symbols - FFS: the number of the consecutive slots/symbols - (Optional) The periodicity of the time window, which is defined similar to IE PeriodicitySRS in “Requested SRS Transmission Characteristics” in TS 38.455. - FFS: the maximum number of the windows Agreement To enable LMF to request the serving gNB and neighboring gNBs of a UE to measure the UL SRS resources from the UE within indicated time window(s), each time window is defined with the following parameters: - The start of the time window, which is indicated by a combination of subframe number, slot offset and symbol index with respect to the SFN initialization time - The duration of the time window, which is given by a number of consecutive slots/symbols - FFS: the number of consecutive slots/symbols - (Optional) The periodicity of the time window, which is defined similar to IE Measurement Periodicity in MEASUREMENT REQUEST in TS 38.455. - FFS: the maximum number of the windows Agreement To enable LMF to request the UEs, including target UE and PRU(s), to perform measurements on indicated DL PRS resource set(s) occurring within indicated time window(s), each time window is defined with the following parameters: - The start of the time window, which is indicated by a combination of subframe number, slot offset and symbol index - The duration of the time window, which is given by a number of consecutive slots/symbols - FFS: the number of consecutive slots/symbols - (Optional) The periodicity of the time window, which is defined similar to IE NR-DL-PRS-Periodicity-and-ResourceSetSlotOffset in TS 37.355.FFS: the maximum number of the windows

RAN1#114:

Agreement - Confirm the following working assumption with modification made in RAN1#113: Working assumption - To enable LMF to optionally request the serving gNB of a UE to configure the transmission of the UL positioning SRS resources from the UE within indicated time window(s), support: - Option 1D: Each of the time windows is defined with the following parameters: - The start of the time window, which is indicated by a combination of system frame number, slot offset and symbol index with respect to the SFN initialization time - The duration of the time window, which is given by a number of consecutive slots/symbols - FFS: the number of the consecutive slots/symbols - (Optional) The periodicity of the time window, which is defined similar to IE PeriodicitySRS in “Requested SRS Transmission Characteristics” in TS 38.455. - FFS: the maximum number of the windows Agreement - When a LMF requests the serving gNB of a UE to configure the transmission of the UL positioning SRS resources from the UE within indicated time window(s), - the duration of a time window can be configured by one of the following values: - {1, 2, 4, 8, 12} OFDM symbols - {1, 2, 4, 6, 8, 12, 16} slots - FFS: additional values - the number of the time windows can be configured as: - {1, 2, …, 16} Agreement When a LMF requests the serving gNB and neighboring gNBs of a UE to measure the UL SRS resources from the UE within indicated time window(s): - The duration of a time window can be configured as follows: - {1, 2, 4, 6, 8, 12, 16} slots. - the number of the time windows can be: - {1, 2, …, 16} Agreement When an LMF requests the UEs, including target UE and PRU(s), to perform measurements on indicated DL PRS resource set(s) occurring within indicated time window(s) - The duration of a time window can be configured as follows: - {1, 2, 4, 6, 8, 12, 16} slots. - the number of the time windows can be: - {1, 2} - FFS: {4, 8} Agreement Each DL RSCP/RSCPD measurement instance is obtained with sample only. Conclusion From RAN1’s perspective, the granularity and the range of the RSCP/RSCPD measurements can be defined by RAN4. Agreement For the timestamp associated with a reported RSCP/RSCPD measurement, NR-TimeStamp, with the granularity of a slot, currently defined in TS 37.355, can be reused as the timestamp. - Subject to UE capability, a UE may optionally provide an OFDM symbol index in the timestamp. - Note: It is up to RAN2/RAN3 how to signal the timestamp Agreement When DL RSCPD/RSCP measurements are reported together with the DL RSTD/ UE Rx – Tx time difference measurements, the DL RSCPD/RSCP measurements are obtained from a single DL PFL only. Note: From RAN1’s perspective, the reporting of the carrier phase measurements from one DL PFL has no impact on the reporting of the DL RSTD and/or UE Rx – Tx time difference measurements from the same DL PFL or other DL PFLs. Agreement For UE-based carrier phase positioning, when LMF forwards the DL carrier phase measurement reported by a PRU to a target UE, the timestamp associated with the PRU carrier phase measurements should also be forwarded in positioning assistance data. Agreement Support UE/TRP to report the phase quality indication for the RSCP/RSCPD measurements. The phase quality indication includes the following fields: - phase quality index - phase quality resolution The values of the phase quality index and phase quality resolution are left for RAN4.