3GPP TSG-WG2 Meeting #124	R2-2311868

# Chicago, USA, Nov. 13th – 17th, 2023

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 37.355 | CR | 0477 | rev | 1 | Current version: | 17.6.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | x | Radio Access Network |  | Core Network | x |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Clarification on the field description of dl-prs-ResourceSetPeriodicityReq |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | vivo |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R2 |  |  |  |  |  |  |  |  |  |
| Work item code: | NR_pos_enh-Core |  |  |  |  |  | Date: |  |  | 2023-10-31 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-17 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | In the field description of dl-prs-ResourceSetPeriodicityReq, the requested periodicity of the on-demand DL-PRS takes values 2μ4, 5, 8, 10, 16, 20, 32, 40, 64, 80, 160, 320, 640, 1280, 2560, 5120, 10240 slots, where μ=0, 1, 2, 3 for SCS of 15, 30, 60 and 120 kHz respectively. μ refers to the target device’s current primary cell. However, there are multiple resources with different SCS for the target device’s current PCell, e.g., SCS of SSB, SCS of activated BWP. The explicit resource the μ refers to should be clarified, otherwise, the gNB and UE/LMF may have different understandings on the requested periodicity. For on-demand PRS, the actual PRS changes are requested by the LMF irrespective of whether the procedure is UE- or LMF-initiated. As the LMF is only aware of the SSB SCS of the serving PCell, it can be further clarified that the μ refers to the SCS of SSB of target device’s current primary cell. |  |  |  |  |  |  |  |  |
| Summary of change: |  | For the field description of dl-prs-ResourceSetPeriodicityReq, further clarify that the μ refers to the SCS of SSB of target device’s current primary cell. Impact analysis Impacted 5G architecture options: NR SA, (NG)EN-DC, NE-DC,NR-DC Impacted functionality: - On-demand PRS Inter-operability: 1. if the Network supports the change and the UE does not: No interoperability problems are foreseen. 2. if the UE supports the change and the network does not: No interoperability problems are foreseen. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Ambiguous about the value of μ/SCS, which may lead to different understandings on the requested periodicity of on-demand PRS between gNB and UE/LMF. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 6.4.3 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

Start changes

–	NR-On-Demand-DL-PRS-Information

The IE NR-On-Demand-DL-PRS-Information defines the requested on-demand DL-PRS.

-- ASN1START

NR-On-Demand-DL-PRS-Information-r17 ::= SEQUENCE (SIZE (1..nrMaxFreqLayers-r16)) OF

NR-On-Demand-DL-PRS-PerFreqLayer-r17

NR-On-Demand-DL-PRS-PerFreqLayer-r17 ::= SEQUENCE {

dl-prs-FrequencyRangeReq-r17			ENUMERATED { fr1, fr2, ...},

dl-prs-ResourceSetPeriodicityReq-r17	ENUMERATED { p4, p5, p8, p10, p16, p20, p32, p40,

p64, p80, p160, p320, p640, p1280, p2560,

p5120, p10240, p20480, p40960, p81920, ...,

p128-v1760, p256-v1760, p512-v1760}

OPTIONAL,

dl-prs-ResourceBandwidthReq-r17			INTEGER (1..63)							OPTIONAL,

dl-prs-ResourceRepetitionFactorReq-r17	ENUMERATED {n2, n4, n6, n8, n16, n32, ...}

OPTIONAL,

dl-prs-NumSymbolsReq-r17				ENUMERATED {n2, n4, n6, n12, ...}		OPTIONAL,

dl-prs-CombSizeN-Req-r17				ENUMERATED {n2, n4, n6, n12, ...}		OPTIONAL,

dl-prs-QCL-InformationReqTRPlist-r17	DL-PRS-QCL-InformationReqTRPlist-r17	OPTIONAL,

...

}

DL-PRS-QCL-InformationReqTRPlist-r17 ::= SEQUENCE (SIZE (1..nrMaxTRPsPerFreq-r16)) OF

DL-PRS-QCL-InformationReqPerTRP-r17

DL-PRS-QCL-InformationReqPerTRP-r17 ::= SEQUENCE {

dl-PRS-ID-r17						INTEGER (0..255),

nr-PhysCellID-r17					NR-PhysCellID-r16							OPTIONAL,

nr-CellGlobalID-r17					NCGI-r15									OPTIONAL,

nr-ARFCN-r17						ARFCN-ValueNR-r15							OPTIONAL,

dl-prs-QCL-InformationReqSet-r17	SEQUENCE (SIZE (1..nrMaxSetsPerTrpPerFreqLayer-r16)) OF

DL-PRS-QCL-InfoReq-r17,

...

}

DL-PRS-QCL-InfoReq-r17 ::= SEQUENCE {

nr-DL-PRS-ResourceSetID-r17			NR-DL-PRS-ResourceSetID-r16,

dl-prs-QCL-InformationReq-r17		CHOICE {

dl-prs-QCL-InfoRecPerResourceSet-r17	DL-PRS-QCL-Info-r16,

dl-prs-QCL-Info-requested-r17			NULL

},

...,

[[

dl-prs-QCL-InfoRecPerResource-r17	SEQUENCE  (SIZE (1..nrMaxResourcesPerSet-r16)) OF

DL-PRS-QCL-Info-r16			OPTIONAL

]]

}

-- ASN1STOP

NR-On-Demand-DL-PRS-Information field descriptions

dl-prs-FrequencyRangeReq This field specifies the frequency range for which the on-demand DL-PRS is requested.

dl-prs-ResourceSetPeriodicityReq This field specifies the requested periodicity of the DL-PRS Resource Set in slots. The periodicity depends on the subcarrier spacing (SCS) and takes values 2μ4, 5, 8, 10, 16, 20, 32, 40, 64, 80, 160, 320, 640, 1280, 2560, 5120, 10240 slots, where μ=0, 1, 2, 3 for SCS of 15, 30, 60 and 120 kHz respectively. μ refers to the SCS of SSB of target device's current primary cell.

dl-prs-ResourceBandwidthReq This field specifies the requested number of PRBs allocated for the DL-PRS Resource (allocated DL-PRS bandwidth) in multiples of 4 PRBs. Integer value 1 corresponds to 24 PRBs, value 2 corresponds to 28 PRBs, value 3 corresponds to 32 PRBs and so on.

dl-prs-ResourceRepetitionFactorReq This field specifies the requested DL-PRS Resource repetition. Enumerated values n2, n4, n6, n8, n16, n32 correspond to 2, 4, 6, 8, 16, 32 resource repetitions, respectively.

dl-prs-NumSymbolsReq This field specifies the requested number of symbols per DL-PRS Resource within a slot.

dl-prs-CombSizeN-Req This field specifies the requested Resource Element spacing in each symbol of the DL-PRS Resource.

dl-prs-QCL-InformationReqTRPlist This field specifies the recommended or requested QCL indication with other DL reference signals. - dl-PRS-ID indicates the DL-PRS ID of the TRP for which the QCL information is recommended. - nr-PhysCellID indicates the physical Cell-ID of the TRP for which the QCL information is recommended, as defined in TS 38.331 [35]. - nr-CellGlobalID indicates the NCGI, the globally unique identity of a cell in NR, of the TRP for which the QCL information is recommended, as defined in TS 38.331 [35]. - nr-ARFCN indicates the NR-ARFCN of the TRP's CD-SSB (as defined in TS 38.300 [47]) corresponding to nr-PhysCellID. - dl-prs-QCL-InformationReqSet indicates the recommended QCL information per DL-PRS Resource Set. - nr-DL-PRS-ResourceSetID indicates the DL-PRS Resource Set ID for which the QCL information is recommended. - dl-prs-QCL-InformationReq - dl-prs-QCL-InfoRecPerResourceSet indicates a single recommended QCL source for the DL-PRS Resource Set. - dl-prs-QCL-Info-requested indicates that the UE requests to provide the QCL information in the assistance data. - dl-prs-QCL-InfoRecPerResource indicates a list of recommended QCL sources for the DL-PRS Resource Set. If this field is present, the dl-prs-QCL-InformationReg shall be ignored by the receiver.

End of change