3GPP TSG-RAN WG2 Meeting #124	R2-2312248

Chicago, USA, November 13th - 17th, 2023

Agenda Item:	7.6.4

Source:	ZTE Corporation, Sanechips

Title:	Paging window alignment in discontinuous coverage

Document for:	Discussion and Decision

Introduction

In the IoT NTN WID [1], the objective of further enhancement to discontinuous coverage is listed below:

-	Study and specify, if needed, mobility management enhancements and power saving enhancements for discontinuous coverage, taking into account the conclusions from the SA2 study FS_5GSAT_Ph2.  [RAN2, RAN3].

For enhancements on discontinuous coverage, SA2 has finished their study on Integration of satellite components in the 5G architecture (Phase 2) and 23 solutions for two key issues (Key Issue #1: Mobility Management enhancement with discontinuous satellite coverage and Key Issue #2: Power saving enhancement for UE in discontinuous coverage) have been captured in the TR 23.700-28 [2].

Under the new WID [3] proposed in SA2#154AH meeting on January, 2023, SA2 has already started the normative work for this topic. Meanwhile, RAN3 has had initial discussion on the RAN3 impacts of some solutions.

In the recent RAN2 meetings, RAN2 has had discussion on this topic and the following agreements have been achieved:

RAN2#121 meeting Agreements: - RAN2 can continue to check whether dedicated RRC signalling can be used for providing satellite information corresponding to discontinuous coverage. - RAN2 will support enhancements in paging and eDRX, in alignment with the work in SA2 and CT1. FFS on the details - RAN2 may consider enhancements for connected UE upon detecting discontinuous coverage (e.g., suspend RLM, RLF detection, and RRC re-establishment process) - Companies supporting the store and forward approach can bring a proposal to the plenary for TEI18 or for updating the WID RAN2#121bise meeting agreements: - RAN2 will not introduce any enhancement to allow a UE in RRC Connected to stay in RRC_CONNECTED during/after a coverage gap (e.g. suspend RLM/RLF, activation time in RRC Reconfiguration, CHO enhancement) - RAN2 to introduce enhancement to RRC Release using one of the following options (FFS which one): - Explicit RRC Release using a new RRC Release cause - UE Autonomous release (e.g. timer based or upon detection of coverage gap) RAN2#122 meeting, no further agreements. RAN2#123 meeting agreements: - RAN2 understands that UE may directly go to RRC_IDLE after RLF is triggered, if there is not enough time for the UE to finish the procedure of RRC re-establishment due to the discontinuous coverage (FFS whether this needs to be captured in the specs, e.g. a NOTE) RAN2#123bis meeting agreements: - Provide carrier frequency for the existing satellite list in SIB32 to facilitate cell selection and reduce service interruption after an NTN coverage gap (FFS if the information can be considered as valid after the validity of SI).

In this document, we will mainly discuss one remaining issue about enhancements in eDRX scheme and paging alignment between UE and NW in the discontinuous coverage case.

Discussion

In RAN2#123 meeting, there was much discussion for the issue about misalignment between PTW and Coverage Window and finally a simple LS (R2-2309283) has been sent to SA2. However, this LS has not been handled yet in last SA2 meeting and therefore no response LS has been sent out. In RAN2#123bis meeting, some contributions [4] [5] [6] were submitted to further discuss issue, but due to limited time, they haven’t been treated.

During RAN2#123 meeting discussion, some companies think all paging needs to be determined by CN and therefore no need to introduce enhancements in RAN2 spec. However, with reference to the SA2 specification description mentioned in Annex section 5.2, it can be seen that the core network processes, e.g., details on how to determine H-SFN, Paging H-SFN and PTW, also refer to TS 36.304.

Since no matter for UE side or CN side, the determination of PH and PTW_start refers to the RAN2 spec, we understand whether and how to address the misalignment between PTW and Coverage Window for the discontinuous coverage case in TS 36.304 should be discussed and decided by RAN2.

Observation 1: According to SA2 specification, the details on how to determine Paging H-SFN and PTW, also refer to TS 36.304.

As mentioned in the Annex section 5.1, SA2 has defined the Unavailability Period within Mobility Management and UE Power Saving Optimization functionality for Enhanced Discontinuous Coverage. SA2 also agreed NAS layer negotiation between UE and CN to support discontinuous coverage, especially on how to (re)negotiate the unavailability period. Furthermore, SA2 specification indicates that, when MME provides the UE with timers (e.g. periodic TAU timer, extended idle mode DRX, and PSM mode configuration), it may also consider the Unavailability Period Duration (if available) and Start of Unavailability Period (if available).

According to the above progress in SA2, we can assume the UE and core network can negotiate the suitable timer length (e.g., long enough) for the related PSM/eDRX schemes in the scenario of discontinuous network coverage.

However, taking eDRX scheme as example, even CN can configure a suitable eDRX cycle for the UE, the position of Paging Hyperframe (PH), PTW_start and PTW_end are determinate (mainly determined by eDRX cycle, PTW length and UE_ID) according to the formula given in TS 36.304 as below:

TS 36.304 - 7.3 Paging in extended DRX ……The PTW is UE-specific and is determined by a Paging Hyperframe (PH), a starting position within the PH (PTW_start) and an ending position (PTW_end). PH, PTW_start and PTW_end are given by the following formulae: The PH is the H-SFN satisfying the following equation: H-SFN mod TeDRX,H= (UE_ID_H mod TeDRX,H), where - UE_ID_H: - 10 most significant bits of the Hashed ID, if P-RNTI is monitored on PDCCH or MPDCCH - 12 most significant bits of the Hashed ID, if P-RNTI is monitored on NPDCCH - T eDRX,H : eDRX cycle of the UE in Hyper-frames, (TeDRX,H =1, 2, …, 256 Hyper-frames) (for NB-IoT, TeDRX,H =2, …, 1024 Hyper-frames) and configured by upper layers. PTW_start denotes the first radio frame of the PH that is part of the PTW and has SFN satisfying the following equation: SFN = 256* ieDRX, where - ieDRX = floor(UE_ID_H /TeDRX,H) mod 4 PTW_end is the last radio frame of the PTW and has SFN satisfying the following equation: SFN = (PTW_start + L*100 - 1) mod 1024, where - L = Paging Time Window length (in seconds) configured by upper layers

It’s easy to understand that the network cannot prevent the PTW from falling into the Unavailability Period via configuration to avoid useless paging monitor, and network also cannot ensure PTW appears as soon as possible after the Unavailability Period ends via configuration in order that the UE can monitor paging in timely.

In the general cases, the eDRX cycle may be partially overlapped with the Unavailability Period and the position of Paging Hyperframe (PH) and PTW_start can be earlier than the end of the Unavailability Period. Therefore, if just following the existing calculation for PH and PTW_start, the UE would start monitoring PTW within Unavailability Period, this is undesired. According the agreements in R17, when out of coverage, the UE is not required to perform AS functions. That means, if the calculated start of such PH and PTW are within the Unavailability Period, the UE (and also NW) can skip the PH and PTW, or at least skip some POs in the time duration where Unavailability Period and PTW overlap.

It’s obvious that such PH/PTW/POs skipping will result in the reduced paging occasions. At the end of the Unavailability Period, the NW would have no or very little chances to page the UE. The NW has to wait till the next eDRX cycle. Such process will further increase the paging delay. This issue will become even worse in the following cases:

- If the discontinuous coverage occurs approximately periodically and the configured eDRX cycle is very similar as the Unavailability Period, it will be highly possible for the UE to just miss a PTW or part of it at each time when the Unavailability Period ends.

- If the Unavailability Period is long, the eDRX cycle can be configure long accordingly, e.g., dozens of hyper frames. With reference to the following value range of eDRX cycle and PTW length and considering the maximum value of PTW length is only 4 hyper frames (for NB-IoT), once the (part or most of) the paging opportunities in the current PTW are skipped and paging fails to be delivered to the UE, the paging latency caused by waiting PTW till next eDRX cycle can be considerably large.

TS 36.413 9.2.1.115 NB-IoT Paging eDRX Information This IE indicates the NB-IoT Paging eDRX parameters as defined in TS 36.304 [20]. IE/Group Name; Presence; Range; IE type and reference; Semantics description. NB-IoT Paging eDRX Cycle; M; ENUMERATED (hf2, hf4, hf6, hf8, hf10, hf12, hf14, hf16, hf32, hf64, hf128, hf256, hf512, hf1024, …); TeDRX defined in TS 36.304 [20]. Unit: [number of hyperframes].. NB-IoT Paging Time Window; O; ENUMERATED (s1, s2, s3, s4, s5, s6, s7, s8, s9, s10, s11, s12, s13, s14, s15, s16, …); Unit: [2.56 seconds]. …

Observation 2: In the general cases in DC, the eDRX cycle may be partially overlapped with the Unavailability Period and the position of Paging Hyperframe (PH) and PTW_start can be earlier than the end of the Unavailability Period. The UE (and also NW) will skip the PH and PTW, or at least skip some POs in the time duration where Unavailability Period and PTW overlap. This will result in the reduced paging occasions at the end of the Unavailability Period and may further cause the paging to be delayed till next eDRX cycle.

In order to address the above issues caused by R17 UE implementation of skipping PH/PTW/POs, a straightforward option is to try to adjust the PTW_start to align it with or shortly after the end of the Unavailability Period within the eDRX cycle which is partially overlapped with Unavailability Period. Such adjustment is explained in the following Figure 1.

Figure 1: To adjust the PTW_start to align it with the end of the Unavailability Period

Considering the granularity of Unavailability Period may be large, e.g., minutes or hours, the offset between PTW_start and the end of the Unavailability Period may be also large, e.g., larger than a Paging Hyperframe. Since the adjustment for PTW_start can only be within one PH, only to adjust PTW_start is not enough. We may firstly need to align PH with the end of the Unavailability Period coarsely, and then to align PTW_start with the end of the Unavailability Period precisely.

Based on the above consideration, we give the following proposals:

Proposal 1a: If the PTW for a UE configured with eDRX is overlapped with the H-SFN where the end of the Unavailability Period, the UE can adjust PH with an offset_ph to make it aligned with the H-SFN where the end of the Unavailability Period is located.

Proposal 1b: If the PTW_start for a UE configured with eDRX is earlier than the end of the Unavailability Period in the PH which is overlapped with the H-SFN where the end of the Unavailability Period is located, the UE can adjust PTW_start with an offset_ptw to make it aligned with the end of the Unavailability Period.

The adjustment of PH/PTW_start would have impacts at least on the TS 36.304 specification. Furthermore:

- If UE and eNB can determine the offset between PTW_start and the end of the Unavailability Period, they can delay the PTW_start by this offset respectively and still keep consistence on PTW_start. By this way, there is no need for eNB to explicitly configure offset to the UE. But at least the eNB also needs to acquire the information of the Unavailability Period in advance, e.g., from CN.

- However, even the Unavailability Period is a UE-specific parameters, it’s still possible for several or many UEs to be very close on the end of the Unavailability Period. Therefore, in order to distribute the PTW_start for different UEs (e.g., distributing the pending paging when coverage is resumed), it may still need eNB to configure different UE-specific offset for different UEs (which may be similar or a bit larger than the calculated offset between the PTW_start and the end of the Unavailability Period).

For simplicity in R18, we prefer to adopt the former way.

Proposal 1c: UE and eNB calculate the offset_ph and/or offset_ptw, which are respectively equal to the difference between the PTW_start/the start of PH and the end of the Unavailability Period.

With the proposal 1a and proposal 1b, the pending paging during Unavailability Period can be delivered to the UE as soon as possible. One remaining issue is that the possible DL and UL congestion as the PTW_start of (possible) many UEs are adjusted to a similar place which is at or close to the end of Unavailability Period. RACH congestion may happen if many UEs receive the paging during the adjusted PTW, but it can already be addressed by the new-introduced NAS solution, e.g., DCW timer. How to address the DL paging congestion can be left to NW implementation, at least in this release. Whether other enhancement is needed can be discussed in future.

Conclusion

In previous sections, the following observations and proposals were made:

Observation 1: According to SA2 specification, the details on how to determine Paging H-SFN and PTW, also refer to TS 36.304.

Observation 2: In the general cases in DC, the eDRX cycle may be partially overlapped with the Unavailability Period and the position of Paging Hyperframe (PH) and PTW_start can be earlier than the end of the Unavailability Period. The UE (and also NW) will skip the PH and PTW, or at least skip some POs in the time duration where Unavailability Period and PTW overlap. This will result in the reduced paging occasions at the end of the Unavailability Period and may further cause the paging to be delayed till next eDRX cycle.

Proposal 1a: If the PTW for a UE configured with eDRX is overlapped with the H-SFN where the end of the Unavailability Period, the UE can adjust PH with an offset_ph to make it aligned with the H-SFN where the end of the Unavailability Period is located.

Proposal 1b: If the PTW_start for a UE configured with eDRX is earlier than the end of the Unavailability Period in the PH which is overlapped with the H-SFN where the end of the Unavailability Period is located, the UE can adjust PTW_start with an offset_ptw to make it aligned with the end of the Unavailability Period.

Proposal 1c: UE and eNB calculate the offset_ph and/or offset_ptw, which are respectively equal to the difference between the PTW_start/the start of PH and the end of the Unavailability Period.

References

- RP-223519, Revised WID: IoT (Internet of Things) NTN (non-terrestrial network) enhancements, MediaTek Inc., December 2022

- TR 23.700-28 V18.1.0 Study on Integration of satellite components in the 5G architecture; Phase 2 (Release 18), 2023-03.

- S2-2210221, New WID: 5GC/EPC enhancement for satellite access Phase 2, SA2#154AH, January 2023

- R2-2310021, Discussion on enhancement to discontinuous coverage for IoT NTN, Transsion Holdings, 3GPP RAN2#123bis, October 2023

- R2-2310071, Considerations on Supporting Discontinuous Coverage, NEC, 3GPP RAN2#123bis, October 2023

- R2-2310826, Remaining issues of discontinuous coverage, ZTE, 3GPP RAN2#123bis, October 2023

Annex

Progress in SA2 for R18 discontinuous coverage

SA2 has agreed the following aspects to support discontinuous coverage in NAS layer, especially on how to (re)negotiate the unavailability period between UE and core network:

TS 23.401 i30 - Mobility Management and UE Power Saving Optimization For satellite access that provides discontinuous network coverage, and in case both the UE and the network support Enhanced Discontinuous Coverage, then the Mobility Management and UE Power Saving Optimization functionality may be used. If both the UE and the network indicate support for "Enhanced Discontinuous Coverage Support" and if the UE determines it will lose coverage and will become unavailable, and decides to remain in no service during that time, then: - The UE may be able to determine, including considering current and expected future locations of the UE, a Start of Unavailability Period and/or Unavailability Period Duration for when it expects to be out of coverage. NOTE 1: A UE, based on implementation, can combine successive periods of no satellite coverage into a single continuous period that is notified to the network if the UE does not require network access during this period. NOTE 2: UE informing the network of coverage gaps would increase signalling and UE power consumption if coverage gaps are more frequent than UE's communication period. - The UE triggers a TAU procedure and includes an indication of upcoming loss of coverage in the TAU Request message to the MME. If the UE is able to determine a Start of Unavailability Period and/or Unavailability Period Duration it also includes them in the TAU Request message. - The UE should trigger the TAU procedure early enough such that the procedure, under normal conditions, is able to complete before the start of the unavailability period. - The UE and the MME re-negotiate unavailability at every TAU procedure, if it is required. If Start of Unavailability Period and/or Unavailability Period Duration is not included in a TAU Request message any pending loss of coverage configuration stored in the UE context at MME is discarded. - If the UE determines an upcoming loss of coverage to the network no longer applies or determines a new Start of Unavailability Period or Unavailability Period Duration related to the upcoming loss of coverage, the UE sends a new TAU Request to the MME to update the Start of Unavailabililty Period and/or Unavailability Period Duration. Upon receiving a TAU Request message from the UE including an indication of upcoming loss of coverage: - If the UE did not include a Start of Unavailability Period, the MME considers implicitly the Start of Unavailability Period to be the time at which it has received the TAU Request message from the UE. If the UE included a Start of Unavailability Period, the Start of Unavailability Period indicates the time at which the UE determines it expects to lose coverage, i.e. time until which the UE determines it is available. - The MME may determine, if not provided by the UE, or update the Unavailability Period Duration. If the MME knows an Unavailability Period Duration (e.g. based on information available to the MME as described in clause 4.13.8.4) for the UE, and the UE not include an Unavailability Period Duration or included an Unavailability Period Duration different to the Unavailability Period Duration known to the MME, the MME may use either the Unavailability Period Duration known to the MME or the Unavailability Period Duration from the UE as the Unavailability Period Duration. The MME should include the Unavailability Period Duration known to the MME in the TAU Accept. How the UE treats the MME provided Unavailability Period Duration is up to UE implementation e.g. to help to determine when to return to coverage after a discontinuous coverage period, whether to listen to paging in eDRX, not to initiate any NAS signalling (including Service Request for MO data) within the discontinuous coverage period in case of any UL signalling/data request or the UE may deactivate its Access Stratum functions for satellite access in order to optimise power consumption until coverage returns, etc. The MME indicates to the UE in the Attach Accept or TAU Accept whether the UE is not required to perform a TAU procedure when the unavailability period has ended. - The MME stores the information that the UE is unavailable at the Start of Unavailability Period in the UE context, and considers the UE is unreachable from then until the UE enters ECM_CONNECTED state. If the UE requests power saving features the MME uses procedures defined in other clauses to provide the UE with timers (e.g. periodic TAU timer, extended idle mode DRX (see clause 5.13a), and PSM mode configuration (see clause 4.3.22)), and may also consider the Unavailability Period Duration (if available) and Start of Unavailability Period (if available). Unless the MME indicated that the UE is not required to perform a TAU procedure when the unavailability period has ended, then once the event which makes the UE unavailable is completed in the UE, the UE triggers a TAU procedure. If the event which makes the UE unavailable is delayed to a future time or cancelled or unavailability period deviates from negotiated value in the UE, the UE triggers TAU procedure. The MME should adjust the mobile reachable timer or Implicit Detach timer or both such that the MME does not implicitly detach the UE while the UE is out of coverage, see clause 3,4.2. Features described for High latency communication in clause 4.3.14.7 may be used to handle mobile terminated (MT) communication with UEs being unreachable due to satellite access with discontinuous coverage and the Unavailability Period Duration (if available) and Start of Unavailability Period (if available) may be used when determining the Estimated Maximum Wait Time. The UE may send Tracking Area Update request message to inform the network of its UE unavailability period even if Mobility Management back-off timer is running.

Paging occasions determination in SA2 spec

TS 23.501 V18.3.0 2 References [52] 3GPP TS 36.304: "Evolved Universal Terrestrial Radio Access (E-UTRA); User Equipment (UE) procedures in idle mode". <skip unrelated parts> 5.31.7.2 Extended Discontinuous Reception (DRX) for CM-IDLE and CM-CONNECTED with RRC-INACTIVE 5.31.7.2.1 Overview The UE and the network may negotiate over non-access stratum signalling the use of extended idle mode DRX for reducing its power consumption, while being available for mobile terminating data and/or network originated procedures within a certain delay dependent on the DRX cycle value. Extended DRX in CM-IDLE is supported for E-UTRA and NR connected to 5GC. Extended DRX in CM-CONNECTED with RRC_INACTIVE mode is supported for WB-E-UTRA, LTE-M and NR connected to 5GC. RRC_INACTIVE is not supported by NB-IoT connected to 5GC. The negotiation of the eDRX parameters for NR, WB-E-UTRA and LTE-M is supported over any RAT. Applications that want to use extended idle mode DRX need to consider specific handling of mobile terminating services or data transfers, and in particular they need to consider the delay tolerance of mobile terminated data. A network side application may send mobile terminated data, an SMS, or a device trigger, and needs to be aware that extended idle mode DRX may be in place. A UE should request for extended idle mode DRX only when all expected mobile terminating communication is tolerant to delay. NOTE 1: The extended idle mode DRX cycle length requested by UE takes into account requirements of applications running on the UE. Subscription based determination of eDRX cycle length can be used in those rare scenarios when applications on UE cannot be modified to request appropriate extended idle mode DRX cycle length. The network accepting extended DRX while providing an extended idle mode DRX cycle length value longer than the one requested by the UE, can adversely impact reachability requirements of applications running on the UE. UE and NW negotiate the use of extended idle mode DRX as follows: If the UE decides to request for extended idle mode DRX, the UE includes an extended idle mode DRX parameters information element in the Registration Request message. The UE may also include the UE specific DRX parameters information element for regular idle mode DRX according to clause 5.4.5. The extended DRX parameters information element includes the extended idle mode DRX cycle length. The AMF decides whether to accept or reject the UE request for enabling extended idle mode DRX. If the AMF accepts the extended idle mode DRX, the AMF based on operator policies and, if available, the extended idle mode DRX cycle length value in the subscription data from the UDM, may also provide different values of the extended idle mode DRX parameters than what was requested by the UE. The AMF taking into account the RAT specific Subscribed Paging Time Window, the UE's current RAT and local policy also assigns a Paging Time Window length to be used, and provides this value to the UE during Registration Update procedures together with the extended idle mode DRX cycle length in the extended DRX parameter information element. If the AMF accepts the use of extended idle mode DRX, the UE shall apply extended idle mode DRX based on the received extended idle mode DRX length, the UE's current RAT (NR, NB-IoT, WB-E-UTRA or LTE-M) and RAT specific Paging Time Window length. If the UE does not receive the extended DRX parameters information element in the relevant accept message because the AMF rejected its request or because the request was received by AMF not supporting extended idle mode DRX, the UE shall apply its regular discontinuous reception as defined in clause 5.4.5. For NR, Paging Time Window applies for extended DRX lengths greater than 10.24s as defined in TS 38.304 [50]. For WB-E-UTRA, Paging Time Window applies for extended DRX lengths of 10.24s and greater as defined in TS 36.304 [52]. <skip unrelated parts> 5.31.7.2.2 Paging for extended idle mode DRX in E-UTRA and NR connected to 5GC 5.31.7.2.2.0 General For WB-E-UTRA and LTE-M connected to 5GC, the extended idle mode DRX value range will consist of values starting from 5.12s (i.e. 5.12s, 10.24s, 20.48s, etc.) up to a maximum of 2621.44s (almost 44 min). For NB-IoT, the extended idle mode DRX value range will start from 20.48s (i.e. 20.48s, 40.96s, 81.92, etc.) up to a maximum of 10485.76s (almost 3 hours) (see TS 36.304 [52]). For NR, the extended idle mode DRX value range will consist of values starting from 2.56s (i.e. 2.56s, 5.12s, 10.24s, 20.48s, etc.) up to a maximum of 10485.76s (almost 3 hours) (see TS 38.304 [50]). The extended idle mode DRX cycle length is negotiated via NAS signalling. The AMF includes the extended idle mode DRX cycle length for NR, WB-E-UTRA, LTE-M or NB-IoT in paging message to assist the NG-RAN node in paging the UE. For NR, Paging Time Window applies for extended DRX lengths longer than 10.24s as defined in TS 38.304 [50]. For WB-E-UTRA, LTE-M and NB-IoT, Paging Time Window applies for extended DRX lengths of 10.24s and longer as defined in TS 36.304 [52]. The network follows the regular paging strategy as defined in clause 5.4.5 when the extended idle mode DRX cycle length is 5.12s or less for WB-E-UTRA, LTE-M and NB-IoT, or 10.24s or less for NR. Clauses 5.31.7.2.2.1, 5.31.7.2.2.2 and 5.31.7.2.2.3 apply when the extended idle mode DRX cycle length is 10.24s or longer for WB-E-UTRA, LTE-M and NB-IoT, or longer than 10.24s for NR. 5.31.7.2.2.1 Hyper SFN, Paging Hyperframe and Paging Time Window length A Hyper-SFN (H-SFN) frame structure is defined on top of the SFN used for regular idle mode DRX. Each H-SFN value corresponds to a cycle of the legacy SFN of 1024 radio frames, i.e. 10.24s. When extended idle mode DRX is enabled for a UE, the UE is reachable for paging in specific Paging Hyperframes (PH), which is a specific set of H-SFN values. The PH computation is a formula that is function of the extended idle mode DRX cycle, and a UE specific identifier, as described in TS 36.304 [52]. This value can be computed at all UEs and AMFs without need for signalling. The AMF includes the extended idle mode DRX cycle length and the PTW length in paging message to assist the NG-RAN nodes in paging the UE. The AMF also assigns a Paging Time Window length, and provides this value to the UE during Registration Update procedures together with the extended idle mode DRX cycle length. The UE first paging occasion is within the Paging Hyperframe as described in TS 36.304 [52]. The UE is assumed reachable for paging within the Paging Time Window. The start and end of the Paging Time Window is described in TS 36.304 [52]. After the Paging Time Window length, the AMF considers the UE unreachable for paging until the next Paging Hyperfame.