3GPP TSG-RAN WG2 Meeting #124	R2-2312521

# Chicago, US, November 13th – 17th 2023

Agenda Item:	7.25.1.7

Source:	Ericsson

Title:	Discussion on RAN4 LS R4-2317342

Document for:	Discussion, Decision

# 1	Introduction

We discuss issues covered by RAN4 LS R4-2317342.

# 2	Discussion

## 2.1 RAN4 LS R4-2317342

In the RAN4 LS R4-2317342, it is described as the below

Under the release 18 work item on enhanced NR support for high speed train scenario in frequency range 2 (NR_HST_FR2_enh), RAN4 has agreed to specify the enhanced RRM requirements for intra-band carrier aggregation (CA) scenario. The target velocity is up to 350 km/h. Furthermore, RAN4 has not introduced any inter-RAT measurement requirements for NR HST FR2.

RAN4 had agreed to reuse existing Rel-17 network signaling to indicate new Rel-18 UE behavior as well.

- For Rel-18 inter-frequency measurement enhancement for FR2 HST

- RAN4 made the agreement as below:

- Reuse Rel-17 IE highSpeedMeasFlagFR2-r17 in SIB to inform UE whether to apply the enhanced RRM requirements for inter-frequency measurement for FR2 HST in idle/inactive mode. - Reuse Rel-17 signalling highSpeedMeasFlagFR2 to inform UE whether to apply the enhanced RRM requirements for inter-frequency measurement for FR2 HST in Connected mode.

- For CA enhancement

  - For Rel-18 intra-frequency measurement on SCC for FR2 HST

- RAN4 made the agreement as below:

- Reuse Rel-17 signalling highSpeedMeasFlagFR2 to inform UE whether to apply the enhanced RRM requirements for intra-frequency measurement on SCC for FR2 HST in Connected mode.

RAN4 respectfully asks RAN2 to check if there is any change required in RAN2 specifications based on the above-mentioned RAN4 agreements for the NR HST FR2 enhancement features introduced in Rel-18.

2. Actions:

To RAN2 group:

ACTION1: RAN4 respectfully asks RAN2 to check if there is any change required in RAN2 specifications based on the above-mentioned RAN4 agreements for the NR FR2 HST enhancement features introduced in Rel-18.

## 2.2 Discussion

According to the LS, the below observations can be made.

Observation 1: Enhanced RRM requirements for intra-band carrier aggregation (CA) has been extended for inter-frequency and intra-frequency measurements.

Observation 2: Reusing existing Rel-17 network signalling to indicate new Rel-18 UE behaviour indicates that the network doesn’t need to distinguish the feature of enhanced RRM requirements between Rel-17 UEs and Rel-18 UEs, so that the network can use the common signalling to control both Rel-17 UEs and Rel-18 UEs.

Enhanced RRM requirements for intra-band carrier aggregation (CA) has been extended for inter-frequency and intra-frequency measurements.

Reusing existing Rel-17 network signalling to indicate new Rel-18 UE behaviour indicates that the network doesn’t need to distinguish the feature of enhanced RRM requirements between Rel-17 UE and Rel-18 UE, so that the network can use the common signalling to control both Rel-17 UEs and Rel-18 UEs.

By checking the existing RRC parameter highSpeedMeasFlagFR2-r17 as shown in the below

HighSpeedConfigFR2-r17 ::=  SEQUENCE {

highSpeedMeasFlagFR2-r17                    ENUMERATED {set1, set2}                       OPTIONAL,   -- Need R

highSpeedDeploymentTypeFR2-r17              ENUMERATED {unidirectional, bidirectional}    OPTIONAL,   -- Need R

highSpeedLargeOneStepUL-TimingFR2-r17       ENUMERATED {true}                             OPTIONAL,   -- Need R

...

}

highSpeedMeasFlagFR2

If the field is present, the UE shall apply enhanced intra-NR RRM requirement set one to support high speed up to 350 km/h for FR2 as specified in TS 38.133 [14], if the field value is set1 or RRM requirement set two if the field value is set2.

The existing field description has specified the UE behaviours concerning intra-NR RRM requirements in HST scenario with high speed up to 350 km/h. The texts are generic to be applicable for the UE regardless of the measurement scenario is intra-frequency or inter-frequency and the RRC state is RRC_IDLE/INACTIVE or RRC_CONNECTED. Therefore, no change to the above text is required for the RAN4 agreements captured in the LS.

The field description of highSpeedMeasFlagFR2-r17 is generic to be applicable for the UE supporting HST regardless of the measurement scenario is intra-frequency or inter-frequency and the RRC state is RRC_IDLE/INACTIVE or RRC_CONNECTED.

Therefore, we make the below proposal,

RAN2 to confirm that there is no change required in RAN2 specifications based on the RAN4 agreements captured in the LS R4-2317342.

In addition, if P1 is agreed, RAN2 sends a LS response to RAN4 based on P1,

RAN2 sends a LS response to RAN4 informing that there is no change required in RAN2 specifications based on the RAN4 agreements captured in the LS R4-2317342.

We have prepared an LS draft on the response in the Annex.

Adopt the LS draft captured in the Annex.

# 3 Conclusion

In the previous sections we made the following observations:

Observation 1	Enhanced RRM requirements for intra-band carrier aggregation (CA) has been extended for inter-frequency and intra-frequency measurements.

Observation 2	Reusing existing Rel-17 network signalling to indicate new Rel-18 UE behaviour indicates that the network doesn’t need to distinguish the feature of enhanced RRM requirements between Rel-17 UE and Rel-18 UE, so that the network can use the common signalling to control both Rel-17 UEs and Rel-18 UEs.

Observation 3	The field description of highSpeedMeasFlagFR2-r17 is generic to be applicable for the UE supporting HST regardless of the measurement scenario is intra-frequency or inter-frequency and the RRC state is RRC_IDLE/INACTIVE or RRC_CONNECTED.

Based on the discussion in the previous sections we propose the following:

Proposal 1	RAN2 to confirm that there is no change required in RAN2 specifications based on the RAN4 agreements captured in the LS R4-2317342.

Proposal 2	RAN2 sends a LS response to RAN4 informing that there is no change required in RAN2 specifications based on the RAN4 agreements captured in the LS R4-2317342.

Proposal 3	Adopt the LS draft captured in the Annex.

# 4 Annex

3GPP TSG-RAN WG2 Meeting #124	 R2-230xxxx

Chicago, US, Nov 13 – 17, 2023

Title:	[Draft] LS reply to RAN4 LS R4-2317342

Response to:	R4-2317342

Release:	Rel-18

Work Item:	NR_HST_FR2_Enh

Source:	RAN2

To:	RAN4

Cc:

Contact person:	Min Wang

min.w.wang@ericsson.com

Send any reply LS to:	3GPP Liaisons Coordinator, mailto:3GPPLiaison@etsi.org

Attachments:	-

# 1	Overall description

Based on the RAN4 agreements captured in the LS R4-2317342, RAN2 has checked the existing Rel-17 RRC parameter highSpeedMeasFlagFR2-r17.

The existing RRC parameter highSpeedMeasFlagFR2-r17 as shown in the below

HighSpeedConfigFR2-r17 ::=  SEQUENCE {

highSpeedMeasFlagFR2-r17                    ENUMERATED {set1, set2}                       OPTIONAL,   -- Need R

highSpeedDeploymentTypeFR2-r17              ENUMERATED {unidirectional, bidirectional}    OPTIONAL,   -- Need R

highSpeedLargeOneStepUL-TimingFR2-r17       ENUMERATED {true}                             OPTIONAL,   -- Need R

...

}

highSpeedMeasFlagFR2

If the field is present, the UE shall apply enhanced intra-NR RRM requirement set one to support high speed up to 350 km/h for FR2 as specified in TS 38.133 [14], if the field value is set1 or RRM requirement set two if the field value is set2.

The existing field description has specified the UE behaviours concerning intra-NR RRM requirements in HST scenario with high speed up to 350 km/h. The texts are generic to be applicable for the UE regardless of the measurement scenario is intra-frequency or inter-frequency and the RRC state is RRC_IDLE/INACTIVE or RRC_CONNECTED. Therefore, RAN2 confirms that no change in RAN2 specifications is required for the RAN4 agreements captured in the LS R4-2317342.

RAN2 respectively informs RAN4 of the above RAN2 confirmation and asks RAN4 to consider the above confirmation in further work.

# 2	Actions

To RAN4:

ACTION: 	RAN2 respectfully inform RAN4 that RAN2 confirms no change in RAN2 specifications is required for the RAN4 agreements captured in the LS R4-2317342 and asks RAN4 to consider this in further work.

# 3	Dates of next TSG-RAN WG2 meetings

TSG-RAN2 Meeting #125	Feb. 26 – Mar. 01, 2024 		Athens, GR

TSG-RAN2 Meeting #125-bis	Apr. 15 – Apr. 19, 2024 		CN