3GPP TSG RAN WG2#124	R2-2312653

Chicago, USA, Nov. 13th – 17th, 2023

Agenda Item:	7.8.5

Source:	CMCC

Title:	Discussion on UAV identification broadcast

Document for:	Discussion, Decision

Introduction

In last RAN2 meeting, there were some discussions on UAV identification broadcast and achieved some agreements as follows [1]:

Agreements:

- Configure the separate resource pools one-level higher on SL-BWPPoolConfigCommon/SL-BWPPoolConfig. Adopt the TP for 38.331.

- Add additional flag to indicate whether the pool is for BRID, DAA, or both (check if this introduces a problem)

In this contribution, we would like to provide some considerations on the separate SL resource pool configuration enhancements and the specific remote ID content.

Discussion

Separate SL resource pool configuration

In last meeting, we have agreed that configure the separate resource pools one-level higher on SL-BWPPoolConfigCommon/SL-BWPPoolConfig. However, regarding the detail configuration, we need to consider the specific characteristics of UAV system. Different from V2X communications, which is on the ground, UAV UEs move in the sky, having high altitude above the ground. Further, UAV UEs also have flight path information, which could also be considered to help configure the resource pool. Therefore, for UAV UEs, height and/or flight path information could be used to configure the separate SL resource pool for BRID and DAA.

As we agreed height-dependent more-than-one MO/MR configuration, maybe we could also consider similar solution for BRID and DAA resource pool configuration to apply the characteristics of the UAV communication better. For example, the network could (pre)-configures a certain part of the resource pool to the UE for different height ranges, then UE selects resources in the subset of the whole resource pool when it moves to corresponding height.

Proposal 1: Height and/or flight path information could be used to configure the separate SL resource pool for BRID and DAA.

Proposal 2: It is proposed to consider resource pool configuration enhancement for BRID and DAA similar to height-dependent more-than-one MO/MR.

Remote ID content

Then, we can find detail information about remote ID in FAA(Federal Aviation Administration) website [2], for the usage of remote ID, Remote ID helps the FAA, law enforcement, and other federal agencies find the control station when a drone appears to be flying in an unsafe manner or where it is not allowed to fly. Remote ID also lays the foundation of the safety and security groundwork needed for more complex drone operations. And we also could find the information to be broadcasted. Thence, regarding the specific information included in UAV identification, the carried information mentioned in FAA website could be the baseline, that is a Standard Remote ID Drone or a drone with a remote ID broadcast module must transmit the following message elements:

- A unique identifier for the drone;

- The drone's latitude, longitude, geometric altitude, and velocity;

- An indication of the latitude, longitude, and geometric altitude of control station (standard) or take-off location (broadcast module);

- A time mark; and

- Emergency status (Standard Remote ID Drone only)

Proposal 3: It is proposed that the the carried information mentioned in FAA website could be the baseline, that is a Standard Remote ID Drone or a drone with a remote ID broadcast module must transmit the following message elements:

- A unique identifier for the drone;

- The drone's latitude, longitude, geometric altitude, and velocity;

- An indication of the latitude, longitude, and geometric altitude of control station (standard) or take-off location (broadcast module);

- A time mark; and

- Emergency status (Standard Remote ID Drone only)

Conclusion

Based on the discussions mentioned above, in this contribution we provide some discussions on UAV identification broadcast for NR UAV and have the following proposals:

Proposal 1: Height and/or flight path information could be used to configure the separate SL resource pool for BRID and DAA.

Proposal 2: It is proposed to consider resource pool configuration enhancement for BRID and DAA similar to height-dependent more-than-one MO/MR.

Proposal 3: It is proposed that the the carried information mentioned in FAA website could be the baseline, that is a Standard Remote ID Drone or a drone with a remote ID broadcast module must transmit the following message elements:

- A unique identifier for the drone;

- The drone's latitude, longitude, geometric altitude, and velocity;

- An indication of the latitude, longitude, and geometric altitude of control station (standard) or take-off location (broadcast module);

- A time mark; and

- Emergency status (Standard Remote ID Drone only)

Reference

- R2_123bis_ChairNotes_23-10-13_13-00_final

- https://www.faa.gov/uas/getting_started/remote_id