3GPP TSG-RAN WG2 Meeting #124	R2-2312706

Chicago, USA, 13 – 17 November 2023

Agenda item:	7.14.3

Source:	Nokia, Nokia Shanghai Bell

Title:	Remaining issues on QoE for NR-DC

WID/SID:	NR_QoE_enh-Core - Release 18

Document for:	Discussion and Decision

# 1	Introduction

During the ongoing Rel-18 discussions, RAN2 and RAN3 have developed generic framework for QoS support in NR-DC. In the post-meeting email discussion [1], the open issue list has been discussed and the following issues are to be discussed in RAN2-124 meeting.

Remaining open issues to be discussed in next meeting: Proposal 2b: When UE cannot send RVQoE report because the configured RVQoE specific SRB is not available, RAN2 to further clarify whether the RVQoE report should be discarded and not stored and reported later. Proposal 3b: When SN is released, FFS how to treat the unsent QoE report configured to be reported over SRB5.

In this paper, we elaborate more on the left open issues.

# 2	Discussion

In RAN2-123bis meeting, RAN2 agreed UE is not required to buffer the RVQoE report if the report cannot be sent because the configured SRB is not available.

RAN2-123bis Agreement: When UE cannot send RVQoE report because the configured RVQoE specific SRB is not available, UE is not required to buffer the RVQoE report.

However, at the email discussion[1], some company think the agreement leaves the freedom to UE implementation which may store the RVQoE report. Therefore it may lead unexpected UE behaviour such as UE report the delayed and outdated RVQoE to gNB once the SRB is available. Since the RVQoE report is time-critical for RAN optimization, the outdated RVQoE report may result in incorrect gNB actions. Hence we think it is reasonable for UE to discard the RVQoE report if the configured RVQoE specific SRB is not available.

Proposal 1: When UE cannot send RVQoE report because the configured RVQoE specific SRB is not available, the UE should discard the RVQoE report.

Another open issue is that, when SN is released, it is FFS on how to treat the unsent QoE report configured to be reported over SRB5. RAN3 has agreed that all SN configured QoE measurements should be released by UE during SN release as indicated in LS[2].

For NR-DC - Release all SN configured QoE measurements during SN release

It is our understanding that, if network thinks the QoE data of the affected QoE configuration is important, before the SN release, network should reconfigure the reporting leg from SRB5 to SRB4 to get further QoE reports. Otherwise (i.e,, if the QoE configuration is configured to report QoE data via SRB5 during SN release), the UE should discard the unsent QoE reports.

Proposal 2: When SN is released, the UE should discard the unsent QoE report configured to be reported via SRB5.

# 3	Conclusion

This document has proposed the following:

Proposal 1: When UE cannot send RVQoE report because the configured RVQoE specific SRB is not available, the UE should discard the RVQoE report.

Proposal 2: When SN is released, the UE should discard the unsent QoE report configured to be reported via SRB5.

# Reference

[1] R2-2312704	[Post123bis][618][QoE] 37.340 CR update and open issues (Nokia)

[2] R3-235912	Reply LS on Priority information and NR-DC