3GPP TSG-RAN WG2 Meeting #124	R2-2312851

# Chicago, USA, 13-17 November 2023

| CR-Form-v12.1 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.306 | CR | 0965 | rev | 1 | Current version: | 17.6.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | X | Radio Access Network | X | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Introduction of NR Support for UAV |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Huawei, HiSilicon |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R2 |  |  |  |  |  |  |  |  |  |
| Work item code: | NR_UAV-Core |  |  |  |  |  | Date: |  |  | 2023-09-21 |
| Category: | B |  |  |  |  |  | Release: |  |  | Rel-18 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-15 (Release 15) Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) |  |
| Reason for change: |  | Introduction of capabilities for Rel-18 NR Support for UAV WI |  |  |  |  |  |  |  |  |
| Summary of change: |  | Add capability bit indication for triggering based on number of cells, height-dependent measurement configurations, flight path plan report/update and dedicated resource pool. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Features of Rel-18 NR Support for UAV WI not supported |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 2, 4.2.9, 4.2.16.1 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  | X |  | Other core specifications |  |  |  | TS 38.331 CR … |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS 38.300 CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

START OF CHANGE

# 2	References

<<<<<<<<<<<skipped>>>>>>>>>>>>>

[35]	3GPP TS 38.104: "NR; Base Station (BS) radio transmission and reception".

[x]	3GPP TS 23.256: "Support of Uncrewed Aerial Systems (UAS) connectivity, identification and tracking; Stage 2".

END OF CHANGE

START OF CHANGE

### 4.2.9	MeasAndMobParameters

Editor’s note: once agreed, the following parameters needs to be implemented on top of the existing ones in 4.2.9 in alphabethical order.

| Definitions for parameters | Per | M | FDD-TDD DIFF | FR1-FR2 DIFF |
| --- | --- | --- | --- | --- |
| height-meas-r18 This field defines whether the UE supports height-based measurement reporting as specified in TS 38.331 [9]. It is mandatory to support this feature for UEs which have Aerial UE subscription as defined in TS 23.256 [x]. | UE | Yes | No | No |
| height- dependent-configurations- r18 This field defines whether the UE supports height-dependent measurement configurations as specified in TS 38.331 [9]. It is mandatory to support this feature for UEs which have Aerial UE subscription as defined in TS 23.256 [x]. | UE | Yes | No | No |
| multipleCellsMeasExtension-r18 This field defines whether the UE supports measurement reporting triggered based on a number of cells. | UE | No | No | No |
| flightPathPlan-r18 This field defines whether the UE supports reporting/updating of the flight path plan and its update through the procedure defined in TS 38.331 [9]. | UE | No | No | No |

Editor’s note: The height-meas-r18 is used for event Hx and the height-dependent-configurations-r18 is used for event Ax + Hx, height-dependent SSB-ToMeasure and height-dependent NumberOfTriggeringCells.

NEXT CHANGE

#### 4.2.16.1	Sidelink Parameters in NR

##### 4.2.16.1.1	Sidelink General Parameters

| Definitions for parameters | Per | M | FDD-TDD DIFF | FR1-FR2 DIFF |
| --- | --- | --- | --- | --- |
| accessStratumReleaseSidelink-r16 Indicates the access stratum release for NR sidelink communication the UE supports as specified in TS 38.331 [9]. | UE | Yes | No | No |
| relayUE-Operation-L2-r17 Indicates whether NR L2 sidelink relay UE operation is supported by the UE. | UE | No | No | No |
| remoteUE-Operation-L2-r17 Indicates whether NR L2 sidelink remote UE operation is supported by the UE. | UE | No | No | No |
| remoteUE-PathSwitchToIdleInactiveRelay-r17 Indicates whether L2 sidelink remote UE supports direct to indirect path switch with target relay in RRC_IDLE or RRC_INACTIVE state. | UE | No | No | No |
| sl-A2X-Service-r18 This field defines whether the UE supports dedicated resource pool for A2X service as specified in TS 38.331 [9]. | UE | No | No | No |

END OF CHANGE