3GPP TSG-RAN WG2 Meeting #124		R2-2313274

Chicago, USA, November 13th – 17th, 2023

Source:	CATT

Title:	Further considerations on LP-WUS in RRC_IDLE&INACTIVE states

Agenda Item:	7.22.2

Document for:	Discussion and Decision

Introduction

In RAN2#123bis meeting, there is no on-line discussion on LP-WUS in RRC_IDLE and INACTIVE. And during post-meeting email discussion [1], some limited scope for LP-WUS in RRC_IDLE and INACTIVE was discussed. In this contribution, we would like to provide our further consideration on the topic.

Discussion

2.1	Network awareness of whether the UE is monitoring LP-WUS or not

As summarized in the table in [2], for the network awareness of whether UE monitoring LP-WUS, it would require the UE to inform the network when it switches receivers. The table lists some associated drawbacks e.g. this will lead to more UE power consumption caused by more signalling overhead, which somehow defeats the purpose of UE power saving.

Moreover, considering the RAN2 agreement that it would be beneficial to use subgrouping for LP-WUS, in such case, the gNB may still need to transmit the LP-WUS even if only one UE in the sub-group can receive it. Thus the energy saving on the network side may be reduced in practice even if the network can be aware of whether the UE is monitoring LP-WUS or not.

Observation 1: When LP-WUS is used with subgrouping, the gNB may still need to transmit the LP-WUS even if only one UE in the sub-group can receive it. Thus the energy saving on the network side may be reduced in practice even if the network can be aware of which UEs use/don’t use LP-WUS.

Finally it remains to be discussed how an Idle UE can report such information to the network. If it requires that it switches to Connected, then it is quite ineffective.

Observation 2: If an Idle UE is required to report to the network when it switches receiver, it also requires the UE to switch to Connected, which adds further overhead and power consumption, and is ineffective, from RRC state management perspective.

According to the discussion in [1], all companies support to capture the corresponding pros/cons in the TR whether the network is aware of whether the UE is monitoring LP-WUS or not, and continue to discuss the details in WI, if it is included. Hence, we propose updating the comparison table in the TR to add the above observations:

Table X: Pros and Cons for network awareness of using LP-WUS Network knows whether UE monitors LR or MR; Network does not know whether UE monitors LR or MR. Pros; Reduce Uu resource consumption: NW only sends LP-WUS when the target UE is monitors LP-WUS; Lower false wake-up rate: When LP-WUS is not sent, the other UE monitoring LP-WUS, which is in the same group with the target paging UE, will not be waken up as a result of false wake up.; Since the UE needs not to inform the NW whether its MR is monitoring or not, the signalling overhead, Uu resource consumption, UE power consumption caused by MR state report does not exist.. Cons; More signalling overhead: UE needs to inform the NW when it starts/stops monitoring with MR. Uu resource consumption caused by more signalling overhead. More UE power consumption caused by more signalling overhead. When LP-WUS is used with subgrouping, the gNB may still need to transmit the LP-WUS even if only one UE in the sub-group can receive it. Thus the energy saving on the network side may be reduced in practice even if the network can be aware of which UEs use/don’t use LP-WUS. If an Idle UE is required to report to the network when it switches receiver, it also requires the UE to switch to Connected, which adds further overhead and power consumption and is ineffective, from RRC state management perspective.; More Uu resource consumption：NW always send LP-WUS signal given it always assume the target UE is monitoring the LP-WUS. More alarm rate of LP-WUS: in case the target UE is not monitoring LP-WUS, the other UE (monitoring the same LP-WUS as the target UE) will be waken up..

Proposal 1: RAN2 recommends updating the comparison table in the TR with the observation1&2 for the cons for network awareness of using LP-WUS.

2.2	LP-WUS monitoring and waking up procedure

In RAN2#123 meeting [3], it was agreed:

After waking up by a LP-WUS, capture the below solutions in the TR: - Alt 1.1: UE could monitor paging DCI/paging; - Alt 1.2: UE could monitor PEI, if configured and supported; FFS details on using LP-WUS and PEI together, e.g. subgrouping - FFS Alt 2: UE could perform random access directly, FFS on whether and what condition/requirement is needed. R2 assumes that this require that LP-WUS includes UE_ID or equivalent. (Depends on LP-WUS capacity to carry information)

But there is no summary of impact or comparison among the above alternatives.

For Alt 1.1, LP-WUS is used to replace PEI with lower power consumption. False paging is reduced by use of UE subgrouping for LP-WUS. The payload of LP-WUS is low even with UE subgroup indication in it. Therefore, the complexity of LP-WUS design is low. The access latency is medium which is determined by LP-WUS reception, paging reception and random access procedure.

For Alt 1.2, LP-WUS and PEI are used together. Compared with Alt 1.1, one company [4]thinks the false alarm probability may be further reduced if UE subgrouping mechanism is used in LP-WUS and PEI together. However, if the LP-WUS subgroup is larger, then there will be useless UEs wake-up, which is not desirable. In addition, the complexity of UE subgrouping mechanism is increased. Compared with Alt 1.1, the access latency is increased which is determined by LP-WUS reception, PEI reception, paging reception and random access procedure.

For Alt 2, UE performs random access directly since LP-WUS replaces both PEI and Paging altogether. The access latency is reduced which is determined by LP-WUS reception and random access procedure. But in order to support the UE to perform random access directly after reception of LP-WUS, a unique UE identifier, e.g., the 48-bit 5G-S-TMSI or 24/40-bit I-RNTI, needs to be included in the LP-WUS. This increases the payload of LP-WUS. The following issues need to be carefully studied with large payload of LP-WUS, e.g., whether the reliability can be guaranteed for this kind of LP-WUS, whether it may further reduce the coverage performance of LP-WUS. The complexity of LP-WUS design is high. There is no false alarm considering a unique UE identifier is included in LP-WUS. However, in legacy, Both Paging messages and Short Messages are addressed with P-RNTI on PDCCH. And the latter is for notifying UEs of SI change or ETWS/CMAS indications, which do not trigger RACH. So with Alt 2, it remains to sort out how SI change and ETWS/CMAS would be carried out.

The pros and cons of the above alternatives are summarized in the table below.

Table 1 pros and cons of three possible wake up procedures

|  | Alt1.1 LP-WUS+ Paging | Alt 1.2 LP-WUS+PEI+paging | Alt 2 LP-WUS+Random Access procedure directly |
| --- | --- | --- | --- |
| Access latency | LP-WUS reception+ paging reception+ random access procedure | LP-WUS reception+PEI reception+paging reception +random access procedure | LP-WUS reception +random access procedure |
| False Alarm | Can be reduced by use of UE subgrouping for LP-WUS. | Unclear if and how it can be further reduced if UE subgrouping mechanism is used in LP-WUS and PEI together. | No false alarm issue |
| Complexity of LP-WUS Design | Low | It is higher if UE subgrouping mechanism is used in LP-WUS and PEI together | High considering a unique UE identifier needs to be contained in LP-WUS |
| Integration with legacy | Smooth as LP-WUS replaces PEI | Unclear what PEI functionality adds on top of LP-WUS | It remains to sort out how SI change and ETWS/CMAS would be carried out |

Proposal 2: RAN2 recommends capturing pros and cons of three possible waking up procedures in the table in TR, as summarized in the Table 1.

2.3 LP-WUS coverage and trigger condition

It is FFS whether the entry/exit condition(s) for using LP-WUS is configured via RRC dedicated signalling, e.g. by RRC release. We see no obvious benefit to provide the entry/exit condition(s) of using LP-WUS via RRC dedicated signaling. On the other side, if we support to provide via RRC dedicated signaling, we need to further discuss which info needs to be provided by the UE to assist the network to configure the dedicated entry/exit condition(s) and which condition can be provided via RRC dedicated signaling. Hence, we proposed to deprioritize it.

Proposal 3: Configuring entry/exit condition(s) for using LP-WUS in RRC dedicated signalling is deprioritized.

2.4 RRM measurement

In last RAN2 meeting, it was agreed RRM measurement on serving cell via MR is relaxed (may include no measurement) if RRM measurement on LR is feasible/supported. It is FFS on the details, e.g. how to relax, in which condition. Ideally, when it uses LP-WUR the UE does not use MR and that is where the power saving comes from. If it still has to rely on MR to perform some relaxed measurement, then the power saving is questionable. So the best situation is when UE uses LP-WUR, it performs all RRM measurements of serving cell on LP-WUS (or LP-SS) and no measurement on MR.

Proposal 4: From RAN2 perspective, it is preferred that when UE uses LP-WUR, it performs all RRM measurements of serving cell on LP-WUS (or LP-SS) and no measurement on MR. Pending feasibility to be assessed in RAN1.

It is still FFS on the feasibility of RRM measurements performed by LP for neighboring cell in RAN1. Therefore, we think it is still too early for RAN2 to discuss neighbor cell measurements based on LP-WUR. Hence, it is proposed:

Proposal 5: RAN2 waits for more inputs from RAN1 to study RRM for neighbor cells in RC_IDLE/INACTIVE mode.

Conclusion

According to the analysis in section 2, it is observed and proposed:

Observation 1: When LP-WUS is used with subgrouping, the gNB may still need to transmit the LP-WUS even if only one UE in the sub-group can receive it. Thus the energy saving on the network side may be reduced in practice even if the network can be aware of which UEs use/don’t use LP-WUS.

Observation 2: If an Idle UE is required to report to the network when it switches receiver, it also requires the UE to switch to Connected, which adds further overhead and power consumption, and is ineffective, from RRC state management perspective.

Proposal 1: RAN2 recommends updating the comparison table in the TR with the observation1&2 for the cons for network awareness of using LP-WUS.

Proposal 2: RAN2 recommends capturing pros and cons of three possible waking up procedures in the table in TR, as summarized in the Table 1.

Table 1 pros and cons of three possible wake up procedures

|  | Alt1.1 LP-WUS+ Paging | Alt 1.2 LP-WUS+PEI+paging | Alt 2 LP-WUS+Random Access procedure directly |
| --- | --- | --- | --- |
| Access latency | LP-WUS reception+ paging reception+ random access procedure | LP-WUS reception + PEI reception +paging reception +random access procedure | LP-WUS reception +random access procedure |
| False Alarm | Can be reduced by use of UE subgrouping for LP-WUS. | Unclear if and how it can be further reduced if UE subgrouping mechanism is used in LP-WUS and PEI together. | No false alarm issue |
| Complexity of LP-WUS Design | Low | It is higher if UE subgrouping mechanism is used in LP-WUS and PEI together | High considering a unique UE identifier needs to be contained in LP-WUS |
| Integration with legacy | Smooth as LP-WUS replaces PEI | Unclear what PEI functionality adds on top of LP-WUS | It remains to sort out how SI change and ETWS/CMAS would be carried out |

Proposal 3: Configuring entry/exit condition(s) for using LP-WUS in RRC dedicated signalling is deprioritized.

Proposal 4: From RAN2 perspective, it is preferred that when UE uses LP-WUR, it performs all RRM measurements of serving cell on LP-WUS (or LP-SS) and no measurement on MR. Pending feasibility to be assessed in RAN1.

Proposal 5: RAN2 waits for more inputs from RAN1 to study RRM for neighbor cells in RC_IDLE/INACTIVE mode.

Reference

- R2-2311915, Summary of discussions on open issues for LP-WUS, vivo, RAN2#124

- R2-2311914, Update of TR 38.869 for LP-WUS, vivo (Rapporteur)

- R2-2309401, RAN2#123 meeting report;

- R2-2307082, Use of low-power receiver in RRC Idle/Inactive, Qualcomm Incorporated, discussion