3GPP TSG-RAN WG2 #124	R2-2313488

# Chicago, USA, Nov. 13th-17th, 2023

Agenda Item:	7.19.3

Source:	Ericsson

Title:	UE capability and relaxed processing timeline for eRedCap UEs

Document for:	Discussion

# Introduction

The previous RAN2 meeting agreed on many open issues [1][2]. This contribution addresses a remaining issue concerning details on UE capability for random access.

# 2	UE capability and relaxed processing timeline

In RAN2#123bis there was a discussion on capabilities for eRedCap feature group 48-1. A subset of the discussed list comprises item that only apply to Rel-18 eRedCap UEs:

The following components are new compared to supportOfRedCap-r17 (28-1):

11. DL/UL peak data rate target of 10 Mbps corresponding to vLayers·Qm·f = 3.2

12. Maximum number of PDSCH/PUSCH PRBs that can be scheduled for unicast per slot of 25 PRBs for 15 kHz SCS and 12 PRBs for 30 kHz SCS

13. Relaxed processing timeline of 1/0.5 ms for 15/30 kHz SCS when the RAR PDSCH and MsgB PDSCH (if supported) is larger than 25/12 PRBs for 15/30 kHz SCS

14. Network-configurable additional separate early indication in Msg1 for Rel-18 eRedCap UEs

FFS whether to add additional components

In the following item #13 is of particular interest where processing timeline was considered. The outcome from the discussion was the proposal below [4]:

Proposal 1.1.                  FFS whether “contention based random access” should be added in relation to the following functional component of “Relaxed RAR-PDSCH processing timeline” to indicate that is only applicable during initial access and contention based random access when eRedCap UE also supports notReducedBB-BW-r18.

In the running CR for 38.306 on UE capabilities for Rel-18 eRedCap UEs, the following editor’s note is captured:

- Relaxed RAR-PDSCH processing timeline of 1/0.5 ms for 15/30 kHz SCS when the RAR PDSCH and MsgB PDSCH (if supported) is larger than 25/12 PRBs for 15/30 kHz SCS. - If UE supporting this feature also indicates eRedCapNotReducedBB-BW-r18, this component is only applicable during initial access and contention based random access. Editor’s note: FFS whether “contention based random access” should be added in relation to the following functional component of “Relaxed RAR-PDSCH processing timeline” to indicate that is only applicable during initial access and contention based random access when eRedCap UE also supports notReducedBB-BW-r18

This issue was also raised in a contribution from the last RAN2 meeting [5]. In short, the possible confusion is that it is not clear if “applicable during initial access and contention based random access” means that it is limited to initial access, or one should also consider the contention based random access in connected mode. As [5] concludes, since the gNB allocates RAR PDSCH and MsgB PDSCH larger than 25/12 PRBs for 15/30kHz during CBRA procedure, also in RRC_CONNECTED mode, the statement applies also to “contention based random access”.

# 3	Conclusion

Based on the discussion in the previous section we propose the following:

Proposal 1	The correct interpretation is “applicable during initial access and applicable during contention based random access”.

# References

R2-123bis Chair notes for RAN2#123bis. 3GPP TSG-RAN2 Meeting #123bis, Xiamen, China, Oct 9th – 13th, 2023

R2-2311280 Report from maintenance and eRedCap breakout session. 3GPP TSG-RAN2 Meeting #123bis, Xiamen, China, Oct 9th – 13th, 2023

R2-2309734 Discussion on 2-step RACH for eRedCap. 3GPP TSG-RAN2 Meeting #123bis, Xiamen, China, Oct 9th – 13th, 2023

R2-2310211 Outcome of email discussion [POST123][753][eRedCap] on UE Capabilities for Rel-18 eRedCap. 3GPP TSG-RAN2 Meeting #123bis, Xiamen, China, Oct 9th – 13th, 2023

R2-2310832 Remaining issues of eRedCap UE capabilities. 3GPP TSG-RAN2 Meeting #123bis, Xiamen, China, Oct 9th – 13th, 2023

R2-2311428 WF on selection of RA resource set for eRedCap. 3GPP TSG-RAN2 Meeting #123bis, Xiamen, China, Oct 9th – 13th, 2023

R2-2311438 Running MAC CR for eRedCap. 3GPP TSG-RAN2 Meeting #123bis,Xiamen, China, Oct 9th – 13th, 2023

R2-2311439 Running CR 38.331 for eRedCap. 3GPP TSG-RAN2 Meeting #123bis, Xiamen, China, Oct 9th – 13th, 2023