3GPP TSG-RAN WG2 Meeting #124	R2-2313571

Chicago, USA, 13-17 Nov 2023

Source: 	Session chair (ZTE)

Title:	Report from Further NR coverage enhancements session

## Status of At-Meeting Email Discussions

[AT124][850][CE_enh]  Organisational (Session chair: ZTE)

Scope:

- Share plans and list ongoing email discussions and their status

- Share meeting notes for any comments

Status: Ongoing

[AT124][851][CE_enh]  MAC CR updates (ZTE)

Scope:

  - F2F offline discussion to discuss the Editor’s notes and any other issues in the MAC CR implementation

Intended outcome:

  - Set of agreeable proposals for MAC CR updates

Deadline:  Wednesday 15-11-2023

## Summary of Post-Meeting Email Discussions

[POST124][850][CE_enh] 38.300 CR (China Telecom)

Intended outcome: agreeable CR

Deadline:  Short (for plenary)

[POST124][851][CE_enh] 38.331 CR (Huawei)

Intended outcome: agreeable CR

Deadline:  Short (for plenary)

[POST124][852][CE_enh] 38.321 CR (ZTE)

Intended outcome: agreeable CR

Deadline:  Short (for plenary)

## 7.21	Further NR coverage enhancements

(NR_cov_enh2-Core; leading WG: RAN1; REL-18; WID: RP-221858)

Time budget: 0.5 TU

Tdoc Limitation: 2 tdoc

### 7.21.1	Organizational

Incoming LSs, Rapporteur input etc, including reports from [Post123bis][851] and [Post123bis][853].

Incoming LSs:

R2-2311710	Reply LS on RAN1 impacts regarding enhancements to realize increasing UE power high limit for CA and DC (R1-2310518; contact: Nokia)	RAN1	LS in	Rel-18	NR_cov_enh2-Core	To:RAN4	Cc:RAN2

Noted

R2-2311757	LS reply on further clarifications on enhancements to realize increasing UE power high limit for CA and DC (R4-2317768; contact: Huawei)	RAN4	LS in	Rel-18	NR_cov_enh2	To:RAN1, RAN2

Huawei indicate that there could be some dependency between the DPC and ul-FullPowerTransmission and may have some impact on the UE capability discussion.

Noted

Endorsement of Running CRs

R2-2312732	Introduction of Further NR coverage enhancements to 38.300	China Telecom	CR	Rel-18	38.300	17.6.0	0733	-	B	NR_cov_enh2-Core

Update the WI code to NR_cov_enh2-Core

Endorsed as baseline

R2-2312573	Introduction of Further NR coverage enhancements in RRC	Huawei, HiSilicon	CR	Rel-18	38.331	17.6.0	4433	-	B	NR_cov_enh2-Core	R2-2310197

Endorsed as baseline

R2-2312772	Introduction of Further NR Coverage Enhancements in MAC spec	ZTE Corporation, Sanechips	CR	Rel-18	38.321	17.6.0	1711	-	B	NR_cov_enh2-Core

Endorsed as baseline

[AT124][851][CE_enh]  MAC CR updates (ZTE)

Scope:

  - F2F offline discussion to discuss the Editor’s notes and any other issues in the MAC CR implementation

Intended outcome:

  - Set of agreeable proposals for MAC CR updates

Deadline:  Wednesday 15-11-2023

Status: Ongoing

R2-2313761	Report of [AT124][851][CE_enh] MAC CR updates (ZTE)		ZTE Corporation	CR	Rel-18	38.321	17.6.0	1711	-	B	NR_cov_enh2-Core

Proposal 1  Adopt below MAC CE format for Multiple Entry PHR with assumed PUSCH MAC CE.

DISCUSSION

Nokia indicate that there is a typo in the above format, the wrong byte was removed.

Huawei think we also need to agree the LCID and LCP for this new MAC CE.  LG would like to make sure this is the one octet eLCID.

Update the above format to remove the correct byte from the PHR MAC CE and finalise this in the post meeting email discussion

Use one Octet eLCID for this new PHR MAC CE and the same priority as the legacy PHR MAC CE will apply for this new PHR MAC CE

Proposal 2  On how to capture the triggering of new PHR MAC CE, to take LG’s TP (R2-2313463) as a baseline, with following considerations:

- Change “assumedPUSCHInfo” into “phr-AssumedPUSCH-Reporting” (to align with RRC spec)

- Update the sentence like: “4>	obtain the value for the corresponding PCMAX,f,c field for assumed PUSCH from the physical layer, if provided if available, as specified in clause 7.7 of TS 38.213 [6].”

- Because it is per CG configured, so whether/how to update the sentence “this Serving Cell is configured with assumed PUSCH” can be further discussed during CR reviewing phase.

For the specification of triggering of the new PHR MAC CE, the MAC CR can use twoPHRMode type of implementation from MAC spec as base line, details FFS.

Proposal 3	Agree the intention of the second change in R2-2313430, how to capture it in MAC spec can be further discussed during CR reviewing phase (e.g. whether to split it into two separate paragraphs, whether to change the term “set of ROs”).

DISCUSSION

ZTE indicate that the MAC spec is complicated for this sentence and may be worth splitting this.

Nokia think we need to agree that the next available

For Issue 2, it was not discussed during the official Wednesday offline, but based on some offline feedbacks, the Option 2 TP from HW/Samsung looks reasonable. So, if time allows, rapporteur suggest to discuss it online.

Proposal 4	To take the Option 2 TP of issue 2 as a baseline.

- In 5.1.1d, the Note is removed, instead, add following texts to normative text:

1>	else if more than one set of Random Access resources is identified:

2>	if all the identified sets of Random Access resources are configured with Msg1 repetition indication and the same featureCombination.

3> select the set of Random Access resources that associated with highest repetition number among the identified sets of Random Access resources.

2> else:

3>	repeat the procedure taking as an input the identified sets of Random Access resources and the feature applicable to the current Random Access procedure with the highest priority assigned in featurePriorities among all the features applicable to this Random Access procedure, except the features considered already.

- In 5.1.1b, add the following texts in order to address the case that more than one identified RACH resources sets are associated with all features that triggering the RACH procedure (e.g. Msg1 repetition, or RedCap+Msg1 repetition).

2>	else if there is one set of Random Access resources available which can be used for indicating all features triggering this Random Access procedure:

3>	select this set of Random Access resources for this Random Access procedure.

2>	else if there are more than one set of Random Access resources available which can be used for indicating all features triggering this Random Access procedure (i.e. for Msg1 repetition):

3>	select the set of Random Access resources that associated with highest repetition number among the sets of Random Access resources.

2>	else (i.e. there are one or more sets of Random Access resources available that are configured with indication(s) for a subset of all features triggering this Random Access procedure):

3>	select a set of Random Access resources from the available set(s) of Random Access resources based on the priority order indicated by upper layers as specified in clause 5.1.1d for this Random Access Procedure.

DISCUSSION

ZTE indicate that the above TP may be acceptable.

For the selection of set of RACH resources associated with highest repetition number when more than one set of RACH resources available, convert the note into normative text – Details FFS and can be worked offline during the CR finalisation.

For issue 1, it was not discussed during the official Wednesday offline, but several companies (Samsung/LG/ZTE) have provided TPs for this, however, companies still need more time to discuss those TPs and find a way forward, so it is recommended to continue the discussion and the outcome will be taken as a baseline for follow-up CR reviewing.

Proposal 5	Continue discussing the TPs for capturing Msg1-based SI request with Msg1 repetition in MAC/RRC specs. Based on the outcome of the discussion, Rapporteur will provide the updated MAC CR for reviewing.

Open issues and rapporteur proposals

R2-2312572	Summary of [POST123bis][851][CE_enh] CP running CR and open issues (Huawei)	Huawei, HiSilicon	discussion	NR_cov_enh2-Core

Easy proposals:

Proposal 2 (9/10): The value of rsrp-ThresholdSSB for MSG1 repetition is common to different repetition number.

Discussion

LG are okay with the proposal but the current CR supports different values to be configured. So, what is the impact to the running CR?

HW explain that some restriction in field description may be needed if we agree this.

Samsung why this restriction is needed.

ZTE and Ericsson also think this is not needed

Proposal 3(9/10): Separate MSG3 repetition parameter (e.g. numberOfMsg3-RepetitionsList and mcs-Msg3-Repetitions) when MSG1 repetition is applicable is not supported.

Discussion

Separate MSG3 repetition parameter (e.g. numberOfMsg3-RepetitionsList and mcs-Msg3-Repetitions) when MSG1 repetition is applicable is not supported as implemented in the current running CR

Proposal 4(10/10): The values of preambleTransMax-Msg1Repetition are { n1, n2, n4, n6, n8, n10, n20, n50, n100, n200}.

Discussion

The values of preambleTransMax-Msg1Repetition are { n1, n2, n4, n6, n8, n10, n20, n50, n100, n200}

Proposal 6(9/10): CFRA configured with one MSG1 repetition number can be applied to CHO. No further optimization of CFRA is needed in this case.

Discussion

ZTE wonder if this  means that for fallback case we will still use the same repetition number, companes seem to agree with this understanding.

CFRA configured with one MSG1 repetition number can be applied to CHO. No further optimization of CFRA is needed in this case (and in this case the same repetition will be used upon fallback to CBRA as already agreed in the past)

May need online discussion:

Proposal 1(7/10): RAN2 to discuss if numberOfRA-PreamblesGroupA can be configured separately for different repetition number.

Discussion

LG think that there is no need to have any restriction for this either (similar to the RSRP threshold). Ericsson, ZTE also support,

HW indicate that the issue is if there is a different configuration for different repetition number then there may be issues in case of fallback.

Samsung think if we agree that it is up to the network configuration to allow that there is no need for preamble group reselection. Especially there should be no need to specify rebuilding.

numberOfRA-PreamblesGroupA can be configured separately for different repetition number.

Proposal 5(6/10): From RAN2 CE perspective, the maximum number of RACH configuration that the network is allowed to configure can be extended. To decide the maximum value between 32 and 64.

Huawei highlight that the current restriction is for maximum of 16.

Samsung think 16 is already too many even considering MSG1 repetition. Nokia and CATT agrees.

ZTE think 64 will be preferable. There will be other features requiring eREDCAP also.

LG agree that some extension may be needed but can be discussed further.

Huawei think that even without considering eREDCAP we need to extend this.

From CE perspective, the maximum number of RACH configurations that the network is allowed to configure may need to be extended to 32. Can be revisited if other features need other number

R2-2312771	UP open issue list for R18 CE	ZTE Corporation, Sanechips	Work Plan	Rel-18	NR_cov_enh2-Core

DISCUSSION on overall completion of the WI

ZTE indicate that capabilities and RAN1/RAN4 parameters will be implemented in the RRC once they are finalised.

Further NR coverage enhancements WI is considered to be complete from RAN2 perspective

### 7.21.2	Control plane issues

Details of RACH configuration and RACH partitioning signalling and any other impacts to CP from RAN1 agreements.

SI request period/RA association period index with repetition

R2-2311830	SI request and CFRA Aspects	Samsung Electronics Co., Ltd	discussion	Rel-18	NR_cov_enh2-Core

Proposal 1: Adopt the SI-RequestConfigRepetition structure above in the RRC CR.

Delete si-RequestResourcesRepetition-r18

Add si-RequestResourcesRepetitionTwo-r18, si-RequestResourcesRepetitionFour-r18 and si-RequestResourcesRepetitionEight-r18, optionally, where each is SEQUENCE (SIZE (1..maxSI-Message)) OF SI-RequestResources

Focus on P1

LG support the new structure as it reduces the overhead.

HW clarify that there is no functionality change and agree that this is the simplest one.

Delete si-RequestResourcesRepetition-r18

Add si-RequestResourcesRepetitionTwo-r18, si-RequestResourcesRepetitionFour-r18 and si-RequestResourcesRepetitionEight-r18, optionally, where each is SEQUENCE (SIZE (1..maxSI-Message)) OF SI-RequestResources

R2-2311816	Discussion on Remaining Issues for PRACH Repetition	vivo	discussion	Rel-18	NR_cov_enh2-Core

Proposal 2: SI request period is not applicable for Msg-1 based SI request with Msg1 repetition.

Focus on P2

R2-2313163	Discussion on Coverage Enhancements CP	Ericsson	discussion	NR_cov_enh2-Core

Proposal 5: RAN2 to discuss the alternatives described and the impact on RAN1/RAN2 specifications.

Opt A) Collision handling in RAN1 specification handles this with a restriction that multiple PRACH transmissions are restrained in the indicated association period in one si-RequestPeriod.

Opt B) The ra-AssociationPeriodIndex denotes the first association period in every si-RequestPeriod, and the UE implicitly determines a number of association periods starting from the indicated association period within si-RequestPeriod for the multiple PRACH transmissions.

Opt C) Multiple PRACH transmissions can be performed in ROs in the indicated association period in more than one si-RequestPeriod.

Opt D) a UE doesn't expect a configured PRACH repetition factor for SI request to be larger than the number of ROs permitted by ra-ssb-OccasionMaskIndex in an association period.

Focus on P5

Combined discussion on P12 and P5 above

Huawei think another alternative is to highlight that in this case the RA association period is associated with a set of RACH occasions. This can be clarified in field description.

Samsung agree with the proposal and think that solving this issue might be too late for this issue.

LG think one option is to apply the SI request occasion only for the first period and rest can be in RAN1 specifications.

Ericsson and Chinatelecom indicate that they prefer option D

Samsung indicate that it is still not clear how the options A – D in R2-2313163 work.

SI request period is not applicable for Msg-1 based SI request with Msg1 repetition (can comeback if there is a critical issue with this agreement)

LTM applicability

R2-2311830	SI request and CFRA Aspects	Samsung Electronics Co., Ltd	discussion	Rel-18	NR_cov_enh2-Core

Proposal 3: Msg1 repetition is supported for both CBRA and CFRA based LTM cell switch. Msg1 repetition number can be signalled in cell switch command MAC CE.

Focus on P3

R2-2312574	Remaining issues of CP aspects for CE	Huawei, HiSilicon	discussion	NR_cov_enh2-Core

Proposal 2: CFRA with repetition is applicable to LTM with no additional change.

Focus on P2

Combined discussion on P3 and P2 above

Vivo, LG think this discussion should happen in LTM session.

ZTE indicate that the dedicated configuration for LTM can also be provided in RRC signalling. And some companies in LTM session think this should be discussed in CE.

Nokia think we can indicate this is feasible from CE point of view and leave the MAC CE design to LTM session.

Samsung indicate that they are fine to discuss it in LTM session.

Huawei don’t support the proposal.

From CE perspective, Msg1 repetition is feasible for both CBRA and CFRA based LTM cell switch assuming the MSG1 repetition configuration is in the RACHConfigDedicated.

Open if time allows (after all other discussions)

R2-2311816	Discussion on Remaining Issues for PRACH Repetition	vivo	discussion	Rel-18	NR_cov_enh2-Core

R2-2311830	SI request and CFRA Aspects	Samsung Electronics Co., Ltd	discussion	Rel-18	NR_cov_enh2-Core

R2-2312511	Discussion on the remaining CP issues	NEC Corporation.	discussion	Rel-18	NR_cov_enh2-Core

R2-2312574	Remaining issues of CP aspects for CE	Huawei, HiSilicon	discussion	NR_cov_enh2-Core

R2-2312750	Discussion on numberOfRA-PreamblesGroupA for Msg1 repetition	CATT	discussion	Rel-18	NR_cov_enh2-Core

R2-2312773	Remaining CP issues for CE	ZTE Corporation, Sanechips	discussion	Rel-18	NR_cov_enh2-Core

R2-2313163	Discussion on Coverage Enhancements CP	Ericsson	discussion	NR_cov_enh2-Core

R2-2313462	Remaining CP issues on Msg1 repetition	LG Electronics Inc.	discussion	Rel-18	NR_cov_enh2-Core

### 7.21.3	User plane issues

Overall RACH procedure and any other MAC impacts

Open issues for RA procedure

CFRA fallback

R2-2313164	Discussion on Coverage Enhancements UP	Ericsson	discussion	NR_cov_enh2-Core

Proposal 2 RAN2 does not support the fallback from lower to higher number of multiple PRACH Transmissions if UE has performed fallback from CFRA to CBRA.

Focus on P2

R2-2312575	Remaining issues of UP aspects for CE	Huawei, HiSilicon	discussion	NR_cov_enh2-Core

Proposal 1: Fallback from lower repetition number to higher repetition is also applicable when CFRA with repetition is configured.

Focus on P1

Combined discussion on P2 and P1 above

Huawei indicate that they are also fine with Proposal from Ericsson.

Fallback from lower to higher number of multiple PRACH Transmissions is not supported if UE has performed fallback from CFRA to CBRA

Fallback for SI request

R2-2312774	Remaining UP issues for CE	ZTE Corporation, Sanechips	discussion	Rel-18	NR_cov_enh2-Core

Proposal 1 Fallback from lower number to higher number is not supported for Msg1-based SI request with Msg1 repetition.

Focus on P1

Fallback from lower number to higher number is not supported for Msg1-based SI request with Msg1 repetition.

DWS open issues

R2-2312725	Discussion on PHR for dynamic waveform switching	Xiaomi	discussion	Rel-18

Proposal 1: RAN2 agree to introduce Ei field for each serving cell to indicate the existence of PH information for assumed PUSCH in multiple entry PHR with assumed PUSCH MAC CE.

Focus on P1

R2-2313018	PHR for assumed PUSCH	InterDigital	discussion	Rel-18	NR_cov_enh2-Core

Proposal 2: If a value for Pcmax for assumed PUSCH is not available from lower layers for an NR cell or for any LTE cell, UE reports “R” bits instead of Pcmax.

Focus on P2

Discussion on P1 from R2-2312725 and P2 from R2-2313018

Nokia think we don’t need either of these proposals and highlight that the alternative according to P9 in can be used R2-2313431 instead. Nokia also indicate that there is this 1 byte overhead unnecessarily.

HW think for CA case this may be unambiguous but for DC case there may be a problem.

Interdigital think that Nokia proposal is acceptable assuming there is no ambiguity at network side. IDC indicate that the DC case can be also solved by network implementation.

introduce Ei field for each serving cell to indicate the existence of PH information for assumed PUSCH in multiple entry PHR with assumed PUSCH MAC CE (can double check the implementation in MAC offline and comeback on Thursday if needed)

Proposal 1: If DWS is configured, the UE reports the new PHR format for assumed PUSCH for any triggered PHR.

Discussion on P1

IDC highlight that this is needed as we already agreed no discussion on new triggers at last meeting.

LG wonder how this can work if the gNB doesn’t support the DWS feature.

IDC indicate that there is an RRC configure the transmission of assumed PUSCH.

Vivo think that there is DCI indication that should enable this. Session chair indicate that the DCI only switches between the waveforms.

LG wonder what “any triggered PHR means”. IDC explain that this refers to any PHR triggered by legacy triggers.

LG ask how this is impacted if the 2 PHR mode is configured.

If DWS is configured for the MAC entity transmitting PHR, the UE uses the new PHR format for PHR reporting (details on how to implement this in MAC CR is FFS can be discussed as part of 851 offline)

Proposal 3: The new PHR format for assumed PUSCH is not reported if twoPHRmode is configured.

The new PHR format for assumed PUSCH is not reported if twoPHRmode is configured unless RAN1 indicates us otherwise

DPC open issues

R2-2313431	Delta Power Class and assumed PUSCH reporting	Nokia, Nokia Shanghai Bell	discussion	Rel-18	NR_cov_enh2-Core

Proposal 1: ΔPPowerClass is reported in a PHR MAC CE upon a trigger to report ΔPPowerClass.

Discussion

Ericsson support P1

LG think the final decision depends on whether 2 bits are enough or not. Think that we can only agree this as a baseline.

Qualcomm think 2 bits for DPC and 2 bits for change in full power MIMO transmission

Ericsson highlight that even if we need more than 2 bits it will still be in the PHR MAC CE.

As a baseline ΔPPowerClass is reported in a PHR MAC CE upon a trigger to report ΔPPowerClass.

- DISCUSSION on Thursday post RAN4 progress

LG think that 2 bits may be sufficient but there may be further impacts to our design if there are more band combinations.

ZTE indicate that RAN4 agreements mean that 2 bits is sufficient.

LG think 2 bits may not work in case of band combination for CA/DC

QC agree that 2 bits per serving cell would mean that band combinations case will cover all cases. Ericsson agree. Nokia agree and think we can make progress based on this and revisit if RAN4 indicate to us something else is needed on top.

RAN2 assumes that 2 bit MPE field can be reused for indicating the ΔPPowerClass. Can be revisited if RAN4 design needs some updates for this assumption.

Proposal 2: MAC entity triggers PHR upon ΔPPowerClass reporting is triggered based on the conditions specified by the RAN4 into RAN4 specifications.

Ericsson think some triggering specification work is needed in MAC.

RAN2 preference is that triggering of PHR for ΔPPowerClass reporting is based on the power class change conditions specified by RAN4 and we will add a reference to RAN4 specs in the MAC spec.

Proposal 3: ΔPPowerClass reporting is provided per Serving Cell.

ZTE indicate that this is still being discussed in RAN4. So, we can wait. But we can still reuse the legacy MAC CE format regardless, but final discussion can wait until RAN4 conclusion.

Nokia indicate that per serving cell is simpler but we can wait for RAN4 input.

- Discussion on Thursday

ZTE indicate that there may be different terminology used in RAN2 and RAN4 for this and the proposal above covers both CA and DC cases.  Nokia agree and suggest to add “Any” to cover CA/DC case

Huawei need more time to check the latest RAN4 final agreement before we can confirm this.

RAN2 assumes that any ΔPPowerClass reporting is provided per Serving Cell and this can also be revisited if the RAN4 design is not compatible with this assumption.

Observation 1: The two MPE bits used for reporting P-MPR could be used for reporting ΔPPowerClass given the MPE reporting is applicable only for FR2 while ΔPPowerClass reporting is relevant for FR1 only.

Proposal 4: If the reporting of full power MIMO transmission capability per ΔPPowerClass would be agreed to be supported, the UE can report the full power MIMO transmission capability per ΔPPowerClass in UE capabilities and it does not need to be included in the PHR MAC CE.

QC don’t think that from a UE implementation this proposal is acceptable. So we should wait for RAN4 feedback.

Vivo clarify that this is not just for MAC signalling but any dynamic signalling is precluded. Nokia confirm this understanding.

DISCUSSION

Nokia indicate that full power MIMO transmission capability is now not agreed for Rel-18. RAN4 may still be discussing the band combinations.

Proposal 5: Use the two MPE bits for ΔPPowerClass reporting in the PHR MAC CE.

Focus on P1-5

R2-2311817	Discussion on RAN2 Impacts of DWS and DPC Reporting	vivo	discussion	Rel-18	NR_cov_enh2-Core

R2-2311829	Fallback from lower repetition number to higher repetition number	Samsung Electronics Co., Ltd	discussion	Rel-18	NR_cov_enh2-Core

R2-2311993	Open issues of power domain enhancements for CE	China Telecom	discussion	Rel-18	NR_cov_enh2-Core

R2-2312575	Remaining issues of UP aspects for CE	Huawei, HiSilicon	discussion	NR_cov_enh2-Core

R2-2312725	Discussion on PHR for dynamic waveform switching	Xiaomi	discussion	Rel-18

R2-2312751	Discussion on remaining UP issues for Msg1 repetition	CATT	discussion	Rel-18	NR_cov_enh2-Core

R2-2312774	Remaining UP issues for CE	ZTE Corporation, Sanechips	discussion	Rel-18	NR_cov_enh2-Core

R2-2312954	Open Issues in PRACH Repetition	Qualcomm Incorporated	discussion

R2-2312956	DPC and DWS UE reporting	Qualcomm Incorporated	discussion	Rel-18

R2-2313018	PHR for assumed PUSCH	InterDigital	discussion	Rel-18	NR_cov_enh2-Core

R2-2313164	Discussion on Coverage Enhancements UP	Ericsson	discussion	NR_cov_enh2-Core

R2-2313430	Miscellaneous issues with PRACH repetition	Nokia, Nokia Shanghai Bell	discussion	Rel-18	NR_cov_enh2-Core

R2-2313431	Delta Power Class and assumed PUSCH reporting	Nokia, Nokia Shanghai Bell	discussion	Rel-18	NR_cov_enh2-Core

R2-2313463	Remaining issues on Coverage Enhancement in UP aspects	LG Electronics Inc.	discussion	Rel-18	NR_cov_enh2-Core