3GPP TSG RAN WG2 Meeting #124                                                                         R2-2313956

Chicago, USA, 13-17 Nov, 2023

Agenda Item:	7.12.2.4

Source:	Intel Corporation

Title:	UE cell (re)selection TP to TS38.304

Document for:	Discussion and Decision

# Introduction

In this contribution, as rapporteur, we propose a way-forward of mobile IAB-cell prioritization.

# Discussion

## TP to TS38.304

A UE on a vehicle with a mobile IAB-cell may consider the frequency for which a mobile IAB cell is the best cell to be the highest priority.  The UE may identify a mobile IAB cell by the mIAB-cell type indicator [ref 38.331] in SIB1 or the assistance information in SIB4. The UE may narrow its search scope for mobile IAB cell(s) by assistance information (frequency and PCI list) if broadcasted in SIB4. A cell not indicated by the assistance information may be excluded from mobile IAB frequency prioritization for up to 300 seconds.

# Conclusion

In this contribution, we continue discuss the remaining open issues of UE cell reselection frequency prioritization.

In summary, we have following proposals:

Proposal 1: RAN2 to agree below TP as baseline to TS38.304 for UE cell re-selection frequency prioritization:

A UE on a vehicle with a mobile IAB-cell may consider the frequency for which a mobile IAB cell is the best cell to be the highest priority.  The UE may identify a mobile IAB cell by the mIAB-cell type indicator [ref 38.331] in SIB1 or the assistance information in SIB4. The UE may narrow its search scope for mobile IAB cell(s) by assistance information (frequency and PCI list) if broadcasted in SIB4. A cell not indicated by the assistance information may be excluded from mobile IAB frequency prioritization for up to 300 seconds.

# References