3GPP TSG-RAN WG2 Meeting #124	R2-2313991

# Chicago, US, 13 - 17 November 2023	was R3-238086

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 37.340 | CR | 0379 | rev | - | Current version: | 17.6.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network | x | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Introduction of subsequent CPAC |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | R3 (ZTE, China Telecom, Huawei, China Unicom, LG Electronics, Samsung, Ericsson, Qualcomm Incorporated, NEC, Nokia, Nokia Shanghai Bell, Lenovo, CATT) |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R2 |  |  |  |  |  |  |  |  |  |
| Work item code: | NR_Mob_enh2-Core |  |  |  |  |  | Date: |  |  | 2023-11-22 |
| Category: | B |  |  |  |  |  | Release: |  |  | Rel-18 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | Introduce subsequent CPAC |  |  |  |  |  |  |  |  |
| Summary of change: |  | #121: - Capture RAN3 related part from endorsed RAN2’s running CR 2306952 - Introduce texts to reflect RAN3’s agreements #121bis: - Revise texts to describe MN behaviour after CPC execution #122: - Use new signalling flow chart and procedural texts to capture subsequent CPAC |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Subsequent CPAC is not supported |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 10.X (new) |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  | X |  | Other core specifications |  |  |  | TS 38.423 CR 1051 TS 38.473 CR 1227 TS 37.483 CR 0097 |  |  |
| affected: |  |  | x | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | x | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  | The words highlighted in gray are copied from RAN2’s running CR R2-2312235 |  |  |  |  |  |  |  |  |
| This CR's revision history: |  | This CR was endorsed by RAN3 in R3-238086. |  |  |  |  |  |  |  |  |

START OF CHANGES

## 10.X	Subsequent Conditional PSCell Addition or Change

A Subsequent Conditional PSCell Addition or Change (subsequent CPAC) is defined as a conditional PSCell addition or change procedure that is executed after a PSCell addition, a PSCell change or an SCG release based on pre-configured subsequent CPAC configuration of candidate PSCell(s) without reconfiguration and re-initiation of CPC/CPA. The UE keeps the configured subsequent CPAC configuration and evaluates the execution conditions of candidate PSCells after completion of a PSCell addition or a PSCell change. Intra-SN subsequent CPAC, inter-SN subsequent CPAC initiated either by MN or SN are supported.

The following principles apply to subsequent CPAC:

-	For MN initiated subsequent CPAC, the MN initially triggers the candidate cell preparation of subsequent CPAC procedure and generates the execution conditions for CPA execution and initial CPC execution.

-	For SN initiated subsequent CPAC, the source SN initially triggers the candidate cell preparation of subsequent CPAC procedure and generates the execution conditions for initial CPC execution.

-	For both MN and SN initiated subsequent CPAC, the candidate SN generates the execution conditions for the following execution of the subsequent CPAC when the candidate SN prepares the candidate SCG configuration(s) for candidate PSCell(s).

-	The subsequent CPAC configuration contains the configuration of candidate PSCell(s), execution conditions for both the initial CPAC execution and the following subsequent CPAC execution, and may contain the MCG configuration (to be applied when CPAC execution is triggered), the reference configuration and the security update configuration.

-	The subsequent CPAC configuration for CPA or inter-SN CPC candidate PSCell(s) is provided in MN format. The subsequent CPAC configuration for intra-SN CPC candidate PSCell(s) is provided in MN format or SN format.

-	For one UE, the subsequent CPAC configuration for all CPC candidate PSCells (including inter-SN and/or intra-SN) is provided in the same format, i.e., the MN format, or the SN format. If the configured CPC candidate PSCell(s) includes at least one inter-SN CPC candidate PSCell, the subsequent CPAC configuration can only be provided in MN format. If only intra-SN CPC candidate PSCell(s) is configured, the subsequent CPAC configuration can be provided in either MN format or SN format.

-	Each candidate PSCell configuration can be provided as delta configuration on top of a reference configuration, which is used to form a complete candidate cell configuration. Only one reference configuration is supported.

-	The MN generates the MCG part of the reference configuration (if any), while the SN (source or candidate) generates the SCG part of the reference configuration. The MN can request an SCG reference configuration from any one of the involved SNs.

-	The network explicitly configures a subsequent CPAC configuration for the current serving PSCell if the network wants to use that PSCell as a candidate PSCell for subsequent CPAC.

-	The network always explicitly releases the subsequent CPAC configuration for candidate PSCells after an inter-MN PCell change.

-	Upon the release of SCG, the UE releases the stored subsequent CPAC configuration in SN format.

-	A candidate PSCell configuration for CPA can be used for the subsequent CPC execution, but with different execution conditions of the candidate PSCell.

-	The subsequent CPAC configuration with CPA execution condition(s) maintained after SCG release can be used for the subsequent CPA execution.

-	Upon inter-SN subsequent CPAC execution, the UE uses a first unused sk-Counter value for S-KgNB generation, based on the per-SN pre-configured sk-Counter value list.

-	Upon PCell change, PSCell change or SCG release, if the subsequent CPAC configuration is maintained, the UE also maintains the unused sk-Counter values.

MN initiated subsequent CPAC

The subsequent CPAC procedure is initiated by the MN for inter-SN subsequent CPAC configuration and inter-SN CPC execution.

Figure 10.X-1: Inter-SN subsequent CPAC - MN initiated

Figure 10.X-1 shows an example signalling flow for the inter-SN subsequent CPAC initiated by the MN:

1/2/3/4.	The MN initiates the inter-SN subsequent CPAC by requesting the candidate SN(s) to allocate resources for the UE by means of the SN Addition procedure, indicating that the request is for subsequent CPAC. The MN also provides the candidate cells recommended by MN via the latest measurement results for the candidate SN(s) to choose and configure the SCG cell(s), provides the lists of candidate PSCells proposed to other candidate SN(s) and provides the upper limit for the number of PSCells that can be prepared by the candidate SN. Within the list of cells as indicated within the measurement results indicated by the MN, the candidate SN decides the list of PSCell(s) to prepare (considering the maximum number indicated by the MN) and, for each prepared PSCell, the candidate SN decides other SCG SCells and provides the new corresponding SCG radio resource configuration to the MN in an NR RRCReconfiguration** message contained in the SN Addition Request Acknowledge message with the prepared PSCell ID(s). For each prepared PSCell, the candidate SN also decides the execution conditions for the following execution of the subsequent CPAC within the lists of candidate PSCells proposed to other candidate SN(s). If data forwarding is needed, the candidate SN provides data forwarding addresses to the MN. The candidate SN may also propose data forwarding to the MN or other candidate SN(s) for subsequent CPAC. The candidate SN includes the indication of the full or delta RRC configuration. The candidate SN can either accept or reject each of the candidate cells listed within the measurement results indicated by the MN, i.e. it cannot configure any alternative candidates.

The MN may select one of the candidate SN(s) and requests providing the reference configuration as part of the SN Addition procedure. Once obtained, the MN provides the reference configuration to other candidate SN(s).

NOTE 1:	If the UE is configured with SN-1 in Dual Connectivity operation, then the MN starts the subsequent CPAC operation with MN-initiated SN Modification procedure instead of the SN Addition procedure.

NOTE 2:	If the UE is configured with SN-1 in Dual Connectivity operation, then the MN may trigger the MN-initiated SN Modification procedure (to the source SN) to retrieve the current SCG configuration or request a reference configuration for the subsequent CPAC, and to allow provision of data forwarding related information before step 1.

NOTE 3:	If applicable, the MN stores the data forwarding addresses and data forwarding proposals provided from all the candidate SN(s).

5.	For SN terminated bearers using MCG resources, the MN provides Xn-U DL TNL address information in the Xn-U Address Indication message to the candidate SN(s).

6/7.	If the lists of prepared PSCells received from the candidate SN(s) in steps 2 and 4 are different than the lists of proposed PSCells, the MN initiates the SN Modification procedures towards all candidate SN(s) to inform them about the updated lists of prepared PSCells in other candidate SN(s).

8.	The MN sends to the UE an RRCReconfiguration message including the subsequent CPAC configuration, i.e. a list of RRCReconfiguration* messages and associated execution conditions for the subsequent CPAC, in which each RRCReconfiguration* message contains the SCG configuration in the RRCReconfiguration** message received from one of the candidate SN(s) in steps 2 and 4, and possibly an MCG configuration. Besides, the RRCReconfiguration message can also include an updated MCG configuration, e.g., to configure the required conditional measurements.

9.	The UE applies the RRCReconfiguration message received in step 3, stores the subsequent CPAC configuration and replies to the MN with an RRCReconfigurationComplete message. In case the UE is unable to comply with (part of) the configuration included in the RRCReconfiguration message, it performs the reconfiguration failure procedure.

11.	The UE starts evaluating the execution conditions. If the execution condition of one candidate PSCell is satisfied, the UE applies RRCReconfiguration* message corresponding to the selected candidate PSCell, and sends an MN RRCReconfigurationComplete* message, including an RRCReconfigurationComplete** message for the selected candidate PSCell, and information enabling the MN to identify the SN of the selected candidate PSCell. The RRCReconfigurationComplete* message may also include the sk-Counter value associated with the selected candidate PSCell if a new sk-Counter is selected.

12.	The MN informs the SN of the selected candidate PSCell that the UE has completed the reconfiguration procedure successfully via SN Reconfiguration Complete message, including the RRCReconfigurationComplete** message.

13.	The UE performs synchronisation towards the PSCell indicated in the RRCReconfiguration* message applied in step 12. The order the UE sends the MN RRCReconfigurationComplete* message and performs the Random Access procedure towards the SCG is not defined. The successful RA procedure towards the SCG is not required for a successful completion of the RRC Connection Reconfiguration procedure.

14.	If PDCP termination point is changed to the SN for bearers using RLC AM, and when RRC full configuration is not used, the MN sends the SN Status Transfer message.

15.	For SN terminated bearers or QoS flows moved from the MN, dependent on the characteristics of the respective bearer or QoS flow, the MN may take actions to minimise service interruption due to activation of MR-DC (Data forwarding).

16.	If data forwarding is needed, the MN may send the Xn-U Address Indication message to the selected candidate SN. The SN may decide to perform, if applicable, early data forwarding for SN-terminated bearers, together with the sending of an Early Status Transfer message to the source MN.

NOTE 4:	Separate Xn-U Address Indication procedures may be initiated to provide different forwarding addresses of the prepared subsequent CPAC. In this case, it is up to the MN and the candidate SN implementations to make sure that the Early Status Transfer message(s) from the selected SN, if any, is forwarded to the right other candidate SN.

18.	The UE starts evaluating the execution conditions. If the execution condition of one candidate PSCell is satisfied, the UE applies RRCReconfiguration* message corresponding to the selected candidate PSCell, and sends an MN RRCReconfigurationComplete* message, including an RRCReconfigurationComplete** message for the selected candidate PSCell, and information enabling the MN to identify the SN of the selected candidate PSCell. The RRCReconfigurationComplete* message may also include a sk-Counter value associated with the selected candidate PSCell if a new sk-Counter is selected.

19.	The MN informs the SN of the selected candidate PSCell that the UE has completed the reconfiguration procedure successfully via SN Reconfiguration Complete message, including the RRCReconfigurationComplete** message.

20.	The UE performs synchronisation towards the PSCell indicated in the RRCReconfiguration* message applied in step 19. The order the UE sends the MN RRCReconfigurationComplete* message and performs the Random Access procedure towards the SCG is not defined. The successful RA procedure towards the SCG is not required for a successful completion of the RRC Connection Reconfiguration procedure.

21/22/23.	The MN triggers the MN initiated SN Modification procedure to inform the last serving SN to stop providing user data to the UE, to switch to the prepared state, and if applicable, to allow provisioning of new data forwarding addresses. If applicable, the MN triggers the Xn-U Address Indication procedure to inform the last serving SN the address of the SN of the selected candidate PSCell, to start late data forwarding.

24/25.	If PDCP termination point is changed for bearers using RLC AM, and when RRC full configuration is not used, the SN sends the SN Status Transfer message to MN, which the MN sends then to the SN of the selected candidate PSCell, if needed.

26.	If applicable, data forwarding from the last serving SN takes place. It may be initiated as early as the the last serving SN receives the early data forwarding address in step 4a.

27.	If data forwarding is needed, the MN may send the Xn-U Address Indication message to the selected candidate SN. The SN may decide to perform, if applicable, early data forwarding for SN-terminated bearers, together with the sending of an Early Status Transfer message to the source MN.

NOTE 5:	Separate Xn-U Address Indication procedures may be initiated to provide different forwarding addresses of the prepared subsequent CPAC. In this case, it is up to the MN and selected SN implementations to make sure that the Early Status Transfer message(s) from the selected SN, if any, is forwarded to the right other candidate SN.

SN initiated subsequent CPAC

The subsequent CPAC procedure is initiated by the SN for inter-SN subsequent CPAC configuration and inter-SN CPC execution. The procedure is initiated with SN Change Required procedure, which is then followed with the steps as presented above for the MN-initiated subsequent CPAC.

END OF CHANGES