3GPP TSG-RAN WG2 Meeting #124	R2-2313563

Chicago, USA, Nov 13-17, 2023

Agenda item:	8.1

Source: 	Vice Chairman (ZTE Corporation)

Title:	Report from Break-out session on NR-NTN and IoT-NTN

Document for:	Approval

Organizational

- All organization emails and notes will be shared over the following email discussion throughout the meeting:

[AT124][300] Organizational – NR-NTN and IoT-NTN session

Scope:

    - Share plans for the meeting and list of ongoing email discussions for the sessions related to NR-NTN and IoT-NTN

    - Share meetings notes and agreements for review and endorsement

Schedule/Plan

|  | Main room | Brk 2 room | Brk 1 room | Brk 3 room |
| --- | --- | --- | --- | --- |
| Monday November 13th |  |  |  |  |
| 09:00 – 10:30 | [1], [2], [3], [7.0] R18 common (Diana) NR18 Network Energy Saving [1] (Diana) - 7.3.1 Organizations (including reports from Running CRs email discussions) - 7.3.2 DTX/DRX - 7.3.5 Mobility | Breakout to start after common session MUSIM [1] (Erlin) - 7.17.1 (running CRs, LSin, etc.) - 7.17.2 (report for email [205], other issues) - 7.17.3 - 7.17.4 NR18 MIMO evo [0.75] - 7.20.1 (reports for email [203], running CRs, LSin, etc) MAC aspects - 7.20.2 - 7.20.3 (if time allows) | Breakout to start after formal opening of meeting in main room: NR18 Pos early items (Nathan) - LSs and rapporteur inputs, email reports [404] and [405], kickoff of early offline discussions NR18 SL Relay early items (Nathan) - LSs and rapporteur inputs, email report [420], kickoff of early offline discussions NR17 SL Relay if time (Nathan) |  |
| 11:00 – 13:00 |  |  |  |  |
| 14:30 – 16:30 | EUTRA&NR151617 (Mattias) 4.1.1 5.1.1.1 5.1.3.0 5.1.3.1 5.1.3.2 5.1.3.3 6.1.1.0 6.1.1.1 May continue with the following if time allows 6.1.3.0 6.1.3.1 6.1.3.2 6.1.3.3 | NR MIMO con’t RRC aspects (including report for email [204], other issues in 7.20.1/2/3 if time allows) @15:00 NR151617 UP (Diana) NR18 MT-SDT(Diana) IDC (Yi) (email discussion only) NCR(Sasha) (email discussion only) | NRLTE1516 V2X/SL (Kyeongin) NR17 SL (Kyeongin) NR18 SL (if time allows) 5.2.0, 5.2.1 6.6.0, 6.6.1 7.15.1, 7.15.2 (if time allows) |  |
| 17:00 – 19:00 |  | NR18 fCovEnh [0.5] (Eswar) - 7.21.1 Organizational: - LSin - Endorse the running CRs - open issues and rapporteur proposals from running CR discussions CP and UP AIs - 7.21.2: - 7.21.3: | NR18 SL evolution [1] (Kyeongin) 7.15.2 7.15.3 (if time allows) |  |
| Tuesday November 14th |  |  |  |  |
| 08:30 – 10:30 | NR18 feMob [2] (Johan) [7.4.1/7.4.2] Kick-off LTM CR limited offline (brief). [7.4.3] Subsequent CPAC [7.4.4] CHO with Cand SCG | NR 18 MBS [0.5] (Dawid): - 7.11.1: Organizational - 7.11.2.1: CP issues - 7.11.2.2: UP issues - 7.11.3: Shared processing and UE capabilities If time allows, MBS TEI 18: - RedCap CFR - PTM retransmissions | NR18 SL Relay [1.5] (Nathan) - 7.9.1 Organizational if anything left - 7.9.2 UE-to-UE - 7.9.3 Service continuity if time |  |
| 11:00 – 13:00 | NR18 Mobile IAB [0.5] (Johan) NR18 LP WUS [0.5] (Johan) | NR18 UAV [1] (Diana) (all AIs in order) | NRLTE1516 Pos (Nathan) NR17 Pos (Nathan) NR17 SL Relay if needed (Nathan) |  |
| 14:30 -16:30 | NR18 XR [2] (Diana) - 7.5.1 Organizational (including reports from running CRs email discussions) - 7.5.4.1 BSR - 7.5.4.2 discard operation - 7.5.4.3 configured grant | NR17 NTN Maint (Sergio) - 4.2 - 6.3 NTN Self evaluation (Sergio) - 7.25.4 NR18 NTN enh [1] (Sergio) - 7.7.1 - 7.7.2 - 7.7.3 | NR18 Pos [2] (Nathan) - 7.2.1 Organizational if anything left - 7.2.3 RAT-dependent integrity - 7.2.4 LPHAP - 7.2.5 RAN1-led objectives if time |  |
| 17:00– 19:00 | NR18 Other [2] Diana | NR18 NTN enh [1] (Sergio) - 7.7.4.1 - 7.7.4.2.1: report of [301],[302] - 7.7.4.2.2 | NR18 Pos [2] (Nathan) - 7.2.5 RAN1-led objectives - 7.2.2 SL positioning |  |
| Wednesday November 15th |  |  |  |  |
| 08:30 – 10:30 | NR18 feMob [2] (Johan) [7.4.2] LTM cont [7.4.1] UE caps [7.4.5] eEMR, incl Inc LS, initial disc | NR18 eQoE [0.5] (Dawid): - 7.14.1: LSin, running CRs, work plan, open issues - 7.14.2: QoE in IDLE/INACTIVE - 7.14.3: QoE in NR-DC - 7.14.4: UE capabilities and others | NR18 SL relay [1.5] (Nathan) - 7.9.3 Service continuity - 7.9.4 Multi-path TEI Relay/POS (Nathan) (30minutes) |  |
| 11:00 – 13:00 | NR18 URLLC [0.5] (Diana) NR18 Network Energy Saving [1] (Diana) - 7.3.4 Cell selection/reselection - 7.3.6 Others | NR17 (Mattias) 6.1.3.0 6.1.3.1 6.1.3.2 6.1.3.3 | NR17 SONMDT (HuNan) NR18 SONMDT [1] (HuNan) |  |
| 14:30 – 16:30 | NR18 XR [2] (Diana) - 7.5.3 XR specific power saving - 7.5.2 XR awareness - 7.5.5 UE capabilities | R18 IoT-NTN [1] (Sergio) - 7.6.1 - 7.6.2.1 - 7.6.2.2: report of [304] - 7.6.3.1 - 7.6.3.2 - 7.6.4 | NR18 SONMDT [1] (HuNan) |  |
| 17:00 – 19:00 | NR18 AIML [1] (Diana) All AIs | NR18 RedCap [1] (Mattias) 7.19.1 7.19.2 7.19.3 | Offlines (to be scheduled during the week) |  |
| Thursday November 16th |  |  |  |  |
| 08:30 – 10:30 | CB Eswar Cov. Enhc. | CB Sergio NR18 NTN Enh | CB Kyeongin Comebacks |  |
| 11:00 – 13:00 | NR18 TEI [1] (Diana) | CB Erlin MU-SIM MIMO evo | CB Kyeongin Comebacks |  |
| 14:30 – 16:30 | NR18 Other [2] (Diana) | CB Dawid: - QoE - MBS - MBS TEI18 | CB Nathan Prioritise positioning CBs (TBR) |  |
| 17:00 – 19:00 | CB Diana XR UAV NES AI/ML if time allows | CB Johan - LP-WUS - mIAB - feMob | CB Nathan Prioritise relay CBs (TBR) |  |
| Friday November 17th |  |  |  |  |
| 08:30 – 10:30 | CB Johan FeMob | CB EUTRA&NR151617 Mattias CB eRedCap Mattias | 8:30-9:30 CB Kyeongin 9:30-11:30 CB Nathan |  |
| 11:00 – 13:00 | CB Diana AI/ML (if not done on Thursday) | CB Sergio R18 IoT-NTN | 11:00-11:30 Nathan CB 11:30 – 12:00 CB Hunan |  |
| 14:30 – 16:00 | Reports of breakout sessions |  |  |  |
| 16:00 – 17:00 |  |  |  |  |

List and details of [AT124] offline discussions

NOTE:  No offline email discussions will be kicked off before Monday Nov 13th, 09:00 local time

[AT124][301][NR-NTN Enh] RACH-less HO (Interdigital)

Scope: Discuss the remaining open issues for RACH-less HO, based primarily on R2-2313004 and R2-2312105 (if time allows)

Intended outcome: offline discussion summary

F2F schedule: Monday 2023-11-13 16:30-17:00 Brk3

Deadline for rapporteur's summary (in R2-2313784):  Tuesday 2023-11-14 12:00

[AT124][302][NR-NTN Enh] CHO enhancements (Nokia)

Scope: Discuss the remaining open issues for CHO enhancements, based primarily on section 2.2 of R2-2313051 (and section 2.1 if time allows)

Intended outcome: offline discussion verbal summary (the discussion will continue online)

F2F schedule: Tuesday 2023-11-14 16:30-17:00 Brk2

[AT124][303][NR-NTN Enh] Unchanged PCI (Apple)

Scope: Discuss the remaining open issues for Unchanged PCI (exact scope to be clarified after the online session on Tuesday)

Intended outcome: offline discussion summary

F2F schedule: Wednesday 2023-11-15 10:30-11:00 Brk3

Deadline for rapporteur's summary (in R2-2313785):  Wednesday 2023-11-15 22:00

[AT124][304][IoT-NTN Enh] GNSS enhancements (ZTE)

Scope: Start discussing the remaining open issues for GNSS operation enhancements, based on the list of open issues identified in R2-2311892 and R2-2312116 and the submitted contributions in AI 7.6.2.2

Intended outcome: offline discussion summary

F2F schedule: Tuesday 2023-11-14 10:30-11:00 Brk3

Deadline for rapporteur's summary (in R2-2313786):  Wednesday 2023-11-15 12:00

[AT124][305][NR-NTN Enh] UE Caps CRs (Intel)

Scope: Update the running drafts CRs with meeting agreements

Intended outcome: Endorsed draft CRs

Deadline for companies' feedback:  Thursday 2023-11-16 20:00

Deadline for endorsed CRs (in R2-2313775 and R2-2313776):  Friday 2023-11-17 08:00

Status: to be started after the online discussion on Tuesday

## 4.2	NB-IoT and eMTC support for NTN Rel-17

(LTE_NBIOT_eMTC_NTN; leading WG: RAN1; REL-17; WID: RP-211601)

Tdoc Limitation: 1 tdocs

This Agenda Item is treated in the Breakout session that includes NTN

A single CR per TS with miscelaneous corrections is encouraged.  Small editorial corrections should be sent directly to rapporteur.  Big open issues can be discussed with contributions with CR in the appendix of the contribution

### 4.2.0	In Principle Agreed CRs

R2-2313161	Clarification on ul-SyncValidityDuration in SIB31	ZTE Corporation, Sanechips	CR	Rel-17	36.331	17.6.0	4975	-	F	LTE_NBIOT_eMTC_NTN-Core

Agreed??

### 4.2.1	Other

Koffset handling

R2-2313550	Correction to 36.321 on Koffset handling during handover	Huawei, Ericsson, Samsung, OPPO, Nokia, Qualcomm, HiSilicon	CR	Rel-17	36.321	17.6.0	1573	1	F	LTE_NBIOT_eMTC_NTN	R2-2311597

R2-2313485	Correction on the Koffset handling during RRC connection re-establishment	Google Inc.	CR	Rel-17	36.321	17.6.0	1579	-	F	LTE_NBIOT_eMTC_NTN-Core

R2-2313357	Correction on Koffset when receiving dedicated SIB31	ZTE Corporation, Sanechips	CR	Rel-17	36.321	17.6.0	1578	-	F	LTE_NBIOT_eMTC_NTN-Core

Corrections to SIB31

R2-2313395	Corrections to SystemInformationBlockType31 for IoT NTN	Huawei, HiSilicon	CR	Rel-17	36.331	17.6.0	4978	-	F	LTE_NBIOT_eMTC_NTN

SIB31 in a non NTN cell

R2-2313008	Correction on SIB31 signalling only in NTN cell	Samsung	CR	Rel-17	36.331	17.6.0	4972	-	F	LTE_NBIOT_eMTC_NTN

Withdrawn

R2-2313370	Correction to 36.321 on Koffset handling during MAC reset	Huawei, Ericsson, Samsung, OPPO, Nokia, Qualcomm, HiSilicon	CR	Rel-17	38.321	17.6.0	1722	-	F	LTE_NBIOT_eMTC_NTN	R2-2311597	Withdrawn

R2-2313547	Correction to 36.321 on Koffset handling during MAC reset	Huawei, Ericsson, Samsung, OPPO, Nokia, Qualcomm, HiSilicon	CR	Rel-17	38.321	17.6.0	1722	1	F	LTE_NBIOT_eMTC_NTN	R2-2313370	Withdrawn

## 6.3	NR Non-Terrestrial Networks (NTN)

(NR_NTN_solutions-Core; leading WG: RAN2; REL-17; WID: RP-211557)

Tdoc Limitation: 1 tdocs

A single CR per TS with miscellaneous corrections is encouraged.  Small editorial corrections should be sent directly to rapporteur.  Big open issues can be discussed with contributions with CR in the appendix of the contribution

### 6.3.0	In Principle Agreed CRs

R2-2312626	Notes in the RRC release procedure for NR-NTN	Google Inc., Qualcomm Inc., LG Electronics	CR	Rel-17	38.331	17.6.0	4351	2	F	NR_NTN_solutions-Core	R2-2311313

Agreed??

### 6.3.1	Other

Koffset handling

R2-2313369	Correction to 38.321 on Koffset handling during MAC reset	Huawei, Ericsson, Samsung, OPPO, Nokia, Qualcomm, HiSilicon	CR	Rel-17	38.321	17.6.0	1692	1	F	NR_NTN_solutions-Core	R2-2311598

Misc

R2-2313081	Miscellaneous corrections to 38.331 for NR NTN	Huawei, HiSilicon	CR	Rel-17	38.331	17.6.0	4463	-	F	NR_NTN_solutions-Core

Event D1

R2-2311964	Correction on Event D1	OPPO	CR	Rel-17	38.331	17.6.0	4402	-	F	NR_NTN_solutions-Core

Reference point

R2-2312211	Consideration on UTC reference point and correction on CondEvent T1 in NR NTN R17	ZTE Corporation, Sanechips	discussion	Rel-17	NR_NTN_solutions-Core

Proposal 1: RAN2 understands that, for referenceTimeInfo, in NR NTN R17 it’s anyway not possible to achieve the same level of accuracy as in a TN network.

Proposal 2: In NR NTN, the indicated time in referenceTimeInfo and timeInfoUTC is referenced at gNB, i.e., UE should take into account the propagation delay in service link and feeder link when determining the time at the UE.

Proposal 3: Remove “measured” from the definition of Mt for CondEvent T1:

Mt is the time measured at UE.

Proposal 4: Agree the TP as included in the Appendix.

R2-2313298	UTC reference point in NR NTN R17	Ericsson	discussion	Rel-17	NR_NTN_solutions

Proposal 1	Align the NR NTN UTC reference point to the IoT NTN reference point for UTC.

Proposal 2	Consider the text proposal in the Appendix.

R2-2313486	RP of epoch time for neighbor and target cells / RP of t-Service	Sequans Communications	discussion	Rel-17	NR_NTN_solutions-Core

- Revised in R2-2313554

R2-2313554	RP of epoch time for neighbor and target cells / RP of t-Service	Sequans Communications	discussion	Rel-17	NR_NTN_solutions-Core

Proposal 1: Confirm that the reference point for epoch time of neighbour cell is the serving cell ULTSRP

Proposal 2: Confirm that the reference point for epoch time of target cell is the target cell ULTSRP

Proposal 3: The reference point for t-Service is the ULTSRP of the cell

Other

R2-2313194	Clarification on cellBarredNTN in RRC_CONNECTED	Qualcomm Technologies Ireland	discussion	Rel-17	NR_NTN_solutions-Core

Proposal 1	Clarify the cellBarredNTN bit applies to only UE in RRC_IDLE, RRC_INACTIVE and RRC_CONNECTED while T311 is running.

## 7.6	IoT NTN enhancements

(IoT_NTN_enh-Core; leading WG: RAN1; REL-18; WID: RP-223519)

Time budget: 1 TU

Tdoc Limitation: 4 tdocs

### 7.6.1	Organizational

LSs, rapporteur inputs and other organizational documents. Rapporteur inputs and other pre-assigned documents in this AI do not count towards the tdoc limitation.

Including, for each affected spec:

- Updated running CR

- List of open issues to be addressed by company contributions

- (where applicable) CR rapporteur input with proposals for stage-3 issues (e.g. signaling details, parameter values/ranges) where company contributrions should be avoided

based on the outcome of:

[Post123bis][301][IoT-NTN Enh] 36.300 running CR (Ericsson)

[Post123bis][302][IoT-NTN Enh] 36.331 running CR (Huawei)

[Post123bis][303][IoT-NTN Enh] 36.321 running CR (Mediatek)

[Post123bis][304][IoT-NTN Enh] 36.304 running CR (Nokia)

[Post123bis][305][IoT-NTN Enh] 36.306 running CR (Qualcomm)

Incoming LSs

R2-2311716	LS on Rel-18 RAN1 UE features list for LTE after RAN1#114bis (R1-2310634; contact: NTT DOCOMO, AT&T)	RAN1	LS in	Rel-18	IoT_NTN_enh	To:RAN2	Cc:RAN4

(RAN1 UE feature list in R1-2310632)

Stage 2 CR

R2-2313301	Introduction of IoT NTN enhancements	Ericsson	CR	Rel-18	36.300	17.5.0	1387	1	B	IoT_NTN_enh-Core	R2-2311244

Endorsed??

Revised in R2-2313779

R2-2313779	Introduction of IoT NTN enhancements	Ericsson	CR	Rel-18	36.300	17.5.0	1387	2	B	IoT_NTN_enh-Core

[Post124][307][IoT-NTN Enh] 36.300 CR (Ericsson)

Scope: update the running CR with meeting agreements

Intended outcome: Agreed CR

Deadline for agreed CR (in R2-2313779): short

R2-2313304	Stage 2 open issues	Ericsson	discussion	Rel-18	IoT_NTN_enh-Core

The following issues need to be addressed:

•	Adding a new trigger for random access procedure by GNSS validity duration MAC CE in Section 10.1.5.0

•	Upon failed GNSS acquisition, shall the UE be allowed to stay in CONNECTED if it still has a valid GNSS position? This can be captured in 23.21.2.2.

•	To discuss whether “Provide carrier frequency for the existing satellite list in SIB32 to facilitate cell selection and reduce service interruption after an NTN coverage gap (FFS if the information can be considered as valid after the validity of SI)” needs to be captured in stage 2.

36.331 CR

R2-2311891	Introduction of IoT NTN enhancements	Huawei, HiSilicon	CR	Rel-18	36.331	17.6.0	4964	-	B	IoT_NTN_enh-Core

Endorsed??

Revised in R2-2313780

R2-2313780	Introduction of IoT NTN enhancements	Huawei, HiSilicon	CR	Rel-18	36.331	17.6.0	4964	1	B	IoT_NTN_enh-Core

[Post124][308][IoT-NTN Enh] 36.331 CR (Huawei)

Scope: update the running CR with meeting agreements

Intended outcome: Agreed CR

Deadline for agreed CR (in R2-2313780): short

R2-2311892	Report of [Post123bis][302][IoT-NTN Enh] 36.331 running CR (Huawei)	Huawei, HiSilicon	discussion	Rel-18	IoT_NTN_enh-Core

Stage-3 issue proposals:

Proposal 1: Remove the references to 5.5.x in Section 5.3.3.4, 5.3.3.4a, 5.3.5.3, 5.3.5.4 and 5.3.7.5, considering that in the updated running CR 5.5.x is already referenced in clause 5.3.3.21.

Proposal 2: maxSat-r18 is 4.

Proposal 3: Value range of SatelliteId-r18 is “INTEGER (0..255)”.

Open issue list:

-	GNSS

Issue 1-1: How to determine GNSS invalid (considering duration X and Y), this affects 1) condition for entering RRC_IDLE, and 2) the start of autonomous gap

-	Option 1: It is up to RAN1 whether/how to decide GNSS validity duration considering X and Y.

-	Option 2: Even if duration X is provided, the remaining GNSS validity duration keeps unchanged.

-	Option 3: UE considers the GNSS position as outdated and goes to RRC_IDLE, upon the expiry of X on top of the expiry of the GNSS validity duration.

To be discussed in 7.6.2.2

Issue 1-2: Whether to suspend T317, T318 during measurement gap

To be discussed in 7.6.2.2

-	Mobility

Issue 2-1: Regarding RLF based measurement enhancements for eMTC UEs in RRC_CONNECTED, which frequencies to measure (frequencies in MeasObjects, or frequencies in SIB, or both), whether measurement report will be triggered

To be discussed in 7.6.3

Issue 2-2: Whether time/location based CHO can be configured simultaneously for the same target cell

To be discussed in 7.6.3

Issue 2-3: Whether to allow joint configuration among time/location/RSRP-based measurements in RRC Idle/Connected, and if allowed, the intended UE behaviour

To be discussed in 7.6.3

Issue 2-4: Regarding reacquisition of SIBxx during T318, clarify the intended behavior:

-	Option 1: UE acquires SIBxx during T318 if the stored SIBxx has expired (as in the current CR)

-	Option 2: Keep the possibility that UE does not acquire SIBxx even if the stored SIBxx expires

-	Option 3: Allow the UE to acquire SIBxx even if the stored SIBxx has not expired (e.g. close to expiry)

To be discussed in 7.6.3

Issue 2-5: How to solve the case where T318 is stopped before successful acquisition of SIBxx

-	Option 1: UE stops T318 when both SIB31 and SIBxx have been acquired

-	FFS whether to clarify in the spec that RLF is not triggered if T318 expires and SIB31 has been obtained

-	Other solutions

To be discussed in 7.6.3

Issue 2-6: Whether satellite IDs in SIB31/SIB32/SIBxx are unique

To be discussed in 7.6.3

-	Discontinuous coverage

Issue 3-1: Whether to capture a note in RRC about “UE may directly go to RRC_IDLE after RLF is triggered, if there is not enough time for the UE to finish the procedure of RRC re-establishment due to the discontinuous coverage”

To be discussed in 7.6.4

Issue 3-2: Whether and how to apply the “early stop of T310 and early start of T311 due to t-Service expiry” to discontinuous coverage scenario

To be discussed in 7.6.4

36.321 CR

R2-2312116	Stage-3 running CR for TS 36.321 for Rel-18 IoT-NTN	MediaTek Inc.	draftCR	Rel-18	36.321	17.6.0	F	IoT_NTN_enh-Core

Endorsed??

Draft a formal CR in R3-2313781

List of Open Issues:

- Configuring GNSS timers using X and Y introduced by RAN1

To be discussed in 7.6.2.2

- MAC action related to UL transmission after GNSS validity duration expires with duration X, Y

To be discussed in 7.6.2.2

- The use of UL LCID for GNSS Validity Duration

To be discussed in 7.6.2.2

R2-2313781	Stage-3 running CR for TS 36.321 for Rel-18 IoT-NTN	MediaTek Inc.	draftCR	Rel-18	36.321	17.6.0	F	IoT_NTN_enh-Core

[Post124][309][NR-NTN Enh] 36.321 CR (Mediatek)

Scope: update the running CR with meeting agreements

Intended outcome: Agreed CR

Deadline for agreed CR (in R2-2313781): short

36.304 CR

R2-2313320	Introduction of IoT-NTN Enhancements	Nokia Solutions & Networks (I)	CR	Rel-18	36.304	17.4.0	0869	-	B	IoT_NTN_enh-Core

Endorsed??

Revised in R2-2313782

R2-2313782	Introduction of IoT-NTN Enhancements	Nokia Solutions & Networks (I)	CR	Rel-18	36.304	17.4.0	0869	1	B	IoT_NTN_enh-Core

[Post124][310][IoT-NTN Enh] 36.304 CR (Nokia)

Scope: update the running CR with meeting agreements

Intended outcome: Agreed CR

Deadline for agreed CR (in R2-2313782): short

R2-2313321	Report of  [Post123bis][304][IoT-NTN Enh] 36.304 running CR (Nokia)	Nokia Solutions & Networks (I)	discussion	Rel-18

Proposal 1 (4/5) : The following EN can be removed in TS36.304.

Editor Note: FFS whether RSS-based measurement condition check is applicable for IoT-NTN.

Proposal 2 (4/5) : No update is needed in 36.304 related to cell reselection aspects in TS36.304 due to the introduction of SIBXX.  RAN2 to discuss the need to capture the following UE behavior in TS36.304.

“For a UE in Idle/Inactive mode it's up to UE implementation whether to perform NTN neighbor cell measurements on a cell indicated in SIB4 but not included in SIBXX.”

Proposal 3: RAN2 to wait for SA2 LS response to conclude on paging-related impacts in RAN2 specification

Proposal 4: RAN2 to discuss how to capture frequency information in SIB32 for cell selection in DC scenario. Whether to capture the changes as part of the SIB32 reception or in TS36.304 to be decided.

Proposal 5: Need to specify any NAS-AS layer interactions related to unavailability period to be discussed based on contributions in RAN2-124.

36.306 CR

R2-2312281	Introduction of Rel-18 IoT NTN UE capabilities	Qualcomm Incorporated	CR	Rel-18	36.306	17.4.0	1872	-	B	IoT_NTN_enh-Core

Endorsed??

Revised in R2-2313783

R2-2313783	Introduction of Rel-18 IoT NTN UE capabilities	Qualcomm Incorporated	CR	Rel-18	36.306	17.4.0	1872	1	B	IoT_NTN_enh-Core

[Post124][311][IoT-NTN Enh] 36.306 CR (Qualcomm)

Scope: update the running CR with meeting agreements

Intended outcome: Agreed CR

Deadline for agreed CR (in R2-2313783): short

R2-2312282	Rapporteur input to open issues on the introduction of Rel-18 IoT NTN UE capabilities	Qualcomm Incorporated	draftCR	Rel-18	36.306	17.4.0	B	IoT_NTN_enh-Core

### 7.6.2	Performance Enhancements

#### 7.6.2.1	HARQ enhancements

R2-2311958	Discussion on HARQ enhancement for IoT NTN	OPPO	discussion	Rel-18	IoT_NTN_enh-Core

Proposal 1	(Missing from summary of offline#308 @RAN2#123bis with unanimous support) For multiple TB scheduling with mixed HARQ feedback enabled/disabled configuration, if HARQ-ACK bundling is not configured, HARQ RTT Timer for HARQ process with HARQ feedback enabled is calculated based on the number of scheduled TBs with HARQ feedback enabled.

Proposal 2	For DL multiple TB scheduling for a NB-IoT UE, if both HARQ processes are configured with disabled HARQ feedback, UE starts drx-InactivityTimer in the subframe containing the last repetition of the PDSCH corresponding to the last scheduled TB plus 12 subframes plus deltaPDCCH.

Proposal 3	For DL multiple TB scheduling for a NB-IoT UE, if only one of the HARQ processes is configured with disabled HARQ feedback, UE starts drx-InactivityTimer in the subframe containing the last repetition of the PDSCH corresponding to the last scheduled TB plus 12 subframes plus deltaPDCCH.

Proposal 4	For UL multiple TB scheduling for a NB-IoT UE, if both HARQ processes are configured with HARQ mode B, UE starts drx-InactivityTimer in the subframe containing the last repetition of the PUSCH corresponding to the last scheduled TB plus 1 subframe plus deltaPDCCH.

Proposal 5	For UL multiple TB scheduling for a NB-IoT UE, if only one of the HARQ processes is configured with HARQ mode B, UE starts drx-InactivityTimer in the subframe containing the last repetition of the PUSCH corresponding to the last scheduled TB plus 1 subframe plus deltaPDCCH.

Proposal 6	For multiple TB scheduling, for a HARQ process configured as disabled HARQ feedback by RRC and further reversed to enabled HARQ feedback by DCI, RAN2 waits for RAN1 progress before discussing UE DRX behaviour in this case.

Proposal 7	RAN2 confirms that for both single TB scheduling and multiple TB scheduling. for a HARQ process configured as HARQ feedback enabled by RRC and further reversed to HARQ feedback disabled by DCI, UE behaviour on DRX follows the case when HARQ feedback is disabled.

R2-2311838	Remaining Issues on HARQ Enhancement for IoT NTN	vivo	discussion	Rel-18	IoT_NTN_enh-Core

R2-2312114	Remaining Issues on HARQ Enhancements in IoT-NTN	MediaTek Inc.	discussion

R2-2312244	Remaining issues of HARQ enhancements	ZTE Corporation, Sanechips	discussion	IoT_NTN_enh-Core

R2-2312283	Open issues on HARQ enhancements	Qualcomm Incorporated	discussion	Rel-18	IoT_NTN_enh-Core

R2-2312700	Remaining issues on HARQ enhancements for IoT NTN	Nokia, Nokia Shanghai Bell	discussion	Rel-18	IoT_NTN_enh-Core

R2-2312714	Remaining issues on HARQ enhancement	Huawei, Turkcell, HiSilicon	discussion	Rel-18	IoT_NTN_enh-Core

R2-2312722	Discussion on HARQ enhancement open issues	Xiaomi	discussion	Rel-18

R2-2313300	R18 IoT NTN HARQ enhancements	Ericsson	discussion	Rel-18	IoT_NTN_enh-Core

R2-2313317	Discussion on HARQ enhancements in IoT NTN	CATT	discussion

#### 7.6.2.2	GNSS operation enhancements

[AT124][304][IoT-NTN Enh] GNSS enhancements (ZTE)

Scope: Start discussing the remaining open issues for GNSS operation enhancements, based on the list of open issues identified in R2-2311892 and R2-2312116 and the submitted contributions in AI 7.6.2.2

Intended outcome: offline discussion summary

F2F schedule: Tuesday 2023-11-14 10:30-11:00 Brk3

Deadline for rapporteur's summary (in R2-2313786):  Wednesday 2023-11-15 12:00

R2-2313786	Summary of [AT124][304][IoT-NTN Enh] GNSS Enhancements	ZTE	discussion

R2-2311839	Remaining Issues on GNSS Operation for IoT NTN	vivo	discussion	Rel-18	IoT_NTN_enh-Core

R2-2311962	Discussion on GNSS operation for IoT NTN	OPPO	discussion	Rel-18	IoT_NTN_enh-Core

R2-2311963	DRAFT LS on GNSS validity duration	OPPO	LS out	Rel-18	IoT_NTN_enh-Core	To:RAN1

R2-2312046	Leftover issues on the GNSS operation enhancements	Google Inc.	discussion

R2-2312054	Discussion on GNSS operation enhancements	CATT	discussion

R2-2312115	Remaining GNSS Enhancement Issues in IoT-NTN	MediaTek Inc.	discussion

R2-2312246	Remaining issues of GNSS enhancements	ZTE Corporation, Sanechips	discussion	IoT_NTN_enh-Core

R2-2312286	Open issues on GNSS fix in RRC_CONNECTED	Qualcomm Incorporated	discussion	Rel-18	IoT_NTN_enh-Core

R2-2312353	Leftover issues in improved GNSS operation	Apple	discussion	Rel-18	IoT_NTN_enh

R2-2312458	Views on timer handling during GNSS measurement gap	Lenovo	discussion	Rel-18

R2-2312608	GNSS operation enhancement	NEC	discussion	Rel-18	IoT_NTN_enh-Core

R2-2312673	Discussion on GNSS enhancement for IoT-NTN	CMCC	discussion	Rel-18	IoT_NTN_enh-Core

R2-2312701	Remaining issues on GNSS operation enhancement for IoT NTN	Nokia, Nokia Shanghai Bell	discussion	Rel-18	IoT_NTN_enh-Core

R2-2312715	Remaining issues on GNSS measurement	Huawei, Turkcell, HiSilicon	discussion	Rel-18	IoT_NTN_enh-Core

R2-2312721	Discussion on GNSS operation enhancement open issues	Xiaomi	discussion	Rel-18

R2-2312879	GNSS acquisition and reporting for IoT NTN	Interdigital, Inc.	discussion	Rel-18	IoT_NTN_enh-Core

R2-2313010	GNSS measurement procedures in connected mode	Samsung R&D Institute UK	discussion	Rel-18	IoT_NTN_enh

R2-2313299	R18 IoT NTN GNSS operation enhancements	Ericsson	discussion	Rel-18	IoT_NTN_enh-Core

### 7.6.3	Mobility Enhancements

#### 7.6.3.1	Enhancements for neighbour cell measurements

R2-2313078	Remaining issues on mobility enhancements	Huawei, HiSilicon, Turkcell	discussion	Rel-18	IoT_NTN_enh-Core

- Revised in R2-2313586

R2-2313586	Remaining issues on mobility enhancements	Huawei, HiSilicon, Turkcell	discussion	Rel-18	IoT_NTN_enh-Core

R2-2313011	Enhancements for neighbour cell measurements	Samsung R&D Institute UK	discussion	Rel-18	IoT_NTN_enh

R2-2312860	Further analysis on open issues for IoT-NTN Mobility Enhancements	Nokia, Nokia Shanghai Bell	discussion

R2-2312764	Discussion on the remaining issues for the mobility enhancements	Xiaomi	discussion

R2-2312247	Remaining issues of mobility enhancements	ZTE Corporation, Sanechips	discussion	IoT_NTN_enh-Core

R2-2311959	Discussion on mobility enhancement for IoT NTN	OPPO	discussion	Rel-18	IoT_NTN_enh-Core

R2-2312055	Discussion on leftover issues of mobility enhancements	CATT	discussion

R2-2312285	Open issues on measurement and Mobility enhancements	Qualcomm Incorporated	discussion	Rel-18	IoT_NTN_enh-Core

R2-2312355	Neighbour cell measurements before RLF for eMTC-NTN	Apple	discussion	Rel-18	IoT_NTN_enh

R2-2312880	Fast RLF and re-establishment in the discontinuous coverage scenario	Interdigital, Inc.	discussion	Rel-18	IoT_NTN_enh-Core

R2-2313078	Remaining issues on mobility enhancements	Huawei, HiSilicon, Turkcell	discussion	Rel-18	IoT_NTN_enh-Core

- Revised in R2-2313586

R2-2313586	Remaining issues on mobility enhancements	Huawei, HiSilicon, Turkcell	discussion	Rel-18	IoT_NTN_enh-Core

R2-2313228	Neighbour cell measurements in IoT NTN	Ericsson	discussion	Rel-18	IoT_NTN_enh-Core

R2-2313229	Discussion on triggering RA for RRC connection re-establishment in IoT NTN	Ericsson	discussion	Rel-18	IoT_NTN_enh-Core

#### 7.6.3.2	Other

R2-2311840	Discussion on CHO Enhancement for IoT NTN	vivo	discussion	Rel-18	IoT_NTN_enh-Core

R2-2312354	Leftover issues for mobility enhancement in IoT NTN	Apple	discussion	Rel-18	IoT_NTN_enh

R2-2312459	Views on providing NB-IoT UE location information	Lenovo	discussion	Rel-18

R2-2312878	CHO enhancement for earth-moving cells	Interdigital, Inc.	discussion	Rel-18	IoT_NTN_enh-Core

R2-2313012	On other mobility enhancements for IoT NTN	Samsung R&D Institute UK	discussion	Rel-18	IoT_NTN_enh

### 7.6.4	Enhancements to discontinuous coverage

R2-2312056	Discussion on open issues for discontinuous coverage	CATT	discussion

R2-2312284	UE Autonomous release in discontinuous coverage	Qualcomm Incorporated	discussion	Rel-18	IoT_NTN_enh-Core

R2-2311841	Discussion on Discontinuous Coverage	vivo	discussion	Rel-18	IoT_NTN_enh-Core

R2-2312048	Leftover issues on the discontinuous coverage	Google Inc.	discussion

R2-2312199	Considerations on Supporting Discontinuous Coverage	NEC	discussion	Rel-18	IoT_NTN_enh-Core

R2-2312248	Paging window alignment in discontinuous coverage	ZTE Corporation, Sanechips	discussion	IoT_NTN_enh-Core

R2-2312460	Views on some remaining issues for discontinuous coverage	Lenovo	discussion	Rel-18

R2-2312631	Discussion on enhancement to discontinuous coverage for IoT NTN	Transsion Holdings	discussion	Rel-18

R2-2312716	Remaining issues on discontinuous coverage	Huawei, Turkcell, HiSilicon	discussion	Rel-18	IoT_NTN_enh-Core

R2-2312723	Discussion on Discontinuous coverage open issues	Xiaomi	discussion	Rel-18

R2-2312861	Discussion on remaining issues discontinuous coverage Enhancements	Nokia, Nokia Shanghai Bell	discussion

R2-2312881	RRC Release in discontinuous coverage	Interdigital, Inc.	discussion	Rel-18	IoT_NTN_enh-Core

R2-2313296	Enhancements to Discontinuous Coverage	SHARP Corporation	discussion

R2-2313397	Enhancements to discontinuous coverage	Samsung	discussion	Rel-18	IoT_NTN_enh-Core

## 7.7	NR NTN enhancements

(NR_NTN_enh -Core; leading WG: RAN1; REL-18; WID: RP-232669)

Time budget: 1 TU

Tdoc Limitation: 4 tdocs

### 7.7.1	Organizational

LSs, rapporteur inputs and other organizational documents. Rapporteur inputs and other pre-assigned documents in this AI do not count towards the tdoc limitation.

Including, for each affected spec:

- Updated running CR

- List of open issues to be addressed by company contributions

- (where applicable) CR rapporteur input with proposals for stage-3 issues (e.g. signaling details, parameter values/ranges) where company contributrions should be avoided

based on the outcome of:

[Post123bis][306][NR-NTN Enh] 38.300 running CR (Thales)

[Post123bis][307][NR-NTN Enh] 38.331 running CR (Ericsson)

[Post123bis][308][NR-NTN Enh] 38.321 running CR (Interdigital)

[Post123bis][309][NR-NTN Enh] 38.304 running CR (ZTE)

[Post123bis][310][NR-NTN Enh] EU caps running CRs (Intel)

[Post123bis][311][NR-NTN Enh] 37.355 running CR (CATT)

Stage 2 CR

R2-2312858	Introduction of NTN enhancements	THALES	CR	Rel-18	38.300	17.6.0	0734	-	B	NR_NTN_enh-Core

Endorsed??

Revised in R2-2313771

R2-2313771	Introduction of NTN enhancements	THALES	CR	Rel-18	38.300	17.6.0	0734	1	B	NR_NTN_enh-Core

[Post124][301][NR-NTN Enh] 38.300 CR (Thales)

Scope: update the running CR with meeting agreements

Intended outcome: Agreed CR

Deadline for agreed CR (in R2-2313771): short

R2-2312857	Remaining Issues on NR Non-Terrestrial Networks (NTN)	THALES	discussion	Rel-18	NR_NTN_enh-Core

Noted

38.331 CR

R2-2313531	Introduction of Rel-18 NR NTN enhancements	Ericsson	CR	Rel-18	38.331	17.6.0	4501	-	B	NR_NTN_enh-Core

Endorsed??

Revised in R2-2313772

R2-2313772	Introduction of Rel-18 NR NTN enhancements	Ericsson	CR	Rel-18	38.331	17.6.0	4501	1	B	NR_NTN_enh-Core

[Post124][302][NR-NTN Enh] 38.331 CR (Ericsson)

Scope: update the running CR with meeting agreements

Intended outcome: Agreed CR

Deadline for agreed CR (in R2-2313772): short

R2-2313533	TS 38.331 Open Issue List for NR NTN Rel-18	Ericsson	discussion	Rel-18	NR_NTN_enh-Core

Issue 1. Configured grant in RACH-less HO takes CG-SDT as a baseline. There are several RAN1 related parameters in the configured grant settings which may not be applicable or have a different configuration for NTN RACH-less HO. The following have been identified:

-	ntn-NrofDMRS-Sequences

-	ntn-DMRS-Ports

-	antennaPort

-	cg-RetransmissionTimer

-	harq-ProcID-Offset

-	pathlossReferenceIndex

-	phy-PriorityIndex

-	precodingAndNumberOfLayers

-	srs-ResourceIndicator

-	uci-OnPUSCH

As a solution, the rapporteur suggests sending an LS to RAN1 to check the applicability of these parameters with the exception of cg-RetransmissionTimer and harq-ProcID-Offset. These are within RAN2’s scope and should not apply to RACH-less handover.

…

Issue 2. Regarding the association of the Configured Grant with an SSB (ntn-SSB-Subset-r18), can it be optional? What is the UE’s behaviour if this information is not provided?

-	Option 1. The association of Configured Grant and SSB is mandatory for NTN RACH-less HO.

-	Option 2. The association is optional. If the field is absent, the UE assumes the SSB set includes all actually transmitted SSBs.

To be discussed in 7.7.4.2.1

Issue 3. Signalling details to indicate in the handover command a single beam associated with the dynamic grant for initial UL transmission.

-	Option 1: TCI state ID. Similar mechanism to LTM.

-	Option 2: SSB position in burst. Similar mechanism to dynamic grant.

To be discussed in 7.7.4.2.1

Issue 4. MAC level is configured with the Configured Grant by RRC. This configuration has a “Need N” code which means one-shot configuration that is not maintained. Given that the UE shall not continue using the grant once the handover is completed, a few companies have raised attention of whether this configuration should be released and which layer should be responsible.

-	Option 1. Release the configuration explicitly in RRC. Similar approach to LTE.

-	Option 2: Release in MAC, i.e., the configured grant is no longer valid after HO completion. Similar approach to LTM.

To be discussed in 7.7.4.2.1

38.321 CR

R2-2313014	Introduction of RACH-less handover to TS 38.321	InterDigital, Samsung	CR	Rel-18	38.321	17.6.0	1716	-	B	NR_NTN_enh-Core, NR_mobile_IAB-Core	R2-2309345	Late

Endorsed??

Revised in R2-2313773

R2-2313773	Introduction of RACH-less handover to TS 38.321	InterDigital, Samsung	CR	Rel-18	38.321	17.6.0	1716	1	B	NR_NTN_enh-Core, NR_mobile_IAB-Core

[Post124][303][NR-NTN Enh] 38.321 CR (Interdigital)

Scope: update the running CR with meeting agreements

Intended outcome: Agreed CR

Deadline for agreed CR (in R2-2313773): short

R2-2313002	MAC open issues in NTN	InterDigital	discussion	Rel-18	NR_NTN_enh-Core

Unchanged PCI switch scenario

Open issue 1: timeAlignmentTimer handling during RACH-less unchanged PCI switch

To be discussed in 7.7.4.2.2

Open issue 2: Impacts of unchanged PCI switch on MAC

To be discussed in 7.7.4.2.2

RACH-less HO procedure

Open issue 3: Beam indication for dynamic grant

To be discussed in 7.7.4.2.1

Open issue 4: Whether UE can trigger RACH when SR is triggered and rach-lessHO is configured

To be discussed in 7.7.4.2.1

Open issue 5: Release of CG after completion of RACH-less HO

To be discussed in 7.7.4.2.1

38.304 CR

R2-2312210	Introduction of NR NTN enhancements in 38.304	ZTE Corporation, Sanechips	CR	Rel-18	38.304	17.6.0	0357	-	B	NR_NTN_enh-Core

Endorsed??

Revised in R2-2313774

R2-2313774	Introduction of NR NTN enhancements in 38.304	ZTE Corporation, Sanechips	CR	Rel-18	38.304	17.6.0	0357	1	B	NR_NTN_enh-Core

[Post124][304][NR-NTN Enh] 38.304 CR (ZTE)

Scope: update the running CR with meeting agreements

Intended outcome: Agreed CR

Deadline for agreed CR (in R2-2313774): short

UE capabilities

R2-2312163	UE capabilities for Rel-18 NR NTN Enh. WI	Intel Corporation	draftCR	Rel-18	38.306	17.6.0	NR_NTN_enh-Core

Endorsed??/Noted??

Revised in R3-2313775

R2-2313775	UE capabilities for Rel-18 NR NTN Enh. WI	Intel Corporation	draftCR	Rel-18	38.306	17.6.0	NR_NTN_enh-Core

R2-2312164	UE capabilities for Rel-18 NR NTN Enh. WI	Intel Corporation	draftCR	Rel-18	38.331	17.6.0	NR_NTN_enh-Core

Endorsed??/Noted??

Revised in R3-2313776

R2-2313776	UE capabilities for Rel-18 NR NTN Enh. WI	Intel Corporation	draftCR	Rel-18	38.331	17.6.0	NR_NTN_enh-Core

[AT124][305][NR-NTN Enh] UE Caps CRs (Intel)

Scope: Update the running drafts CRs with meeting agreements

Intended outcome: Endorsed draft CRs

Deadline for companies' feedback:  Thursday 2023-11-16 20:00

Deadline for endorsed CRs (in R2-2313775 and R2-2313776):  Friday 2023-11-17 08:00

Status: to be started after the online discussion on Tuesday

R2-2312162	Open topics on UE capabilities for Rel-18 NR NTN Enh. WI including summary report of email discussion [Post123bis][310]	Intel Corporation	discussion	Rel-18	NR_NTN_enh-Core

Proposal 1.	To define a new UE capability, e.g., locationBasedCondHandoverNTN-r18, to indicate whether the UE supports location-based conditional handover for moving cell in NTN bands (which involves the calculation of the present reference location from ephemeris and one reference location at epoch time, as specified in TS 38.331).

Proposal 1.1.	This locationBasedCondHandoverNTN-r18 capability is defined per Band, optional with signaling capability, and N/A for FDD/TDD DIFF and FR1/FR2 Diff. This is defined as part of §4.2.7.2 BandNR parameters in TS 38.306.

Proposal 1.2.	An editor’s note is added to locationBasedCondHandoverNTN-r18 to capture “FFS whether any change or update is needed considering how locationBasedCondHandover-r17 is defined”, or whether location-based conditional handover for moving cell refers to source cell, target cell or both.

Proposal 2.	To define a two new UE capability, e.g., unchangedPCI-NTN-SoftSwitch-r18 and unchangedPCI-NTN-HardSwitch-r18, to indicate whether UE support unchanged PCI with soft or hard switch, as specified in TS 38.331.

Proposal 2.1.	unchangedPCI-NTN-SoftSwitch-r18 and unchangedPCI-NTN-HardSwitch-r18 capabilities are defined per UE, optional with signaling capability, and No for FDD/TDD DIFF and FR1/FR2 Diff. This is defined as part of §4.2.2 General parameters in TS 38.306.

Proposal 2.2.	An editor’s note is added to unchangedPCI-NTN-SoftSwitch-r18 and unchangedPCI-NTN-HardSwitch-r18 to capture “FFS whether further changes may be needed after further progressing on the design to support unchanged PCI with soft and hard switch”.

Proposal 3.	If Proposal 1 and Proposal 2 are agreeable, to endorse the UE Capability draftCRs to TS 38.306 and 38.331 provided in R2-2312163 and in R2-2312164 for Rel-18 NR NTN Enh. WI.

37.355 CR

R2-2313225	Introduction of network verification of UE location in TS 37.355	CATT	CR	Rel-18	37.355	17.6.0	0482	-	B	NR_NTN_enh-Core

Endorsed??

Revised in R2-2313777

R2-2313777	Introduction of network verification of UE location in TS 37.355	CATT	CR	Rel-18	37.355	17.6.0	0482	1	B	NR_NTN_enh-Core

[Post124][305][NR-NTN Enh] 37.355 CR (CATT)

Scope: update the running CR with meeting agreements

Intended outcome: Agreed CR

Deadline for agreed CR (in R2-2313777): short

R2-2313226	LPP stage-3 issue and open issue status for Rel-18 NR NTN	CATT (Rapporteur)	Work Plan	Rel-18	NR_NTN_enh-Core

-	Stage-3 issues (not requiring functionality discussion)

During the email discussion in [1], the following two LPP stage-3 issues were identified (which are also listed in the LPP running CR):

[Stg.3 Issue 1] FFS on the value range of RRC parameter nr-NTN-DL-TimingDrift-r18 (introduced by RAN1 RRC parameter list).

[Stg.3 Issue 2] FFS whether HAPS operating bands need to be added into the field description of UE capability nr-NTN-MeasAndReport (introduced by RAN1 UE feature list).

Rapp’s proposal: RAN2 waits for RAN1 final conclusion to resolve the remaining LPP Stage-3 issues listed above.

38.305 CR

R2-2312276	Multi-RTT positioning in NTN	Qualcomm Incorporated	draftCR	Rel-18	38.305	17.6.0	B	NR_NTN_enh-Core

Endorsed??/Noted??

Draft a formal CR in R3-2313778

R2-2313778	Multi-RTT positioning in NTN	Qualcomm Incorporated	CR	Rel-18	38.305	17.6.0		XXXX	-	B	NR_NTN_enh-Core

[Post124][306][NR-NTN Enh] 38.305 CR (Qualcomm)

Scope: update the running CR with meeting agreements

Intended outcome: Agreed CR

Deadline for agreed CR (in R2-2313778): short

### 7.7.2	Coverage Enhancements

R2-2312702	Msg3 indication for PUCCH repetition for Msg4 HARQ-ACK	Nokia, Nokia Shanghai Bell	discussion	Rel-18	NR_NTN_enh-Core

Proposal 1: Use the LCID codepoint within the Rel-18 extension space to indicate the request/capability of PUCCH repetition for Msg4 HARQ-ACK.

Proposal 2: Feature combination of NTN, RedCap and eRedCap should be supported for Msg3-based early indication via LCID.

Proposal 3: Discuss the LCID allocation for feature combinations in common session.

Proposal 4: From NTN WI point of view, there is no need to use explicit NW indication to indicate LCID extension. If NW implicitly indicate the support of PUCCH repetition for Msg4 HARQ-ACK in SIB, UE can deduce the NW support LCID extension.

R2-2312908	Further consideration on PUCCH repetition for Msg4 HARQ-ACK	Huawei, HiSilicon	discussion	Rel-18	NR_NTN_enh-Core

Observation 1: For random access during RRC connected state, there is no case requiring common PUCCH resources thus the current PUCCH repetition mechanism for Msg4 HARQ-ACK is not needed.

Proposal 1: PUCCH repetition for Msg4 HARQ-ACK does not apply to random access procedures during RRC connected state.

R2-2311960	Discussion on PUCCH enhancement for Msg4 HARQ-ACK in NR NTN	OPPO	discussion	Rel-18	NR_NTN_enh-Core

R2-2312052	Discussion on remaining issue for NR NTN coverage enhancement	CATT	discussion

R2-2312280	UE capability indication for Msg4 ACK repetition	Qualcomm Incorporated	discussion	Rel-18	NR_NTN_enh-Core

R2-2312649	Considerations on the coverage enhancements	CMCC	discussion	Rel-18	NR_NTN_enh-Core

R2-2312789	Consideration on remaining coverage enhancements issues	ZTE Corporation, Sanechips	discussion

R2-2313003	Coverage enhancement in Non-Terrestrial Networks	InterDigital	discussion	Rel-18	NR_NTN_enh-Core

R2-2313294	Indication for Msg3 based request for PUCCH repetition	LG Electronics Inc.	discussion	NR_NTN_enh-Core

### 7.7.3	Network verified UE location

R2-2312517	Discussion on network verified UE location	Ericsson	discussion	Rel-18	NR_NTN_enh-Core

Observation 1	it is beneficial to capture in specs that the same satellite at different time instances is modelled as a separate TRP, which can show the difference between NTN positioning with a single satellite in this release and the existing RAT dependent positioning framework.

Proposal 1	Capture in Stage 2 spec that both the UE and the serving gNB/satellite need to provide multiple RX-TX time difference measurements at multiple time instances to the LMF for positioning, where the satellite at each time instance is modelled as a separate TRP.

Proposal 2	Adopt the TP for 38.300 running CR captured in Appendix.

Observation 2	The UE can already provide RX-TX measurement results at different time instances in a one-shot report.

Based on the discussion in the previous sections we propose the following:

Proposal 3	There is no spec change in LPP foreseen for UE to report RX-TX time difference measurements at different time instances in separate or the same report message(s).

R2-2312713	Remaining issues on UE location verification	Huawei, Turkcell, HiSilicon	discussion	Rel-18	NR_NTN_enh-Core

Observation 1: For legacy handover, the POSITIONING INFORMATION UPDATE message can be relied on to indicated LMF that handover happens.

Proposal 1: Legacy procedure can be reused to indicate the LMF about the happening of CHO.

R2-2312121	Remaining Issues in Network verified UE Location	MediaTek Inc.	discussion

R2-2312461	Views on cell change during UE location verification	Lenovo	discussion	Rel-18

R2-2312650	Discussion on network verified UE location	CMCC	discussion	Rel-18	NR_NTN_enh-Core

R2-2312948	UE location verification by Network	NEC Telecom MODUS Ltd.	discussion	R2-2310985

R2-2313007	Network Verified UE Location in NTN	Samsung Electronics Iberia SA	discussion	Rel-18	NR_NTN_enh-Core

R2-2313050	Remaining Aspects on Network Verified UE Location	Nokia, Nokia Shanghai Bell	discussion	Rel-18	NR_NTN_enh-Core

Withdrawn?

R2-2313346	Discussion on network verified UE location in NR NTN	IPLOOK	discussion	Rel-18

### 7.7.4	NTN-TN and NTN-NTN mobility and service continuity enhancements

NTN neighbour cell information in TN cells

R2-2313530	NTN neighbour cell information in TN cells	Ericsson, Thales, Apple, Samsung, Deutsche Telekom, Qualcomm	discussion	Rel-18	NR_NTN_enh-Core

Observation 1	Broadcast NTN satellite assistance information in TN cells to facilitate service continuity is a feature which fits into Release 18 scope.

Observation 2	RAN2 has already discussed and identified a potential solution which would fit within Release 18 timeframe and has limited specification impact.

Observation 3	Supporting the broadcast of SIB19 in TN cells gives network operators the flexibility to selectively provide this information where there is a beneficial impact.

Observation 4	Information exchange between TN and NTN system is not precluded by existing specification. In fact, other Rel-18 enhancements assume a certain level of information exchange.

Observation 5	UE implementation can secure maintaining a valid copy of SIB19 in a TN cell.

Proposal 1	SIB19 can be broadcast in TN cells to provide satellite assistance information for NTN neighbour cells (e.g., ntn-NeighCellConfigList-r17).

Proposal 2	Adopt the text proposal to TS 38.331.

Moved here from 7.7.4.1

R2-2312462	Views on providing NTN information in TN cell	Lenovo	discussion	Rel-18

Proposal 1: Providing NTN neighbor cell information in a TN serving cell is supported.

Proposal 2: NTN neighbor cell information is provided in SIB19 in a TN serving cell.

Proposal 3: SIB19 is not an essential SIB when provided in a TN serving cell, i.e. UE does not consider the TN serving cell as barred if it fails to acquire SIB19.

Proposal 4: UE in RRC_IDLE/INACTIVE is not required to ensure having a valid version of SIB19 in a TN serving cell.

Proposal 5: The exact time of reacquiring SIB19 for UE in RRC_IDLE/INACTIVE in TN serving cell is up to UE implementation.

Proposal 6: UE in RRC_CONNECTED does not start T430 when SIB19 is provided in a TN cell.

Proposal 7: The SFN and subframe numbers of epoch time indicated in SIB19 in TN serving cell are based on the timing of the serving cell.

Proposal 8: If the epoch time indicated in SIB19 in TN serving cell is absent, UE considers the epoch time as the end of SI window where this SIB19 is scheduled in the TN serving cell.

Proposal 9: If the validity duration indicated in SIB19 in TN serving cell is absent, one of the following options is adopted:

A) UE considers the validity duration as infinity or not applicable;

B) UE considers the validity duration as a default value.

Moved here from 7.7.4.1

R2-2313481	Support of NTN neighbor cell info in TN cell	Sequans Communications	discussion	Rel-18	NR_NTN_enh-Core

Observation 1: For a TN cell, the ULTSRP is the gNB, whose location is not known

Proposal 1: Assuming the gNBUE propagation delay can be neglected in propagation calculations, the RPepochTime for NTN-config in TN cell is the gNB

Proposal 2: If the gNBUE propagation delay cannot be neglected in propagation calculations, the RPepochTime can be indicated by an additional field (UTC time)

Proposal 3: Reuse SIB19 to broadcast NTN neighbor cell info in TN cell

R2-2313079	Discussion on TN broadcasting NTN assistance information	Huawei, HiSilicon, Turkcell	discussion	Rel-18	NR_NTN_enh-Core

Observation 1: For UEs in RRC_CONNECTED, and UEs in RRC_IDLE/RRC_INACTIVE that are still in TN coverage, the provision of NTN neighbour cell information is not useful.

Observation 2: Providing NTN neighbour cell information requires the UE to maintain the time-variant information, leading to excessive power consumption.

Observation 3: The NTN frequency priorities will not be higher than TN frequencies, so UEs need not measure NTN frequencies if TN serving cell fulfils Srxlev > SnonIntraSearchP and Squal > SnonIntraSearchQ.

Observation 4: For the UE to perform measurement towards NTN neighbour cell, the NTN frequencies need to be provided in SIB3/4, thus affecting legacy UEs.

Proposal 1: Provision of NTN neighbour cell assistance information by a TN cell is not considered in this release.

Also the following papers are moved here from 7.7.4.1

R2-2311888	Cell (re)selection – discussion on broadcasting SIB19 in terrestrial networks	PANASONIC	discussion

R2-2311968	Discussion on support of NTN neighbor cell info in TN cell	OPPO	discussion	Rel-18	NR_NTN_enh-Core

R2-2312547	Discussions on providing NTN neighbor cell information in TN cell	ITRI	discussion	NR_NTN_enh-Core

R2-2312841	Support of NTN neighbour cell info in TN cells	Sony	discussion	Rel-18	NR_NTN_enh

#### 7.7.4.1	Cell reselection enhancements

R2-2312104	Remaining issues of cell reselection enhancement	Samsung	discussion	Rel-18	NR_NTN_enh-Core

Proposal 1: Legacy SI update procedure will be used for earth moving cell when the network updates the TN coverage information.

Proposal 2: The new SIB containing TN coverage area information belongs to Other SI, either periodically broadcast, broadcast on-demand, or sent in a dedicated manner.

Proposal 4: If TN cell broadcasting SIB19 is supported, SIB19 belongs to Other SI in TN, and is provided by either periodically broadcast, broadcast on-demand, or a dedicated manner.

R2-2312644	Remaining issues on cell reselection enhancements	ZTE Corporation, Sanechips	discussion	Rel-18	NR_NTN_enh-Core

Proposal 1: Separate capability description for location-based measurement initiation for quasi-earth-fixed system and earth moving system is used in 38.304 to align with the capability definition in 38.306.

Proposal 2: Choose one term from the following for skipping or relaxing measurements on TN cells where there is no coverage and such term should be aligned in both 38.306 and 38.304:

Term 1: TN measurement relaxation

Term 2: Skipping TN measurement

R2-2313532	Cell reselection enhancements for hard switch	Ericsson	discussion	Rel-18	NR_NTN_enh-Core

Observation 1	In NTN, for service or feeder link switch, the UE should be aware of the type of switch (hard or soft) to perform neighbour measurements accordingly.

Proposal 1	In case of NTN cell hard switch, UE needs not to start neighbour cell measurements of the new cell before t-service expires.

Proposal 2	The network informs (either implicitly or explicitly) the UE whether the next NTN cell switch is a soft or a hard switch.

Proposal 3	Adopt the text proposal to TS 38.304.

R2-2313506	Discussion on mobility enhancements for VSAT	THALES	discussion	Rel-18	NR_NTN_enh	R2-2310046	Late

R2-2311834	Remaining Issues on Cell Reselection for NR NTN	vivo	discussion	Rel-18	NR_NTN_enh-Core

R2-2311967	Discussion on the change of TN coverage information	OPPO	discussion	Rel-18	NR_NTN_enh-Core

R2-2312277	Cell coverage info and measurements	Qualcomm Incorporated	discussion	Rel-18	NR_NTN_enh-Core

R2-2312291	NTN-TN cell reselection enhancement	Apple	discussion	Rel-18	NR_NTN_enh-Core

R2-2312651	Discussion on NTN-TN cell reselection	CMCC	discussion	Rel-18	NR_NTN_enh-Core

R2-2312949	TN-NTN Mobility	NEC Telecom MODUS Ltd.	discussion

R2-2312950	On the use of TN coverage signalling to indicate non-TN areas	NEC Telecom MODUS Ltd.	discussion	R2-2310986

R2-2313401	Remaining issues on NTN-TN cell reselection enhancement	LG Electronics France	discussion	Rel-18	38.331	NR_NTN_enh	R2-2309862

- Revised in R2-2313552

R2-2313552	Remaining issues on NTN-TN cell reselection enhancement	LG Electronics France, Google Inc., Thales	discussion	Rel-18	38.331	NR_NTN_enh

R2-2313411	Discussion on NTN-TN cell reselection enhancements	ETRI	discussion	Rel-18	NR_NTN_enh

#### 7.7.4.2	Connected mode enhancements

R2-2313080	Discussion on HO enhancements	Huawei, HiSilicon, Turkcell	discussion	Rel-18	NR_NTN_enh-Core

##### 7.7.4.2.1	Handover enhancements

RACH-less HO

R2-2313004	Remaining open issues: RACH-less handover	InterDigital	discussion	Rel-18	NR_NTN_enh-Core

Proposal 1a:	If cg-NTN-RACH-less-Configuration is not configured, beam information to monitor target cell PDCCH for dynamic grant for initial UL transmission is always provided in the RACH-less HO command.

Proposal 1b:	An SSB index (not TCI-state ID) is provided in the RACH-less HO command to monitor target cell PDCCH for dynamic grant for initial UL transmission.

Proposal 2:	UE does not initiate a Random Access procedure if SR is triggered and rach-lessHO is configured (similar to LTE).

Proposal 3:	Specify in MAC that upon successful completion of RACH-less HO, UE releases the configured grant configuration used for initial UL transmission.

R2-2312105	Remaining issues on Handover enhancements	Samsung	discussion	Rel-18	NR_NTN_enh-Core

Proposal 1: An SSB index is indicated in RACH-less HO command for PDCCH monitoring for DG if CG for initial UL transmission is not configured.

Proposal 2: If CG for initial UL transmission is configured, CG occasions mapping to SSB is configured mandatorily.

Proposal 3: If CG for initial UL transmission is configured, UE starts to monitor PDCCH using the selected SSB after initial UL transmission.

Proposal 4: If CG is configured in RACH-less HO, UE uses the earliest available CG occasion associated to the selected SSB for the initial UL transmission.

Proposal 5: If CG is configured in RACH-less HO, RRC releases the CG with SSB association after RACH-less HO completion.

Proposal 6: Support autonomous retransmission with a CG- retransmission timer for the initial UL transmission.

Proposal 7: Both HARQ mode A and B can be configured for the HARQ process of the initial UL transmission using CG, which is up to NW implementation.

Proposal 8: Confirm SUL is not applied in NTN.

[AT124][301][NR-NTN Enh] RACH-less HO (Interdigital)

Scope: Discuss the remaining open issues for RACH-less HO, based primarily on R2-2313004 and R2-2312105 (if time allows)

Intended outcome: offline discussion summary

F2F schedule: Monday 2023-11-13 16:30-17:00 Brk3

Deadline for rapporteur's summary (in R2-2313784):  Tuesday 2023-11-14 12:00

R2-2313784	Summary of [AT124][301][NR-NTN Enh] RACH-less HO	Interdigital	discussion

R2-2312356	Open issues on RACH-less in NR NTN	Apple	discussion	Rel-18	NR_UAV

Proposal 1: For RACH-less HO with dynamic grant, support the threshold-based fallback to RACH-based HO mechanism, i.e., same as the usage of RACH-less HO with pre-allocated grant.

R2-2311836	Remaining Issues on RACH-less for R18 NR NTN	vivo	discussion	Rel-18	NR_NTN_enh-Core

R2-2311859	Remaining Issues on RACH-less for R18 NR NTN	Quectel	Work Plan	Rel-18

R2-2312057	Discussion on RACH-less HO in NR NTN	CATT	discussion

R2-2312500	Remaining issue for RACH-less	Sharp	discussion	Rel-18	NR_NTN_enh-Core

R2-2312790	Consideration on RACH-less HO remaining issues	ZTE Corporation, Sanechips	discussion

R2-2313297	Remaining open issues on RACH-less HO for NTN	ETRI	discussion	Rel-18

CHO Enhancements

R2-2313051	Remaining issues for IDLE and CONNECTED mode mobility in Rel-18 NTN	Nokia, Nokia Shanghai Bell	discussion	Rel-18	NR_NTN_enh-Core

Observation 1: The issue related to beam quality for completing the RACH-less CHO is equally applicable to Configured Grant and Dynamic Grant.

Proposal 1: Target cell provides the DG so that the UE can complete the RACH-less CHO within the (t1, t2) time window. FFS when the UE starts monitoring for DG considering the CHO-related execution conditions.

Proposal 2: Threshold (i.e. quality threshold) for dynamic grant in RACH-less CHO is not introduced in Rel-18 NTN.

Proposal 3: ReferenceLocation for source and for each candidate target cell is included in RRC Reconfiguration (CHO command) in EMC.

Proposal 4: In EMC CHO, the UE obtains the ephemeris information for both the serving. and candidate CHO cells from SIB19.

Proposal 5: Alternatively (if Proposal 4 is not pursued), the UE obtains also the ephemeris and epochTime for each candidate CHO cell from RRC Reconfiguration. The corresponding information for the serving cell is still taken from SIB19.

[AT124][302][NR-NTN Enh] CHO enhancements (Nokia)

Scope: Discuss the remaining open issues for CHO enhancements, based primarily on section 2.2 of R2-2313051 (and section 2.1 if time allows)

Intended outcome: offline discussion verbal summary (the discussion will continue online)

F2F schedule: Tuesday 2023-11-14 16:30-17:00 Brk1

R2-2311835	Remaining Issues on CHO Enhancements for NR NTN	vivo	discussion	Rel-18	NR_NTN_enh-Core

R2-2312053	Configuration for location-based CHO for earth-moving cell	CATT	discussion

R2-2312292	CHO enhancement to earth moving target cell	Apple	discussion	Rel-18	NR_NTN_enh-Core

R2-2312840	Signaling overhead reduction during NTN-NTN HOs	Sony	discussion	Rel-18	NR_NTN_enh

R2-2313005	Remaining open issues: CHO for Earth-moving cells	InterDigital	discussion	Rel-18	NR_NTN_enh-Core

R2-2313190	Discussion on CHO configuration for moving cell location	ASUSTeK	discussion	Rel-18	NR_NTN_enh-Core

All issues

R2-2311966	Discussion on handover enhancement for NR NTN	OPPO	discussion	Rel-18	NR_NTN_enh-Core

R2-2312278	Open issues for handover enhancements	Qualcomm Incorporated	discussion	Rel-18	NR_NTN_enh-Core

R2-2312463	Some remaining issues for CHO and RACH-less HO in NTN	Lenovo	discussion	Rel-18

R2-2312763	Discussion on the remaining issues for the handover enhancements	Xiaomi	discussion

R2-2313399	Remaining issues on handover enhancements	LG Electronics France	discussion	Rel-18	38.331	NR_NTN_enh

##### 7.7.4.2.2	Unchanged PCI satellite switch

Including report of [Post123bis][312][NR-NTN Enh] Unchanged PCI (CMCC/Apple). Company contributions on aspects handled in [Post123bis][312] might be down-prioritized.

R2-2313206	Report of [Post123bis][312][NR-NTN Enh] Unchanged PCI	CMCC, Apple	discussion	Rel-18	NR_NTN_enh-Core

List of proposals for agreement:

Proposal A-1: it is proposed to introduce one new target satellite configuration, e.g. ntn-TargetSatConfig, and provide the NTN-config of the target satellite in it for the the specific signaling format about the target satellite information in SIB19. (14/15)

Proposal A-2 : At least for soft switch, support  SMTC configuration of target satellite can be different from that in source. (15/15)  FFS for hard switch.(10/15)

Proposal A-3: SMTC configuration adjustment should be handled by UE. (12/15)

Proposal A-4: the SMTC configuration of target satellite should be provided in SIB19 (if needed). FFS whether an indication to indicate the SMTC configuration of target satellite is same with that of serving satellite can be provided in SIB19.

Proposal A-5:  Support implicit indication (e.g. soft switching if T-start is configured) to inform UE it is hard switch or soft switch case.

Proposal A-7: For hard satellite switch, there is no need to provide the SSB information of the  target satellite. (13/15)

< UE operation during satellite switching>

Proposal B-1: PHR procedure is not impacted by the satelliete switching procedure.

Proposal B-2: During satellite switching procedure, UE should reset the L3 filter for serving cell RRM measurement and RLM, and it’s up to UE implementation (i.e. no RAN2 spec impact).

Proposal B-3: The satellite switching failure is detected by the legacy RLF mechanism, i.e no need to introduce new timer based failure detection mechanism.

Proposal B-4: UE initiates the UE connection reestablishment procedure when the satellite switching failure is detected.

Proposal B-8: For RACH-less satellite switching, no additional beam information is provided to UE for the UL transmission in target satellite.

Proposal B-9: For RACH-less satellite switching, there is no dedicated preallocated UL grant  provided to UE for the 1st UL transmission in target satellite.

Proposal B-10: For RACH-less satellite switching, there is no case to fallback to RACH-based satellite switching procedure based on radio quality.

Proposal B-11: For RACH-less satellite switching procedure, UE sets Nta value to 0.

Proposal B-12: For RACH-less satellite switching, UE resumes the UE dedicated UL/DL transmission after UE aquires the DL sync in target satellite.

< Coexistence with L3 mobility scheme>

Proposal B-13: If UE receive the HO command before UE initiates the satellite switching procedure (i.e. before the time point of satellite switching), UE will initiate the HO procedure immediately.

Proposal B-14: Both CHO and satellite switching procedure can be configured simultaneously.

Proposal B-15a : Intra-cell CHO and satellite switching procedure should not be configured simultaneously.

Proposal B-15b : When both CHO and satellite switching procedure are configured, RAN2 is proposed to discuss which option should be adopted:

-	Option 1: UE always follows CHO;

-	Option 2: UE initiates the procedure that triggers earlier; it's up to UE implementation if both procedures are triggered at the same time.

List of proposals that require online discussions:

Proposal A-6:  For soft satellite switch, regarding the target satellite SSB information, we could further discuss the following options:

Option 1: Indicating a time offset/information  for the target satellite

Option 2: Indicating a different SSB index for the target satellite

Option 3: ssb-PositionsInBurst of the target satellite

Option 4: option 1+2

Option 5: SMTC configuration is enough

< proposal from session Chair (along the lines of option 1+2):

Proposal A-6rev:  For soft satellite switch, as a baseline, the target satellite SSB information is provided via STMC configuration of the target satellite. Optionally a different SSB index for the target satellite can also be provided.

>

Proposal A-8: The detail signaling of t-star can be discussed online.

< proposal from session Chair:

Proposal A-8rev:  Adopt one of the following solutions:

- T-start is explicitly signalled (same format as T-service). If T-start is not signalled, T-start is assumed to be equal to T-service, i.e. hard switch.

- T-start is derived from “T-offset” (T-start = T-service-T-offset), where T-offset only has positive values (or zero) and reserved values for possible future use (e.g. negative values). If T-overlap is not signalled, T-overlap is assumed to be zero, i.e. hard switch

>

Proposal A-9: Further discuss whether T-start is needed for hard satellite switch.

< proposal from session Chair:

Proposal A-9rev: If T-start will be explicitly signalled, discuss whether to specify that the NW should not signal a T-start higher than T-service (If T-offset will be signalled, we simply don’t introduce negative values for it)

>

Proposal B-1a (for discussion): Discuss whether UE triggers TA reporting upon satellite switching.

<RACH-less satellite switching>

< proposal from session Chair:

Proposal B-1a_rev: Upon satellite switching the triggering of TA reporting is based on legacy procedures

>

Proposal B-5 (for discussion): Discuss whether UE that supports satellite switching is mandatory to support RACH-less satellite switching.

< proposal from session Chair:

Proposal B-5_rev: UE supporting satellite switching with re-sync mandatorily supports RACH-less satellite switching.

>

Proposal B-6 (for discussion): Support the explicit configuration of RACH-less satellite switching in SIB19.

Proposal B-6a (for discussion): If proposal B-6 is not agreed, RAN2 is proposed to discuss the following two issues:

-	Issue 1: How does the UE determine whether to trigger RACH-less or RACH-based procedure?

-	Issue 2: Does the network always support the RACH-less apprach for satellite switching?

Proposal B-7 (discussed after proposal B-5 and B-6):  If UE does not support RACH-less satellite switching, UE will only perform RACH-based procedure regardless of whether the network configured RACH-less or RACH-based satellite switching procedure.

Moved here from 7.7.4.2

R2-2313529	Remaining issues with connected mode enhancements	Ericsson	discussion	Rel-18	NR_NTN_enh-Core

- Unchanged PCI

  - Feature name

Proposal 4	Adopt the name satellite switch with re-sync for the unchanged PCI feature.

  - TAT handling

Proposal 7	For satellite switch with re-sync, network implementation can solve the issue with TAT expiration before the UE is synchronized to the target satellite.

- Duplicated info in SIB19

Proposal 10	UE shall assume when a PCI associated to an NTN carrier frequency is included in a measurement object but it is not present in the neighbour cell list in SIB19, this PCI is provided by the serving satellite.

Proposal 11	Adopt the text proposal to TS 38.331 in Section 3.

[AT124][303][NR-NTN Enh] Unchanged PCI (Apple)

Scope: Discuss the remaining open issues for Unchanged PCI (exact scope to be clarified after the online session on Tuesday)

Intended outcome: offline discussion summary

F2F schedule: Wednesday 2023-11-15 10:30-11:00 Brk3

Deadline for rapporteur's summary (in R2-2313785):  Wednesday 2023-11-15 22:00

R2-2313785	Summary of [AT124][303][NR-NTN Enh] Unchanged PCI	Apple	discussion

R2-2311837	Remaining Issues on Service Link Switching with Unchanged PCI	vivo	discussion	Rel-18	NR_NTN_enh-Core

R2-2311849	  Discussion on unchanged PCI mechanism	Quectel	discussion	Rel-18

R2-2311989	Signalling design of satellite switching with PCI unchanged	China Telecom	discussion	Rel-18	NR_NTN_enh-Core

R2-2312047	Leftover issues on the unchanged PCI satellite switch	Google Inc.	discussion

R2-2312058	Discussion on unchanged PCI mechanism	CATT	discussion

R2-2312106	Remaining issues on PCI unchanged satellite switch	Samsung	discussion	Rel-18	NR_NTN_enh-Core

R2-2312120	On Outstanding Issues in Unchanged PCI in LEO NTN	MediaTek Inc.	discussion

R2-2312279	Major issues for satellite switch with PCI unchanged	Qualcomm Incorporated	discussion	Rel-18	NR_NTN_enh-Core

R2-2312293	Satellite switching with unchanged PCI	Apple	discussion	Rel-18	NR_NTN_enh-Core

R2-2312464	On some remaining issues for PCI-unchanged scenario	Lenovo	discussion	Rel-18

R2-2312546	Discussions on SMTC configuration for satellite switch without PCI change	ITRI	discussion	NR_NTN_enh-Core

Moved here from 7.7.4.2

R2-2312609	FFS issues of unchanged PCI solution	NEC	discussion	Rel-18	NR_NTN_enh-Core

R2-2312645	Usage and signaling of t-start	ZTE Corporation, Sanechips	discussion	Rel-18	NR_NTN_enh-Core

R2-2312646	Discussion on remaining issues of soft and hard satellite switch with PCI unchanged	Transsion Holdings	discussion	Rel-18

R2-2313006	Remaining open issues: Satellite switching without PCI change	InterDigital	discussion	Rel-18	NR_NTN_enh-Core

Moved here from 7.7.4.2

R2-2313052	Remaining Issues for Satellite Switching without L3 Mobility	Nokia, Nokia Shanghai Bell	discussion	Rel-18	NR_NTN_enh-Core

R2-2313191	Discussion on remaining issue for unchanged PCI switch	ASUSTeK	discussion	Rel-18	NR_NTN_enh-Core

R2-2313279	Remaining issues on Unchanged PCI	ITL	discussion	Rel-18

R2-2313400	Remaining issues on unchanged PCI	LG Electronics France	discussion	Rel-18	38.331	NR_NTN_enh

R2-2313475	Unchanged PCI satellite switch considerations	Sequans Communications	discussion	Rel-18	NR_NTN_enh-Core

Withdrawn

R2-2312632	Discussion on remaining issues of soft and hard satellite switch with PCI unchanged	Transsion Holdings	discussion	Rel-18	Withdrawn

### 7.25.4	Self-Evaluation NTN

(FS_IMT-2020_Sat_eval; leading Group: TSG RAN; REL-18; WID: RP-230736)

This will be treated in NTN breakout session (Sergio).

Study on Self-Evaluation towards the 3GPP submission of a IMT-2020 Satellite Radio Interface Technology, including both NR NTN and IoT-NTN. Note that the time allocated will be very limited, and this is expected to be mostly an offline activity.

R2-2312865	Discussion on IMT-2020 Satellite self-evaluation for Latency	THALES	discussion	Rel-18	NR_NTN_enh-Core

Observation 1:	The maximum RTD for GEO scenario with an elevation angle of 10° for service link and feeder link is 541 ms

Proposal 1:	The Satellite Radio Interface supports larger latencies up to 650 ms for user plane dataflows

Proposal 2:		The Satellite Radio Interface supports larger latencies up to 1.15 s for control plane signalling

Proposal 3:		Consider the above text proposal for TP for TR 37.911

# Summary

In-Principle Agreed CRs

NR-NTN

IoT-NTN

Approved LSs out

[Post124] Email discussions

Short

[Post124][301][NR-NTN Enh] 38.300 CR (Thales)

Scope: update the running CR with meeting agreements

Intended outcome: Agreed CR

Deadline for agreed CR (in R2-2313771): short

[Post124][302][NR-NTN Enh] 38.331 CR (Ericsson)

Scope: update the running CR with meeting agreements

Intended outcome: Agreed CR

Deadline for agreed CR (in R2-2313772): short

[Post124][303][NR-NTN Enh] 38.321 CR (Interdigital)

Scope: update the running CR with meeting agreements

Intended outcome: Agreed CR

Deadline for agreed CR (in R2-2313773): short

[Post124][304][NR-NTN Enh] 38.304 CR (ZTE)

Scope: update the running CR with meeting agreements

Intended outcome: Agreed CR

Deadline for agreed CR (in R2-2313774): short

[Post124][305][NR-NTN Enh] 37.355 CR (CATT)

Scope: update the running CR with meeting agreements

Intended outcome: Agreed CR

Deadline for agreed CR (in R2-2313777): short

[Post124][306][NR-NTN Enh] 38.305 CR (Qualcomm)

Scope: update the running CR with meeting agreements

Intended outcome: Agreed CR

Deadline for agreed CR (in R2-2313778): short

[Post124][307][IoT-NTN Enh] 36.300 CR (Ericsson)

Scope: update the running CR with meeting agreements

Intended outcome: Agreed CR

Deadline for agreed CR (in R2-2313779): short

[Post124][308][IoT-NTN Enh] 36.331 CR (Huawei)

Scope: update the running CR with meeting agreements

Intended outcome: Agreed CR

Deadline for agreed CR (in R2-2313780): short

[Post124][309][NR-NTN Enh] 36.321 CR (Mediatek)

Scope: update the running CR with meeting agreements

Intended outcome: Agreed CR

Deadline for agreed CR (in R2-2313781): short

[Post124][310][IoT-NTN Enh] 36.304 CR (Nokia)

Scope: update the running CR with meeting agreements

Intended outcome: Agreed CR

Deadline for agreed CR (in R2-2313782): short

[Post124][311][IoT-NTN Enh] 36.306 CR (Qualcomm)

Scope: update the running CR with meeting agreements

Intended outcome: Agreed CR

Deadline for agreed CR (in R2-2313783): short

Medium

Long