3GPP TSG RAN WG2#124	R2-2311709

Chicago, USA, 13th - 17th November 2023

3GPP TSG RAN WG 1 Meeting #114bis	R1-2310502

Xiamen, China, October 9th – October 13th, 2023

Title:	Reply LS on XR capacity enhancements

Response to:	R1-2308825(R2-2309007) Reply LS on XR capacity enhancements

Release:	Release 18

Work Item:	NR_XR_enh-Core

Source:	RAN1

To:	RAN2

Cc:

Contact Person:

#### Name:	Umut Ugurlu

E-mail Address:	umut.ugurlu [at] mediatek.com

Send any reply LS to:	3GPP Liaisons Coordinator, mailto:3GPPLiaison@etsi.org

Attachments:	-

1. Overall Description:

RAN1 would like to thank RAN2 for the LS on specification of HARQ process ID determination for a multi-PUSCHs configured grant.

Regarding the question on where the definition of invalid CG PUSCH is captured in RAN1 specification, RAN1 would like to provide the following feedback.

RAN1 discussed this question in RAN1#114bis meeting in October 2023. RAN1 endorsed the following TP for physical layer procedures for data for TS 38.214:

Agreement

Adopt TP1-1 below for Clause 6.1 of 38.214:

## 6.1 UE procedure for transmitting the physical uplink shared channel ************** Unchanged parts omitted************** When the UE is configured dl-OrJointTCI-StateList or ul-TCI-StateList, the UE shall perform PUSCH transmission corresponding to a Type 1 configured grant or a Type 2 configured grant or a dynamic grant according to the spatial relation, if applicable, with a reference to the RS for determining UL Tx spatial filter. The RS is determined based on an RS configured with qcl-Type set to 'typeD' of the indicated TCI-State or an RS in the indicated TCI-UL-State. The reference RS in the indicated TCI-State can be a CSI-RS resource in a NZP-CSI-RS-ResourceSet configured with higher layer parameter repetition, or a CSI-RS resource in an NZP-CSI-RS-ResourceSet configured with higher layer parameter trs-Info. The reference RS in the indicated TCI-UL-State can be a CSI-RS resource in a NZP-CSI-RS-ResourceSet configured with higher layer parameter repetition, a CSI-RS resource in an NZP-CSI-RS-ResourceSet configured with higher layer parameter trs-Info, an SRS resource in an SRS resource set with the higher layer parameter usage set to 'beamManagement', or SS/PBCH block associated with the same or different PCI from the PCI of the serving cell. When [nrofSlots_InCGperiod] is configured for Type 1 configured grant or Type 2 configured grant, HARQ process ID for the Kth (1 < K ≤ [nrofSlots_InCGperiod]) valid configured PUSCH grant is determined as in clause 5.4.1 of [10, TS 38.321], excluding invalid configured PUSCH grant(s) that are not transmitted due to collision with the DL symbol(s) indicated by tdd-UL-DL-ConfigurationCommon or tdd-UL-DL-ConfigurationDedicated if provided, or a symbol(s) of an SS/PBCH block with index provided by ssb-PositionsInBurst as described in clause 11.1 of [6, TS 38.213]. ************** Unchanged parts omitted**************

RAN1 would like to inform RAN2 that the description in Clause 6.1 of TS38.214 can be used as the reference for the definition of invalid CG PUSCH.

2. Actions:

To RAN2:

ACTION: 	RAN1 respectfully asks RAN2 to take the above information into account.

3. Dates of Next RAN1 Meetings:

TSG WG RAN1 #115	November 13th -17th, 2023	Chicago, U.S.A.

TSG WG RAN1 #116	February 26th – March 1st, 2024	Athens, GR