3GPP TSG-RAN WG2 Meeting #125                                                        R2-2400104

Athens, Greece, 26th Feb - 1st Mar, 2024

Source:              CATT

Title:                Clarification on More Than One Leg on Direct Uu Path in Multi-path

Agenda Item:	7.9.6

Document for:	Discussion and Decision

Introduction

In RAN2#124 meeting, RAN2 agreed that more than one leg (i.e., CA configuration with 2 or 3 legs) on direct Uu path in MP is supported. But in subclause 5.2.1 and 5.6 of TS38.323, the current description didn’t reflect the above latest RAN2 agreement completely. This issue is discussed in the current contribution.

Discussion

In RAN2#124 meeting, the bellow agreement was reached for Multi-path case:

Agreement:

More than one leg (i.e., CA configuration with 2 or 3 legs) on direct Uu path in MP is supported.  Capability for this feature to be discussed under the general capability discussion.

The subclause 5.2.1 of TS38.321 records the transmit operation for PDCP entity. When submitting a PDCP PDU to lower layer, the multi-path scenario was added under the below branch:

-	else, if the transmitting PDCP entity is associated with one or more RLC entities and, either one SRAP entity or the N3C:

The above green marked part is related to the direct path which include the above RAN2#124 meeting’s agreement.

But in the NOTE of the current subclause, there is one part that more than one leg on direct Uu path is missed. Similar situation happens in subclause 5.6 of TS38.321.

Proposal 1: Clarification on more than one leg on direct Uu path in multi-path is needed.

Conclusion

According to the analysis in section 2, it is proposed:

Proposal 1: Clarification on more than one leg on direct Uu path in multi-path is needed.

The corresponding TP for Proposal 1 is attached in section 4.

Text Proposal of TS38.323

Start Change

## 5.2	Data transfer

### 5.2.1	Transmit operation

At reception of a PDCP SDU from upper layers, the transmitting PDCP entity shall:

-	if discardTimerForLowImportance is configured and PSI based SDU discard is activated, and the PDCP SDU belongs to a low importance PDU Set:

-	start the discardTimerForLowImportance associated with this PDCP SDU;

-	else:

-	start the discardTimer associated with this PDCP SDU (if configured).

NOTE 0:	Identification of PSI of a PDU Set and determination of low importance PDU Set are left up to UE implementation.

For a PDCP SDU received from upper layers, the transmitting PDCP entity shall:

-	associate the COUNT value corresponding to TX_NEXT to this PDCP SDU;

NOTE 1:	Associating more than half of the PDCP SN space of contiguous PDCP SDUs with PDCP SNs, when e.g., the PDCP SDUs are discarded or transmitted without acknowledgement, may cause HFN desynchronization problem. How to prevent HFN desynchronization problem is left up to UE implementation.

-	perform header compression of the PDCP SDU using ROHC as specified in the clause 5.7.4 and/or using EHC as specified in the clause 5.12.4;

-	perform uplink data compression of the PDCP SDU as specified in clause 5.14.4;

-	perform integrity protection, and ciphering using the TX_NEXT as specified in the clause 5.9 and 5.8, respectively;

-	set the PDCP SN of the PDCP Data PDU to TX_NEXT modulo 2[pdcp-SN-SizeUL];

-	increment TX_NEXT by one;

-	submit the resulting PDCP Data PDU to lower layer as specified below.

When submitting a PDCP PDU to lower layer, the transmitting PDCP entity shall:

-	if the transmitting PDCP entity is associated with one SRAP entity:

-	submit the PDCP PDU to the associated SRAP entity;

-	else, if the transmitting PDCP entity is associated with one RLC entity:

-	submit the PDCP PDU to the associated RLC entity;

-	else, if the transmitting PDCP entity is associated with one or more RLC entities and, either one SRAP entity or the N3C:

-	if PDCP duplication is activated for the RB:

-	if the PDCP PDU is a PDCP Data PDU:

-	duplicate the PDCP Data PDU and submit the PDCP Data PDU to both the primary path and secondary path, including any associated Uu RLC entities activated for PDCP duplication;

-	else:

-	submit the PDCP Control PDU to the primary path;

-	else (i.e., PDCP duplication is deactivated for the RB):

-	if the total amount of PDCP data volume, RLC data volume pending for initial transmission (as specified in TS 38.322 [5]) in the RLC entity, and data volume pending for either transmission in the N3C (if available) or mapped SL RLC entity associated with the SRAP entity, is equal to or larger than ul-DataSplitThreshold:

-	submit the PDCP PDU to either the primary path or secondary path;

-	else:

-	submit the PDCP PDU to the primary path;

-	else, if the transmitting PDCP entity is associated with at least two RLC entities:

-	if the PDCP duplication is activated for the RB:

-	if the PDCP PDU is a PDCP Data PDU:

-	duplicate the PDCP Data PDU and submit the PDCP Data PDU to the associated RLC entities activated for PDCP duplication;

-	else:

-	submit the PDCP Control PDU to the primary RLC entity;

-	else (i.e. the PDCP duplication is deactivated for the RB or the RB is a DAPS bearer):

-	if the split secondary RLC entity is configured; and

-	if the total amount of PDCP data volume and RLC data volume pending for initial transmission (as specified in TS 38.322 [5]) in the primary RLC entity and the split secondary RLC entity is equal to or larger than ul-DataSplitThreshold:

-	submit the PDCP PDU to either the primary RLC entity or the split secondary RLC entity;

-	else, if the transmitting PDCP entity is associated with the DAPS bearer:

-	if the uplink data switching has not been requested:

-	submit the PDCP PDU to the RLC entity associated with the source cell;

-	else:

-	if the PDCP PDU is a PDCP Data PDU:

-	submit the PDCP Data PDU to the RLC entity associated with the target cell;

-	else:

-	if the PDCP Control PDU is associated with source cell:

-	submit the PDCP Control PDU to the RLC entity associated with the source cell;

-	else:

-	submit the PDCP Control PDU to the RLC entity associated with the target cell;

-	else:

-	submit the PDCP PDU to the primary RLC entity.

NOTE 2:	If the transmitting PDCP entity is associated with two RLC entities, or with one or more RLC entities and either an SRAP entity or the N3C, the UE should minimize the amount of PDCP PDUs submitted to lower layers before receiving request from lower layers and minimize the PDCP SN gap between PDCP PDUs submitted to two associated RLC entities, or to one or more RLC entities and either the SRAP entity or the N3C, to minimize PDCP reordering delay in the receiving PDCP entity.

Next Change

## 5.6	Data volume calculation

For the purpose of MAC buffer status reporting, the transmitting PDCP entity shall consider the following as PDCP data volume:

-	the PDCP SDUs for which no PDCP Data PDUs have been constructed;

-	the PDCP Data PDUs that have not been submitted to lower layers;

-	the PDCP Control PDUs;

-	for AM DRBs, the PDCP SDUs to be retransmitted according to clause 5.1.2 and clause 5.13;

-	for AM DRBs, the PDCP Data PDUs to be retransmitted according to clause 5.5.

If the transmitting PDCP entity is associated with at least two RLC entities, or with one or more RLC entities and either an SRAP entity or the N3C, when indicating the PDCP data volume to a MAC entity for BSR triggering and Buffer Size calculation (as specified in TS 38.321 [4] and TS 36.321 [12]), the transmitting PDCP entity shall:

End of Change