3GPP TSG-RAN WG2 #125	R2-2400648

Athens, Greece, February 2024

Agenda Item:	6.2

Source:	OPPO

Title:	Discussion on AS condition checking for SUI transmission

Document for:	Discussion, Agreement

Introduction

In the current RRC specification, when performing U2N Relay communication, there is no AS threshold condition defined, i.e., Relay/Remote UE doesn’t need to check any signal strength to perform U2N relay communication. But for reporting the SUI for U2N Relay communication, the UE will check whether the discovery threshold condition configured in SIB12 is satisfied, which seems a gap and has not been discussed in RAN2 before. This paper will discuss whether the SUI for U2N Relay communication transmission needs to check the discovery transmission conditions.

Discussion

In the current RRC specification, sidelink communication transmission (as TS 38.331 clause 5.8.8) includes both L2/3 U2N Relay communication and non-relay communication, and there is no AS threshold condition defined for sidelink communication transmission, i.e., Tx UE (e.g., U2N Relay/Remote UE) doesn’t need to check the discovery transmission condition defined for U2N Relay/Remote UE to perform U2N relay communication.

For U2N Relay communication transmission, based on clause 5.8.8 in RRC specification, Tx UE doesn’t need to check any AS threshold condition.

For SUI initiation, when reporting the U2N Relay communication transmission-related information to gNB, the UE needs to check whether the relay discovery transmission condition is satisfied or not:

2> if configured by upper layer to transmit NR sidelink L2 U2N relay communication on the frequency included in sl-FreqInfoList in SIB12 of the PCell including sl-L2U2N-Relay; or if configured by upper layer to transmit NR sidelink L3 U2N relay communication on the frequency included in sl-FreqInfoList in SIB12 of the PCell including sl-L3U2N-RelayDiscovery: 3> if the UE did not transmit a SidelinkUEInformationNR message since last entering RRC_CONNECTED state; or 3> if since the last time the UE transmitted a SidelinkUEInformationNR message the UE connected to a PCell not providing SIB12 including sl-ConfigCommonNR; or connected to a PCell providing SIB12 but not including sl-L2U2N-Relay in case of L2 U2N relay operation; or connected to a PCell providing SIB12 but not including sl-L3U2N-RelayDiscovery in case of L3 U2N relay operation; or 3> if the last transmission of the SidelinkUEInformationNR message did not include sl-TxResourceReqL2U2N-Relay; or if the information carried by the sl-TxResourceReqL2U2N-Relay has changed since the last transmission of the SidelinkUEInformationNR message; or if the last transmission of the SidelinkUEInformationNR message did not include sl-TxResourceReqL3U2N-Relay; or if the information carried by the sl-TxResourceReqL3U2N-Relay has changed since the last transmission of the SidelinkUEInformationNR message; or 3> if configured by upper layers not to transmit either NR sidelink L2 U2N relay communication or NR sidelink L3 U2N relay communication, and if the last transmission of the SidelinkUEInformationNR message includes both sl-TxResourceReqL2U2N-Relay and sl-TxResourceReqL3U2N-Relay: 4> if the UE is capable of U2N Relay UE, and if SIB12 includes sl-RelayUE-ConfigCommon, and if the U2N Relay UE threshold conditions as specified in 5.8.14.2 are met; or 4> if the UE is selecting a U2N Relay UE / has a selected U2N Relay UE, and if SIB12 includes sl-RemoteUE-ConfigCommon, and if the U2N Remote UE threshold conditions as specified in 5.8.15.2 are met: 5> initiate transmission of the SidelinkUEInformationNR message to indicate the NR sidelink relay communication transmission resources required by the UE in accordance with 5.8.3.3;

As shown above, UE cannot report the SUI for U2N Relay communication if the U2N Relay/Remote AS condition is not satisfied.

Based on clause 5.8.3.2 in RRC specification, UE needs to check AS condition for the SUI report to indicate U2N Relay communication transmission.

This will lead to a scenario in which U2N Remote UE and Relay UE can perform U2N Relay communications but the related configurations/resources are not valid/available since the SUI cannot be reported.

The gap in checking AS condition for SUI transmission and U2N Relay communication transmission causes non-valid/available configuration/resource for U2N Relay communications.

Besides, there is no RAN2 conclusion on AS condition for SUI transmission for U2N Relay communication.

There is no RAN2 conclusion on AS condition for SUI transmission indicating U2N Relay communication.

Therefore, it is proposed to remove the AS condition checking of SUI transmission for U2N Relay service as follows:

2> if configured by upper layer to transmit NR sidelink L2 U2N relay communication on the frequency included in sl-FreqInfoList in SIB12 of the PCell including sl-L2U2N-Relay; or if configured by upper layer to transmit NR sidelink L3 U2N relay communication on the frequency included in sl-FreqInfoList in SIB12 of the PCell including sl-L3U2N-RelayDiscovery: 3> if the UE did not transmit a SidelinkUEInformationNR message since last entering RRC_CONNECTED state; or 3> if since the last time the UE transmitted a SidelinkUEInformationNR message the UE connected to a PCell not providing SIB12 including sl-ConfigCommonNR; or connected to a PCell providing SIB12 but not including sl-L2U2N-Relay in case of L2 U2N relay operation; or connected to a PCell providing SIB12 but not including sl-L3U2N-RelayDiscovery in case of L3 U2N relay operation; or 3> if the last transmission of the SidelinkUEInformationNR message did not include sl-TxResourceReqL2U2N-Relay; or if the information carried by the sl-TxResourceReqL2U2N-Relay has changed since the last transmission of the SidelinkUEInformationNR message; or if the last transmission of the SidelinkUEInformationNR message did not include sl-TxResourceReqL3U2N-Relay; or if the information carried by the sl-TxResourceReqL3U2N-Relay has changed since the last transmission of the SidelinkUEInformationNR message; or 3> if configured by upper layers not to transmit either NR sidelink L2 U2N relay communication or NR sidelink L3 U2N relay communication, and if the last transmission of the SidelinkUEInformationNR message includes both sl-TxResourceReqL2U2N-Relay and sl-TxResourceReqL3U2N-Relay: 4> if the UE is capable of U2N Relay UE, and if SIB12 includes sl-RelayUE-ConfigCommon; or 4> if the UE is selecting a U2N Relay UE / has a selected U2N Relay UE, and if SIB12 includes sl-RemoteUE-ConfigCommon: 5> initiate transmission of the SidelinkUEInformationNR message to indicate the NR sidelink relay communication transmission resources required by the UE in accordance with 5.8.3.3;

RAN2 to discuss removing the AS condition checking of SUI transmission for U2N Relay service as the above TP.

Conclusion

We have the following observations:

Observation 1	For U2N Relay communication transmission, based on clause 5.8.8 in RRC specification, Tx UE doesn’t need to check any AS threshold condition.

Observation 2	Based on clause 5.8.3.2 in RRC specification, UE needs to check AS condition for the SUI report to indicate U2N Relay communication transmission.

Observation 3	The gap in checking AS condition for SUI transmission and U2N Relay communication transmission causes non-valid/available configuration/resource for U2N Relay communications.

Observation 4	There is no RAN2 conclusion on AS condition for SUI transmission indicating U2N Relay communication.

We have the following proposals:

Proposal 1	RAN2 to discuss removing the AS condition checking of SUI transmission for U2N Relay service as the above TP.