3GPP TSG-RAN WG2 Meeting #125                                         R2-2400679

Athens, Greece, Feb 26th – Mar 1st, 2024

Source:         ZTE Corporation

Title:           Discussion on SA2 LS on partial coverage

Agenda item:    7.2.1

Document for:	Discussion and Decision

- Introduction

SA2 has sent a new LS[1] to ask RAN2 the questions about coverage:

1. Overall Description: SA2 has approved that the coverage condition was taken into consideration (see 23.586 clause 5.2.2 and 5.2.3) during Ranging/Sidelink Positioning UE Discovery & Selection. During SA2 discussion, companies are not sure whether the solutions in clause 6.20 of 23.273 also work in the partial coverage scenario, i.e. when Target UE has a NAS connection with a LMF supporting Ranging/Sidelink Positioning, while other UEs (UE 2 to n) are out of Uu coverage or does not have connection with a LMF. The following questions were raised: Question 1: Is there any potential RAN2 impact to support network based or network assisted SL positioning as defined in the 23.273 clause 6.20 when the target UE is in coverage while some of the SL reference UE(s) are out of coverage or does not have connection to a LMF? Question 2: Is there any potential RAN2 impact to support UE only SL positioning as defined in 23.586 clause 6.8 when the target UE is out of coverage while some of the SL reference/Located UE(s) are in coverage of a network that supports Ranging/Sidelink Positioning?

This paper discusses the question that SA2 raises, and provides a draft reply.

- Discussion

The revised Rel-18 WID[2] is specified below:

- Specify unicast session-based signalling and procedures to facilitate support of SL positioning for single target UE (it is not precluded to apply the procedures to multiple target UEs but no signaling optimizations will be considered for this case) [RAN2, RAN3]: - Specify the protocol and procedures for SL positioning between UEs (Protocol for Sidelink positioning procedures (SLPP)). - Specify the protocol and procedures for SL positioning between UEs and a single LMF for in coverage scenario only, including joint PC5-Uu scenarios. - NOTE: Assumes all involved UEs are served by same LMF. - For SL-TDOA, RAN2 will not work on procedures for synchronization of the anchor UEs. RAN2 can discuss and implement agreed RAN1 parameters related to synchronization.

According to the new WID, 38.305-i00[3] has the following description:

5.1 Architecture Figure 5.1-1 shows the architecture in 5GS applicable to positioning of a UE with NR or E-UTRA access. The positioning architecture also supports the NR PC5 interface as illustrated in Figure 5.1-1. Sidelink positioning can be supported when the UE is inside NG-RAN coverage (UE A and UE B in Figure 5.1-1) and when the UE is outside NG-RAN coverage (UE C and UE D in Figure 5.1-1). NOTE: Sidelink positioning for UEs in partial coverage is not supported in this Release of the specification.

There are totally four kinds of coverage scenarios in SL positioning:

- Session is in coverage(IC), case 1: Target UE is in coverage, and all anchor UEs are in coverage; (RAN2 currently supports)

- Session is partial coverage(PC), case 2-1: Target UE is in coverage, while at least one of the anchor UE is out of coverage; (Q1 in SA2’s LS)

- Session is partial coverage(PC), case 2-2: Target UE is out of coverage, while at least one of the anchor UE is in coverage; (Q2 in SA2’s LS)

- Session is out of coverage(OOC), case 3: Target UE is out of coverage, and all anchor UEs are out of coverage; (RAN2 currently supports)

From RAN2 current specification and assumption, only IC target UE and IC anchor UEs can be included in a SL positioning session and perform SL positioning; only OOC target UE and OOC anchor UEs can be included in a SL positioning session and perform SL positioning; so from RAN2 perspective, RAN2 think there is no case 2-1 and 2-2 in Rel-18.

The WID is updated to ‘only in coverage’ because RAN2 runs out of time and does not want to support SLPP-level forwarding, so RAN2 adds ‘not to support PC’ in RAN2 specification. As we know, forwarding functionality is finally supported in SA2, that implies SA2 supports PC from SA2 perspective, since having at least one UE in coverage to be the forwarding node(UE1) is enough. So, this is the misalignment between SA2 specification and RAN2 specification.

Observation 1: The misalignment between SA2 specification and RAN2 specification is, RAN2 spec says RAN2 does not support PC scenario, but current SA2 spec naturally supports PC scenario using forwarding functionality.

Two ways to solve the misalignment issue: SA2 to follow RAN2 that only IC and OOC can be supported, or RAN2 to follow SA2 that PC can also be supported.

If SA2 follows RAN2’s spec that only IC and OOC can be supported:

The principle is, when IC target UE performs discovery and anchor UE selection, IC target UE should only find IC anchor UEs. When OOC target UE performs discovery and anchor UE selection, OOC target UE should only find OOC anchor UEs.

For IC, as specified in 38.305-i00 below, the UE1 in the figure discovers the anchor UE with application ID, the anchor UEs are determined by LMF and tell to UE1. so here it can be assumed that LMF can give UE1 the ID of anchor UEs which are all in coverage, by implementation. (Or we add a note in the procedure to make it more clear)

38.305-i00: Figure 7.3A.2-1: UE Positioning Operations to support an SL-MT-LR. 1. The AMF serving UE1 sends a location request to the LMF including information on the required location results of the n UEs (e.g., absolute location, relative location or ranges and directions between pairs of UE(s)), the required LCS QoS and the Application layer IDs of the UEs when available, as described in TS 23.273 [35]. 2. The LMF sends an SL-MT-LR request to UE1 (via serving AMF) including the types of required location results (e.g., absolute location, relative locations or distances and/or directions) and the Application layer IDs of the other UEs 2 to n. 3. UE1 attempts to discover the other UEs 2 to n using their Application layer IDs if not already discovered, as described in TS 23.273 [35].

For OOC, target UE should discover the OOC anchor UEs first. So the discovery message should contain filter or indication of ‘in coverage or not’ to let OOC target UE filter the OOC anchor UEs. However current RAN2 does not contain the indication of ‘in coverage or not’ in RSPP metadata field, so this can be added to discovery message directly, based on SA2’s decision.

However some SA2 companies believe the PC is an important scenario, specification should not restrict all UEs to be in the same coverage. In addition, PC can be naturally supported by forwarding function in current SA2 spec. So if RAN2 follows SA2’s spec that PC can also be supported:

Firstly for PC, there is no resource allocation issue in RAN2. For example, if target UE is in coverage, target UE will get the SL PRS or SL data resource allocation from its serving RAN node by mode 1, or performs resource sensing/selection by mode 2, if anchor UE is out of coverage, anchor UE will get the SL PRS or SL data resource allocation by mode 2 resource sensing/selection in pre-configured radio resources. Therefore, there should be no stage-3 impact in RAN2 to support PC.

Current RAN2 spec has the note saying that partial coverage is not supported in this specification. If PC should be supported, stage-2 specification should be modified, to be specific, the note in 38.305 section 5.1 should be deleted, and PC description should be added, for example as the following TP:

5.1 Architecture Figure 5.1-1 shows the architecture in 5GS applicable to positioning of a UE with NR or E-UTRA access. The positioning architecture also supports the NR PC5 interface as illustrated in Figure 5.1-1. Sidelink positioning can be supported when the UE is inside NG-RAN coverage (UE A and UE B in Figure 5.1-1) and when the UE is outside NG-RAN coverage (UE C and UE D in Figure 5.1-1). Sidelink positioning can be supported when some of UEs are inside NG-RAN coverage (UE A and UE B in Figure 5.1-1) and some of UEs are out of NG-RAN coverage ( UE C and UE D in Figure 5.1-1) as specified in 23.273 and 23.586.

Proposal 1: RAN2 to discuss the following options to solve the RAN2-SA2 misalignment issue:

- Option 1: SA2 should follow RAN2 spec that only IC and OOC can be supported, then RAN2 recommends SA2 to add indication of ‘in coverage or not’ in discovery message.

- Option 2: RAN2 should follow SA2 spec that PC can also be supported, then RAN2 should add the description to support IC/PC/OOC in 38.305.

Proposal 2: Reply SA2’s LS that there is no additional stage-3 impact for RAN2 to support PC for both two questions, but there should be some stage-2 impact to support PC.

- Conclusion

In this contribution, we propose the following observation and proposals:

Observation 1: The misalignment between SA2 specification and RAN2 specification is, RAN2 spec says RAN2 does not support PC scenario, but current SA2 spec naturally supports PC scenario using forwarding functionality.

Proposal 1: RAN2 to discuss the following options to solve the RAN2-SA2 misalignment issue:

- Option 1: SA2 should follow RAN2 spec that only IC and OOC can be supported, then RAN2 recommends SA2 to add indication of ‘in coverage or not’ in discovery message.

- Option 2: RAN2 should follow SA2 spec that PC can also be supported, then RAN2 should add the description to support IC/PC/OOC in 38.305.

Proposal 2: Reply SA2’s LS that there is no additional stage-3 impact for RAN2 to support PC for both two questions, but there should be some stage-2 impact to support PC.

- References

- S2-2401383, LS on coverage condition for Ranging/Sidelink Positioning, SA2, 02-2024

- RP-232670, Revised WID on Expanded and Improved NR Positioning, Intel, 09-2023

- TS38.305-i00, Stage 2 functional specification of User Equipment (UE) positioning in NG-RAN, 01-2024

- Annex: draft LS reply

3GPP TSG RAN WG2#125	R2-24xxxxx

Athens, Greece, 26th, Feb. - 1st , Mar. 2024

Title:	(draft) Reply LS on coverage condition for Ranging/Sidelink Positioning

Response to:	S2-2401383

Release:	Release 18

Work Item:	     NR_pos_enh2

Source:	RAN WG2

To:	SA WG2

Cc:	RAN WG3

Contact Person:

Name:	Yu Pan

E-mail Address:	pan.yu24@zte.com.cn

Send any reply LS to:	3GPP Liaisons Coordinator, mailto:3GPPLiaison@etsi.org

Attachments:	-

1. Overall Description:

RAN2 discussed the SA2’s question in the LS and provides the following answer:

Question 1: Is there any potential RAN2 impact to support network based or network assisted SL positioning as defined in the 23.273 clause 6.20 when the target UE is in coverage while some of the SL reference UE(s) are out of coverage or does not have connection to a LMF?

RAN2’s answer: there is no any stage-3 RAN2 impact to support partial coverage scenarios in network assisted SL positioning, but some stage-2 modification to the current RAN2 spec may be needed.

Question 2: Is there any potential RAN2 impact to support UE only SL positioning as defined in 23.586 clause 6.8 when the target UE is out of coverage while some of the SL reference/Located UE(s) are in coverage of a network that supports Ranging/Sidelink Positioning?

RAN2’s answer: there is no any stage-3 RAN2 impact to support partial coverage scenarios in UE only SL positioning, but some stage-2 modification to the current RAN2 spec may be needed.

2. Actions:

To SA WG2

ACTION:	RAN2 kindly requests SA2 to take the above information into account.

3. Date of Next RAN2 Meetings:

TSG RAN WG2 #125-bis            2024-04-15 to 2024-04-19	             Changsha, CN

TSG RAN WG2 #126                2024-05-20 to 2024-05-24	             Fukuoka, JP