3GPP TSG RAN WG2 #125	                            				     R2-2400837

Athens, Greece, 26th February – 1st March 2024

Agenda Item:	7.4.1.4

Source:	InterDigital, Inc.

Title:	MAC CE to activate/deactivate semi-persistent PUCCH report for LTM.

Document for:	Discussion

# Introduction

The LS from RAN1 in [1] states the following.

With an independent configuration of LTM CSI reporting which RAN2 has agreed, the legacy MAC CE command cannot be used for LTM semi-persistent reporting on PUCCH because the MAC CE is used for the legacy CSI report configuration. It is not clear how the activation/deactivation of semi-persistent PUCCH report for LTM CSI reporting can be supported.

We assume a solution needs to be found in RAN2.

# Discussion

The existing MAC CEs for CSI reporting on PUCCH Activation/Deactivation are defined as follows

#### 6.1.3.16 SP CSI reporting on PUCCH Activation/Deactivation MAC CE The SP CSI reporting on PUCCH Activation/Deactivation MAC CE is identified by a MAC subheader with LCID as specified in Table 6.2.1-1. It has a fixed size of 16 bits with following fields: - Serving Cell ID: This field indicates the identity of the Serving Cell for which the MAC CE applies. The length of the field is 5 bits; - BWP ID: This field indicates a UL BWP for which the MAC CE applies as the codepoint of the DCI bandwidth part indicator field as specified in TS 38.212 [9]. The length of the BWP ID field is 2 bits; - Si: This field indicates the activation/deactivation status of the Semi-Persistent CSI report configuration within csi-ReportConfigToAddModList, as specified in TS 38.331 [5]. S0 refers to the report configuration which includes PUCCH resources for SP CSI reporting in the indicated BWP and has the lowest CSI-ReportConfigId within the list with type set to semiPersistentOnPUCCH, S1 to the report configuration which includes PUCCH resources for SP CSI reporting in the indicated BWP and has the second lowest CSI-ReportConfigId and so on. If the number of report configurations within the list with type set to semiPersistentOnPUCCH in the indicated BWP is less than i + 1, MAC entity shall ignore the Si field. The Si field is set to 1 to indicate that the corresponding Semi-Persistent CSI report configuration shall be activated. The Si field is set to 0 to indicate that the corresponding Semi-Persistent CSI report configuration i shall be deactivated. If the Semi-Persistent CSI report configuration i is configured with csi-ReportSubConfigList, the Si field is set to 0 to additionally indicate that all SubConfigurations within csi-ReportSubConfigList shall be deactivated; - R: Reserved bit, set to 0. NOTE: If a Semi-Persistent CSI report configuration i is configured with csi-ReportSubConfigList, the corresponding Si field is not set to 1. Figure 6.1.3.16-1: SP CSI reporting on PUCCH Activation/Deactivation MAC CE

#### 6.1.3.80 Enhanced SP CSI reporting on PUCCH Activation/Deactivation MAC CE The enhanced SP CSI reporting on PUCCH Activation/Deactivation MAC CE is identified by a MAC subheader with eLCID as specified in Table 6.2.1-1b. It has a variable size and consists of the following fields: - Serving Cell ID: This field indicates the identity of the Serving Cell for which the MAC CE applies. The length of the field is 5 bits; - BWP ID: This field indicates a UL BWP for which the MAC CE applies as the codepoint of the DCI bandwidth part indicator field as specified in TS 38.212 [9]. The length of the BWP ID field is 2 bits; - Si: This field indicates the activation/deactivation status of the Semi-Persistent CSI report configuration within csi-ReportConfigToAddModList, as specified in TS 38.331 [5]. S0 refers to the report configuration which includes PUCCH resources for SP CSI reporting in the indicated BWP and has the lowest CSI-ReportConfigId within the list with type set to semiPersistentOnPUCCH, S1 to the report configuration which includes PUCCH resources for SP CSI reporting in the indicated BWP and has the second lowest CSI-ReportConfigId and so on. If the number of report configurations within the list with type set to semiPersistentOnPUCCH in the indicated BWP is less than i + 1, MAC entity shall ignore the Si field. The Si field is set to 1 to indicate that the corresponding Semi-Persistent CSI report configuration shall be activated. The Si field is set to 0 to indicate that the corresponding Semi-Persistent CSI report configuration i shall be deactivated; - Ni,x: this field indicates the activation/deactivation status of the Semi-Persistent CSI report SubConfiguration x within csi-ReportSubConfigList of CSI-ReportConfigId i, as specified in TS 38.331 [5]. If Si set to 1, the octet corresponding to Ni,0 to Ni,7 is present. If Si set to 0, the octet corresponding to Ni,0 to Ni,7 is not present. N0,0 refers to the report SubConfiguration which has the lowest csi-ReportSubConfigID within the list, N0,1 to the report SubConfiguration which has the second lowest csi-ReportSubConfigID and so on. If the number of report SubConfigurations within the list with type set to csi-ReportSubConfigList in the indicated BWP is less than x + 1, the MAC entity shall ignore the Ni,x field. The Ni,x field is set to 1 to indicate that the corresponding Semi-Persistent CSI report SubConfiguration x shall be activated. The Ni,x field is set to 0 to indicate that the corresponding Semi-Persistent CSI report SubConfiguration x shall be deactivated; - R: Reserved bit, set to 0. Figure 6.1.3.80-1: Enhanced SP CSI reporting on PUCCH Activation/Deactivation MAC CE

Although there are several “R” bits available, the IE names referred to (e.g. csi-ReportConfigToAddModList) are not applicable to LTM, as LTM introduced LTM-specific corresponding IEs. The MAC CE description would need to be significantly modified to refer to the different IE names for the LTM case and the legacy cases and the text will become complex.

In addition, there may be some cases in the future where a specific enhancement is introduced for LTM but not other cases, or vice-versa.

Hence, for readability and for future maintenance, it seems that a new MAC CE would be more convenient.

Proposal 1: New MAC CE should be introduced for LTM SP CSI reporting on PUCCH Activation/Deactivation

# Conclusion

In this paper we discuss the issue of MAC CE for LTM SP CSI reporting on PUCCH Activation/Deactivation, and make the following recommendation:

Proposal 1: New MAC CE should be introduced for LTM SP CSI reporting on PUCCH Activation/Deactivation

# References

- R2-2400029,  “LS on MAC CE to activate/deactivate semi-persistent PUCCH report for LTM”