3GPP TSG-RAN WG2 Meeting #125	R2-2400908

Athens, Greece, 26 February – 01 March 2024

Title:	[DRAFT] Reply LS on RedCap UE MBS Broadcast reception

Response to:	R2-2400078/S2-2401506

Release:	Release 18

Work Item:	TEI18, 5MBS_Ph2

Source:	Nokia [TSG RAN WG2]

To:	SA2

Cc:	RAN3, CT3, CT4

Contact Person:

#### Name:	Mani Thyagarajan

E-mail Address:	mani.thyagarajan@nokia.com

Send any reply LS to:	3GPP Liaisons Coordinator, mailto:3GPPLiaison@etsi.org

Attachments:	-

1. Overall Description:

RAN2 thanks SA2 for the latest developments in SA2 on RedCap UE MBS Broadcast reception discussions and for the CR to TS 23.247.

Regarding the new questions from SA2 about whether a single MBS Frequency Selection Area (FSA) ID applies for RedCap UEs and non-RedCap UEs in the same MBS session, RAN2 provides the following answers to the SA2 questions directed to RAN2:

Q1: SA2 would like to ask RAN2 to confirm the feasibility of having the same MBS FSA ID for the RedCap UEs and non-RedCap UEs in the same MBS session.

RAN2 Answer: Yes, it is feasible to broadcast the same MBS broadcast session to both RedCap UEs and non-RedCap UEs in the same MBS FSA ID and the current RAN2 specification already supports this. It is RAN2 understanding that, based on TS 23.247, how the FSAIs are used for a service intended for RedCap UE only or non-RedCap UE only or for both types of UE depends on operators’ deployment decisions and agreements with content providers.

Q2: If the answer to Q1 is no, could RedCap UEs and non-RedCap UEs in the same MBS session use separate MBS FSA ID(s)?

RAN2 Answer: Again, as it is a deployment choice as to how the services are mapped to FSAIs, it is also feasible for RedCap UEs and non-RedCap UEs to receive the same MBS session on separate MBS FSA IDs. In fact from RAN perspective it is beneficial to deploy the services for RedCap UE and non-RedCap UE on separate FSAIs and frequencies so that the frequencies of interest determination and MBS interest indication by UE would help with cell reselection of the RedCap and non-RedCap UEs to different frequency layers for better load management.

2. Actions:

To SA2.

ACTION: 	RAN2 respectfully asks SA2 to take the above information into account in further work on RedCap UE MBS Broadcast reception.

3. Date of Next TSG-RAN WG2 Meeting:

RAN2#125bis	from 2024-04-15	to 2024-04-19		China, CN

RAN2#126	from 2024-05-20	to 2024-05-24		Fukuoka, Japan