3GPP TSG-RAN WG2 Meeting #125                                R2-2400967

Athens, Greece, 26th February – 1st March, 2024

Agenda item:		7.2.1

Source:				Samsung

Title:					Support of SRS pre-configuration in RAN3

Document for:		Discussion & Decision

1	Introduction

At the last RAN2#124, RAN2 made the following agreement for multiple SRS pre-configurations.

For preconfigured multiple SRS configurations, confirm the WA, i.e. UE sends a new ResumeCause of RRCResumeRequest message to indicate the change or activations of SRS configuration when different SRS configuration is selected due to change of validity area, or when a new SRS configuration is selected where none was previously in use.

According to the latest TS 38.331 v18.0.0 [1], the NW can configure one or more SRS pre-configurations to the UE via RRCRelease message and the UE can indicate the activation/application of the pre-configured SRS by sending RRCResumeRequest message. However, when we check the current implementation status [2][3], the SRS pre-configuration feature seems not supported from RAN3’s perspective. Thus, in this contribution, we would like to propose to send the LS to RAN3 to request the support of SRS pre-configuration feature.

2	Discussion

From RAN2 perspective, the SRS pre-configuration feature can be supported as below.

- The NG-RAN node provides the UE with SRS pre-configuration that can be used for SRS transmission in RRC_INACTIVE state.

- The SRS pre-configuration (srs-PosRRC-InactiveValidityAreaConfigList-r18) is provided to the UE via RRCRelease message and it can include one or more SRS configurations each of which is associated with one validity area.

- The UE can indicate the change or activations of SRS configuration by sending RRCResumeRequest message with the resume cause ‘srs-PosConfigOrActivationReq’ when different SRS configuration is selected due to change of validity area, or when a new SRS configuration is selected where none was previously in use.

The above are already implemented in the latest 38.331 [1] and the SRS pre-configuration feature is supported from RAN2’s perspective. However, when we check the latest BL CRs for TS 38.455 [2] and for TS 38.305 [3], the SRS pre-configuration feature seems not be supported from RAN3’s perspective considering the following aspects.

- No procedure for the LMF to acquire multiple SRS pre-configurations with multiple validity area.

- No procedure for the LMF to provide the serving gNB with the multiple SRS pre-configurations.

- No procedure for the serving gNB to report the activation/application of pre-configured SRS to the LMF.

Based on the observation above, we would like to propose the following.

Proposal 1: RAN2 to send the LS to RAN3 to request the support of SRS pre-configuration feature. The draft LS in Annex can be used as baseline for the LS.

3	Conclusion

Based on the above, RAN2 is requested to discuss the following proposals:

Proposal 1: RAN2 to send the LS to RAN3 to request the support of SRS pre-configuration feature. The draft LS in Annex can be used as baseline for the LS.

4	Reference

- 3GPP TS 38.331 v18.0.0, ‘Radio Resource Control (RRC) protocol specification’

- R3-238100, ‘(BL CR to TS 38.455) Support of NR Positioning Enhancements’, CATT et al.

- R3-238097, ‘(BL CR to TS 38.305) Support of NR Positioning Enhancements’, Nokia et al.

5	Annex

3GPP TSG RAN WG2#125	R2-231xxxx

Athens, Greece, 26th February – 1st March, 2024

Title:	Draft LS to RAN3 on SRS pre-configuration

Response to:	-

Release:	Rel-18

Work Item:	NR_pos_enh2

Source:	Samsung (to be RAN2)

To:	RAN3

Cc:	-

Contact person:	Taeseop Lee

Taeseop.lee(at)samsung.com

Send any reply LS to:	3GPP Liaisons Coordinator, mailto:3GPPLiaison@etsi.org

Attachments: R2-231xxxx (if needed)

1	Overall description

SRS pre-configuration is introduced in Rel-18. From RAN2 perspective, the SRS pre-configuration feature is already supported in the latest TS 38.331 v18.0.0 as below:

- The NG-RAN node provides the UE with SRS pre-configuration that can be used for SRS transmission in RRC_INACTIVE state.

- The SRS pre-configuration (srs-PosRRC-InactiveValidityAreaConfigList-r18) is provided to the UE via RRCRelease message and it can include one or more SRS configurations each of which is associated with one validity area.

- The UE can indicate the change or activations of SRS configuration by sending RRCResumeRequest message with the resume cause ‘srs-PosConfigOrActivationReq’ when different SRS configuration is selected due to change of validity area, or when a new SRS configuration is selected where none was previously in use.

However, RAN2 assume that the support of SRS pre-configuration would require RAN3 work on coordination between NG-RAN node and LMF.

2	Actions

To RAN3

ACTION: RAN2 respectfully asks RAN3 to take the above into account for your further corresponding work.

3	Dates of next TSG-RAN WG4 meetings

3GPP TSG RAN WG2#125bis	15 April - 19 April 2024		Changsha, CN

3GPP TSG RAN WG2#126  	20 May - 24 May 2024		Fukuoka, JP