3GPP TSG-RAN WG2 Meeting #125	R2-2400987

Athens, Greece, 26 February – 01 March 2024

Agenda item:	7.2.2

Source:	Nokia, Nokia Shanghai Bell

Title:	Solution for some key RIL issues impacting stage-2

WID/SID:	NR_pos_enh2-core - Release 18

Document for:	Discussion and Decision

# 1	Introduction

In this paper we discuss some key RIL issues submitted during ASN.1 review of TS 37.355 that has impacts to TS 38.305 and propose solutions. Text proposals for these solutions are provided in the Annex in this paper.

# 2	Discussion

## 2.1	SRS for Positioning transmission in RRC_INACTIVE in a Validity Area (RILs C401, X046, H578)

RILs C401, X046 and H578 discussed the need for clause 5.7.18 in TS 38.331 and suggested to not have PHY layer procedures described in RRC specification and to move the description of SRS for positioning transmission in SRSRRC_INACTIVE to TS 38.305. There were other comments that suggested the handling of the release of (pre-)configuration of SRS for positioning under the clause where the RRC connection release procedure is specified in TS 38.331. The involved RILs are shown verbatim for reference:

[RIL]: C401 [Delegate]: CATT (Jianxiang) [WI]: POS [Class]:1 [Status]: ToDo [TDoc]: R2-24xxxxx [Proposed Conclusion]: v12 [Description]: The desrion of stage-2 can be moved to TS 38.305 [Proposed Change]: Move these description to TS 38.305 [Comments]:

[RIL]: X046 [Delegate]: Xiaomi (Xiaolong) [WI]:Pos [Class]: 1[Status]: ToDo [TDoc]: None [Proposed Conclusion]: v073 [Description]: The procedure here is related to the RAN1 agreement on the SRS transmission suspending and resuming, we think it should be caputured in the RAN1 spec. [Proposed Change]: Remove the crossponding procedure and caputure it in the RAN1 spec. [Comments]:

[RIL]: H578 [Delegate]: Huawei (YinghaoGuo) [WI]: Pos [Class]: 1 [Status]: ToDo [TDoc]: [Proposed Conclusion]: v014 [Description]: These are PHY procedures?? And not sure why they are part of the RRC spec [Proposed Change]: Remove this paragraph [Comments]:

We agree with the comments C401, X046 and H578. In this paper we provide a TP for inclusion in TS 38.305.

Proposal 1: RAN2 is kindly requested to consider the text proposal for SRS for positioning transmission in RRC_INACTIVE that is shown in the Annex.

# 3	Conclusion

This document has made the following proposals:

Proposal 1: RAN2 is kindly requested to consider the text proposal for SRS for positioning transmission in RRC_INACTIVE that is shown in the Annex.

# Annex A

Text proposal for 38.305 on SRS for positioning transmission in RRC_INACTIVE.

--- Start of Text Proposal to 38.305

## 7.9	Positioning in RRC_INACTIVE state

Positioning may be performed when a UE is in RRC_INACTIVE state. Any uplink LCS or LPP message can be transported in RRC_INACTIVE state. If the UE initiated data transmission using UL SDT, the network can send DL LCS, LPP, RRC Release message and SP Positioning SRS Activation/Deactivation MAC CE to the UE without the need of state transition.

Periodic and Semi-persistent Positioning SRS transmission can be supported in RRC_INACTIVE.

NOTE:	Aperiodic Positioning SRS transmission is not supported in RRC_INACTIVE.

The Positioning SRS configuration in RRC_INACTIVE state may be associated with the serving cell or may be associated with an SRS Validity Area. An SRS Validity Area consists of a list of cells in which the Positioning SRS configuration in RRC_INACTIVE state is valid. There is only one Positioning SRS configuration in RRC_INACTIVE state for an SRS Validity Area. The UE may be configured with Positioning SRS configuration for the current SRS Validity Area the UE is in or may be pre-configured with Positioning SRS configurations for multiple SRS Validity Areas.

If the UE reselects to another cell within the same SRS Validity Area during Positioning SRS transmission the UE continues the Positioning SRS transmission, subject to time alignment validation for Positioning SRS transmission as specified in TS 38.321 [39] and TS 38.331 [14]. If the UE reselects out of the current SRS Validity Area during Positioning SRS transmission, the UE may send an "RRC Resume Request" message to the network to request the network to either configure or pre-configure Positioning SRS configuration or may request the network to activate a pre-configured Positioning SRS configuration.

--- End of Text Proposal to 38.305